Sydex

HP150.SYS and HP150II.SYS

Two Device Drivers for Hewlett-Packard HP 150 MS-DOS Diskettes

Sydex

P.O. Box 5700

Eugene, OR  97405

Voice:  (503)  683-6033

FAX:    (503)  683-1622

HP150.SYS and HP150II.SYS

Two Device Drivers for Hewlett-Packard HP 150 MS-DOS Diskettes

Copyright 1990, Sydex

All Rights Reserved

Version 2.0 June, 1990

Sydex

P.O. Box 5700

Eugene, OR  97405

Voice:  (503)  683-6033

FAX:    (503)  683-1622

NOTICE

THIS IS NOT FREE SOFTWARE!   If you paid a "public domain" vendor

for  this  program,  you  paid  for  the  service  of copying the

program, and not for the program itself.  Rest assured that noth-

ing ever gets to the originators of  this  product  from  such  a

sale.   You may evaluate this product, but if you make use of it,

you must register your copy.

We offer several inducements to you for registering.    First  of

all,  you receive the most up-to-date copy of the program that we

have without the advertising copy embedded.  You also receive sup-

port for HP150.SYS and HP150II.SYS--which can be  quite  valuable

at times.   And finally,  we include a few utilities not included

in the evaluation package.  Make no mistake  however--we  do  not

"cripple"  our evaluation software--our sample programs are fully

functional and complete.

REGISTRATION INFORMATION

The non-commercial single-user registration fee for HP150.SYS and

HP150II.SYS is $20.00 US.   The commercial and  multisystem  site

fee is $150.00 for HP150.SYS and HP150II.SYS.

Send a check or company P.O. for the appropriate amount to:

SYDEX

P.O. Box 5700

Eugene, OR  97405

Voice:  (503)  683-6033

FAX:    (503)  683-1622

Data:   (503)  683-1385

Major Credit Cards Accepted

Please  indicate  what product you are ordering and if you have a

requirement for 3.5" media.   We normally ship  5.25"  360K  dis-

kettes, but we will furnish 3.5" 720K media upon request.  Orders

placed  outside  of  the U.S.  and Canada should include an addi-

tional $5.00 for airmail.  We can accept U.S. funds only, please.

DISTRIBUTION NOTICE

This is "user-supported"  software.  You  are  hereby  granted  a

license  by Sydex to distribute this evaluation copy of HP150.SYS

and HP150II.SYS and its documentation,  subject to the  following

conditions:

1.   HP150.SYS  and HP150II.SYS may be distributed freely without

charge in evaluation form only.

2.   HP150.SYS and HP150II.SYS may not be sold,  licensed,  or  a

fee  charged for their use.   If a fee is charged in connec-

tion with HP150.SYS and/or HP150II.SYS,  it must  cover  the

cost of copying or dissemination only.  Such charges must be

clearly identified as such by the originating party.   Under

no circumstances may the purchaser be given  the  impression

that he is buying HP150.SYS or HP150II.SYS themselves.

3.   HP150.SYS  and  HP150II.SYS  must  be  presented as complete

units, including this documentation.  This documentation may

be not amended or altered in any way.

4.   In granting you the right to distribute the evaluation  form

of HP150.SYS and HP150II.SYS, Sydex does not acknowledge you

as the owner of HP150.SYS or HP150II.SYS in any form.

Any   other   use   or  distribution  of  HP150.SYS  and

HP150II.SYS is expressly forbidden without  the  written

consent of SYDEX.

________________________________________________________________

"HP  150" and "HP" are trademarks of Hewlett-Packard Corporation.

"IBM", "PC DOS" and "Personal System/2" are trademarks of Interna-

tional Business Machines Corporation.  "MS-DOS" is a trademark of

Microsoft Corporation.

What are HP150.SYS and HP150II.SYS?

HP150.SYS and HP150II.SYS are two device  drivers  which  support

reading  and  writing  of  MS-DOS  diskettes from Hewlett-Packard

Corporation's HP 150 series computers on PC XT- or  AT-compatible

machines.  Under DOS 3.2 or later, the utilities FORMAT, DISKCOPY

and DISKCOMP are supported,  as well as normal DOS read and write

operations.  HP150.SYS is used for single-sided HP 150 diskettes;

HP150II.SYS is used for the double-sided diskettes of the  Series

II HP 150.

To  use  HP150.SYS or HP150II.SYS,  your computer must be PC AT-,

XT- or IBM Personal System/2 compatible.   Computers such as  the

Tandy  2000 or the Zenith Z-150 are not suitable because of their

relative  incompatibility.    Further,   your  computer  must  be

equipped  with a 3.5" 720K or 1.44M diskette drive.   To read and

write HP 150 5.25" diskettes, your computer must be equipped with

high-density 1.2 Megabyte drive or a 5.25" "quad" density  drive,

such as the Teac 55F.  The diskette adapter on your computer must

match  the  diskette  drive  characteristics.   That is,  a high-

density drive must be used with a  high-density  adapter.    Many

manufacturers, such as Western Digital, produce high-density dis-

kette adapters for XT-style computers, and these may be used suc-

cessfully with HP150.SYS and HP150II.SYS.   Currently, we support

only the first (or primary) diskette adapter in a system.

And finally,  your computer needs to be running PC-DOS or  MS-DOS

Version  2.0 or later.   DOS 3.2 or later is preferable,  but not

required.   Version 2 DOS users may find that some HP150.SYS  and

HP150II.SYS functions, such as formatting, are not supported.

Installing and Using HP150.SYS and HP150II.SYS

Installation  of  HP150.SYS  and  HP150II.SYS is straightforward.

First, find the file CONFIG.SYS in the root directory of the disk

that your computer boots (loads DOS) from.   It may  be  possible

that  you  do not have a CONFIG.SYS file,  in which case the next

few steps will create one.    If  you  are  unfamiliar  with  the

CONFIG.SYS  file,  take time out now and read the section in your

DOS manual dealing with it, and in particular, the DEVICE= state-

ment.

You'll need to add a line to CONFIG.SYS of the following  general

form:

DEVICE=HP150.SYS /D:unit  /F:format

Or:

DEVICE=HP150II.SYS /D:unit  /F:format

Page 1

This  can  be  done with the program EDLIN which is supplied with

DOS, or with a text editor of your choosing.   If you use a word-

processing editor, make sure that you indicate that CONFIG.SYS is

a nondocument file.   If you didn't find a CONFIG.SYS,  the above

line will be the only line in the file which is created.

The value of unit can range from 0 to 3 and represents the physi-

cal diskette unit to be used to read and write the  HP  150  dis-

kettes.    If  this  unit is normally called A: on your computer,

then the value of unit is 0;  if B:,  then unit is 1.    External

drives are generally units 2 and 3.

The  value  of  format depends on the type of diskette drive that

you're using.  If it's a high-density 1.2 megabyte unit, then the

value of format is 1.   If you're using a 3.5" drive or a  "quad"

density drive, such as the Teac 55F, then format is 2.

If /D:unit is omitted in the configuration line,  unit is assumed

to be 1. If /F:format is omitted, format is assumed to be 1 also.

The last step is to copy the file HP150.SYS or  HP150II.SYS  into

the root directory of your system or "boot" drive.   Finally, re-

boot your computer.   If your computer is configured with a  ver-

sion of DOS 3, the following message will be displayed:

HP 150 (or HP 150II) diskette driver, Copyright 1990, Sydex

All Rights Reserved.

HP 150 (or HP 150II) DOS diskette unit x is accessible as

drive y:

The  letters x and y will take on values which depend on the con-

figuration of your computer.

If your computer is configured with a version of DOS 2,  the fol-

lowing message will be displayed:

HP 150 (or HP 150II) diskette driver, Copyright 1990, Sydex

All Rights Reserved.

HP150 (or HP 150II) DOS diskette unit x installed.

In this case,  the drive letter which is used to read HP 150 dis-

kettes will generally be one greater than the last  drive  letter

in use before you modified CONFIG.SYS.   For example, if drive C:

was the last drive on your computer,  HP150.SYS  and  HP150II.SYS

will use drive letter D:.

Page 2

As an example of the whole process described above,  suppose that

we wished to add HP150.SYS or  HP150II.SYS  to  a  PC-AT  "clone"

which  has  one  3.5"  diskette  drive  and  a  hard  disk.   Our

CONFIG.SYS file on  drive  C:  might  look  like  this  after  we

modified it using EDLIN:

FILES = 30

BUFFERS = 10

DEVICE = HP150.SYS  /D:0  /F:2     (This line is added)

If we are adding HP150II.SYS to this system,  our CONFIG.SYS file

on drive C: might look like this after we modified it  using  ED-

LIN:

FILES = 30

BUFFERS = 10

DEVICE = HP150II.SYS  /D:0  /F:2   (This line is added)

We then copy HP150.SYS or HP150II.SYS to drive C:...

COPY /B A:HP150.SYS  C:\

Or:

COPY /B A:HP150II.SYS  C:\

We  then  press  the  keys  Ctrl,  Alt  and Del simultaneously to

"reboot" our computer.   After the  rebooting  process  has  com-

pleted, the following message is displayed:

HP 150 (or HP 150II) diskette driver, Copyright 1989, Sydex

All Rights Reserved.

HP 150 (or HP 150II) DOS diskette unit 0 is accessible as

drive D:

We  then find that HP 150 diskettes placed in our A: drive can be

read and written if we refer to that drive as drive D:.

FINAL COMMENTS...

HP150.SYS and HP150II.SYS were written in response  to  inquiries

from  customers  who had our CP/M diskette transfer package,  but

who had no way to easily handle the HP 150  DOS  diskettes.    We

hope  that  this  situation  is now remedied.   If you experience

problems in getting HP150.SYS or HP150II.SYS to work for you,  or

if  you have questions about any Sydex product,  please give us a

call.

Page 3

Other Sydex Products

AnaDisk -- The  compleat  diskette  utility.    Nothing  like  it

anywhere else; scan, edit, repair and copy just about any kind of

diskette.  $25.00 ($150.00 site)  registration fee.

Con>Format  --  Concurrent "background" diskette formatter.  Fea-

tures "pop-up" operation and "hot key" activation.  You've got to

see it to believe.   Supports all  current  DOS  formats.  $15.00

($50.00 site) registration fee.

22DISK  --  Transfer  files,  format,  examine and erase files on

"foreign" CP/M diskettes on your PC.  Includes tips on supporting

8" and 5.25" single-density diskettes.   Contains definitions for

over  250 different formats.   "Roll your own" option for unusual

formats.  $25.00 ($100.00 site) registration fee.

22NICE -- A  CP/M  2.2  emulation  package.    Supports  the  NEC

V-series  chips  or  performs  emulation by software for both the

8080 and Z80 processors.   Includes terminal emulation  and  dis-

kette handling for common CP/M systems.  Includes 22DISK.  $40.00

($150.00 site) registration fee.

TeleDisk  -- Our diskette FAX machine.   Turn any diskette into a

compressed data file and vice-versa.   Allows  you  to  send  and

receive entire diskettes via modem.   Even works with some "copy-

protected" diskettes.  $20.00 ($150.00 site) registration fee.

COPYQM -- Mass diskette duplicator.  Format, copy and verify mul-

tiple diskettes from a single master.   Implements "no  keyboard"

interaction mode and drive "round robin" servicing.  Supports all

standard  DOS  formats.   Copy even non-DOS diskettes with "blind

copy" option  in  the  latest  release.    $15.00  ($50.00  site)

registration fee.

FORMATQM  --  Mass diskette formatter - format a box of diskettes

at a single sitting.   Implements "no keyboard" interaction  mode

and drive "round robin" servicing.  Supports all standard DOS for-

mats.  $10.00 ($40.00 site) registration fee.

Information on any of these products can be obtained  from  Sydex

by  calling or writing us at:

Sydex

P.O. Box 5700

Eugene, OR  97405

Voice:  (503)  683-6033

FAX:    (503)  683-1622

Data:   (503)  683-1385

Page 4