CARPENTER'S DREAM

The construction program for contractors, carpenters, and home-

owners. Measurements are expressed in feet, inches and fractions

of an inch; and angles as pitch. Everything for residential

construction as well as commercial and heavy industrial applica-

tions.

Version 2.11S

Nov. 1991

Workhorses Inc.

805 B 14th Street

Golden  CO 80401

(303) 279-8551

(800) 777-2477

Dan Heilman         Editor in Chief

Kata McCarville     Editor

Peggy Heilman       Assistant Editor

Joe Applegate       Principal Programmer

Layout and Design   IT'sNEWS! Publications

Orv Heilman         Construction Consultant

Copyright c 1990 WORKHORSES, INC.

All rights reserved.

1

Carpenter's Dream is not free software!  This program has been made

available to you as shareware.  If you intend to use Carpenter's

Dream you should register your copy by sending in the registration

form with a payment of $25 to:

WORKHORSES INC.

805 B 14th Street

Golden, CO 80401

To charge your registration (MasterCard, Visa, or American Express)

call 1-800-777-2477 (orders only), or fill in the appropriate blanks

on the registration form.

You will receive the latest version of the program, telephone

support, Quick Reference Guide, and hard copy of the Educational

Manual, and you will be notified of future updates.

Registration will also allow you to purchase our latest program,

Contractor's Dream for far less than the normal price, which is

currently $275.  Contractor's Dream is a greatly enhanced version

of Carpenter's Dream with a cursor controlled menu, full screen

field entry of parameters, a material data base, prices for

materials, printed estimates, Lotus (C) 123 and ASCII output,

a Pop-Up carpenter's calculator, "what-if" modeling, and many

more features of use to carpenters and contractors.

That's why we are making Carpenter's Dream into a shareware product.

Since 1989, Workhorses, Inc., has sold thousands of copies of

Carpenter's Dream at $99.  We have received favorable reviews in

numerous construction and computer magazines.  But Carpenter's Dream,

while still a useful program, is surpassed by Contractor's Dream.

We have included a demo of Contractor's Dream with this disk so that

you can see for yourself how powerful Contractor's Dream is.

REMEMBER:  As a registered owner of Carpenter's Dream you can buy

Contractor's Dream for only $199!

Please refer to the READ.ME file on the diskette for any last

minute changes, any new files added to the disk, as well as other

information that may not be included in the printed material.

IBM is a trademark of International Business Machines, Inc.

MS-DOS is a trademark of Microsoft Corporation.

2

Limited Warranty

Limited warranty. Workhorses, Inc. warrants that (a) the software

will perform substantially in accordance with the accompanying

written materials for a period of 90 days from the date of re-

ceipt.  Any implied warranties on the software are limited to 90

days.  Some states do not allow limitations on duration of an

implied warranty, so the above limitation may not apply to you.

Customer remedies. The liability of Workhorses, Inc., and your

exclusive remedy, shall be, at the option of Workhorses, Inc.,

either (a) return of the price paid or (b) repair or replacement

of the software that does not meet the limited warranty of Work-

horses, Inc., and which is returned to Workhorses, Inc. with a

copy of your receipt.  This limited warranty is void if failure

of the software has resulted from accident, abuse, or misapplica-

tion.  Any replacement software will be warranted for the remain-

der of the original warranty period or 30 days, whichever is

longer.

No other warranties. Workhorses, Inc. disclaims all other warran-

ties, either express or implied, including but not limited to

implied, including but not limited to implied warranties of

merchantability and fitness for a particular purpose, with re-

spect to the software and the accompanying written materials.

This limited warranty gives you specific legal rights. You may

have others, which may vary from state to state.

No liability for consequential damages. In no event shall Work-

horses, Inc. or its suppliers be liable for any damages whatsoev-

er including, without limitation, damages for loss of business

profits, business interruption, loss of business information, or

other pecuniary loss arising out of the use of or inability to

use this Workhorses, Inc. product, even if Workhorses, Inc. has

been advised of the possibility of such damages. Because some

states do not allow the exclusion or limitation of liability for

consequential or incidental damages, the above limitation may not

apply to you.

3

Table of Contents

GENERAL USAGE                      6

STARTING THE PROGRAM               6

ENTERING FRACTIONS                 6

ENTERING PITCHES                   6

FUNCTION CONTROLS                  8

ENTERING SHAPES                    8

HELPFUL HINT WITH COMPLEX SHAPES   9

PROGRAM 1

CONCRETE                   10

LIQUIDS                    10

TRUCK LOADS                10

TANDEM LOADS       10

SEMI END DUMP      10

BLOWN INSULATION

CEILINGS           11

WALLS              11

CUBICS                     11

YARDS, FEET, AND METERS  11

PROGRAM 2

DECKS                      12

SHEETS                     12

AREA                       12

PROGRAM 3

TILES, BRICKS, CERAMICS    12

PROGRAM 4

ESTIMATE ROOFS             13

MAIN STYLE ROOF    13

DORMER STYLE       14

MULTI-SIDED        14

COMPLEX ROOFS      14

ROLLS OF HALF LAP ROOFING  16

REBAR                      16

PRICE OR QUANTITY          16

ON CENTERS                 16

PROGRAM 5

CUTTING LIST FOR RAFTERS & TRUSSES   18

ABBREVIATIONS              18

2x RIDGE OR TRUSS          18

2x6 RIDGE ONLY             19

2x RIDGE AND HIP ONLY      19

PROGRAM 6

STAIRS                     20

MAIN RUNNER        20

PROGRAM 7

CUTTING LIST FOR WINDOWS   21

PROGRAM 8

CROSS CORNER LENGTHS       22

PROGRAM 9

SPACING BETWEEN POSTS      23

PROGRAM 10

4

ADD                        24

SUBTRACT                   24

MULTIPLY                   24

DIVIDE                     24

CONVERSIONS

FEET               24

INCHES             24

METERS             24

DECIMAL FEET       24

QUANTITY           24

PITCH TO DEGREES   25

PROGRAM 11

GENERAL INFORMATION        26

PROGRAM 12

REGISTRATION FORM          26

TELEPHONE SUPPORT                  26

GLOSSARY                           27

Index                              29

5

GENERAL USAGE

STARTING THE PROGRAM

To start the program, type in: DREAM and press the <ENTER> key.

The program works in feet, inches, and fractions, but if you

prefer, it will also work in decimal feet or decimal inches.

EXAMPLE:

7,6 = 7.5 = 7 feet 6 inches = 7 1/2 feet

0,3,1/2 = 0,3.5 = 3 1/2 inches

0,96 = 8 feet

The program understands both forms with no other command neces-

sary.

ALL MEASUREMENTS EXACT TO

1/16 OF AN INCH

ENTERING FRACTIONS:

1/16, 1/8, 3/16, 1/4, 5/16, 3/8, 7/16, 1/2, 9/16, 5/8, 11/16,

3/4, 13/16, 7/8, and 15/16.

FOLLOW THESE EXAMPLES:

For 10 feet 11 3/16 inches, TYPE IN:  10,11,3/16

For 9 feet 5/8 inches, TYPE IN:  9,0,5/8

For 3 1/2 inches, TYPE IN:  0,3,1/2

For 38 feet 4 inches, TYPE IN: 38,4

For 7 feet, TYPE IN:  7

ALWAYS SEPARATE FEET, INCHES, AND FRACTIONS WITH COMMAS

ENTERING PITCHES:

1-12, 1.5-12, 2-12, 2.5-12, 3-12, 3.5-12, 4-12, 4.5-12, 5-12,

5.5-12, 6-12, 6.5-12, 7-12, 7.5-12, 8-12, 8.5-12, 9-12, 9.5-12,

10-12, 10.5-12, 11-12, 11.5-12, 12-12, 12.5-12, 13-12, 13.5-12,

14-12, 14.5-12, 15-12, 15.5-12, 16-12, 16.5-12, 17-12, 17.5-12,

18-12, 18.5-12, 19-12, 19.5-12, 20-12, 20.5-12, 21-12, 21.5-12,

22-12, 22.5-12, 23-12, 23.5-12, and 24-12.

Read "4.5-12" as "four and a half-twelve pitch," and so forth.

When working with roof rafters, hips, and jacks, the program has

already been adjusted for angular thickness of ridge, subfascia,

and hip material using standard 2x material. There is no ridge in

trusses only 2x subfascia. 2x means "2 by material" = 1 1/2

inches.

NO OTHER DEDUCTIONS NECESSARY

WHAT YOU READ IS WHAT YOU CUT

6

If you have a printer attached to your PC, press the <PRINT

SCREEN> key to get a copy of the screen.  All the programs are

designed to accept input and produce output on a single screen.

On some PCs, you must press both <SHIFT> and <PRINT SCREEN>

together.

ESTIMATION PROGRAMS

All programs figure exact amounts. You need to add in any waste

you might expect for a job.

7

FUNCTION  KEYS

F1 = Help Screen

You can press F1 at any time for help. Return to where you were

working by pressing any key.

F2 = Main Menu

You can press F2 to go to the main menu at any time but, will not

be able to return to where you were.

F3 = Quick Graphic Reference Guide-STAIRS & POSTS

This screen requires a CGA, MCGA, EGA, or VGA color monitor. It

shows you what to enter for measurements and how to read the

measurements that the program gives you. Return to where you were

working by pressing any key.

F4 = Quick Graphic Reference Guide-MULTI-SIDED &

ROOF ESTIMATIONS

This screen requires a CGA, MCGA, EGA, or VGA color  monitor. It

shows you what to enter for measurements and how to read the

measurements that the program gives you. Return to where you were

working by pressing any key.

F5 = Quick Graphic Reference-RAFTER CUT LIST

This screen requires a CGA, MCGA, EGA, or VGA color monitor. It

shows you what to enter for measurements and how to read the

measurements that the program gives you. Return to where you were

working by pressing any key.

F10 = Exit Program

Press F10 at any time to exit the program.

ENTERING SHAPES

CIRCLE

For the Diameter-give the distance across the circle.

TRIANGLE

This program works for any triangle.

For the Base-give the distance across the bottom.

For the Height-give the vertical distance from the base to the

highest point above the base.

Rectangle

For the Length-give the distance on one side.

For the Width-give the distance on the other side.

8

MULTI-SIDED

For Length of One Side-Give the distance on one side.

For Cross Corner Length-Give largest distance across the shape

(labled "CC" on diagram).

WORKS GREAT FOR GAZEBOS

Examples: 3 sides = equilateral triangle

Examples:

Examples:

4 sides = square

5 sides = pentagon

6 sides = hexagon

7 sides = heptagon

8 sides = octagon

HELPFUL HINT FOR COMPLEX SHAPES

If you have a complex shape, break it up into circles, triangles,

rectangles, and multi-sided shapes. Run the program for all of

the pieces and save the figures for each by printing your re-

sults. Then total all of your figures together.

THICKNESS

For the thickness-Give the average distance from top to bottom.

Example-4 inches thick, type in: 0,4.

9

PROGRAM 1-CONCRETE, GALLONS,  AND MORE

Make sure you have looked at the SHAPES AVAILABLE section before

you try to use this program.

All programs figure exact amounts. You need to add any waste that

you might expect on that job.

CONCRETE

The program gives you the number of cubic yards of concrete to

order.

LIQUIDS

U.S. GALLONS and LITERS

These programs work identically. They give you the amount of

liquid in U.S. Gallons or LITERS contained in the shape you

choose. This is helpful for picking the size of pump that you

will need to pump out a pond or basement. Check to see how many

gallons (or liters) the pump puts out per hour or minute to

decide which pump is right for you. You may just want to know the

cost of filling the pool or how much water to haul to that dove

pond. The program will also work for oil, gas, chemicals, or any

other liquid.

TRUCK LOADS

All weights are based on the OPTIMUM MOISTURE level, just right

for compaction. Be sure to check local load limits and your truck

specifications.

TANDEM LOADS: SAND, TOP SOIL, FILL

(or anything with similar density)

The program gives you the number of tandem loads you need to

order. Specifications: 1.4 tons per cubic yard and 15 tons per

load. If this doesn't meet your specifications, you must adjust

accordingly.

TANDEM LOADS: WHITE ROCK, ROAD GRAVEL,

RIP RAP (or anything with similar density)

RIP RAP

RIP RAP

The program gives you the number of tandem loads you need to

order. Specifications: 1.6 tons per cubic yard and 15 tons per

load. If this doesn't meet your specifications, you must adjust

accordingly.

SEMI END DUMP: SAND, TOP SOIL, FILL

(or anything with similar density)

10

The program gives you the number of tandem loads you need to

order. Specifications: 1.4 tons per cubic yard and 22 tons per

load. If this doesn't meet your specifications you must adjust

accordingly.

SEMI END DUMP: WHITE ROCK, ROAD GRAVEL,

SEMI END DUMP: WHITE ROCK, ROAD GRAVEL,

SEMI END DUMP: WHITE ROCK, ROAD GRAVEL,

RIP RAP (or anything with similar density)

RIP RAP

RIP RAP

The program gives you the number of tandem loads you need to

order. Specifications: 1.6 tons per cubic yard and 22 tons per

load. If this doesn't meet your specifications you must adjust

accordingly.

BLOWN INSULATION

CEILINGS

The program gives you the number of 30 pound bags to order.

Specifications: 2.5 pounds per cubic feet. If this doesn't met

your specifications, you must adjust accordingly.

R VALUE FOR CEILINGS

(Settled density depth in inches)

R-50 = 13.9  R-32 = 8.9  R-24 = 6.7  R-18 = 5.0  R-10 = 2.8

R-44 = 12.2  R-31 = 8.6  R-23 = 6.4  R-16 = 4.5  R-9  = 2.5

R-40 = 11.1  R-30 = 8.3  R-22 = 6.1  R-14 = 3.9  R-8  = 2.2

R-38 = 10.6  R-28 = 7.8  R-21 = 5.8  R-13 = 3.6  R-6  = 1.7

R-36 = 10.0  R-27 = 7.5  R-20 = 5.6  R-12 = 3.3  R-4  = 1.1

R-34 =  9.5  R-26 = 7.2  R-19 = 5.3  R-11 = 3.1  R-3.6= 1.0

WALLS

The program gives you the number of 30-pound bags to order.

Specifications: 4 pounds per cubic foot. If this doesn't meet

your specifications, you must adjust accordingly.

R VALUE FOR WALLS

For 2x4 studs: The cavity has an R value of 12.25

For 2x6 studs: The cavity has an R value of 19.25

CUBICS

YARDS, FEET, AND METERS

You can use these cubics for any material you choose, such as

peat moss, sand, or fertilizer.  NOTE:  The cubic yards in con-

crete can be used for cubic yards of anything.

11

PROGRAM 2-DECKS, SHEETS,

AND AREAS

Make sure you have looked at the SHAPES AVAILABLE section before

you try to use this program.

All programs figure exact amounts. You need to add in any waste

that you might expect.

DECKS

The program gives you the linear feet needed. To get the number

of boards to order, take this length and divide by the length

board you plan to use.

Specifications:                 2x4 = 1 1/2 inches x 3 1/2 inches

Specifications:

Specifications:

2x6 = 1 1/2 inches x 5 1/2 inches

2x8 = 1 1/2 inches x 7 1/4 inches

SHEETS

The program gives you the number of sheets to order. It works for

any thickness of sheet you choose on materials such as plywood,

drywall, or paneling.

Specification:                       4x8  = 4 feet by 8 feet

Specification:

Specification:

4x9  = 4 feet by 9 feet

4x10 = 4 feet by 10 feet

4x12 = 4 feet by 12 feet

AREA (YARDS, FEET, OR METERS)

Use this for any material you wish, such as paint, linoleum, or

carpet.

PROGRAM 3-TILES, BRICKS, AND CERAMICS

Make sure you have looked at the SHAPES AVAILABLE section before

you try to use this program.

All programs figure exact amounts. You need to add any waste that

you might expect on that job.

The program will ask for the dimensions of a rectangular shape

used to fill an area.  That fill material may be tiles, bricks,

or any rectangular shape.  The fill shape will then be fitted to

the selected shape and area and the program will report the

calulated number of fill shapes required.

Be sure and include mortor on two sides only.  Standard mortor on

12

bricks and blocks is 3/8 of an inch.  Standard mortor on ceramics

is 1/8 of an inch.

PROGRAM 4-ESTIMATE ROOFS, REBAR,  HALF-LAP, AND

16 INCH  ON-CENTERS

All programs figure exact amounts. You need to add in any waste

that you might expect.

ROOFS

Take measurements from Blue Print.

NOTE:  For black paper use the squares of shingles for squares or

black paper.

MAIN STYLE AND DORMER STYLE ROOF

For the Overall Length of Roof-Give the longest distance of that

section of roof along the ridge.

For the Overall Width of Roof-Give the full distance perpendicu-

lar to the ridge.

For the Pitch-Give the slope of the roof using one of the pitch

choices on page 6.

For Hip or Gable-Enter 1 for hip or 2 for gable.

The program gives you the number of 4x8 sheets of plywood needed.

The program gives you the number of squares of shingles needed.

Most roofing material comes in squares. The program gives you the

number of squares of shingles for caps. Add these two numbers to

get the total number of squares of shingles you will need.

The program gives you the length of rafters required and the

length of hip material to order. Length is given to the nearest 2

foot length just as you order them.

The program gives you the number of rafters for both 16 inches

and 2 feet on center. It does not matter if the roof is a hip or

gable. The jacks are included for the hip style. This doesn't

include subfascia or ridge material.  (Read these lengths off of

blue prints.

MAIN STYLE ROOF

This is any section of the roof that can stand alone and is not

13

on another roof.

14

DORMER STYLE

This is any section of the roof that lies on another section of

the roof.

The program gives you the roll valley material needed with the

overlap already included.

The program gives you the deductions for the plywood to take from

the adjoining roof. Deductions are for only the triangular part.

You may not want to take all of this deduction because generally

you go ahead and lay most of the 4x8 sheets of plywood down.

The program gives you the deductions for the shingles to take

from the adjoining roof. You should take this deduction, which

only includes the triangular part.

MULTI-SIDED ROOFS

This is any section of the roof where all of the sides are the

same length and have the same angle between them. Works great for

gazebos.

For the Number of Sides-Give the number of outer edge sides.

For the Pitch-Give the slope of the roof using one of the choices

on page 2.

As for main and dormer roofs, the program gives you the number of

4x8 sheets of plywood, the number of squares of shingles, and the

number of squares of shingles of cap you should order. The pro-

gram also gives you the length of the common rafters required.

The length is given to the 2 feet length just as you would order

them.

COMPLEX ROOFS

For complex roofs, break the roof into sections. Then compute

each section and total the results together.

Dormer roof with different pitch than main roof: Run the dormer

Dormer roof with different pitch than main roof:

Dormer roof with different pitch than main roof:

style program once for each pitch. Then add the results together

and divide by 2. Don't forget to adjust for the overall length of

the roof.

Dormer roof that butts into two different main roofs: Cut the

Dormer roof that butts into two different main roofs:

Dormer roof that butts into two different main roofs:

dormer in half and do it in two sections. Make each end a gable

and deduct two rafters.

Roof with an offset ridge: Treat the roof as though the ridge is

Roof with an offset ridge:

Roof with an offset ridge:

in the center. To get singles, cap, and plywood run the program

twice, once for each for each side of the offset. Make your

overall width twice the distance of one side. This will give you

15

the length of the rafters for each side. Then divide the quantity

of rafters by 2.

Cone shaped roof: Use the multi-sided roof program. Figure how

Cone shaped roof:

Cone shaped roof:

many common rafters the roof will take and use that number for

the number of sides to enter. Measure the distance at the fascia

on the center with the commons and use this measurement for the

length of one side. This will give you the shingles, plywood, and

rafter length. Ignore cap.

16

ROLLS OF HALF-LAP ROOFING

Specifications: One roll of half lap covers 50 square feet. Five

gallons of cold process are required for every 4 rolls of half

lap. NOTE: This will be less in the summer and more in the win-

ter. Each roll weighs 90 pounds. This program works for rectan-

gles only.  See graphic on  p. 10

When asked for the Length of Area to Cover-Give the distance on

one side.

When asked for the Width of Area to Cover-Give the length of the

other side.

The program gives you the number rolls of half lap needed.

The program gives you the number gallons of cold process needed.

REBAR

Specification:

Specification:

Specification:

#2 Rebar = 1/4 inch diameter = 0.167 lb. per linear foot

#3 Rebar = 3/8 inch diameter = 0.367 lb. per linear foot

#4 Rebar = 1/2 inch diameter = 0.668 lb. per linear foot

#5 Rebar = 5/8 inch diameter = 1.043 lb. per linear foot

#6 Rebar = 3/4 inch diameter = 1.502 lb. per linear foot

#7 Rebar = 7/8 inch diameter = 2.044 lb. per linear foot

#8 Rebar =   1 inch diameter = 2.670 lb. per linear foot

For the Linear Footage-Give the total length needed.

The program gives the number of pounds of rebar to order.

PRICE OR QUANTITY

This program calculates the total price for an item, using the

quantity of material multiplied times the unit price.

For quantity-Give number of items, the amount, or the length

needed.

For Price-Give the price per unit.

The program gives you the total price for the material or the

quantity.

ON CENTERS

This program lets you find the studs, joists, rebar, roll insula-

tion, and anything else that is on center, with ends included.

17

For things such as rebar and roll insulation, find out how many

you need on center Then multiply that number times the length

needed to get your linear footage. For rebar you can then go to

rebar estimations and enter the linear footage to find how many

pounds to order.

For Length of Run-Give the overall distance.

The program gives you quantity needed with the ends included.

NOTE: In general, when estimating the number of studs for a house

at 16 inches on center, total the length of all exterior and

interior walls. The number of feet is the quantity of studs to

order. This will include door doublers, partition posts, corner

posts, and 2x4 headers. Next, take this total length and multiply

by 3 for the bottom and double top plates. You still have to

include backing, collar ties, braces, and garage studs to round

out your 2x4 list.

18

PROGRAM 5-CUTTING LIST

FOR RAFTERS AND TRUSSES

ABBREVIATIONS:

LP = Long point

SP = Short point

BM = Bird's mouth

2x = 1 1/2 inches

The angular thickness of 2x ridge, 2x subfascia, and 2x hip

material has already been adjusted for. There is no ridge in

trusses, only 2x subfascia.

NO OTHER DEDUCTIONS NECESSARY

WHAT YOU READ IS WHAT YOU CUT

When the program asks for the Width of the Building-Give the

distance from outside to outside of building where the Bird's

Mouth butts into. This is probably the celotex or something

similar, not the top plates.  For trusses this is the top plates.

For the Overhang

When using rafters-Give the distance from the outside edge of the

subfascia to the celotex or similar covering.

When using trusses-Give the distance from the outside edge of the

subfascia to the stud.

For the Pitch-Give the slope of the roof. See ENTERING PITCHES.

For either 2x Ridge or Truss: Give 2x if you are using 1 1/2 inch

material or truss if there is no ridge.

2x RIDGE OR TRUSS

The program gives you the degree of the common, which is the

slope of the roof.

See Quick Graphic Reference Guide

These measurements are exactly what you cut

The program gives the long point to bird's mouth.

The program gives the long point to short point.

The program gives the  height of ridge from top plate minus

bird's mouth. This is very helpful when setting up scaffolding.

Just take this height and add to the wall height and ground-to-

floor height.

The program gives you level line of bottom of 2x6 subfascia to

outside wall PLUS bird's mouth.

19

This is an IMPORTANT measurement

Check to see where this hits with respect to the window.

EXAMPLE: If you are using:

PLUS                        MINUS

3 in. of top plate          1 in. reveal on brick mold

11 1/4 in. of 2x12 header   1/2 in. aluminum J mold

1/2 in. clearance above     1/4in. of play

This totals 13 inches MINUS bird's mouth. Standard bird's mouth

is 1 1/2 inches. If these measurements don't match PLUS & MINUS

the bird's mouth then either adjust overhang, pitch, or bird's

mouth. EXAMPLE: If the program gave you 10 inches for level line

and you where using the materials above and 1 1/2 inch bird's

mouth this would be a perfect fit for the soffit and brick mold

with a 1/4 inch play. Both would equal 11 1/2 inches. 13 inches

MINUS bird's mouth = 10 inches PLUS bird's mouth = 11 1/2 inches

2x6 RIDGE ONLY

The program gives you cripple height MINUS bird's mouth. This is

a very helpful measurement. Now you can nail up your first two

rafters to the ridge and then set it right on the cripple!

2x RIDGE AND HIP ONLY

The program gives you the degrees of the hip and the pitch. For

the pitch change the 12 to 17. EXAMPLE: If roof is a 6-12 pitch

then the hip pitch is 6-17.

The program gives you long point to bird's mouth of the hip.

Don't precut the hip from long point to short point. This should

be done after it is up by snapping a line from common ends.

The program gives you the difference to cut the jacks for both

two feet and 16 inches on center.

The program gives you what to cut the first jack rafter laid out

at either two feet or 16 inches on center.

20

PROGRAM 6-STAIRS

CAN BE USED ON ANY SET OF STAIRS

The program was designed to find the best rise and run using the

first measurement equal to or over 7 inches. This program can be

used for small stairs and for very large stairs, too.  For con-

crete stairs, make the overlap equal to 0 and ignore the length

of opening. For spiral stairs, use the rise measurement and the

number of tread and ignore everything else.

For Height of Opening-Give it from finished floor to finished

floor.

For Tread Width-Give the width of material you are using. EXAM-

PLE: 2x12 tread would be 11 1/2 inches = 0,11,1/2. For the Over-

lap: Give the distance the tread hangs over the run. EXAMPLE: 1

inch overlap = 0,1.

The program gives you rise, which is the distance from bottom of

tread to bottom of next tread.

The program gives you run, which is the distance of the width of

the tread minus overlap.

The program gives you the length of opening figured using 1/8-

inch floor tile, 3/4-inch plywood flooring, and 2x10 floor joist

on upper floor. If your flooring material is different adjust

accordingly. This measurement is very helpful, as it allows for

needed head height. This measurement is only given if the height

of opening is big enough. You don't want to hit your head.

The program gives you the floor space needed. This is helpful for

finding out if there is enough room for a       3-foot landing at

the bottom. You don't want to run into a wall.

CUTTING YOUR MAIN RUNNER: Don't forget to cut the thickness of

CUTTING YOUR MAIN RUNNER:

CUTTING YOUR MAIN RUNNER:

your tread of the horizontal bottom of the main runner.  When

using kicker boards cut the thickness of your kicker off the

vertical top of the main runner.

21

PROGRAM 7-CUTTING LIST FOR WINDOWS

This program gives you the cutting list for the windows for

framing in. Specifications:  Using standard 92 5/8 inch studs,

single 2x bottom plate, double 2x top plate, and 2x12 header (11

1/4 inch).  Some local codes require that the sill butt into the

window doubler.  If this is the case for you, just subtract 3

inches from sill and ignore window sides. NOTE: The 92 5/8 inch

studs may be different for your area; adjust accordingly.

For the Rough Opening Height:  Give the rough opening height of

the specification of your window.

For the Rough Opening Width:  Give the rough opening width of the

specification of your window.

The program gives header and sill length.

The program gives window side length.

The program gives the length for cripplers, the boards between

the sill and the bottom plate.

22

PROGRAM 8-CROSS CORNER LENGTHS

This program will square up your project for you.  When dealing

with multi-sided shapes, all sides must be the same length and

have the same angle between them.  In other words, it must be a

regular polygon. See ENTERING SHAPES, MULTI-SIDED.

For the Length of One Side-Give the length of one edge.

The program gives you the cross corner length, (CC) on graphics.

This is the distance from one corner to the farthest opposite

corner.

23

PROGRAM 9-FENCING AND SPACING BETWEEN POSTS

This program can be used for spacing support beam posts, deck

railings, stair railings, Fencing, gardens (tomato plants etc.),

Ornamental railings, and more!

For Length of Run - Give the overall distance with end posts

included.  When asked for Width of Posts:  Give the width of a

post. EXAMPLE:  For a 4x4, give 3 1/2 inches = 0,3,1/2.

For approximate spacing between posts-Give the longest distance

desired between posts.

The program gives you the exact spacing between the posts and the

number of posts, ends included.

24

PROGRAM 10-ADD, SUBTRACT, MULTIPLY, DIVIDE, AND CONVERT

ADD

For the Starting Length-Give a distance.

For the Length to Add-Give a distance you want to add to the

first distance.

The program gives you the total of the two.

SUBTRACT

For the Starting Length-Give the longer of two distances.

For the Length to Subtract-Give the other distance.

The program gives you the difference between the longer and

shorter length.

MULTIPLY

For the Starting Length-Give the first distance or quantity.

For the Length to Multiply by-Give the length or quantity to

multiply by.

The program gives you the answer in square units of the same

units you enter.

DIVIDE

This will be handy to find fencing, on centers, and placements.

It is great for figuring blocks or bricks needed.

For the Starting Length:  Give a distance or quantity.

For the Length to Divide- Give  a length or quantity to divide

by.

The program gives you the answer in the same units you enter.

CONVERSION OF FEET, INCHES, AND FRACTIONS OF AN INCH, METERS, AND

DECIMAL FEET OR QUANTITY

For feet, inches, and fraction of an inch, decimal feet, or

meters:  Enter your measurement.

The program gives you the measurement in all three forms: feet,

inches, and fraction of an inch, decimal feet, and meters.

25

CONVERSIONS OF PITCH TO DEGREES

For Pitch-Give one of the pitches listed in ENTERING PITCHES.

Always put a dash (-) between numbers as in 4-12.

The program gives you degrees. This is helpful for setting your

saw.

NOTE: This program only does one function (add, subtract, multi-

ply, and divide) at a time.  To do multiples of a function (or

multiple functions) you need to buy Workhorses' Pop-Up calcula-

tor, "Pop an Inch".

26

PROGRAM 11

GENERAL INFORMATION

This program gives you many standard measurements that are easily

forgotten.  Some of these measurements may be regulated by local

code.  Be sure to check your local code.

PROGRAM 12REGISTRATION FORM

This program gives you the registration form that you should send

in. After you have registered, you will receive information on

updates, as well as telephone support.  Press the print screen

key on your computer to print the form on paper. Fill in all

blanks appropriately.  Your comments are appreciated. Send the

form (and $99 if you have not already paid for the program-Visa,

MasterCard, or COD's accepted, COD's add $5) to:

WORKHORSES, INC.

805 B 14th Street

Golden, CO  80401

1-303-279-8551 for telephone support.

Registration entitles you to a copy of the latest version of

CARPENTER'S DREAM, a hardcopy of the Educational Manual, notifi-

cation of future updates, and telephone support.  Prices subject

to change without notification.

TELEPHONE SUPPORT

After you have sent in your registration, you may call our tele-

phone support line if you have questions or problems with CARPEN-

TER'S DREAM.  The telephone support number is: (303) 279-8551.

Hours:  Monday through Friday, 9:00 a.m.-5:00 p.m. Mountain time.

27

Glossary

angular        slope or pitch

area           amount of surface

bird's mouth   the cutout of the rafter that sits on the wall and

butts into the outside wall

bottom plate   the board that goes across the bottom of the studs

braces         temporary supports

brick mold     type of window trim

butts          edge to edge joints

cap            cut-up shingles that lie on the ridge and hip

cellulose      treated, ground-up paper stock

celotex        a thick insulating sheet

cold process   a tar substance

collar tie     the support between rafters on opposite sides of

the room

common         identical rafters that extend from the subfascia

to the ridge

conversion     changes from one unit of measure to another

cripple        a support board

cubic          three-dimensional volume

decking        flooring

degrees        measurement of the angle

density        of similar weight per size

dormer         a roof section that lies on another section of

roof

doubler        the board under the header butted next to the stud

gable          no pitch on the end of the roof

gazebos        a multi-sided outdoor summer retreat

half-lap       rolled roofing with half-granular and half smooth

header         beam

hip            pitch on the end of the roof

j mold         the soffit enters this at the wall

jack           the short rafters from the hip to the subfascia

joist          main supports of a wall or a ceiling

landing        area at the bottom of the stairs

long point     longest point on an angular end cut board

main runner    the board that extends from floor to floor on

stairs

on center      spacing distance from center to center

optimum

moisture   just right for compaction

overhang       distance from outside wall to the outside of the

subfascia

parallel       running in the same direction

perpendicular  at 90 degrees to each other

pitch          the slope or angle

placements     where something goes

play           clearance

r value        heat storage value

rafters        main angular supports of the roof

rebar          steel reinforcing rods

28

reveal         the exposed part

ridge          upper most part of the roof

rip rap        generally, broken-up concrete

rise           distance form bottom of tread to bottom of next

tread

rough opening  the opening the door or window sit in

run            the distance of the tread minus the overlap

short point    the shortest point on an angular end cut board

sides          a form of doubler

sill           bottom of the rough opening for a window or door

slope          pitch or angle

squares        one unit by one unit area

standard       most widely used

studs          main supports of a wall

subfascia      the board going across the ends of the rafters

top plate      the board that goes across the top of the studs

tread          the board you walk on

trusses        where the common rafters butt into each other

waste          cut offs, bowed, and damaged material

29

Index

Ridge 6, 13, 14, 15, 16, 18, 19

Add 24                           Rip Rap 10

Area 11, 12                      Road gravel 10

Bricks 12                        Roll insulation 16

Blown insulation 11              Roll valley 14

Cap 13, 14                       Sand 10

Carpet 12                        Semi-end dump 10

Ceramics 12                      Sheets 12

Chemicals 10                     Shingles 12, 13, 14

Circle 8                         Soil 10

Cold process 16                  Spiral stairs 20

Complex roofs 14                 Stairs 6, 20

Concrete 10                      Studs 16

Concrete stairs 20               Subfascia 6, 14, 18, 19

Cone shaped roof 14              Subtract 24

Conversion 24, 25                Tandem loads 10

Cross corner lengths 22          Telephone support 26

Decks 12                         Tiles 12

Dormer roof 14                   Top 10

Drywall 12                       Tread Width 20

Fencing 23                       Truck Loads 10

Fill 10                          Trusses 18

Floor joist 20                   U.S. gallons 10

Floor tile 12                    White rock 10

Gable 13                         Windows 21

Gas 10

Gazebos 9, 14

Half lap 16

Hip 6, 13, 14, 18, 29

hips 6

Jack rafter 6, 13, 19

Joists 16

Landing 20

Linoleum 12

Liquids 10

Liters 10

Main & dormer style roof 13, 14

Multi-sided 6, 16

Offset ridge 16

Oil 10

On centers 16, 19, 24

Paint 12

Paneling 12

Placements 24

Plywood 12, 13, 14, 15, 20

Posts 6, 16, 23

Price or quantity 16

R value 11

Rafters 6, 13, 14, 15, 16, 19

Rebar 12, 15, 16

Registration form 26

30

NOTES

31

NOTES

32

NOTES

33