���������������������������������������������������������������������Ŀ

� ASCII Documentation File for the AUDIT.EXE Self-Extracting Archive  �

���������������������������������������������������������������������Ĵ

�                 Quality Assurance Training Tools:                   �

�         Statistics Committee of the QA Section of the PMA           �

�           PMA = Pharmaceutical Manufacturing Association            �

���������������������������������������������������������������������Ĵ

�             Bob Obenchain, CompuServe User [72007,467]              �

�����������������������������������������������������������������������

This archive contains a pair of MS-DOS software systems useful in

setting upper confidence limits on the unknown, true nonconformance

rates in populations that have been audited via random sampling.

- Binomial.EXE computes upper bounds of 50% to 99% confidence

for random sampling (with or without replacement) from an

infinite population.

- HyprGeom.EXE computes upper bounds of 50% to 99% confidence

for random sampling WITHOUT REPLACEMENT from finite

populations of up to 1,000,000 units.

These software systems compute binomial and hypergeometric probabilities

using state-of-the art approximations for logarithms of factorials.

Because all of the parameters of hypergeometric distributions are

integers (discrete), it may be necessary to generate an upper bound of

slightly higher confidence than the requested level.  On the other hand,

the "percentage nonconforming" parameter of binomial distributions is

continuous (0<=p<=1), so upper bounds of the requested confidence are

generated in this case.

Rules-of-Thumb on USAGE of Binomial.EXE and HyprGeom.EXE...

===========================================================

HyprGeom results are especially interesting when the fraction,

100*(n/N)%, of a finite population that is sampled is large.  The

resulting upper confidence bound on total percent nonconforming is also

most "narrow" in these cases.

As the sampling fraction = 100*(n/N)% decreases, the resulting upper

bounds will widen, approaching the corresponding statistical confidence

limits for binomial sampling from infinite distributions.

Both Binomial.EXE and HyprGeom.EXE can be used in lieu of statistical

tables.  They can not only interpolate between the values given in

standard statistical tables but they can also make computations well

beyond the ranges commonly covered in published tabulations.

EXAMPLES: Differences between HyprGeom.EXE and Binomial.EXE calculations

========================================================================

Example One...

��������������������������������������������������Ŀ

�  Sample Size:                         n =  100   �

�  Observed Nonconformances:            x =    1   �

�  Observed Percentage Nonconforming:       1.00%  �

�                                                  �

�  Confidence Interval                             �

�  Probability Content:     100*(1-alpha) =   95%  �

����������������������������������������������������

Exact    Sampling

Finite Population Results (Hypergeometric Distribution):   Content  Fraction

--------------------------------------------------------   -------  --------

N =  200,         95% Upper Bound =   8 /  200 = 4.00%    96.8%     50%

N =  300,         95% Upper Bound =  13 /  300 = 4.33%    96.4%     33%

N =  400,         95% Upper Bound =  17 /  400 = 4.25%    95.3%     25%

N =  500,         95% Upper Bound =  22 /  500 = 4.40%    95.5%     20%

N = 1000,         95% Upper Bound =  45 / 1000 = 4.50%    95.1%     10%

N = 5000,         95% Upper Bound = 231 / 5000 = 4.62%    95.0%      2%

Infinite Population Results (Binomial Distribution):

----------------------------------------------------

N = infinity,     95% Upper Bound =              4.66%               0%

Example Two...

��������������������������������������������������Ŀ

�  Sample Size:                         n =  500   �

�  Observed Nonconformances:            x =    5   �

�  Observed Percentage Nonconforming:       1.00%  �

�                                                  �

�  Confidence Interval                             �

�  Probability Content:     100*(1-alpha) =   95%  �

����������������������������������������������������

Exact    Sampling

Finite Population Results (Hypergeometric Distribution):   Content  Fraction

--------------------------------------------------------   -------  --------

N =   750,        95% Upper Bound =  13 /  750 = 1.73%    96.7%     67%

N =  1000,        95% Upper Bound =  18 / 1000 = 1.80%    95.3%     50%

N =  1250,        95% Upper Bound =  24 / 1250 = 1.92%    96.1%     40%

N =  1500,        95% Upper Bound =  29 / 1500 = 1.93%    95.6%     33%

N =  2000,        95% Upper Bound =  39 / 2000 = 1.95%    95.0%     25%

N =  5000,        95% Upper Bound = 102 / 5000 = 2.04%    95.1%     10%

N = 10000,        95% Upper Bound = 207 /10000 = 2.07%    95.1%      5%

Infinite Population Results (Binomial Distribution):

----------------------------------------------------

N = infinity,     95% Upper Bound =              2.09%               0%

EXAMPLES: Going beyond the commonly tabulated range using Binomial.EXE

======================================================================

95% Upper Confidence

Limits on Percent Defective

x  |  n = 6000      7000      8000      9000      10000

__________|_______________________________________________________

|

0  |      0.50%     0.04%     0.04%     0.03%     0.03%

|

1  |      0.08%     0.07%     0.06%     0.05%     0.05%

|

2  |      0.10%     0.09%     0.08%     0.07%     0.06%

|

3  |      0.13%     0.11%     0.10%     0.09%     0.08%

|

4  |      0.15%     0.13%     0.11%     0.10%     0.09%

|

5  |      0.18%     0.15%     0.13%     0.12%     0.11%

|

6  |      0.20%     0.17%     0.15%     0.13%     0.12%

|

7  |      0.22%     0.19%     0.16%     0.15%     0.13%

|

8  |      0.24%     0.21%     0.18%     0.16%     0.14%

|

9  |      0.26%     0.22%     0.20%     0.17%     0.16%

|

10  |      0.28%     0.24%     0.21%     0.19%     0.17%