������������������������������������������������������Ŀ

�               Control Chart Simulator                �

�              CCSIM.EXE....Version 9402               �

������������������������������������������������������Ĵ

�          A Quality Assurance Training Tool:          �

�  Statistics Committee of the QA Section of the PMA   �

������������������������������������������������������Ĵ

�      Bob Obenchain, CompuServe User [72007,467]      �

�     5261 Woodfield Drive North, Carmel, IN 46033     �

��������������������������������������������������������

PMA = Pharmaceutical Manufacturer's Association

*** Table of Contents ***

Introduction . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

Example CCSIM Walk-Through (use of Menus/Prompts). . . . . . . . . . 3

Smoothing with a Moving Average or an EWMA . . . . . . . . . . . . . 9

Other Quality Monitoring Software in the "QATrainTool" Series. . . .10

REFERENCES and Update History. . . . . . . . . . . . . . . . . . . .12

INTRODUCTION

============

When you see a "blip" or "trend" on a control chart, how do you know

whether it either signals something truly worth worrying about or else

is just a simple artifact of common-cause variation?  And I'm talking

here about PRACTICAL IMPORTANCE, not mere statistical significance.

(After all, some very little things of no practical importance

whatsoever can be statistically significant!)

CCSIM uses state-of-the-art pseudo-random number generator techniques

[L'Ecuyer(1988)] to simulate normally distributed and statistically

independent observations.  CCSIM generates VGA (640 x 480 pixel)

graphics displays of 100 observations each, plotted on a Shewhart

3-Sigma Control Chart designed for processes with MEAN � = 0 and

STANDARD DEVIATION � = 1.  The first 50 observations on each Control

Chart are always generated from an i.i.d. normal processes with � = 0

and � = 1.  But the user of CCSIM controls the values of � and � for

the second 50 observations on each Control Chart.

When the user decides to use � = 0 and � = 1 for the second 50

observations as well as for the first 50, then he/she will be

simulating a PERFECTLY STABLE ("in-control") process!  When this case

is simulated, CCSIM helps the user practice the skill of NOT

OVER-REACTING to what he/she sees on a Control Chart.  After all, this

is the situation where any adjustment you might want to make to the

process as a result of "what you think you see on its control chart"

would be PURE TAMPERING with that process!

When the user of CCSIM gives the 2nd 50 observations a mean different

from � = 0 and/or a variance different from �� = 1, then the process

being simulated has a CHANGE POINT between the 1st 50 and 2nd 50

Control Chart Simulator[ver.9402]                     Page 2 of 12

observations on each screen.  The allowed range of parameter settings

for the 2nd 50 observations is:

-3.0 <= � <= +3.0    and    0.01 <= � <= 6.0.

Change points can imply that the 2nd 50 observations on each screen

come from an off-center and/or higher/lower variability processes that

may have correspondingly lower "capability" than the � = 0, �� = 1

processes for which the CCSIM Shewhart 3-Sigma Control Chart is

designed.  The question here, of course, is... "If you didn't KNOW

that there was a change point with a certain � and/or � shift between

the 1st 50 and 2nd 50 observations, would your eyes DETECT this change

upon visual examination of the Control Chart?"  And, if you do think

you can see a change, how far past observation number 51 did the

change finally become visually "obvious?"

In addition to displaying (statistically independent) process values,

the user also has the option of displaying a process "smoother" defined

by either...

[i] a Moving Average (MA) within a finite Window

or

[ii] an Exponentially Weighted Moving Average (EWMA).

Critics, such as Nelson(1983), of the use of "smoothing" in quality

trend monitoring have suggested that, because adjacent smoothed values

can be highly positively correlated, viewers tend to either

misinterpret or else over-interpret these kinds of charts.  So CCSIM

helps users learn when to expect MUCH LONGER RUNS of smoothed values

(either above or below �.)   Long runs result when either the MA

Window Width is very large (say, > 20) or else when the EWMA Short-

Term Memory Weight is small (say, < 0.1 ).

Files distributed with the CC Simulator are:

============================================

CCSIM.EXE....Main executable module

CCSIM.DOC....This ASCII documentation file

CCSIM.C......C-language source code for CCSIM

Invoking CCSIM.EXE

==================

To start up the CC Simulator, ENTER the command CCSIM at your

MS-DOS (>) prompt:

MyPrompt> ccsim

Control Chart Simulator[ver.9402]                     Page 3 of 12

�������������������������������������Ŀ

� ���������������������������������Ŀ �

� �   Example CCSIM Walk-Through:   � �

� ����������������������������������� �

���������������������������������������

���������������������� Control Chart Simulator ���������������������Ŀ

�                                                                    �

�                      CCSIM.EXE...Version 9402                      �

�                                                                    �

�    *** Pseudo-Random Control Chart Displays in VGA Graphics ***    �

�                                                                    �

�                 A Quality Assurance Training Tool:                 �

�         Statistics Committee of the QA Section of the PMA          �

�                                                                    �

�            Bob Obenchain, CompuServe User [72007,467]              �

�                                                                    �

�  ��������������������������������������������������������������ͻ  �

�  �   Press a key to see the Control Chart Simulator MENU...     �  �

�  ��������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

���������������������� Control Chart Simulator ���������������������Ŀ

�                ������������������������������������ͻ              �

�                �            Main Menu               �              �

�                �   P = set  Parameter values        �              �

�                �   V = View simulated charts        �              �

�                �   S = Save Summary Statistics      �              �

�                �   X = eXit from the CCSIM system   �              �

�                �   choice --> P                     �              �

�                ������������������������������������ͼ              �

����������������������������������������������������������������������

Control Chart Simulator[ver.9402]                     Page 4 of 12

���������������������� Control Chart Simulator ���������������������Ŀ

����������������������� Set Parameter Values ��������������������ͻ  �

��   At colon Prompts : ...press ENTER to get the [default].      �  �

��                                                                �  �

��   CCSIM generates normally distributed processes               �  �

��   with statistically independent observations.                 �  �

��                                                                �  �

��   The 1st 50 Observations generated for EACH SCREEN always     �  �

��   have MEAN = 0.00 and STD.DEVIATION = 1.00.                   �  �

��                                                                �  �

��   The MEAN value for the 2nd 50 observations my be set to any  �  �

��   value in the Range from -3.00 to +3.00.                      �  �

��   Specify the 2nd 50 MEAN value. [  0.00] :      <---Take Default �

��                                                                �  �

��   2nd 50 Process MEAN =   0.00.                                �  �

��                                                                �  �

��   The STANDARD DEVIATION of the 2nd 50 observations may be set �  �

��   to any value in the Range from 0.01 to 6.00                  �  �

��   Specify the 2nd 50 STD.DEVIATION. [ 1.000] :   <---Take Default �

��                                                                �  �

��   2nd 50 Process STD.DEVIATION =  1.000.                       �  �

�����������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

���������������������� Control Chart Simulator ���������������������Ŀ

����������������������� Set Parameter Values ��������������������ͻ  �

��   Create Screen Displays of AVERAGES within a Moving WINDOW?   �  �

��   The Window Width can be from 2 to 50.                        �  �

��   Specify WIDTH (Zero => NO WINDOW). [0] :       <---Take Default �

��                                                                �  �

��   No Moving Average will be formed.                            �  �

��                                                                �  �

��   Create Displays of Exponentially Weighted Moving AVERAGES?   �  �

��   The Short-Term Memory Weight can vary from 0.01 to 1.0       �  �

��   Specify WEIGHT (Zero => NO EWMA). [ 0.000] :   <---Take Default �

��                                                                �  �

��   No EWMA will be formed.                                      �  �

��                                                                �  �

��   Input Simulation Start-up Seed ( 0 means use your Clock ):   �  �

��   Equivalent Start-Up Seed = -24478.                           �  �

��                                                                �  �

��   Press Q to QUIT now...Other Key to Continue...               �  �

�����������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

Control Chart Simulator[ver.9402]                     Page 5 of 12

Note One: The default Start-Up Seed [0] causes CCSIM to use your

personal computer clock setting to generate the seed value;

different runs (and resets) of CCSIM usually start out (and

progress) differently in this case.  Entering a positive seed value

(1<=idum<=32767) will cause each run (or reset) of CCSIM to

start at the exact same point of a pseudo-random sequence; "final"

cumulative results will then depend only upon how many screens

of 100 consecutive values the user displays before pressing the

Q, X or ESCape key...

Note Two: It is essential for the pseudo-random number generation

algorithm used in CCSIM to produce statistically independent

observations, i.e. the raw x-series must be free of any auto-

correlation and/or time-series structure that has any "memory."

Thus, CCSIM uses a "bucket" algorithm, such as that in the ran1()

function of Press et. al.(1988).  Specifically, variates generated

using the 32-bit L'Ecuyer(1988) algorithm are first placed in a

bucket of size 100 values.  The 16-bit L'Ecuyer(1988) algorithm is

then used to draw uniformly from that bucket, replacing drawn

values with the next value in the 32-bit generator's sequence.

���������������������� Control Chart Simulator ���������������������Ŀ

�                ������������������������������������ͻ              �

�                �            Main Menu               �              �

�                �   P = set  Parameter values        �              �

�                �   V = View simulated charts        �              �

�                �   S = Save Summary Statistics      �              �

�                �   X = eXit from the CCSIM system   �              �

�                �   choice --> V                     �              �

�                ������������������������������������ͼ              �

����������������������������������������������������������������������

Control Chart Simulator[ver.9402]                     Page 6 of 12

���������������������� Control Chart Simulator ���������������������Ŀ

������������������ Display RANDOM Control Charts ����������������ͻ  �

��                                                                �  �

��   The VGA screens show a normally distributed process          �  �

��   with statistically independent observations.                 �  �

��                                                                �  �

��   Process is COMPLETELY STABLE with MEAN=0.0 and STD.DEV.=1.0. �  �

��                                                                �  �

��   WARNING: Press the Q, X or ESCape Key to stop Simulating...  �  �

��   WARNING: The Control-Break key combination is disabled!      �  �

��                                                                �  �

��   Press the R key or SPACEBAR to detect Run Rule Violations... �  �

��                                                                �  �

��   SCREEN DUMPS to slave parallel PRINTERS on LPT1:             �  �

��     Press the L key or F10 for HP Laser/Desk Jet.              �  �

��     Press the G key or F09 for IBM graphics dot matrix.        �  �

��     Press the E key or F08 for Epson FX, JX or LQ.             �  �

��                                                                �  �

��   Press a Key to initiate Graphics...                          �  �

�����������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

CCSIM displays Shewhart Control charts in the format shown below...

<--- First 50 Observations ---> � <--- Second 50 Observations --->

�������������������������������������������������������������������Ŀ

�  Beyond A                       �  Normal Theory Probability = 0.001

+3�� �� �� �� �� �� �� �� �� �� �� ��� �� �� �� �� �� �� �� �� �� �� Ĵ

�    Zone A                       �  Normal Theory Probability = 0.022

+2� � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � �

�    Zone B                       �  Normal Theory Probability = 0.136

+1� � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � �

�    Zone C                       �  Normal Theory Probability = 0.341

0�������������������������������������������������������������������Ĵ

�    Zone C                       �  Normal Theory Probability = 0.341

-1� � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � �

�    Zone B                       �  Normal Theory Probability = 0.136

-2� � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � �

�    Zone A                       �  Normal Theory Probability = 0.022

-3�� �� �� �� �� �� �� �� �� �� �� ��� �� �� �� �� �� �� �� �� �� �� Ĵ

�  Beyond A                       �  Normal Theory Probability = 0.001

���������������������������������������������������������������������

Press the "R" key or the SPACEBAR repeatedly while a CCSIMU control

chart is being displayed for on-screen detection/display of "Run-Rule"

violations, as follows...

Control Chart Simulator[ver.9402]                     Page 7 of 12

Rule #1:    1 value beyond zone A.

Rule #2:    9 in a row on same side of the centerline.

Rule #3:    6 in a row strictly increasing or decreasing.

Rule #4:   14 in a row alternating between up and down.

Rule #5:    2 out of 3 in zone A or beyond.

Rule #6:    4 out of 5 in zone B or beyond.

Rule #7:   15 in a row in zone C on either side of center.

Rule #8:    8 in a row within zones A or B (i.e. none in zones C).

Note One: In Run-Rules #3 and #4, CCSIM uses an "efactor" of 0.005 to

decide whether consecutive observations are going UP, DOWN or staying

essentially EQUAL.

if( value[n] > ( value[n-1] + efactor ) )

...going UP

else if( value[n] < ( value[n-1] - efactor ) )

...going DOWN

else

...staying EQUAL

Similarly, in Run-Rule #2, CCSIM again uses the "efactor" of 0.005 to

determine whether observations are strictly ABOVE the centerline

(value[n] > efactor) or strictly BELOW (value[n] < -1.0 * efactor.)

Note Two: View as many graphics screens as you wish before pressing

the Q, X or ESCape key to halt simulation and return to the CCSIM Main

Menu.  You may simply press, say, the ENTER key to generate the next

100 observations.  Press the R key or the SPACEBAR repeatedly for

on-screen display of individual Run-Rule violations.  Six keys (L, G,

E, F10, F09 and F08) cause a screen dump to be sent to your slave

printer on parallel port LPT1; most other key presses simply cause the

next graphics screen to appear.  There is a limit to 30,000 simulated

observations, but that would be 300 display screens!

���������������������� Control Chart Simulator ���������������������Ŀ

�                ������������������������������������ͻ              �

�                �            Main Menu               �              �

�                �   P = set  Parameter values        �              �

�                �   V = View simulated charts        �              �

�                �   S = Save Summary Statistics      �              �

�                �   X = eXit from the CCSIM system   �              �

�                �   choice --> S                     �              �

�                ������������������������������������ͼ              �

����������������������������������������������������������������������

Control Chart Simulator[ver.9402]                     Page 8 of 12

���������������������� Control Chart Simulator ���������������������Ŀ

��������������������� Save Summary Statistics �������������������ͻ  �

��                                                                �  �

��   Saving Control Chart Simulation Results...                   �  �

��                                                                �  �

��   Specify filename to save OUTPUT [ccsim.ccs] : test1          �  �

��   The CCSim Output file is to be: test1.ccs                    �  �

��                                                                �  �

��   Number of Observations Simulated = 30,000.                   �  �

��                                                                �  �

��   All Observations have True MEAN = 0.00.                      �  �

��   Observed Process MEAN =   0.01.                              �  �

��                                                                �  �

��   All Observations have True STD.DEV. = 1.000.                 �  �

��   Observed Process STD.DEVIATION =  1.001.                     �  �

��                                                                �  �

��   Press a Key to Continue...                                   �  �

�����������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

���������������������� Control Chart Simulator ���������������������Ŀ

��������������������� Save Summary Statistics �������������������ͻ  �

��                                                                �  �

��   Observed MAXIMUM =  3.784.                                   �  �

��   Percentage >=  4          was    0.0.                        �  �

��   Percentage >=  3 and <  4 was    0.1.                        �  �

��   Percentage >=  2 and <  3 was    2.2.                        �  �

��   Percentage >=  1 and <  2 was   13.8.                        �  �

��   Percentage >=  0 and <  1 was   34.3.                        �  �

��   Percentage >= -1 and <  0 was   34.0.                        �  �

��   Percentage >= -2 and < -1 was   13.4.                        �  �

��   Percentage >= -3 and < -2 was    2.1.                        �  �

��   Percentage >= -4 and < -3 was    0.2.                        �  �

��   Percentage  < -4          was    0.0.                        �  �

��   Observed MINIMUM = -4.062.                                   �  �

��                                                                �  �

��   Press Q to QUIT now...Other Key to Continue...               �  �

�����������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

Control Chart Simulator[ver.9402]                     Page 9 of 12

���������������������� Control Chart Simulator ���������������������Ŀ

��������������������� Save Summary Statistics �������������������ͻ  �

��                                                                �  �

��   Run-Rule No. 1 :   89 Violations =>   0.297 Percent.         �  �

��   Run-Rule No. 2 :  112 Violations =>   0.406 Percent.         �  �

��   Run-Rule No. 3 :   19 Violations =>   0.067 Percent.         �  �

��   Run-Rule No. 4 :   52 Violations =>   0.199 Percent.         �  �

��   Run-Rule No. 5 :  167 Violations =>   0.568 Percent.         �  �

��   Run-Rule No. 6 : 1028 Violations =>   3.569 Percent.         �  �

��   Run-Rule No. 7 :   69 Violations =>   0.267 Percent.         �  �

��   Run-Rule No. 8 :    0 Violations =>   0.000 Percent.         �  �

��                                                                �  �

��   Press Q to QUIT now...Other Key to Continue...               �  �

�����������������������������������������������������������������ͼ  �

����������������������������������������������������������������������

Note that Run-Rules #6 and #5 have relatively large false-alarm rates.

Exact results for these two rules are as follows:

Pr[ 2 or 3 of 3 in A+ ] = 3*(.0455)^2*(.9545) + (.0455)^3 = 0.00602.

Pr[ 4 or 5 of 5 in B+ ] = 5*(.3173)^4*(.6827) + (.3173)^5 = 0.03782.

���������������������� Control Chart Simulator ���������������������Ŀ

�                ������������������������������������ͻ              �

�                �            Main Menu               �              �

�                �   P = set  Parameter values        �              �

�                �   V = View simulated charts        �              �

�                �   S = Save Summary Statistics      �              �

�                �   X = eXit from the CCSIM system   �              �

�                �   choice --> X                     �              �

�                ������������������������������������ͼ              �

����������������������������������������������������������������������

REMINDER(S) :

CCSIM created an output file named: test1.ccs

Use the DOS invocation... TYPE test1.ccs | MORE

to review detailed information about this

run of the Control Chart Simulator.

��������������������������������������������������Ŀ

�    Smoothing with a Moving Average or an EWMA    �

����������������������������������������������������

Here we denote a series of observed values on a quantitative variable,

x, as follows:

..., x(s-4), x(s-3), x(s-2), x(s-1), x(s), ...

The Moving Average (MA) within a Window of Width W ending at

sequential position s is defined by:

Control Chart Simulator[ver.9402]                     Page 10 of 12

MA(s) = [ x(s-W+1) + ... + x(s-1) + x(s) ] / W.

The Exponentially Weighted Moving Average (EWMA) at sequential

position s is defined recursively in terms of the observed x value at

position s-1 and the previous value of the EWMA:

EWMA(s) = �*x(s-1) + (1-�)*EWMA(s-1),

where the proportion "�" is strictly positive but no greater than one,

0 < � <= 1.  Note that EWMA(s) can be computed before x(s) is actually

observed; EWMA(s) is usually interpreted as a "forecast" for x(s).

The � proportion is referred to here as the "Short-Term Memory Weight"

because the numerical value, x(s-1), observed immediately previously

to x(s) gets weight "�" when computing EWMA(s) [and the weights given

to all previous observations total to (1-�).]

CCSIM simulates a stable ("in-control") process of observations that

are statistically independent with common mean, �, and common

variance, ��.  The expected values of the MA and the EWMA are also �

in this case, and their variances are

Var(MA(s))   =  �� / W ,

and

Var(EWMA(s))  =  �� * �  / ( 2 - � ) .

As is clear from the above formula, the variances of a MA or EWMA will

always be less than  ��, the variance of a single observation, when

W > 1 and � < 1.  And the variance of a MA or EWMA will be much, much

less than �� when W is large or � is rather small, like 0.10 or less.

���������������������������������������Ŀ

�   Other Quality Monitoring Software   �

�   in the "QA Training Tools" Series   �

�����������������������������������������

EWMAS.EXE : Exponentially Weighted Moving Average Simulations

(73kBytes, January 1992)

Quality Assurance Training CGA-graphics and Simulations:  Fits

Exponentially Weighted Moving Average models to input data by

least-squares, providing a very simple form of time-series analysis.

The unique feature is that the "statistical significance" of the

fitted Short-Term-Memory Weight coefficient is informally tested by

simulating its distribution under random permutations of the

sequential order of observations.

EWMARL.EXE : Exponentially Weighted Moving Average Run Lengths

(96kBytes, June 1990)

Quality Assurance Training.  Interactive CGA-graphical displays of

Average Run Lengths (ARL) for Exponentially Weighted Moving Average

(EWMA) control schemes.  The archive includes Fortran source code and

a compilation of the algorithm of Saccucci and Lucas, "Average Run

Lengths for Exponentially Weighted Moving Average Control Schemes

Control Chart Simulator[ver.9402]                     Page 11 of 12

Using the Markov Chain Approach," Journal of Quality Technology 22,

154-162 (1990).

EWSHEW.EXE : Combined EWMA-Shewhart Quality Control Schemes

(88kBytes, April 1992)

Quality Assurance Training VGA Graphics:  Use a combination of

Shewhart limits on both observations from a process and on their

Exponentially Weighted Moving Average (EWMA.)  Automatic detection of

out-of-control situations and calculation of run-length statistics;

extensive literature references.  KeyBoard or BatchFile input of up to

500 sequential observations.  Menus/Prompts interface.

QMPAL.EXE : Quality Measurement Plan & Alternative Trend Charts

(103kBytes, October 1992)

Quality Assurance Training CGA-graphics:  Makes 3 kinds of QMP

Poisson/Gamma Theory Charts (I-plots, Forecasts, and Process Average

Boxes), QMP Normal Theory Charts, Exp.Wgt.Mov.Index Forecasts, and

Shewhart t-Rate Charts.  Easy to use "bounce-bar" menu for BATCH input

file selection.  All prompts have [default] values.

QMPSIM.EXE : Quality Measurement Plan Simulation Scenarios

(87kBytes, February 1992)

Quality Assurance Training CGA-graphics and Simulation...Create

arbitrary True-Quality Scenarios and estimate the resulting

Poisson-theory "coverage probabilities" of Quality Measurement Plan

(nominal 90%) confidence intervals.  Easy to use "bounce-bar" menu for

BATCH input file selection.  All prompts have [default] values.

QTM93.EXE : Quality Trend Monitoring ...by Month

(370kBytes, November 1993)

Quality Assurance Training VGA-graphics:  Major dBASE/Clipper system

for quantifying the long range impact of process deviations from an

intended target value using any cost-of-poor-quality regret (loss)

function from ten general fanilies.  A user-friendly menu system for

all phases of operation ...from process specification, to data entry

for individual batches, to accumulation of results by month, to

composite of component processes, to display of VGA trend charts.

VMASK.EXE : Cumulative Sum Control Charts with Movable V-Mask

(79kBytes, May 1992)

Quality Assurance Training VGA-graphics:  Use a movable V-mask to find

statistically significant shifts on a Cumulative Sum (CUSUM) chart.

Plus automatic (numerical) detection of out-of-control situations,

calculation of run-lengths, and estimation of process averages.

Keyboard or batch-file input of up to 500 sequential observations.

Control Chart Simulator[ver.9402]                     Page 12 of 12

�����������������Ŀ

�    REFERENCES   �

�������������������

Hunter, J.S. (1986).  "The exponentially weighted moving average."

Journal of Quality Technology 18, 203-210.

L'Ecuyer, P. (1988).  "Efficient and Portable Combined Random Number

Generators."  Communications of the ACM 31, 742-749,774.

Lucas, J.M. and Saccucci, M.S. (1990).  "Exponentially weighted moving

average control schemes: properties and enhancements."

Technometrics 32, 1-12. (with discussion: 13-29.)

Nelson, L.S.  (1983).  "The Deceptiveness of Moving Averages."  Journal

of Quality Technology 15, 99-100.

Park, S. K. and Miller, K. W.  (1988).  "Random Number Generators: Good

Ones are Hard to Find."  Communications of the ACM 31, 1192-1201.

Press, William H., Flannery, B. P., Teukolsky, S.A., and Vetterling, W.

T.  (1988). "Numerical Recipes in C: The Art of Scientific

Computing."  [especially Chapter 7:  Random Number Generation.]

Cambridge University Press. { Source Code Disk:  Copyright 1987

by Numerical Recipes Software, P.O.Box 243, Cambridge, MA 02238.}

Roberts, S.W. (1959).  "Control charts based upon geometric moving

averages."  Technometrics 1, 239-250.

CCSIM Update History:

===================

9210 ...Beta test version,

9302 ...Add possible change point after first 50 observations on

each screen.  First Upload to CompuServe.

9311 ...Always display the original data series as well as any

(finite window) Moving Average or Exponentially Weighted MA.

9312 ...Fix bug in reporting minimum of the 2nd-50 observations.

9402 ...Add detection/display of "run-rule" violations.