*****************************

*                           *

*   ARCHITECTURAL SYMBOLS   *

*                           *

*****************************

FOR CURVE DIGITIZER*

On - Disk Reference Manual

Revision A

July 1987

Copyright 1987 by:       Builders Design Service

Suite 721

P.O. Box 2714

Huntington Beach, CA  92647

All rights reserved.

* Curve Digitizer is a product of

West Coast Consultants, San Diego, CA

ARCHITECTURAL SYMBOLS for Curve Digitizer                  Page 1

LICENSE

ARCHITECTURAL SYMBOLS is supported by the users who find it

productive and it is distributed under the "shareware" concept.

It is not public domain software, nor is it free software.  But,

you may give away COMPLETE UNALTERED copies of this two disk

package to your friends and PC user groups for trial use.

Non-registered users are granted a limited license to use

ARCHITECTURAL SYMBOLS on a trial basis for the purpose of

determining whether it is suitable for their needs.

USERS IN A COMMERCIAL ENVIRONMENT must be registered.  In the

case of multiple copies of ARCHITECTURAL SYMBOLS in use in one

business establishment, each copy which is in simultaneous use

must be registered.

Distributors of public domain or user supported software may

distribute copies of ARCHITECTURAL SYMBOLS only after obtaining

written permission from Builders Design Service.  Please write

for details.

A registration form can be printed by placing disk 1 in drive A

and typing  COPY REGIS.DOC PRN  after the A> prompt.

If these symbols are of use to you, become a registered owner to

receive the following benefits:

*  Loose-leaf manual (add to your Curve Digitizer program

manual) including additional information on the use of

symbols, facilitating dimensioning and the design and

creation of useful symbols.

*  The latest version of the software with your own unique

serial number that we keep on file under your name.

*  One year commission entitlement of $5.00 for each new

paid registration which shows your serial number.

Commissions paid quarterly.

*  Symbol charts showing the symbols, their names and

origin points.

*  Cross reference lists of the symbols by file name,

discipline, category and detailed description.

*  One year of phone support and published update notices.

DISCLAIMER

Both ARCHITECTURAL SYMBOLS and this documentation are distributed

without any expressed or implied warranties.  No warranty of

fitness for a particular purpose is offered.  Builders Design

Service assumes NO responsibility for any sort of damage

resulting from the use of ARCHITECTURAL SYMBOLS.  This software

contains no executable programs. All symbol data files are

intended to be used with the Curve Digitizer program which is a

product of West Coast Consultants of San Diego, California.

Compatibility of the symbol data files with versions 2 and 3 of

Curve Digitizer has been demonstrated.

No claim is offered that the symbols agree with accepted

standards of any kind.  The author has used many sources during

the preparation of the various symbols and has in some cases

found conflicts in symbol representations.

ARCHITECTURAL SYMBOLS for Curve Digitizer                  Page 2

INTRODUCTION

ARCHITECTURAL SYMBOLS is a collection of more than 350 data files

which are compatible with Curve Digitizer, a CAD program product

of West Coast Consultants, San Diego, California.

The SYMBOL command (S) of MENU 5 (F5) is used to bring already

stored data into a plot.  This operation is the computer

equivalent of using a template in conventional drafting. However,

the computer symbols can be far more complex than any shape which

can be created by holes and slots in a piece of plastic.

The data files are divided into categories and a disk

subdirectory for each category is provided.  This organization

was chosen to make use of the data files easier when the computer

does not have a hard disk drive.  The categories and disk

subdirectory names are:

DISK 1

*  architectural plan symbols         (ARCHPLAN)

*  architectural elevation symbols    (ARCHELEV)

*  electrical symbols                 (ARCHELEC)

DISK 2

*  mechanical symbols                 (ARCHMECH)

*  plumbing symbols                   (ARCHPLUM)

*  miscellaneous symbols              (ARCHMISC)

*  words of text                      (ARCHTEXT)

The SYMBOL file names are as descriptive as possible within the

limitation of the eight characters of a file name.  Vowels are

often dropped from the spelling of a word and dimensional data

are compressed to make it possible to differentiate two or more

versions of the same symbol.  For example, MWAV2416 is a built-in

microwave oven, 24" wide by 16" high (nominal).  DR2-8LH is a

plan view 2'-8" left-hand door.

Users with a computer that is equipped with a hard disk drive may

copy all the files into a single subdirectory (preferably called

'symbols') that can be specified in the configuration setup for

symbol access.  This will eliminate the relatively slow access

time of a floppy drive and the need to swap floppy disks when the

needed symbol is not on the disk presently in the drive.

ARCHITECTURAL SYMBOLS for Curve Digitizer                  Page 3

SYMBOL DESIGN CRITERIA

Rules or conventions must be established before a universal set

of anything can be created.  The conventions discussed below

represent the outcome of a lot of head scratching by the author

during the early weeks of working with the CAD program.

Three pen sizes and three consistent pen position numbers are

used.  Line weight, not color, is the distinguishing factor in

separating line types, i.e. dimension lines, object lines,

section cut lines.  Three line weights, obtainable with

disposable plotter pens, are used.  They are achieved by using

pen stall positions 1, 3 and 5; with .35mm, .5mm and .7mm pens in

each, respectively.  When using an EGA video board and display,

the different pen stall numbers are shown in corresponding

colors.  When using a monochrome display, the line flash function

can help to identify the 'color' of lines.

One foot is represented by one unit on the plot frame (not

necessarily one tick mark on the border).  12 units are 12 feet,

not 12 inches.  A drawing which is to plot out as 1/4" = 1'-0"

will show a scale ratio of 4.000 in the command message area.

The symbols are designed to be a good size on plots which are

1/4" = 1'-0" scale.

When possible, a data file is "colorless" and does not alter the

color (pen number) that has most recently been specified.

Each symbol has an origin point.  The origin point may be at the

center of the symbol, at a corner, at the end of a line or

completely detached from the symbol.  Common sense was applied to

choosing the origin points.  Generally, symbols which are likely

to be merged with other symbols have a center origin point.

Symbols which are typically referenced to a wall usually have

origins such that the symbol will be produced at its proper

spacing from a wall if the cursor is placed at the edge of the

wall before calling the symbol.

The ROTATE and RE-SCALE features of the SYMBOL command often make

it possible for one symbol file to be the basis for an almost

infinite set of symbol clones.  Rotating a symbol is like

rotating a template, while re-scaling a symbol is like having a

set of templates with different size figures.

Symbols names are necessarily short because of the DOS limitation

of 8 characters in a file name.  (The suffix of all data files is

.DAT).  Where possible, abbreviations of the full name have been

used.  In many cases, however, extreme contraction of the name

has been necessary to permit qualifying information to be

included in the file name.  For example, ERDUPGFI is the file

name for an <E>lectric <R>eceptacle, <DUP>lex convenience style,

<G>round <F>ault <P>rotected which is different than a ERDUP

(<E>lectric <R>eceptacle, <DUP>lex convenience style) which is

the common wall socket found in homes.

ARCHITECTURAL SYMBOLS for Curve Digitizer                  Page 4

SYMBOL MODE VERSUS DIMENSION MODE

Symbols may often be used to accomplish what is otherwise done

with the DIMENSION (D) command of MENU 6 (F6).

Lettering and text may be added to a plot by using the SETUP and

TEXT subcommands of the DIMENSION (D) command of MENU 6 (F6).

However, this may be somewhat awkward and inefficient because of

the time required to set up the TEXT sub-command and the loss of

background layers if one zooms around the plot (known limitations

of version 2 and 3 of Curve Digitizer).

Lettering and text may be added to a plot as symbols, with all

the symbol manipulation capabilities at your disposal.  This can

be significantly more efficient when dealing with individual

letters and words that are routinely used in a set of drawings.

Dimensioning consists of two categories of operations; first,

constructing the witness lines and second, calculating and

placing the numbers of the dimension value.  In the DIMENSION (D)

command mode, both operations can be performed simultaneously.

However, the loss of the visibility of layers when zooming often

makes it impossible to dimension part of a drawing.  And, the

burden of high data point counts slows down screen refresh.

Placement of witness lines without the associated dimensions

allows one to zoom out, view the overall clarity and

effectiveness of the dimensioning scheme, zoom in and edit, etc.

This is similar to doing a quick layout with a hard grade pencil

in manual drafting.  Overall dimensions can be placed

sufficiently far away from the object so that subsidiary

dimensions can be properly located for ease of reading the

drawing.

Witness line constructs can be placed as symbols (while in the

normal drawing modes) and adjusted and edited to suit before the

data point intensive dimensions are added.  Three witness line

construct categories are provided in this version of

ARCHITECTURAL SYMBOLS.  EW1, EW2, EW3 and EW4 are long to short

versions of witness lines for corners and edges.  IW1, IW2, IW3

and IW4 are long to short versions of witness lines for

dimensioning to the center of <I>nternal <W>alls.  CL1, CL2, CL3

and CL4 are long to short versions of <C>enter <L>ines to

dimension window and door opening centerlines.

After the witness lines are satisfactory, go to the DIMENSION (D)

mode of MENU 6 (F6) and use SCALAR dimensioning to put dimension

lines and values between the existing witness lines.

ARCHITECTURAL SYMBOLS for Curve Digitizer                  Page 5

SIMILAR SYMBOLS AND UNIT SYMBOLS

Some symbols are part of a set, such as different size doors, and

some are what will be referred to as "unit" symbols which can be

adjusted to suit.

When a symbol may be needed in different sizes, such as doors,

and when scaling the symbol would distort any of its properties,

a set of similar symbols is provided.  For example, a hinged door

could be scaled to fit any size opening.  But, it would have to

be scaled in both X and Y directions to maintain the swing arc as

a circular segment.  This would cause the door thickness to

change by the scale factor.  If a 2'-0" door was scaled to fit a

3'-0" opening, the thickness of the door would increase by one

half and the resulting symbol would be a 'fat' door.

A "unit" symbol is one which can be stretched in one direction

with the resulting shape being the correct result.  For example,

the common plan view window symbol is typically placed in a

nominal 6" thick wall and of a length to suit.  The symbol

WNFXHOR (<W>i<N>dow, <F>ixed, <HOR>izontal view) fits between two

lines 1/2 unit apart and is one unit long (remember, our symbols

are based on a relationship of one unit = one foot).  It can be

scaled in the length direction when placed on the plot to become

a window of any length.  A 4'-0 window results from a scale

factor of 4.  A 3'-6" window results from a scale factor of 3.5

(oops, did you miss the math?).

THE MISCELLANEOUS (MISC) DIRECTORY

Several versions of drawing borders, title blocks and drawing

names are contained in this directory.  These files are intended

for use with a Houston Instruments DMP 52 plotter and 24" by 36"

paper.  While this may not be directly applicable to your

situation, it may serve as an example of "non-symbol" symbols.

The DGBRD--- files can be called as symbols, but since they are

intended to be borders, they may plot outside the viewable area

on your screen.  Remember that calling a symbol does not adjust

the parameters of the XY LIMITS.  You may use the XYLMTS (X)

command of MENU 4 (F4) or the ZOOM (Z) command of MENU 3 (F3) to

adjust the viewing area such that you can view the border.  As an

alternative, the border files may be read by the READ (R) command

of MENU 4 (F4) and the X and Y limits will be set accordingly.

ARCHITECTURAL SYMBOLS for Curve Digitizer                  Page 6

CUSTOM SYMBOLS BY COMBINATION

Many of the symbols can be combined with symbol parts to create

custom symbols.

Letters and numbers can be easily added to symbols by using the

S0 through S15 and SA through SZ symbol parts.  Each of these

symbol parts has its origin at the center of the letter or

number.  Place the cursor at the appropriate position and call

the symbol.

Words from the ARCHTEXT subdirectory can be added to otherwise

ambiguous shapes to create unique understandable symbols.

Have you created some symbols that you need and that might be

incorporated into future versions of ARCHITECTURAL SYMBOLS?  If

so, send us a copy of your symbols (on paper preferably for

simple symbols and on disk for complicated symbols).  Include

your name and registered serial number so that we can extend your

entitlement period for commissions, phone support and update

notices.

REGISTRATION BENEFITS

If the symbols of ARCHITECTURAL SYMBOLS are of use to you, become

a registered owner to receive the following benefits:

*  Loose-leaf manual (add to your Curve Digitizer program

manual) including additional information on the use of

symbols, facilitating dimensioning and the design and

creation of useful symbols.

*  The latest version of the software with your own unique

serial number that we keep on file under your name.

*  One year commission entitlement of $5.00 for each new

paid registration which shows your serial number.

Commissions paid quarterly.

*  Symbol charts showing the symbols, their names and

origin points.

*  Cross reference lists of the symbols by file name,

discipline, category and detailed description.

*  One year of phone support and published update notices.

A registration form can be printed by placing disk 1 in drive A

and typing  COPY REGIS.DOC PRN  after the A> prompt.

ARCHITECTURAL SYMBOLS for Curve Digitizer                  Page 7

#-- Copyright 1987 by Builder's Design Service --�����������������������������������������������������������������������������������������������������������������������������������������������������