COMPUTER  PROGRAM  FOR BAND-PASS FILTERS USING  CAPACITOR-COUPLED

RESONATORS

Written by Stanley H.  Klug,  ca.  1963.  Edited by D. McGarrett,

1983.  (Based on Stan Klug's notes.)

The  class of filters which this program designs is analogous  to

probe-coupled  reentrant cavity resonators.   The user should  be

aware that high-frequency cut-off characteristics are not as good

as predicted, due to finite inductance in the shunt capacitors.

The  program computes Chebychev or Butterworth filters with up to

N=9  shunt L/C resonators surrounded by series  coupling  capaci-

tors.   The  program  will choose the average value of L  between

theoretical limits,  or,  if L is given and is outside these  li-

mits, the program will use the average value of inductance.

Input impedance is R1,  resistive.   Output load is a  resistance

R2>=R1.   R2  can  be infinite.   If infinity is greater than  R2

which is greater than R1,  then R2 may exceed theoretical limits,

which the program will indicate.

If the number of poles, N, is given as 0, the program will select

up  to  nine poles to give specified attenuation at  a  specified

stop-band frequency.

In order to use the program, enter the following data:

Line  10:   Type "10 DATA A1,F1,F2"  A1 specifies units:   1  for

megacycles,  microhenries,  and  picofarads;   2 for  kilocycles,

millihenries, and microfarads;  3 for cycles per second, henries,

and  microfarads.   Stick with the units you chose when  entering

data.    F1  and F2 are the lower and upper  cutoff  frequencies.

These  are the -3dB points for a Butterworth filter,  and can  be

either the -3dB or the valley frequencies for a Chebychev.

Line  20:   Type  "20 DATA R1,R2,D"  R1 is the source  impedance,

which must be equal to or less than R2.   R2 is the output (load)

impedance,  and can be 1/0,  or infinity,  in which case, enter 0

for  R2.   D  is  any  value up to 3dB  ripple  for  a  Chebychev

filter,and h� 0 for a Butterworth.

Line 30:   Type "30 DATA A2,N,L"   A2 equals 3 for a  Butterworth

filter,   or  for  a  Chebychev,  if  F1  and  F2  are  the  -3dB

frequencies.   A2  equals 0 for Chebychev if F1 and F2 are valley

frequencies.   N is the number of poles,  which can be 2 to 9, or

enter  0 and the program will select the number of  poles,  based

upon  the desired bandwidth and stop-band  attenuation.  Enter  a

selected value for L,  or enter 0 and the program will choose the

value.

Line  40:   Type "40 DATA F9,D9"   (If N was not 0,  do not enter

D9.   The  program will compute attenuation at  F9,  a  frequency

which  may not fall between F1 and F2.)  If N was set equal to 0,

enter  D9  as attenuation desired at F9 (in dB) and  the  program

will select N up to 9 poles.