Sometimes it is necessary to have S-parameters of two port devices, such as

GaAs FETs for example, given as three ports.

FETS are almost always measured as two port devices, with the source

leads grounded.   Published S-parameter data for GaAs FETs is almost always

presented this way.  While two port s-parameters measured this way

are satisfactor for a variety of circuit simulations, there are many

instances where three port s-parameters for the FET are needed.   When the FET

is used in either a common-gate or common-drain configuration, or if the

source is used as part of a feed-back ciruit, or whenever the FET's source

isn't directly grounded are just a few examples of when it would be helpful to

have the FET's three port s-parameters.

In other words, you have a three-terminal device, and you want to use it as a

three port device, but you only have two port S-parameters.  How do you derive

the three-port S-parameters from the two-port S-parameters?

The computer program described in this article provides a means for converting

two port s-parameters into three port s-parameters.  It is completely

general, so of course it will work on any set of two port s-parameters and

convert them into three port s-parameters.

Most top of the line microwave CAD programs, such as TouchStone and Super

Compact, do this for you automatically.  You just enter in the three

circuit nodes the device connects to and the program does the rest.

The algorithim this article is based upon duplicates what is already available

in these other programs.

This article, therefore, is primarily targeted at those who have less

sophisticated programs and need to enter in three port data, or for those who

may have the more expensive programs and just want to know how to do the

derivation.

Step 1: Convert the S-paramters of the two port into Z parameters

Step 2: Add the Z-parameters of a 50 ohm resistor to the Z-parameters

calculated above.  This has the effect of putting a 50 ohm resistor in the

common lead.

Step 3: Converting the  Z-parameters of step 2 back into S-parameters

gives some of the sought after three-port S-parameters, in a two-port

format, specifically; S11, S12, S21, and S22 of the three port.

Step 4: Convert the Z-parameters from step 1 into ABCD parameters.

Step 5: Re-calculate these ABCD parameters by first connecting a resistor of

charactistic impedance with the output port, and then interchanging the ouput

port with the common.

Step 6: Converting the ABCD parameters from Step 5 back into S-parameters

gives additional elements of the the three-port S matrix as follows:   S12

calculated this way is really the S13 for our three-port, similarly, S22 is

S33, and S21 is S31.

Step 7: Going back to the ABCD parameters produced in step 4,

re-calculate the ABCD parameters by interchanging the common port with

the input port after first putting a resistor of characteristic impedance in

the input port.

Step 7: Repeat step 6.  Now S12 is S32, S21 is S23.  Also, S11 will be

S33 and S22 will be S22, but these will be redundant, as they're already

been calculated in previous steps.

At this point all the three-port S-parameters have been calculated.

Step 8:  Convert these S-parameters back into polar form and print them out.

As an example of the us of this program sample output is shown for converting

the published two port S-parameters for a GaAs FET into three port

S-parameters.  The results agreed exactly with the output from TouchStone.

This program was written in compiled Microsoft (TM) FORTRAN for an IBM XT.  It

could be re-written in other languages or for use other compilers.  The

advantage of writing circuit analysis programs in FORTRAN over other

languages such as BASIC, for example, is that most FORTRAN compilers readily

handle complex numbers.  If this algorithim were written in BASIC, extra

subroutines would have been necessary to handle the multiplication, addition,

subtraction and division of complex numbers.