�����������������������������������������������������������������Ŀ

�      *** One, Two, and Three Stage Acceptance Sampling ***      �

�                                                                 �

�                  Stage123.EXE....version 9310                   �

�                                                                 �

�               A Quality Assurance Training Tool:                �

�        Statistics Committee of the QA Section of the PMA.       �

�                                                                 �

�             Bob Obenchain, CompuServe User [72007,467]          �

�������������������������������������������������������������������

Note:  PMA = Pharmaceutical Manufacturer's Association

Stage123.EXE uses a binomial probability model to calculate the exact

"operating characteristic" (expressed as a first-pass yield percentage)

when production lots of given percentages of nonconforming units are

subjected to a specified single-, double-, or triple-stage acceptance

sampling plan.  Highly portable C-language source code is included; no

graphics.

TABLE-OF-CONTENTS

=================

Stage123.EXE.................................Executable Module

STAGE123.CSV and .IN..................ASCII Output/Input Files

PROMPTS and DEFAULTS..................Minimum & Maximum Values

NUMERICAL EXAMPLES..............Schilling, Acceptance Sampling

Stage123.C..............................C-language Source Code

*********************************************************************

Stage123.EXE

This module calculates the expected percentage of acceptable lots

in single, double, or triple stage binomial sampling.  In other

words, Stage123 computes the so-called "Type B" Operating

Characteristic Curve giving the probability of individual lot

acceptance as a function of the true, underlying fraction

nonconforming in a manufacturing process.

Testing of each sampled unit is assumed to yield observance of

one of two possibilities...

...a Conformance to Specifications

(say, a result within 25% of target), or

...a Nonconformance

(say, deviation from target by more than 25%).

Stage123.EXE, ver 9310 . . . . . . . . . . . . . . . . Page 2 of 7

Stage123 then calculates

...Percentage of Lots expected to be Accepted (YIELD), and

...Average Outgoing Quality (for selected Re-Work Efficiency)

for each Within-Lot Percentage of Nonconforming Units that is

specified.

Stage123.EXE is an executable module for IBM-compatible personal

computers.  It was compiled using Microsoft C, version 6.00AX.

Stage123.EXE thus provides automatic detection (and use or

emulation) of an 8087/80287/80387 numeric math coprocessor.  If

your PC has a math chip, Stage123 should run much faster.  If your

PC does NOT have a math chip, Stage123 will still run and should

yield identical numerical results.

Stage123.IN

Stage123 prompts the user to choose between Keyboard (K) and ACSII

Batch file (B) input of sample sizes and accept/reject numbers as

well as input of alternative true values for the underlying

percentage nonconforming in the process.

Batch input must come from a file with the filename extension

".IN" (without the quote marks.)  The format for Stage123 batch

input files is as follows...  The first line of the file contains

the filename (at most 8 characters) of the .CSV output file.  Each

successive line starts with a parameter value; only the first

number on each line of the file is utilized as input.  This number

may be either an integer or a floating point number - but must be

separated from other information on the same line by a comma or at

least one "white" space (blank or tab.)

For example, here is a listing of the EX139.IN batch input file:

ex139 = .CSV filename

13 = First Stage Sample Size

0 = First Stage Accept Number

2 = First Stage Reject Number

13 = Second Stage Sample Size

1 = Second Stage Accept Number

2 = Second Stage Reject Number

100.0 = Testing Efficiency %

100.0 = Re-Work Efficiency %

0.00 = 1st Percentage Nonconforming

1.00 = 2nd Percentage Nonconforming

2.00 = 3rd Percentage Nonconforming

3.00 = 4th Percentage Nonconforming

4.00

5.00

6.00

7.00

Stage123.EXE, ver 9310 . . . . . . . . . . . . . . . . Page 3 of 7

8.00

9.00

10.00

12.00

14.00

16.00

18.00  ...Example of Schilling, page 139

20.00

-1 ...End of Input Signal...

The easiest way to create a Stage123 batch input file is to start

with an existing Stage123 batch input file and to simply change

its parameter settings, deleting any unnecessary lines.

Stage123.CSV

This is the default filename for the ASCII summary file of inputs

and outputs from Stage123.EXE.  You may specify an alternative

filename for this file when prompted.  The filename extension of

.CSV stands for "Comma Separated Values," which puts the output

in a format convenient for importing into a spreadsheet system,

such as Excel, to graph the calculated Operating Characteristic

curve.

You can read/print this file after the program terminates, so you

do not need to make hand-written notes during Stage123 execution.

And the easiest way to create a Stage123 .IN file is to edit an

existing .CSV or .IN file to use whatever new sampling parameters

and/or process characteristics you desire.

For example, here is a listing of the EX139.CSV output file:

13, = First Stage Sample Size,

0, = First Stage Accept Number,

2, = First Stage Reject Number,

13, = Second Stage Sample Size,

1, = Second Stage Accept Number,

2, = Second Stage Reject Number,

100.0, = Testing Efficiency %,

100.0, = Re-Work Efficiency %,

In%NConf,%Yield,Out%NConf

0.00,100.00,  0.00

1.00, 97.86,  0.98

2.00, 92.59,  1.85

3.00, 85.51,  2.57

4.00, 77.56,  3.10

5.00, 69.36,  3.47

6.00, 61.34,  3.68

7.00, 53.76,  3.76

8.00, 46.76,  3.74

9.00, 40.42,  3.64

10.00, 34.75,  3.48

Stage123.EXE, ver 9310 . . . . . . . . . . . . . . . . Page 4 of 7

12.00, 25.36,  3.04

14.00, 18.27,  2.56

16.00, 13.03,  2.08

18.00,  9.22,  1.66

20.00,  6.48,  1.30

-1,...End of Input Signal...,

At the worst calculated incoming %NConf of   7.00,,

the Lower Bound for AOQL, %NConf is,   3.76.

PROMPTS and DEFAULTS

...Type responses to prompts using the number keys along the

top row of the keyboard, and then press the ENTER key.

...If you press the ENTER key without typing a value, your

response will be assigned to the DEFAULT value.

...The prompts are listed below with minimum, default, and

maximum allowed values.

Enter your First Stage Sample Size: n1

Minimum Sample Size =     2

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your First Stage Accept Number: c1

Minimum Acceptance Number =     -1

(i.e. lot acceptance impossible at stage 1

even if zero nonconformances are observed.)

Default Acceptance Number =      0

Maximum Acceptance Number =      min(n1,999)

Enter your First Stage Reject Number: r1

Minimum Reject Number =      max(c1+1,1)

(i.e. single stage sampling)

Default Reject Number =      max(c1+1,1)

Maximum Reject Number =      n1

Enter your Second Stage Sample Size: n2

Minimum Sample Size =     1

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your Second Stage Accept Number: c2

Minimum Acceptance Number =      c1

Default Acceptance Number =      c1

Maximum Acceptance Number =      min(n1+n2,999)

Enter your Second Stage Reject Number: r2

Minimum Reject Number =      c2+1

(i.e. two stage sampling)

Stage123.EXE, ver 9310 . . . . . . . . . . . . . . . . Page 5 of 7

Default Reject Number =      c2+1

Maximum Reject Number =      n1+n2

Enter your Third Stage Sample Size: n3

Minimum Sample Size =     1

Default Sample Size =    50

Maximum Sample Size = 5,000

Enter your Final Accept Number:  c3

Minimum Acceptance Number =      r2 - 1

Default Acceptance Number =      r2 - 1

Maximum Acceptance Number =      min(n1+n2+n3,999)

Specify Testing Efficiency (Percentage):

Minimum Testing Efficiency =    10%

Default Testing Efficiency =   100%

Maximum Testing Efficiency =   100%

Specify Re-Work Efficiency (Percentage):

Minimum Re-Work Efficiency =    10%

Default Re-Work Efficiency =   100%

Maximum Re-Work Efficiency =   100%

FIRST-PASS YIELD and AOQ Calculations...

...The user is repeatedly prompted to specify different

percentages of units within lots that are nonconforming.

These are the values of quality (% NonConf) "incoming" to

the acceptance sampling inspection.

...Enter values between 0.0% and 100.0%.

...Enter any negative value (say, -1) to halt execution of

Stage123.EXE.

...Stage123.EXE will calculate the "First-Pass Yield Percentage"

corresponding to each specified incoming % NonConf.  This

Yield % is 100 times the Probability of Lot Acceptance,

which defines the Operating Characteristic Curve for the

sampling plan.

...Stage123.EXE also calculates the "Average Outgoing Quality"

corresponding to each specified incoming % NonConf.  This

AOQ calculation assumes that lots are very large relative to

potential sample sizes (n1+n2+n3) and that all units in

"rejected" lots are screened (with specified Re-Work

Efficiency.)  In reality, rejected lots are usually scrapped

in the pharmaceutical industry because testing is

destructive.  As a result, the quality level outgoing

from an acceptance sampling inspection is essentially

identical to the incoming quality level (i.e. Re-Work

Efficiency = Zero.)

Stage123.EXE, ver 9310 . . . . . . . . . . . . . . . . Page 6 of 7

LOWER BOUND for the AVERAGE OUTGOING QUALITY LIMIT...

...The largest AOQ value observed using ONLY the specified

incoming % NonConf values is a lower bound  for the Average

Outgoing Quality Limit.  The true AOQL is the maximum AOQ

over all % NonConf values, including those for which

calculations may not have been specified.

NUMERICAL EXAMPLES...

EX139.IN, the 2-stage example of Schilling, pages 137-139.

========

n1 = n2 = 13; c1 = 0, c2 = 1; r1 = r2 = 2.

Testing Efficiency = Re-Work Efficiency = 100%

Using Stage123.EXE with % NonConf = 18% (which is p = 0.18

in Schilling's notation)...

First-Pass Yield = 9.22%

(i.e. Probability of acceptance = 0.092)

AOQ for very large lot size ( N = infinity )

= .18*(.076+.016)

= .01656 or 1.66%

Note: Schilling calculates AOQs for N = 120 and gets smaller AOQ

values because n1 = 13 and n1 + n2 = 26 represent about

10% to 20% of the N = 120 (finite) lot size.

EX141.IN, the 3-stage example of Schilling, page 141.

=====

n1 = n2 = n3 = 10; c1 = -1, c2 = 0, c3 = 1; r1 = r2 = r3 = 2.

Testing Efficiency = Re-Work Efficiency = 100%

Using Stage123.EXE with % NonConf = 1% (which is p = 0.01

in Schilling's notation)...

First-Pass Yield = 96.73%

(i.e. Probability of acceptance = 0.967)

Stage123.C

This is an ASCII file of C-language source code for Stage123.  The

only external functions called by Stage123 are the logarithm and

exponential functions, so the code should be highly portable.  For

example, to compile and run the program on a VAX under VMS, upload

Stage123.C to the VAX in ASCII mode and enter the commands...

Stage123.EXE, ver 9310 . . . . . . . . . . . . . . . . Page 7 of 7

VAX Prompt> cc stage123

VAX Prompt> link stage123

VAX Prompt> run stage123

REFERENCE:  Schilling, E. G., "Acceptance Sampling in Quality Control,"

==========              Marcel Dekker, New York, (1982).

Software Update History

=======================

8806  ...Original distribution

9310  ...Eliminate use of command line arguments

...Write output in .CSV format ideal for importing into

Microsoft Excel (and other spreadsheets.)