���������������������������������������������������������Ŀ

� �����������������������������������������������������Ŀ �

� �    Testing The Supplier Data Validity Hypothesis    � �

� �����������������������������������������������������Ĵ �

� �          A Quality Assurance Training Tool:         � �

� �  Statistics Committee of the QC Section of the PMA  � �

� �����������������������������������������������������Ĵ �

� �      Bob Obenchain, CompuServe User [72007,467]     � �

� ������������������������������������������������������� �

�����������������������������������������������������������

Contents:

[1] ...Introduction

[2] ...Testing Homogeneity of Two Poisson Frequencies

[3] ...New and Existing Tables

[4] ...Remarks on Acceptance Sampling Bias

[5] ...Tests of Validity Hypotheses using Software

[6] ...The Exact Test

[7] ...The Approximate Test

Section 1: ...Introduction

==========================

We consider the situation where the supplier provides the results from

his final sampling inspection on an accepted lot to the customer who

purchases that lot.  The customer then takes an additional sample to

detect any downward bias in the supplier's data.  A procedure that is

particularly well suited for quick, manual analysis of these sorts of

supplier-customer data is given by Ellner(1963).  This is the method

used in DOD Quality Control and Reliability Handbook (Interim) H109,

Office of the Assistant Secretary of Defense (Supply and Logistics).

Published tables for this one-sided testing procedure are rather

limited.

Although this methodology validates the supplier's inspection on any lot

or lots, it does not validate the actual acceptability of that material.

Acceptability is determined by the original supplier's inspection.  If

producer and customer inspection results on a lot are not found to

correspond, action is taken to determine the cause of the discrepancy.

The basic testing philosophy is really very simple...

������������������������������������������������������������������Ŀ

� THE CUSTOMER WILL LOSE FAITH IN DATA PROVIDED BY THE SUPPLIER IF �

� THE SUPPLIER'S DATA IMPLY THAT PRODUCT QUALITY IS SIGNIFICANTLY  �

� BETTER THAN WHAT THE CUSTOMER'S OWN INSPECTIONS REVEAL.          �

��������������������������������������������������������������������

Our notation is as follows.  From a relatively large lot, the supplier

takes a random sample of size ns and reports finding ds defects.  The

customer takes a confirmatory sample of size nc (possibly much smaller

than the supplier's sample) and finds dc defects.  The customer then

asks... "Does my observed result (observing dc defects in a confirmatory

sample of size nc), cast doubt upon the supplier's claim that he saw NO

MORE THAN ds defects in his sample of size ns?"

Obenchain and Hsiang(1985d) developed a new form of table for use with

the "approximate" test upon which H109 is based.  The new and different

aspect of these applications is that the ratio, r = ns/nc, of the

supplier's sample size to the customer's sample size does not need to be

an integer nor any pre-assigned value.  Previous H109 type tables

assumed that r = 1, 2, 3, 5, or 8.

In Section 2, we review the relevant statistical theory.  Section 3

explains how some new, much more extensive tables for H109 and EXACT

testing differ from the original tables of Ellner(1963).  Section 4

discusses the issue of downward bias that the supplier can introduce via

acceptance sampling.  Section 5 explains use of personal computer

software for validating supplier data.  The exact method of Hoel(1945)

and the "approximate" method of H109 are explained in very simple terms

in Sections 6 and 7.

Section 2: ...Testing Homogeneity of Two Poisson Frequencies

============================================================

The results from the Supplier's Final Audit can be used to estimate the

SUPPLIER'S PURPORTED QUALITY as follows...

ds / ns  = Proportion Defective in Supplier's Audit.

EXPECTED ( ds / ns ) = Unknown Purported Quality Level.

The results from the Customer's Re-Sample can be used to estimate the

SUPPLIER'S TRUE QUALITY as follows...

dc / nc  = Proportion Defective in Customer's Re-Sample.

EXPECTED ( dc / nc ) = Unknown Shipped (Outgoing) Quality.

Our objective is to use both supplier and customer data to perform a

statistical TEST of HYPOTHESIS that will allow us to choose between two

mutually exclusive states-of-nature...

����������������������������������������������������������������������Ŀ

�                                                                      �

�    The SUPPLIER DATA VALIDITY HYPOTHESIS states that the Supplier's  �

�    Purported Defect Rate Equals or Exceeds its True Defect Rate.     �

�                                                                      �

����������������������������������������������������������������������Ĵ

�                                                                      �

�     The ACTION ALTERNATIVE HYPOTHESIS states that the Supplier's     �

�     Purported Defect Rate is Lower Than its True Defect Rate.        �

�                                                                      �

������������������������������������������������������������������������

Our TESTING PHILOSOPHY will be as follows...

If Customer Data support the SUPPLIER DATA VALIDITY HYPOTHESIS, then the

Customer should TRUST Supplier Provided Data and should treat

Supplier Reports as if they were made by the Customer's Own Unbiased

Inspectors.

But, if Customer Data favor the ACTION ALTERNATIVE HYPOTHESIS, then the

Customer should QUESTION the worth of Supplier Provided Data and

should take appropriate ACTION to determine causes for discrepancies

between Supplier and Customer sampling results.

------------------------------------------------------------------------

Let ps = expected proportion defective in the supplier's sample and pc =

expected proportion defective in the customer's sample.  As in

Ellner(1963), we assume that ds is Poisson distributed with mean ps/ns

that dc is Poisson distributed with mean pc/nc, and that ds and dc are

stochastically independent.  We are primarily interested in testing the

hypothesis

HV :  ps  >=  pc,                       (2.1)

under which the supplier's data are said to be "valid," against the

one-sided "action" alternative

HA :  ps  <  pc,                        (2.2)

under which investigations must be undertaken to determine the cause of

the discrepancy between supplier and customer results.

As pointed out by Ellner(1963), a very simple test of (2.1) against

(2.2) assuming independent Poisson distributions for ds and dc can be

based upon the conditional distribution of dc given the observed total

number of defects

dt = ds + dc.                         (2.3)

The conditional distribution of dc given dt when pc = ps is Binomial

with parameters 1/(1+r) and dt where r is the ratio of supplier sample

size to customer sample size:

r = ns / nc.                         (2.4)

Specifically,

dt! * ([1/(1+r)]**dc) * ([r/(1+r)]**ds)

P( dc | dt, pc=ps ) = ---------------------------------------      (2.5)

dc! * ds!

for dc = 0,1,2,..., dt, where x**y means x to the power y.  One would

then fail to accept the supplier data validity hypothesis, HV , over the

action alternative, HA, when the observed dc is "too large" (too close

to dt.)

Ferguson(1967), page 233, cites the work of Hoel(1945) in establishing

that the above conditional test is, in fact, uniformly most powerful

unbiased for testing HV against HA.  In practice, however, this

conditional test has the disadvantage that its unconditional

significance level can be much smaller than its conditional level.  We

can attempt to explain this phenomenon as follows:  The conditional

probability of the customer observing k or more defects given dt when pc

= ps is a sum of terms of the form (2.5) and can conveniently be

expressed in terms of the incomplete Beta-function as

------------------------

P( k <= dc <= dt | dt , pc = ps ) =

I sub [1/(1+r)] ( k, dt-k+1 ).                  (2.6)

For a specified significance level, alpha , an "exact" test would thus

be of the form

I sub [1/(1+r)] ( k , dt-k+1 )  <= alpha,           (2.7)

where the inequality frequently applies because the distribution is

discrete rather than continuous. (A technique that statisticians call

"ramdomization on the boundary" can be used to produce a test of exactly

correct "size," but that approach is undesirable in the sense that

validation could then depend upon a "coin-flip" as well as upon the

observed data.)  The unconditional level of the test defined by (2.7)

will depend upon the numerical value of pc = ps and can be much smaller

than its already too small conditional value.  This is especially true

when the total number of observed defects, dt, is small ( dt =

1,2,3,...).

Ellner(1963) also considers an "approximate" test that was motivated by

work of Cox(1953) on "continuity corrections."  The modification

replaces (2.7) with

I sub [1/(1+r)] ( k+0.5, dt-k+0.5 ) <= alpha.            (2.8)

Ellner(1963) studied the properties of this approximation and found not

only that (2.8) leads to unconditional significance levels which are

less dependent upon the numerical value of pc = ps than those from (2.7)

but also that (2.8) leads to significance levels which, when averaged

over dt values, are closer to the nominal alpha than those from (2.7).

Ellner(1963) also gives detailed power comparisons and information on

"cumulative results" omnibus tests that also favor the "approximation"

(2.8) over the "exact" result (2.7).  The approximate test has more

power than the exact test when the total number of observed defects, dt,

is small ( dt = 1,2,3) primarily because its size is bigger in these

extreme cases.  Anyway, DOD H109 is based upon the approximate (rather

than the exact) test.

Section 3: ...New and Existing Tables

=====================================

���������������������������������������������������������Ŀ

�  SUPPLIER DATA VALIDITY ANALYSES: NEW H109 TYPE TABLES  �

���������������������������������������������������������Ĵ

�         Thomas C. Hsiang and Robert L. Obenchain        �

�            Bell Communications Research, Inc.           �

�      Basking Ridge, NJ 07920 and Holmdel, NJ 07733      �

�����������������������������������������������������������

The basic table for applying the approximate test, (2.8), was called

Table I in Ellner(1963) and H109, and it is also reproduced here as

Table I.  Our new tables are given Table II (alpha = 0.05) and Table III

(alpha = 0.10), each of which has three parts. The rows of our new

tables again give values of ds, but columns now correspond to values for

dc (rather than values of r = ns / nc as in Table I.)

The standard table for applying the "approximate test" was called Table

I in Ellner(1963) and H109(1960), and it is included here as Table I.

Note that the rows correspond to 36 values for the number of defects

reported by the supplier (ds=0 through 35); the five columns correspond

to supplier/customer sample size ratios of r = 1, 2, 3, 5 and 8.  The

body of Table I gives the so-called "action limit," which is the

smallest number of customer observed defects that yields an "observed

significance level" that is less than or equal to the pre-specified

alpha of 0.05.  The customer declares a discrepancy when its observed dc

value equals or exceeds this tabulated action threshold.

����������������������������������������Ŀ

�  Original Form of Table for the Test   �

�   of Ellner (1963) and H109 (1960)     �

������������������������������������������

Column Label = r = ratio of supplier sample size to

customer sample size, r = ns/nc.

Note that r = 1, 2, 3, 5, and 8

are the only cases tabulated.

Row Label = leftmost column = ds = number of defects

reported in the supplier's sample.

Cell Entry = Action Limit = Customer declares a data

discrepancy (at significance level

alpha <= 0.05) when his observed

number of defects (dc) equals

or exceeds this threshold.

���������������������������������������������Ŀ

�                  Table I                    �

���������������������������������������������Ĵ

�  ds   r = 1   r = 2   r = 3   r = 5   r = 8 �

���������������������������������������������Ĵ

�   0     3       2       2       1       1   �

�   1     5       3       3       2       2   �

�   2     7       4       3       3       2   �

�   3     9       5       4       3       2   �

���������������������������������������������Ĵ

�   4    11       6       5       3       3   �

�   5    12       7       5       4       3   �

�   6    14       8       6       4       3   �

�   7    15       9       6       5       3   �

���������������������������������������������Ĵ

�   8    17       9       7       5       4   �

�   9    18      10       7       5       4   �

�  10    19      11       8       6       4   �

�  11    21      12       8       6       4   �

���������������������������������������������Ĵ

�  12    22      12       9       6       5   �

�  13    23      13       9       7       5   �

�  14    25      14      10       7       5   �

�  15    26      14      10       7       5   �

���������������������������������������������Ĵ

�  16    27      15      11       7       5   �

�  17    28      16      11       8       6   �

�  18    30      16      12       8       6   �

�  19    31      17      12       8       6   �

���������������������������������������������Ĵ

�  20    32      18      13       9       6   �

�  21    34      18      13       9       6   �

�  22    35      19      14       9       7   �

�  23    36      20      14       9       7   �

���������������������������������������������Ĵ

�  24    37      20      15      10       7   �

�  25    39      21      15      10       7   �

�  26    40      22      15      10       7   �

�  27    41      22      16      11       8   �

���������������������������������������������Ĵ

�  28    42      23      16      11       8   �

�  29    43      24      17      11       8   �

�  30    45      24      17      11       8   �

�  31    46      25      18      12       8   �

���������������������������������������������Ĵ

�  32    47      25      18      12       8   �

�  33    48      26      18      12       9   �

�  34    49      27      19      13       9   �

�  35    51      27      19      13       9   �

�����������������������������������������������

------------------------------------------------------------------------

"Ratio-of-Sample-Sizes" Tables for the

Test of Ellner (1963) and H109 (1960)

------------------------------------------------------------------------

Column Label = dc = Observed number of defects in the

customer's sample.  (No column for

dc = 0 is needed because the customer

would never take action in this case.)

Row Label =  ds  =  Reported number of defects in the

supplier's sample.

Cell Entry = Action Limit = Smallest sample size ratio,

r = ns / nc, for which the observed

numbers of defects imply that the

supplier is displaying downward bias

that is statistically significant at

the alpha = 0.05 or alpha = 0.10

levels.

Tables II and III each consist of three sub-tables that cover different

possible RANGES of values for supplier defects(ds) and customer

defects(dc).  These sub-ranges are shown below...

dc=1...........dc=15

ds=0  +------------------+

.   |                  |

.   |     Part One     |

.   |                  |

ds=25 +------------------+

dc=1...........dc=15  dc=16.........dc=30

ds=26 +------------------+  +-----------------+

.   |                  |  |                 |

.   |     Part Two     |  |    Part Three   |

.   |                  |  |                 |

ds=50 +------------------+  +-----------------+

------------------------------------------------------------------------

Table II - Part One

------------------------------------------------------------------------

MINIMUM SUPPLIER/CONSUMER SAMPLE RATIOS: ALPHA = 0.05

------------------------------------------------------------------------

DC=1 DC=2 DC=3 DC=4 DC=5 DC=6 DC=7 DC=8 DC=9 DC10 DC11 DC12 DC13 DC14 DC15

DS=0 3.38 1.32 .799 .569 .440 .359 .303 .262 .231 .206 .186 .170 .156 .144 .134

DS=1 9.28 3.25 1.86 1.29 .978 .787 .658 .564 .494 .439 .395 .359 .329 .304 .282

DS=2 15.0 5.05 2.84 1.93 1.46 1.16 .967 .827 .721 .639 .574 .521 .476 .440 .407

DS=3 20.7 6.83 3.79 2.56 1.92 1.52 1.26 1.08 .937 .829 .743 .673 .615 .566 .525

DS=4 26.4 8.59 4.73 3.18 2.37 1.88 1.55 1.32 1.15 1.01 .908 .822 .750 .690 .639

DS=5 32.1 10.3 5.66 3.79 2.82 2.23 1.84 1.56 1.35 1.20 1.07 .967 .882 .811 .750

DS=6 37.8 12.1 6.59 4.40 3.26 2.58 2.12 1.80 1.56 1.38 1.23 1.11 1.01 .930 .860

DS=7 43.5 13.9 7.52 5.01 3.71 2.92 2.40 2.04 1.76 1.55 1.39 1.25 1.14 1.05 .969

DS=8 49.2 15.6 8.45 5.62 4.15 3.27 2.68 2.27 1.97 1.73 1.55 1.39 1.27 1.17 1.08

DS=9 54.9 17.4 9.38 6.22 4.59 3.61 2.96 2.51 2.17 1.91 1.70 1.54 1.40 1.28 1.18

DS10 60.6 19.1 10.3 6.83 5.03 3.95 3.24 2.74 2.37 2.08 1.86 1.68 1.53 1.40 1.29

DS11 66.3 20.9 11.2 7.43 5.47 4.30 3.52 2.97 2.57 2.26 2.01 1.82 1.65 1.52 1.40

DS12 72.0 22.6 12.2 8.04 5.91 4.64 3.80 3.21 2.77 2.43 2.17 1.96 1.78 1.63 1.50

DS13 77.6 24.4 13.1 8.64 6.35 4.98 4.08 3.44 2.97 2.61 2.32 2.09 1.90 1.75 1.61

DS14 83.3 26.1 14.0 9.24 6.79 5.32 4.35 3.67 3.17 2.78 2.48 2.23 2.03 1.86 1.72

DS15 89.0 27.8 14.9 9.85 7.23 5.66 4.63 3.91 3.37 2.96 2.63 2.37 2.16 1.98 1.82

DS16 94.7 29.6 15.9 10.5 7.67 6.00 4.91 4.14 3.57 3.13 2.79 2.51 2.28 2.09 1.93

DS17 100. 31.3 16.8 11.1 8.11 6.35 5.19 4.37 3.77 3.31 2.94 2.65 2.41 2.20 2.03

DS18 106. 33.1 17.7 11.7 8.55 6.69 5.46 4.60 3.97 3.48 3.10 2.79 2.53 2.32 2.14

DS19 112. 34.8 18.6 12.3 8.98 7.03 5.74 4.83 4.17 3.66 3.25 2.93 2.66 2.43 2.24

DS20 117. 36.6 19.6 12.9 9.42 7.37 6.02 5.07 4.37 3.83 3.41 3.06 2.78 2.55 2.35

DS21 123. 38.3 20.4 13.5 9.86 7.71 6.29 5.30 4.56 4.00 3.56 3.20 2.91 2.66 2.45

DS22 129. 40.1 21.4 14.1 10.3 8.05 6.57 5.53 4.76 4.18 3.71 3.34 3.03 2.78 2.56

DS23 134. 41.8 22.3 14.7 10.7 8.39 6.85 5.76 4.96 4.35 3.87 3.48 3.16 2.89 2.66

DS24 140. 43.6 23.2 15.3 11.2 8.73 7.12 5.99 5.16 4.52 4.02 3.62 3.28 3.00 2.77

DS25 146. 45.3 24.2 15.9 11.6 9.07 7.40 6.22 5.36 4.70 4.17 3.75 3.41 3.12 2.87

------------------------------------------------------------------------

Table II - Part Two

------------------------------------------------------------------------

MINIMUM SUPPLIER/CONSUMER SAMPLE RATIOS: ALPHA = 0.05

------------------------------------------------------------------------

DC=1 DC=2 DC=3 DC=4 DC=5 DC=6 DC=7 DC=8 DC=9 DC10 DC11 DC12 DC13 DC14 DC15

DS26 152. 47.1 25.1 16.5 12.1 9.41 7.67 6.46 5.56 4.87 4.33 3.89 3.53 3.23 2.98

DS27 157. 48.8 26.0 17.1 12.5 9.75 7.95 6.69 5.76 5.04 4.48 4.03 3.66 3.34 3.08

DS28 163. 50.6 26.9 17.7 12.9 10.1 8.23 6.92 5.95 5.22 4.63 4.17 3.78 3.46 3.18

DS29 169. 52.3 27.9 18.3 13.4 10.4 8.50 7.15 6.15 5.39 4.79 4.30 3.90 3.57 3.29

DS30 174. 54.0 28.8 18.9 13.8 10.8 8.78 7.38 6.35 5.56 4.94 4.44 4.03 3.69 3.39

DS31 180. 55.8 29.7 19.5 14.2 11.1 9.05 7.61 6.55 5.74 5.09 4.58 4.15 3.80 3.50

DS32 186. 57.5 30.6 20.1 14.7 11.4 9.33 7.84 6.75 5.91 5.25 4.72 4.28 3.91 3.60

DS33 191. 59.3 31.6 20.7 15.1 11.8 9.61 8.07 6.94 6.08 5.40 4.85 4.40 4.03 3.71

DS34 197. 61.0 32.5 21.3 15.6 12.1 9.88 8.31 7.14 6.25 5.55 4.99 4.53 4.14 3.81

DS35 203. 62.8 33.4 21.9 16.0 12.5 10.2 8.54 7.34 6.43 5.71 5.13 4.65 4.25 3.91

DS36 208. 64.5 34.3 22.5 16.4 12.8 10.4 8.77 7.54 6.60 5.86 5.27 4.78 4.37 4.02

DS37 214. 66.3 35.2 23.1 16.9 13.1 10.7 9.00 7.74 6.77 6.01 5.40 4.90 4.48 4.12

DS38 220. 68.0 36.2 23.7 17.3 13.5 11.0 9.23 7.94 6.95 6.17 5.54 5.02 4.59 4.23

DS39 225. 69.8 37.1 24.3 17.7 13.8 11.3 9.46 8.13 7.12 6.32 5.68 5.15 4.71 4.33

DS40 231. 71.5 38.0 24.9 18.2 14.2 11.5 9.69 8.33 7.29 6.47 5.81 5.27 4.82 4.44

DS41 237. 73.3 38.9 25.5 18.6 14.5 11.8 9.92 8.53 7.46 6.63 5.95 5.40 4.93 4.54

DS42 242. 75.0 39.9 26.1 19.1 14.8 12.1 10.2 8.73 7.64 6.78 6.09 5.52 5.05 4.64

DS43 248. 76.8 40.8 26.7 19.5 15.2 12.4 10.4 8.93 7.81 6.93 6.23 5.64 5.16 4.75

DS44 254. 78.5 41.7 27.3 19.9 15.5 12.6 10.6 9.12 7.98 7.09 6.36 5.77 5.27 4.85

DS45 259. 80.2 42.6 27.9 20.4 15.9 12.9 10.8 9.32 8.16 7.24 6.50 5.89 5.39 4.96

DS46 265. 82.0 43.6 28.5 20.8 16.2 13.2 11.1 9.52 8.33 7.39 6.64 6.02 5.50 5.06

DS47 271. 83.7 44.5 29.1 21.2 16.5 13.5 11.3 9.72 8.50 7.55 6.77 6.14 5.61 5.16

DS48 277. 85.5 45.4 29.7 21.7 16.9 13.7 11.5 9.92 8.67 7.70 6.91 6.27 5.73 5.27

DS49 282. 87.2 46.3 30.3 22.1 17.2 14.0 11.8 10.1 8.85 7.85 7.05 6.39 5.84 5.37

DS50 288. 89.0 47.2 30.9 22.6 17.6 14.3 12.0 10.3 9.02 8.00 7.19 6.51 5.95 5.48

------------------------------------------------------------------------

Table II - Part Three

------------------------------------------------------------------------

MINIMUM SUPPLIER/CONSUMER SAMPLE RATIOS: ALPHA = 0.05

------------------------------------------------------------------------

DC16 DC17 DC18 DC19 DC20 DC21 DC22 DC23 DC24 DC25 DC26 DC27 DC28 DC29 DC30

DS26 2.76 2.57 2.40 2.26 2.13 2.01 1.91 1.81 1.73 1.65 1.58 1.51 1.45 1.40 1.34

DS27 2.85 2.66 2.49 2.33 2.20 2.08 1.97 1.87 1.79 1.71 1.63 1.56 1.50 1.44 1.39

DS28 2.95 2.75 2.57 2.41 2.27 2.15 2.04 1.94 1.85 1.76 1.69 1.62 1.55 1.49 1.44

DS29 3.05 2.84 2.65 2.49 2.35 2.22 2.10 2.00 1.91 1.82 1.74 1.67 1.60 1.54 1.48

DS30 3.14 2.93 2.74 2.57 2.42 2.29 2.17 2.06 1.97 1.88 1.80 1.72 1.65 1.59 1.53

DS31 3.24 3.02 2.82 2.65 2.50 2.36 2.24 2.13 2.02 1.93 1.85 1.77 1.70 1.64 1.57

DS32 3.34 3.11 2.90 2.73 2.57 2.43 2.30 2.19 2.08 1.99 1.90 1.82 1.75 1.68 1.62

DS33 3.43 3.20 2.99 2.81 2.64 2.50 2.37 2.25 2.14 2.05 1.96 1.88 1.80 1.73 1.67

DS34 3.53 3.29 3.07 2.88 2.72 2.57 2.43 2.31 2.20 2.10 2.01 1.93 1.85 1.78 1.71

DS35 3.63 3.37 3.16 2.96 2.79 2.64 2.50 2.38 2.26 2.16 2.07 1.98 1.90 1.83 1.76

DS36 3.72 3.46 3.24 3.04 2.86 2.71 2.57 2.44 2.32 2.22 2.12 2.03 1.95 1.87 1.80

DS37 3.82 3.55 3.32 3.12 2.94 2.78 2.63 2.50 2.38 2.27 2.17 2.08 2.00 1.92 1.85

DS38 3.91 3.64 3.41 3.20 3.01 2.85 2.70 2.56 2.44 2.33 2.23 2.14 2.05 1.97 1.90

DS39 4.01 3.73 3.49 3.28 3.09 2.92 2.76 2.63 2.50 2.39 2.28 2.19 2.10 2.02 1.94

DS40 4.11 3.82 3.57 3.35 3.16 2.98 2.83 2.69 2.56 2.44 2.34 2.24 2.15 2.07 1.99

DS41 4.20 3.91 3.66 3.43 3.23 3.05 2.89 2.75 2.62 2.50 2.39 2.29 2.20 2.11 2.03

DS42 4.30 4.00 3.74 3.51 3.31 3.12 2.96 2.81 2.68 2.56 2.44 2.34 2.25 2.16 2.08

DS43 4.40 4.09 3.82 3.59 3.38 3.19 3.03 2.87 2.74 2.61 2.50 2.39 2.30 2.21 2.13

DS44 4.49 4.18 3.91 3.67 3.45 3.26 3.09 2.94 2.80 2.67 2.55 2.45 2.35 2.26 2.17

DS45 4.59 4.27 3.99 3.74 3.53 3.33 3.16 3.00 2.86 2.73 2.61 2.50 2.40 2.30 2.22

DS46 4.68 4.36 4.07 3.82 3.60 3.40 3.22 3.06 2.92 2.78 2.66 2.55 2.45 2.35 2.26

DS47 4.78 4.45 4.16 3.90 3.67 3.47 3.29 3.12 2.97 2.84 2.71 2.60 2.50 2.40 2.31

DS48 4.88 4.54 4.24 3.98 3.75 3.54 3.35 3.19 3.03 2.90 2.77 2.65 2.54 2.45 2.35

DS49 4.97 4.63 4.32 4.06 3.82 3.61 3.42 3.25 3.09 2.95 2.82 2.70 2.59 2.49 2.40

DS50 5.07 4.72 4.41 4.13 3.89 3.68 3.49 3.31 3.15 3.01 2.88 2.76 2.64 2.54 2.45

------------------------------------------------------------------------

Note that our new tables do not have a column for dc = 0 because the

customer would never take action against the supplier when the customer

sees no defects in his confirmatory sample.  The numbers in the bodies

of our tables are the values of r = ns / nc that yield equality in (2.8)

at k = dc.  As explained above, this tabulated r value is the smallest

sample size ratio (supplier ns divided by customer nc ) for which the

observed numbers of defects (ds = row,and dc = column) lead to the

inference that there is a statistically significant discrepancy (ps < pc

at alpha = 0.05 or 0.10).  Any larger value of the r=ns/nc ratio for

fixed reported ds and observed dc is, of course, even stronger evidence

of a "supplier data discrepancy."

An example may help explain differences between Tables I and II (both of

which correspond to alpha = 0.05) and also illustrate how the new tables

are generally used.  Suppose ds = 0 and dc = 2.  The "zero" row of Table

I applies to this case because ds = 0, and we see that the customer

should take "action" if r=2,3,5 or 8 (but no "action" if r=1).  The

second cell in row "zero" of Table II gives the much more "detailed"

information that r=1.32 is the smallest ratio for which action is

appropriate.  And Table II applies when r is not an integer (or any

other finite set of values that could be used in tables like Table I).

------------------------------------------------------------------------

Table III - Part One

------------------------------------------------------------------------

MINIMUM SUPPLIER/CONSUMER SAMPLE RATIOS: ALPHA = 0.10

------------------------------------------------------------------------

DC=1 DC=2 DC=3 DC=4 DC=5 DC=6 DC=7 DC=8 DC=9 DC10 DC11 DC12 DC13 DC14 DC15

DS=0 1.85 .812 .513 .373 .293 .241 .205 .178 .157 .141 .128 .117 .108 .100 .093

DS=1 5.39 2.17 1.32 .938 .726 .591 .498 .430 .379 .338 .305 .278 .255 .236 .220

DS=2 8.85 3.45 2.06 1.45 1.11 .903 .758 .652 .573 .501 .460 .418 .384 .355 .329

DS=3 12.3 4.72 2.78 1.95 1.49 1.20 1.01 .865 .758 .674 .607 .552 .506 .467 .434

DS=4 15.7 5.97 3.50 2.44 1.86 1.50 1.25 1.07 .940 .835 .751 .682 .625 .576 .535

DS=5 19.1 7.22 4.22 2.93 2.23 1.79 1.49 1.28 1.12 .993 .892 .810 .742 .684 .634

DS=6 22.6 8.47 4.93 3.41 2.59 2.08 1.73 1.48 1.30 1.15 1.03 .937 .857 .790 .732

DS=7 26.0 9.71 5.64 3.90 2.96 2.37 1.97 1.69 1.47 1.31 1.17 1.06 .971 .895 .830

DS=8 29.4 11.0 6.35 4.38 3.32 2.66 2.21 1.89 1.65 1.46 1.31 1.19 1.09 1.00 .926

DS=9 32.9 12.2 7.06 4.87 3.68 2.94 2.45 2.09 1.82 1.61 1.45 1.31 1.20 1.10 1.02

DS10 36.3 13.4 7.77 5.35 4.04 3.23 2.68 2.29 2.00 1.77 1.59 1.44 1.31 1.21 1.12

DS11 39.7 14.7 8.48 5.83 4.40 3.52 2.92 2.49 2.17 1.92 1.72 1.56 1.42 1.31 1.21

DS12 43.1 15.9 9.18 6.31 4.76 3.80 3.16 2.69 2.34 2.07 1.86 1.68 1.54 1.41 1.31

DS13 46.5 17.2 9.89 6.79 5.12 4.09 3.39 2.89 2.52 2.23 2.00 1.81 1.65 1.52 1.40

DS14 50.0 18.4 10.6 7.28 5.48 4.37 3.63 3.09 2.69 2.38 2.13 1.93 1.76 1.62 1.50

DS15 53.4 19.7 11.3 7.76 5.84 4.66 3.86 3.29 2.86 2.53 2.27 2.05 1.87 1.72 1.59

DS16 56.8 20.9 12.0 8.24 6.20 4.95 4.10 3.49 3.04 2.68 2.40 2.17 1.98 1.82 1.69

DS17 60.2 22.2 12.7 8.72 6.56 5.23 4.33 3.69 3.21 2.84 2.54 2.30 2.10 1.93 1.78

DS18 63.7 23.4 13.4 9.20 6.92 5.52 4.57 3.89 3.38 2.99 2.68 2.42 2.21 2.03 1.88

DS19 67.1 24.6 14.1 9.68 7.28 5.80 4.80 4.09 3.56 3.14 2.81 2.54 2.32 2.13 1.97

DS20 70.5 25.9 14.8 10.2 7.64 6.09 5.04 4.29 3.73 3.29 2.95 2.66 2.43 2.23 2.07

DS21 73.9 27.1 15.5 10.6 8.00 6.37 5.27 4.49 3.90 3.44 3.08 2.79 2.54 2.34 2.16

DS22 77.4 28.4 16.3 11.1 8.36 6.65 5.51 4.69 4.07 3.60 3.22 2.91 2.65 2.44 2.25

DS23 80.8 29.6 17.0 11.6 8.72 6.94 5.74 4.89 4.25 3.75 3.35 3.03 2.76 2.54 2.35

DS24 84.2 30.8 17.7 12.1 9.08 7.22 5.98 5.09 4.42 3.90 3.49 3.15 2.88 2.64 2.44

DS25 87.6 32.1 18.4 12.6 9.44 7.51 6.21 5.28 4.59 4.05 3.62 3.28 2.99 2.74 2.54

------------------------------------------------------------------------

Table III - Part Two

------------------------------------------------------------------------

MINIMUM SUPPLIER/CONSUMER SAMPLE RATIOS: ALPHA = 0.10

------------------------------------------------------------------------

DC=1 DC=2 DC=3 DC=4 DC=5 DC=6 DC=7 DC=8 DC=9 DC10 DC11 DC12 DC13 DC14 DC15

DS26 91.0 33.3 19.1 13.0 9.80 7.79 6.45 5.48 4.76 4.20 3.76 3.40 3.10 2.85 2.63

DS27 94.5 34.6 19.8 13.5 10.2 8.08 6.68 5.68 4.93 4.36 3.89 3.52 3.21 2.95 2.72

DS28 97.9 35.8 20.5 14.0 10.5 8.36 6.92 5.88 5.11 4.51 4.03 3.64 3.32 3.05 2.82

DS29 101. 37.1 21.2 14.5 10.9 8.65 7.15 6.08 5.28 4.66 4.16 3.76 3.43 3.15 2.91

DS30 105. 38.3 21.9 15.0 11.2 8.93 7.38 6.28 5.45 4.81 4.30 3.88 3.54 3.25 3.01

DS31 108. 39.5 22.6 15.4 11.6 9.22 7.62 6.48 5.62 4.96 4.43 4.01 3.65 3.35 3.10

DS32 112. 40.8 23.3 15.9 11.9 9.50 7.85 6.68 5.79 5.11 4.57 4.13 3.76 3.46 3.19

DS33 115. 42.0 24.0 16.4 12.3 9.78 8.09 6.87 5.97 5.26 4.71 4.25 3.87 3.56 3.29

DS34 118. 43.3 24.7 16.9 12.7 10.1 8.32 7.07 6.14 5.42 4.84 4.37 3.98 3.66 3.38

DS35 122. 44.5 25.4 17.4 13.0 10.4 8.56 7.27 6.31 5.57 4.98 4.49 4.10 3.76 3.47

DS36 125. 45.8 26.1 17.8 13.4 10.6 8.79 7.47 6.48 5.72 5.11 4.62 4.21 3.86 3.57

DS37 129. 47.0 26.8 18.3 13.7 10.9 9.02 7.67 6.65 5.87 5.25 4.74 4.32 3.96 3.66

DS38 132. 48.2 27.6 18.8 14.1 11.2 9.26 7.87 6.83 6.02 5.38 4.86 4.43 4.06 3.75

DS39 136. 49.5 28.3 19.3 14.5 11.5 9.49 8.07 7.00 6.17 5.52 4.98 4.54 4.17 3.85

DS40 139. 50.7 29.0 19.8 14.8 11.8 9.73 8.26 7.17 6.32 5.65 5.10 4.65 4.27 3.94

DS41 142. 52.0 29.7 20.2 15.2 12.1 9.96 8.46 7.34 6.48 5.79 5.22 4.76 4.37 4.04

DS42 146. 53.2 30.4 20.7 15.5 12.3 10.2 8.66 7.51 6.63 5.92 5.35 4.87 4.47 4.13

DS43 149. 54.5 31.1 21.2 15.9 12.6 10.4 8.86 7.69 6.78 6.06 5.47 4.98 4.57 4.22

DS44 153. 55.7 31.8 21.7 16.3 12.9 10.7 9.06 7.86 6.93 6.19 5.59 5.09 4.67 4.32

DS45 156. 56.9 32.5 22.2 16.6 13.2 10.9 9.26 8.03 7.08 6.32 5.71 5.20 4.77 4.41

DS46 159. 58.2 33.2 22.6 17.0 13.5 11.1 9.46 8.20 7.23 6.46 5.83 5.31 4.88 4.50

DS47 163. 59.4 33.9 23.1 17.3 13.8 11.4 9.65 8.37 7.38 6.59 5.95 5.42 4.98 4.60

DS48 166. 60.7 34.6 23.6 17.7 14.0 11.6 9.85 8.55 7.53 6.73 6.08 5.53 5.08 4.69

DS49 170. 61.9 35.3 24.1 18.0 14.3 11.8 10.1 8.72 7.69 6.86 6.20 5.64 5.18 4.78

DS50 173. 63.1 36.0 24.6 18.4 14.6 12.1 10.2 8.89 7.84 7.00 6.32 5.76 5.28 4.88

------------------------------------------------------------------------

Table III - Part Three

------------------------------------------------------------------------

MINIMUM SUPPLIER/CONSUMER SAMPLE RATIOS: ALPHA = 0.10

------------------------------------------------------------------------

DC16 DC17 DC18 DC19 DC20 DC21 DC22 DC23 DC24 DC25 DC26 DC27 DC28 DC29 DC30

DS26 2.44 2.28 2.14 2.02 1.90 1.80 1.71 1.63 1.56 1.49 1.43 1.37 1.32 1.27 1.22

DS27 2.53 2.36 2.22 2.09 1.97 1.87 1.77 1.69 1.61 1.54 1.48 1.42 1.36 1.31 1.26

DS28 2.62 2.45 2.29 2.16 2.04 1.93 1.83 1.75 1.67 1.59 1.53 1.46 1.41 1.35 1.31

DS29 2.71 2.53 2.37 2.23 2.11 1.99 1.89 1.80 1.72 1.65 1.58 1.51 1.45 1.40 1.35

DS30 2.79 2.61 2.45 2.30 2.17 2.06 1.95 1.86 1.78 1.70 1.63 1.56 1.50 1.44 1.39

DS31 2.88 2.69 2.52 2.37 2.24 2.12 2.02 1.92 1.83 1.75 1.68 1.61 1.55 1.49 1.43

DS32 2.97 2.77 2.60 2.44 2.31 2.19 2.08 1.98 1.89 1.80 1.73 1.66 1.59 1.53 1.48

DS33 3.05 2.85 2.67 2.52 2.38 2.25 2.14 2.03 1.94 1.85 1.78 1.70 1.64 1.58 1.52

DS34 3.14 2.93 2.75 2.59 2.44 2.31 2.20 2.09 1.99 1.91 1.83 1.75 1.68 1.62 1.56

DS35 3.23 3.01 2.82 2.66 2.51 2.38 2.26 2.15 2.05 1.96 1.88 1.80 1.73 1.67 1.60

DS36 3.31 3.09 2.90 2.73 2.58 2.44 2.32 2.21 2.10 2.01 1.93 1.85 1.78 1.71 1.65

DS37 3.40 3.18 2.98 2.80 2.64 2.50 2.38 2.26 2.16 2.06 1.98 1.90 1.82 1.75 1.69

DS38 3.49 3.26 3.05 2.87 2.71 2.57 2.44 2.32 2.21 2.12 2.03 1.94 1.87 1.80 1.73

DS39 3.58 3.34 3.13 2.94 2.78 2.63 2.50 2.38 2.27 2.17 2.08 1.99 1.91 1.84 1.78

DS40 3.66 3.42 3.20 3.01 2.85 2.69 2.56 2.43 2.32 2.22 2.13 2.04 1.96 1.89 1.82

DS41 3.75 3.50 3.28 3.09 2.91 2.76 2.62 2.49 2.38 2.27 2.18 2.09 2.01 1.93 1.86

DS42 3.84 3.58 3.36 3.16 2.98 2.82 2.68 2.55 2.43 2.32 2.23 2.14 2.05 1.97 1.90

DS43 3.92 3.66 3.43 3.23 3.05 2.89 2.74 2.61 2.49 2.38 2.28 2.18 2.10 2.02 1.95

DS44 4.01 3.74 3.51 3.30 3.11 2.95 2.80 2.66 2.54 2.43 2.33 2.23 2.14 2.06 1.99

DS45 4.10 3.82 3.58 3.37 3.18 3.01 2.86 2.72 2.60 2.48 2.38 2.28 2.19 2.11 2.03

DS46 4.18 3.90 3.66 3.44 3.25 3.08 2.92 2.78 2.65 2.53 2.43 2.33 2.24 2.15 2.07

DS47 4.27 3.98 3.73 3.51 3.32 3.14 2.98 2.84 2.70 2.59 2.48 2.37 2.28 2.20 2.12

DS48 4.36 4.07 3.81 3.58 3.38 3.20 3.04 2.89 2.76 2.64 2.53 2.42 2.33 2.24 2.16

DS49 4.44 4.15 3.89 3.66 3.45 3.27 3.10 2.95 2.81 2.69 2.58 2.47 2.37 2.28 2.20

DS50 4.53 4.23 3.96 3.73 3.52 3.33 3.16 3.01 2.87 2.74 2.63 2.52 2.42 2.33 2.24

------------------------------------------------------------------------

Section 4: ...Remarks on Acceptance Sampling Bias

=================================================

We anticipate that the most common type of H109 test application will be

made to supplier data that come from his final sampling inspection for

attributes.  As a result only the "accepted lots" (lots with relatively

small ds / ns observed values) may be sent to the customer for

"validity" re-sampling.  Bias can be introduced by this supplier

selection process that would average-out (over accepted and rejected

lots) if the customer were to re-sample all of the lots incoming to the

supplier's final inspection.  Unless the lot-to-lot variability of

quality is high, the results of Mood(1943) then imply that the

supplier's "accepted-lot-only" results can have considerable downward

bias when viewed as measures of the defect rate outgoing from his final

inspection.

Due to the supplier's acceptance sampling scheme, we suspect in advance

that the validity hypothesis, (2.1), does not hold given these

conditions. This phenomenon is most obvious when the supplier is using

an acceptance number of zero (defects.)  In this extreme case, the

supplier is not necessarily claiming that his accepted lots are totally

defect-free; he simply finds no defects in his samples from the lots

that he ends up accepting.  Given that the customer sees only results

from accepted lots, the conditional expected value of ds / ns is then

zero because the supplier is using a zero accept number.  But the

expected value of the customer's dc/nc is still positive (pc>0) unless

quality is indeed perfect.  Due simply to acceptance sampling (and not

to any attempt at deception,) the supplier's data can be biased in favor

of the action alternative of (2.2) over the validity hypothesis of

(2.1).  The customer's data remain unbiased under these conditions.

Customer data are likely to detect supplier acceptance sampling bias

whenever it is relatively large (e.g. when the true lot-to-lot quality

variation is small and a large fraction of lots are rejected by the

supplier's final inspection.)

The great advantage of quick, manual "type H109" tests is their

simplicity.  If no downward bias in the supplier's data is detected by

this procedure (even though acceptance sampling bias is known to be

present), then acceptance sampling bias is probably inconsequential

(e.g. almost all lots pass the supplier's final inspection or the true

lot-to-lot quality variation is high.)  But, if downward bias is

frequently detected by H109 in supplier data for a product, then the

customer's "action" must be to search for the "causes" of these

discrepancies.  If acceptance sampling bias is the major discernible

cause of frequent discrepancies, then a simple test procedure such as

H109 should probably not be used with that product.  Although the H109

bias detection procedure is working correctly, it is detecting

"nusiance" bias that really is irrelevant to the validity of the

supplier's data.

The details of exactly how supplier/customer results are to be

accumulated over time is a key issue in administration of a supplier

data validation program, Obenchain (1985e).  One possibility is to start

a validation study at a given time that is to last only a certain number

of production lots or reporting periods.  In this case, only one

cumulative total may be needed, i.e. from the start of the study up to

the present.  But if this approach uncovers significant differences

between supplier reports and customer observations, the study may have

to continue beyond the anticipated completion time in some sort of

"open-ended" validation mode.  As comparison of supplier and customer

results continues, two or three different cumulative totals could be

simultaneously monitored.  One could ask not only "How do supplier and

customer results compare for the single most recent production lot or

reporting period?" but also "Over the six most recent lots or periods?"

and "Over the last quarter or year?"  The statistical theory of

"sequential analysis" and "cumsum tests" are applicable here, but those

topics are beyond the scope of this documentation.

In summary, the highly graphical, computer intensive methodology of

Obenchain (1985a,b,c) is more informative than "cumulative" H109, but

sophisticated methods cannot possibly be applied using only hand

calculations and table look-ups.  Cumulative H109 is much easier to

explain and to apply in the field and is thus the preferred method

for

- quick in-the-field applications, and the

- start-up phase of validation studies before sufficient data to

apply sophisticated methods have been collected.

Section 5: ...Tests of Validity Hypotheses using Software

=========================================================

One module of the PMA/QC Statistics Committee "Quality Assurance

Training Tools" is a software system, SDVALID, for testing the "Supplier

Data VALIDITY Hypothesis."  SDVALID.EXE calculates observed significance

levels and 1%, 5%, 10%, 25%, and 50% quantiles for the supplier/customer

ratio-of-sample-sizes, r = ns/nc, for both the H109 and the

(conditional) EXACT testing procedures.

The data to be analyzed should consist of MATCHED SUPPLIER-CUSTOMER

RESULTS.

ns =  Size of Supplier's Final Audit Sample,

nc =  Size of Customer's Confirmatory Re-Sample, and

ds =  Number of Defects Reported to the Customer,

dc =  Number of Defects Observed by the Customer.

It is essential for these data to be MATCHED in the sense that both sets

of samples are taken at random from the same "lots" of product within

the same quality reporting period, but the same "units" do not need to

be re-sampled.

SDVALID.EXE is invoked by entering the command...

A> sdvalid  #���� End this invocation by pressing Enter (#��).

#

���� DOS system prompt.

The software will now prompt you select between the "Keyboard" and

"Batch" modes of data input.

The easiest way to use SDVALID, especially for someone who is just

beginning to learn about how to use PMA/QC Training Tools, is to let the

software PROMPT you to enter, from your keyboard, all of the input data

and parameter settings needed to calculate results and to display them

on your screen.  So, press the "K" key and then respond to the prompts.

Prompts end with a colon (:), and the square brackets [] contain a

default value that you can accept simply by pressing the enter key...

Output Filename ? [sdvalid] : _

Respond to the next prompt by entering the total number of units that

the Supplier sampled in his final audit of the present batch.

Supplier Sample Size ? [ns=50] : _

Use the number keys along the top of the IBM PC keyboard, or use the Num

Lock key to activate the right-hand-side numeric keypad.  Use the

destructive backspace key to erase mistakes and then enter correct

values.  SDVALID expects inputs in the form of integers, so do not type

a decimal point in your input.  After typing the desired integer, press

the Enter Key (#��).

The total size of the Supplier's final audit must be at least ONE unit

and no more than 2,000,000,000.  Similarly, the total size of the

Customer's re-sample must be at least ONE unit and no more than

2,000,000,000.

The third prompt is of the form...

Supplier Reported Defects ? [ds=0] : _

Zero is a valid resonse to this prompt, but you should not enter a value

that is LARGER than the Supplier Sample Size = ns that you entered at

the first prompt.

REMEMBER: use the number keys to enter an integer, use the backspace key

to erase mistakes, and retype the correct value before pressing Enter

(#��).

The fourth (and last) prompt is of the form...

Customer Observed Defects ? [dc=1] : _

ZERO is usually NOT a very reasonable response to this prompt.  The

Customer should never question the "validity" of the Supplier's Report

unless the Customer finds at least one defect in the confirmatory

re-sample.  The "approximate" H109 test can be applied in this extreme

(actually meaningless) case, but an EXACT test is impossible, of course.

Also, you should not enter a value that is LARGER than the Customer

Sample Size that you entered at the second prompt.

Once you have successfully responded to all four prompts, SDVALID will

make the necessary calculations and display results in the following

format:

The observed sample size ratio of ns/nc =  X.XXX

...has EXACT observed significance level =  .XXX

...and H109  observed significance level =  .XXX

�����������������������������������������������������������Ŀ

�                                                           �

� Verbal description of result...NORMAL, WARNING, or ACTION.�

�                                                           �

�������������������������������������������������������������

Thresholds

for ns/nc: 1%       5%      10%      25%      50%

EXACT:  X.XXX    X.XXX    X.XXX    X.XXX    X.XXX

H109 :  X.XXX    X.XXX    X.XXX    X.XXX    X.XXX

Do you want to specify additional tests ? [Y|n] :

Responding to this prompt by pressing the "N" key will terminate

SDVALID.EXE.  You can also halt SDVALID by holding down the control key

(Ctrl) and then pressing the Break key.

In order to use "Batch" parameter input with SDVALID, you will need to

use your favorite text editor (word processor) to create a file (with a

mandatory filename extention of .INP) that is formatted exactly as shown

in the 11-line example shown below...

BATCH     <== MS-DOS Filename for the SDVALID ASCII output file.

734       <== Supplier Sample Size       #�Ŀ

1030      <== Customer Re-Sample Size       �  Responses to the four

6         <== Supplier Reported Defects     �  prompts for ns,nc,ds,dc.

19        <== Customer Observed Defects  #���

<== Blank Line Needed Here...

33        <== Supplier Sample Size       #�Ŀ

11        <== Customer Re-Sample Size       �  Optional, second

0         <== Supplier Reported Defects     �  set of four...

1         <== Customer Observed Defects  #���

n         <== No more tests; QUIT!

The file for the above example, named BATCH.INP, is included in the

distribution archive for SDVALID.EXE.

Section 6: ...The Exact Test

============================

...assume that the numbers, ds and dc, of Supplier Reported and

Customer Observed defects follow independent Poisson

distributions.

NOTE: This assumption is more reasonable when the Supplier and

Customer sample sizes, ns and nc, are large than when they are

small.

...Given the total number, dt = ds + dc, of defects, the conditional

distribution of dc is then Binomial IF the Supplier's purported

quality level equals its true level.  Furthermore, the

probability that any particular defect would have been observed

by the Customer rather than reported by the Supplier is then

simply nc/(nc+ns).

...Calculate the probability that dc or more defects out of the

fixed total of dt defects would be observed by the customer.

Take ACTION when this probability is less than or equal to 0.05.

The EXACT procedure has some PRACTICAL DISADVANTAGES...

...The assumptions upon which the EXACT test is based are

not always realistic, especially in small samples.

...The statistical "size" of the EXACT test cannot usually

be equated to 0.05 because it is based upon a discrete

distribution (Binomial), not on a continuous distribution.

...In fact, the size is frequently much less than 0.05 in

testing situations where the sample size ratio, r = ns/nc,

is fixed in advance to be r = 1, 2, 3, 5, or 8.  When the

size of a statistical procedure cannot be "controlled"

either at or only slightly below its nominal level, then

its statistical "power" will also deteriorate.

In other words, the EXACT test can lead to

...DECREASED SUPPLIER RISK, and

...INCREASED CUSTOMER RISK.

The exact test is easy to motivate and explain.  The conditional

distribution of supplier and customer defects for a given total number

of defects, ds + dc, is a Binomial distribution when defect rates follow

independent Poisson distributions with equal defect intensities.  The

Bernoulli probability of a defect being observed by the customer rather

than reported by the supplier is pi =nc/(nc+ns)=1/(1+r) under the null

hypothesis of valid supplier data.  Hoel(1945) and Ferguson(1967) show

that the "exact" test based upon this conditional, Binomial distribution

is uniformly most powerful unbiased.  Consider the most simple case

possible: ds + dc = 1.  Then dc = 0 with probability 1 - pi and dc = 1

with probability pi.  Thus dc = 1 (and ds = 0) leads to action at a 5%

significance level using the "exact" test when pi <= 0.5 or r >= 19.

When ds + dc = 2 or 3, exact calculations are not quite as easy as above

for ds + dc = 1, but they still can be done by hand.

Section 7: ...The Approximate Test

==================================

H109 was originally used as follows...

...the value of the Supplier/Customer sample size ratio,

r = ns/nc, is chosen to be r = 1, 2, 3, 5, or 8.

This has the effect of determining the size of the

Customer's confirmatory re-sample to be strictly a

function of the size of the Supplier's Final Audit.

...A table of Action Limits was published.  This table has

36 rows corresponding to ds = 0, 1, 2,..., 35 and 5

columns for r = ns/nc = 1, 2, 3, 5, and 8.

If the number of Customer observed defects in the Customer's Re-Sample

equals or exceeds the value in the appropriate row and column of

the table, the Customer should declare a DATA DISCREPANCY.

If the number of Customer observed defects in the Customer's Re-Sample

is smaller than the value in the table, then the Customer should

make a shipment accept/reject decision based solely upon the

SUPPLIER'S DATA.

Usage of H109 may differ from the traditional approach in

several significant respects...

...the size of the Customer Re-Sample may be optimized

considering all available cost, budget, and quality factors.

...The ratio, r, of the supplier's sample size to the customer's

sample size will usually NOT be an integer nor any other

pre-assigned value.

...As a result, Special Statistical tables or computer algorithms

are needed to apply H109.  Given values for ds and dc,

we specify an upper "Action Limit" on r = ns/nc.

...H109 will primarily be used only for quick, manual analyses

"in-the-field."  Methods which are more computer-intensive and

graphical will be used, instead of H109, whenever practical.

The "approximate" test of Cox(1953), Ellner(1963), and H109 is not as

easy to motivate or explain as is the above "exact" procedure.  But the

"approximate" procedure has been shown to be superior to the "exact"

test in several senses of practical importance--see the above references

and Section 2 of Obenchain and Hsiang(1985d).  The chief argument

against the exact procedure is that its unconditional size can be much

smaller than its conditional size at the fixed total number of reported

and observed defects, ds + dc.  A procedure with actual size closer to

the nominal level (like the Cox-Ellner procedure) can thus be much more

powerful than the discrete, non-randomized exact test.  In stark

contrast with the "exact" value of 19 for the 5% quantile of r = ns/nc

when ds = 0 and dc = 1, the "approximate" test uses 3.376.  Similarly,

ds = dc = 1 (total of 2 defects) leads to action at the 5% significance

level at r = 38.49 with the exact test but at r = 9.28 with the

approximate test.

REFERENCES

Cox, D. R. (1953).  "Some simple approximate tests for Poisson

variates."  Biometrika 40, 354-360.

Ellner, Henry (1963).  "Validating results of sampling inspection by

attributes."  Technometrics 5, 23-46.

Ferguson, T. S. (1967).  Mathematical Statistics:  A Decision Theoretic

Approach (specifically, Section 5.4: Uniformly Most Powerful Unbiased

Tests. pp 232-233.)  New York:  Academic Press.

Hoel, P. G. (1945). "Testing the homogeneity of Poisson frequencies."

Ann. Math. Statist. 16, 362-368.

Mood, A. M. (1943). "On the dependence of sampling inspection plans upon

population distributions."  Ann. Math. Statist. 14, 415-425.

Obenchain, R. L. (1985a).  Supplier Data Validity Analyses: Beta-Gamma-

Poisson Model.  Bell Communications Research.  TM-TSY-001569.

Obenchain, R. L. (1985b).  Supplier Data Validity Analyses: Methods and

Software.  Bell Communications Research.  TM-TSY-001570.

Obenchain, R. L. (1985c).  Supplier Data Validity Analyses: Simulation

Scenarios.  Bell Communications Research.  TM-TSY-001571.

Obenchain, R. L. and Hsiang, T. C. (1985d).  Supplier Data Validity

Analyses: New Tables for H109 Tests.  Bell Communications Research.

TM-TSY-001572.

Obenchain, R. L. (1985e).  Supplier Data Validity Analyses: Cumulative

H109 Procedures.  Bell Communications Research.  TM-TSY-001573.

Quality Control and Reliability Handbook (Interim) H109:  Statistical

Procedures for Determining Validity of Suppliers' Attributes

Inspection. (1960).  Office of the Assistant Secretary of Defense

(Supply and Logistics.) Washington, D.C.

Schilling, E. G. (1982). Acceptance Sampling in Quality Control. New

York:  Marcel Dekker.  (specifically, APPENDIX: pp. 510-517.  Procedure

for determining validity of inspection: H109.)