Documentation for the VGA16.BGI device driver.

by John Sieraski 4/10/90

* This release adds support for the Compro ABLE II VGA (800x600) 16 color

graphics mode.

The VGA16.BGI device driver is a device driver which supports various 16 color

VGA graphics modes which can be used within Turbo C 2.0 and/or Turbo Pascal 5.5

programs.  No support for 256 color graphics display modes are provided by

this driver.

Technical details:

This driver was written by me (John Sieraski).   The shell for the driver is

written in Turbo C 2.0 with the time critical support routines written using

Turbo Assembler 1.0's Ideal mode syntax.  All of the display routines use

the EGA/VGA write mode 2 (the fastest mode) and write directly to the device

without going through the BIOS.  Whenever a particular display routine is done

doing it's job, the card is returned to write mode 0 (The default).  The video

mode switching, display page switching, and palette control is done through

calls to interrupt 10h BIOS functions.  The video mode values used in the

interrupt 10h set video mode calls are listed in the table below, so if your

card uses the same video mode value shown for one of the supported cards,

chances are that selecting that mode will work on your card.  If you have an

enhanced VGA not supported by this driver, drop me a note and I'll see about

adding support for your card.

List of files:

Filename        Description

--------        -----------

VGA16.DOC       Documentation for the VGA16.BGI device driver (This file).

VGA16.BGI       A BGI driver supporting various EGA/VGA 16 color graphics modes.

DEMO.C          A Turbo C 2.0 program which demonstrates some of the enhanced

features supported by the VGA16.BGI device driver.

Display modes supported by the VGA16.BGI driver:

BGI Mode    Display card    Resolution  Interrupt 10h    number of

number                                    video mode     display pages

--------    ------------    ----------  -------------    -------------

0         EGA/VGA         320x200        0Dh               8

1         EGA/VGA         640x200        0Eh               4

2         EGA/VGA         640x350        10h               2

3         VGA             640x480        12h               1

4         ATI VGA         800x600        54h               1

5         Orchid VGA      800x600        29h               1

6         Paradise VGA    800x600        58h               1

7         VEGA VGA        800x600        62h               1

8         Compro VGA      800x600        5Bh               1

9         SOTA VGA-16     800x600        29h               1

The VGA16.BGI driver provides a couple of extra features which are not

supported by the standard BGI drivers supplied with Turbo C and Turbo Pascal.

The following is a description of the enhanced capabilities:

Support for two color fill patterns

-----------------------------------

The standard BGI drivers allow you to set the colors that the 1's in the

fill patterns get plotted in, but do not allow you to alter the color that

the 0's get plotted in.  The 0's in a standard BGI driver are always plotted

in color zero.  The default setting for fills in the VGA16 driver is the same

as it is in a standard driver i.e. zeros in fill patterns are plotted in color

zero.  To change the color that the zeros get plotted in, you need to make a

call to the setwritemode() function.  The mode value that you pass to the

setwritemode() function is used to control the color that the zeros in fill

patterns will be plotted in.  The valid mode values for controlling fill

pattern background colors are as follows:

mode value      action performed

----------      ----------------

6               toggles the plotting of zeros in fill patterns ON in color 0.

7               toggles the plotting of zeros in fill patterns ON in color 1.

8               toggles the plotting of zeros in fill patterns ON in color 2.

9               toggles the plotting of zeros in fill patterns ON in color 3.

10              toggles the plotting of zeros in fill patterns ON in color 4.

11              toggles the plotting of zeros in fill patterns ON in color 5.

12              toggles the plotting of zeros in fill patterns ON in color 6.

13              toggles the plotting of zeros in fill patterns ON in color 7.

14              toggles the plotting of zeros in fill patterns ON in color 8.

15              toggles the plotting of zeros in fill patterns ON in color 9.

16              toggles the plotting of zeros in fill patterns ON in color 10.

17              toggles the plotting of zeros in fill patterns ON in color 11.

18              toggles the plotting of zeros in fill patterns ON in color 12.

19              toggles the plotting of zeros in fill patterns ON in color 13.

20              toggles the plotting of zeros in fill patterns ON in color 14.

21              toggles the plotting of zeros in fill patterns ON in color 15.

For example, the following call to setwritemode() would toggle the plotting

of zeros in fill patterns ON in color 7:

#define SET_BACK 6

setwritemode(SET_BACK+7);

Support for transparent fill patterns

-------------------------------------

In the standard BGI drivers, the zeros in fill patterns are always plotted

in color zero.  I've added an extension into the VGA16.BGI driver which allows

you to toggle OFF the plotting of zeros in fill patterns so that you can get

a transparent effect when filling overlapping regions.  To toggle the plotting

of the zeros in fill patterns OFF you make the following call to

setwritemode():

#define NO_BACK 4

setwritemode(NO_BACK);

Support for two color line patterns

-----------------------------------

The standard BGI drivers allow you to set the colors that the 1's in the

line patterns get plotted in, but the zeros in standard BGI patterned lines

are not plotted.  The default setting for patterned lines in the VGA16 driver

is the same as it is in a standard driver i.e. zeros in line patterns are not

plotted.  To change the color that the zeros get plotted in, you need to

make a call to the setwritemode() function.  The mode value that you pass to

the setwritemode() function is used to control the color that the zeros in line

patterns will be plotted in.  The valid mode values for controlling line

pattern background colors are as follows:

mode value      action performed

----------      ----------------

22              toggles the plotting of zeros in line patterns ON in color 0.

23              toggles the plotting of zeros in line patterns ON in color 1.

24              toggles the plotting of zeros in line patterns ON in color 2.

25              toggles the plotting of zeros in line patterns ON in color 3.

26              toggles the plotting of zeros in line patterns ON in color 4.

27              toggles the plotting of zeros in line patterns ON in color 5.

28              toggles the plotting of zeros in line patterns ON in color 6.

29              toggles the plotting of zeros in line patterns ON in color 7.

30              toggles the plotting of zeros in line patterns ON in color 8.

31              toggles the plotting of zeros in line patterns ON in color 9.

32              toggles the plotting of zeros in line patterns ON in color 10.

33              toggles the plotting of zeros in line patterns ON in color 11.

34              toggles the plotting of zeros in line patterns ON in color 12.

35              toggles the plotting of zeros in line patterns ON in color 13.

36              toggles the plotting of zeros in line patterns ON in color 14.

37              toggles the plotting of zeros in line patterns ON in color 15.

For example, the following call to setwritemode() would toggle the plotting

of zeros in line patterns ON in color 11:

#define LINE_BACK 22

setwritemode(LINE_BACK+11);

Support for transparent line patterns

-------------------------------------

In the standard BGI drivers, the zeros in line patterns are never plotted.

I've added an extension into the VGA16.BGI driver which allows you to toggle ON

and OFF the plotting of zeros in line patterns so that you can get still get

a transparent effect when plotting overlapping patterned lines.  To toggle the

plotting of the zeros in line patterns OFF you make the following call to

the setwritemode() function:

#define NO_LINE_BACK 5

setwritemode(NO_LINE_BACK);

Support for extended line transfer modes

----------------------------------------

The standard mode values supported by the setwritemode() function for

controlling the transfer mode used by the line drawing routines are:

0 : for overwrite transfer mode (COPY_PUT).

1 : for XOR transfer mode (XOR_PUT).

The VGA16 driver adds support for the following additional line transfer

modes:

2 : for AND transfer mode (AND_PUT).

3 : for OR transfer mode (OR_PUT).

Standard BGI functions not supported by VGA16.BGI:

--------------------------------------------------

The only function defined in the standard BGI interface that isn't supported

by the VGA16 driver is support for the floodfill() function.  I'll probably

add support for floodfill() in a future release, but I didn't want to delay

making this driver available until I debug my current floodfill routine.  If

you make a call to floodfill() within a program, the function call will just

do nothing and then return.

Bugs

----

If you experience any problems in using this driver, I'd appreciate it if

you'd let me know.

My BIX account has been canceled, so I can no longer be contacted there.

I can be contacted on Compuserve as John Sieraski 76117,2022.

I also have an MCI MAIL account "jsieraski".

Distribution

------------

This driver may be distributed freely as long as no fee (besides download

costs) are required.  I take no responsibility for any problems you might

encounter when using this driver, but if you have any trouble, I'd

appreciate it if you'd let me know so that I can attempt to correct the

problem(s).

Enjoy!