QRT Users Manual

INTRODUCTION

QRT (Quick Ray Tracer) is an easy  to use image generation system

that uses a ray tracing algorithm  for image rendering.  Versions

exist for the Amiga personal  computer  and Data General MV10000.

QRT overcomes many of the problems of other ray tracing packages,

and is designed to operate nicely in a multi-tasking environment.

COMPARISON TO OTHER SYSTEMS

QRT was developed on the Amiga  personal  computer, so it will be

compared to other Amiga ray tracers.  There are, to my knowledge,

five other Amiga ray  tracers,  each  with  its own strengths and

weaknesses.  I will describe each  system briefly, and compare it

to QRT.  All the  Amiga  ray  tracers  can  operate  in HAM (4096

color) mode.

RT: RT was the first ray tracer written for the Amiga, by Eric

Graham.  It will model a universe  made of only spheres, a

sky, and a checkered or  solid  ground.   It is relatively

fast, but not  generally  useful  for  realistic  modeling

because of the sphere  limitation.   The input language is

cryptic, although some error checking is done.  The system

will only generate low resolution images.

SILVER: I have never seen  SILVER,  so  I cannot say much about

this system.

SCULPT-3D: This package incorporates  an interactive editor for

creating objects, and is  capable  of quickly generating a

preliminary image of  the  scene  by  using hidden surface

techniques.  However, every primitive is made of polygons,

and some primitives such as  spheres  require  hundreds of

polygons for a smooth texture,  so the ray tracing is very

slow.  Also, the package takes a large amount of memory to

run, and is prone to system crashes.  Its chief feature is

the ability to  create  arbitrary  shaped  objects using a

series of triangles.  Mirrored, dull, or shiny objects are

supported.

CLIGHT: This ray tracer  also  has  an  interactive editor, but

produces very poor quality  images.   It is not capable of

any patterning or reflection characteristics.

QRT Ray Tracer               Page 1                   User Manual

DBW: This is possibly the  most  complete  ray  tracer  for the

Amiga.  It will support objects  with arbitrary degrees of

reflection  and  gloss,   depth  of  field  effects,  some

texturing, wavy surfaces, fractals,  transparent surfaces,

diffuse propagation of light from  object to object, and 5

primitive types (sphere, triangle, parallelogram, fractal,

and ring).  The input language,  however, is so cryptic as

to be nearly  incomprehensible,  and if there is any error

in the input file, it will  crash  the system.  It is also

painfully  slow;  some  images  take  16  to  24  hours to

complete.

QRT is meant to be  a  compromise  between  the  fast, simple ray

tracers and the slow powerful systems.   It compares favorably in

speed to RT, and in power  to  Sculpt-3d  or  DBW.  It has a very

friendly input language with extensive  error checking.  Here are

some features of QRT:

o  Multiple primitive types, including user defined quadratic

surfaces

o  Arbitrary   levels   of   diffuse   reflection,   spectral

reflection, transmission, ambient lighting, and gloss

o  User defined pattern information for objects

o  Bounding boxes for groups of objects

o  Shadows

o  Multiple light sources with different characteristics

o  Arbitrary Phong spectral reflection coefficients

o  Color dithering to increase the apparent number of colors

o  Easy  to  use,  free  format  input  language  with  error

checking.  Parameters are by keyword and may appear in any

order.

o  Supports medium resolution (128k dots/screen)

Each of these features of QRT will be discussed is greater detail

in this document.

THE QRT WORLD

QRT constructs an  image  of  the  world  by  performing  certain

manipulations  on  an  internal  representation  of  a  group  of

QRT Ray Tracer               Page 2                   User Manual

objects.  You must provide a description  of these objects to QRT

through the QRT input language.

QRT writes a machine independent bitmap file which can be read by

a computer specific post  processor.   The  post processor either

displays the  image  or  converts  it  into  the  proper form for

display  by  system   programs.     (See  the  Machine  Dependent

Information section of this manual for details).

QRT is capable of dealing  with  5  types  of  primitive objects:

spheres,   parallelograms,   triangles,   rings   (annulus),  and

quadratic  surfaces.   The  latter  can  be  used to model cones,

oblong spheroids, and other interesting  surfaces.  Each of these

objects can have an arbitrary orientation in space, and arbitrary

surface characteristics.

QRT INPUT LANGUAGE

The QRT input language is free  format,  in  that commands may be

placed anywhere on a line,  and  newlines  may  be  placed at any

point.  This makes it easy to use indentation.  Comments are also

supported by surrounding  text  with  curly  braces  '{' and '}'.

Comments may span lines.

Once a file with QRT commands is built  (using any available text

editor), qrt can be made to process these commands:

QRT <commands.qrt

where "commands.qrt" is the file containing  the input.  QRT also

produces some statistics, so this  output can also be redirected.

Since QRT runs for a long time, you  will probably want to run it

as a background task with a low  priority.   So,  the full set of

commands to run QRT would be:

SetTaskPriority -5

run QRT <commands.qrt >qrt.out

SetTaskPriority 0

This set of commands is specific to  the Commodore Amiga; see the

Machine Dependent  Information  section  of  the manual for other

operating systems.

QRT will write a very large file, which may be anywhere from 400K

to  4  megabytes  depending  on  the  specific  computer  display

resolution.  (See the Machine  Dependent  Information  section of

this manual for the exact file  size).   There should be at least

enough room on your disk to  accommodate  this  file. I recommend

sending the output to a RAM disk,  so that the physical disk does

QRT Ray Tracer               Page 3                   User Manual

not get used for 60 minutes while QRT computes the image.

A typical QRT  command  to  create  a  sphere  using  the default

surface attributes would, be:

SPHERE ( loc = (5,10,100)     { this is a white sphere }

radius = 12.5

)

For comparison, the sphere command for DBW looks like this:

s 0 0 0 1 0 0 0 .2 .2 .2 .8 .8 .8 5 10 100 12.5

Or, in RT, the command would be:

<1,1,1> 2 (5,10,100):1;

The QRT input should be much  easier  to understand.  If QRT sees

an error in its  input  file,  it  will  stop  and print the line

number of the offending  line,  along  with  a  descriptive error

message (ILLEGAL PARAMETER, etc).

The QRT input language is not case sensitive.  A full description

of the language and a shorter, context  free grammar can be found

elsewhere in this manual.

THE QRT ILLUMINATION MODEL

Before meaningful images  can  be  created,  the QRT illumination

model must be understood.  The  light  that  reaches the observer

from each object is composed of several components:

o  Diffuse light      - The "color" of the object

o  Ambient light      - The color of the light that falls on

the surface of the object if no lamps

are shining on this spot.  Note that

this specifies the color of light, not

the color of the object itself.

o  Reflected light    - If the object acts as a mirror, some

light is reflected.

o  Specular highlights - The "bright spots" of a shiny object

viewed in a light.

o  Transmitted light  - The object may transmit some of the

light that strikes the back of the

object (glass surfaces)

QRT Ray Tracer               Page 4                   User Manual

All of  these  light  characteristics  can  be  specified for any

object.  If none are specified, the current defaults are assumed.

These defaults can be changed (see  DEFAULT in Language Reference

Guide).

Most of these  light  characteristics  deal  with a percentage of

light.  For example, a  light  may  reflect  80% of the red light

that strikes it, 20% of the green,  and 40% of the blue.  This is

the "color" of the object (diffuse light), and would be specified

in QRT by the following syntax:

diff = (.80, .20, .40)

In QRT, 1.00 is 100%.  The  language  is also free-format, so the

above is equivalent to:

diff = (.80,

.20,

.40 )

The commas between parameters  are  optional,  but make the input

easier to read.  Semicolons can also be used.

Ambient light is specified in a similar manner:

amb = (.20, .20, .20)

The ambient light  values  should  be  fairly small.  Most of the

light hitting the object comes from  lamps, but some parts of the

object may be in the shadow  of  another  object.   If no ambient

light is specified,  these  shadows  will  appear  totally black,

which looks unrealistic.  Ambient light will give the affect of a

small amount of light hitting areas in a shadow, producing a more

realistic looking image.

Reflection is specified using the MIRROR attribute:

mirror = (.90, .90, .90)

If a true mirrored surface is  desired,  the mirror values should

be fairly high.  If one of the values has a higher value than the

others, the mirror will appear red, green, or blue.

Transmission is specified as follows:

trans = (.80, .80, .80)

index = 120

Note the addition of another parameter,  the index of refraction.

An explanation of the index of refraction for an object is beyond

QRT Ray Tracer               Page 5                   User Manual

the scope of this document - see a  book on elementary optics for

details.

In  addition,   specular   highlights   are  specified  with  two

parameters:

reflect  = .50,

sreflect = 45

"REFLECT" is the percentage of  light  reflected  in the specular

highlight,  and  sreflect   is   the  Phong  spectral  reflection

coefficient.  Again, an  explanation  of the Phong coefficient is

beyond the scope of this  document;  however,  a higher value for

the coefficient will result in smaller,  tighter highlights for a

more metallic looking object.  Lower values of sreflect should be

accompanied by lower values in  reflect,  and will produce duller

looking surfaces, such as paper.

QRT also accepts a dithering amount for each object:

dither = 3

Dithering is a mechanism for  simulating  colors not available on

the display by blending  other  colors.   The  default  dithering

coefficient is 3; it should be kept  small.  Values of 1 to 6 are

good.  Larger objects should employ  more dithering, and mirrored

or glass objects should have little or no dithering.

QRT also accepts an attribute "FUZZ", but this is not used in the

present implementation.  In a future  implementation of QRT, FUZZ

will effect small, random perturbations of the normal vector at a

given location on an object.  This  will simulate rough surfaces,

matted glass, or imperfectly reflecting mirrors.

All of the above  light  characteristics  may  be attached to any

object.   The  default  is  for  a  white,  non  reflecting,  non

transmitting dull surface with an average amount of dithering. In

addition, any of  the  above  color  information  (which  will be

referred to from now on as  COLORINFO)  may also be attached to a

pattern structure (see the PATTERN section of this document).

LIGHT SOURCES

Light sources in QRT are called  "LAMPS".   The lamp structure is

simple:

LAMP ( loc = (12, 34, 56),

dist = 50,

radius = 10 )

QRT Ray Tracer               Page 6                   User Manual

The default is for a bright white lamp, but if you wish to change

this, you can  specify  different  values  for the lamp's ambient

light.  The radius value  is  ignored  by  this implementation of

QRT, but it must be supplied.   In  a  future version, the radius

may be used to  implement  penumbral  shadows.   I didn't include

them in  this  version,  since  they  take  a  very  long time to

compute, and don't add much to the image quality.

The "dist" entry in the lamp structure specifies at what distance

the light from the lamp is at full intensity.  Beyond this value,

the light will decrease in  intensity.   This value should be set

so that the nearest object  in  the  scene  is slightly more than

"dist" distance units from the  lamp.   This  does not have to be

exact. If objects are nearer  than  dist"  units, they will be so

bright that shading will not  take  place.  Objects very far away

will be dimly illuminated.

By the way, QRT distance  units  are  not  tied to any real world

unit.  They can stand for feet,  nautical  miles, or furlongs, at

your choice.

THE OBSERVER

After defining the world,  you  must  tell  QRT  the position and

orientation of the observer.  This is done as follows:

OBSERVER ( loc     = (0, 10,  20),

lookat  = (5,  5, 120),

up      = (0,  1,   0)

)

The x,y, and z are the location  of  the  observer.  The "lookat"

variables give a location in  space  that the observer is looking

at.  Most ray tracers  require  you  to  give  two angles for the

observers view  direction,  but  it  is  MUCH  easier to know the

location of an object or point in  space you wish the observer to

look at.  This will be the center of your scene.

The "up" variables define which  direction  is  up.  Usually, you

will wish to use the values  given  above.  QRT uses a right hand

coordinate system: positive  y  is  up,  positive z is out of the

screen, and positive x is to the right  if you are looking in the

negative z direction.   The  "up"  parameter  is optional, and if

omitted, defaults to (0,1,0).

QRT will generate an error message if no observer is defined.

QRT Ray Tracer               Page 7                   User Manual

OUTPUT FILENAME

QRT places its output in a file, so  you must give it the name of

this file:

FILE_NAME = outfile.tmp

This file must  observe  any  file  naming  restrictions  of  the

operating system and  computer  you  are  using.  See the Machine

Dependent Information section  of  this manual for information on

file naming restrictions.

FOCAL LENGTH

This is the focal length  of  the  "lens"  used  by the observer.

Think of it as a 35mm  camera  lens  -  higher  numbers produce a

telephoto effect, and smaller numbers  are for wide angle lenses.

Note that small numbers may produce some distortion of the  image

around the edges.

FOC_LENGTH = 60

SCAN RANGE

If you do not want QRT to produce  a whole image, you can specify

the first and last scan  lines.    The  parameters should be less

than the maximum Y resolution of your display.

FIRST_SCAN = 100

LAST_SCAN  = 300

This is useful for quickly generating part of an image.

SKY AND GROUND

QRT has facilities for generating  the sky and ground.  To define

the sky, you must give it two  colors  - one for the sky overhead

(zenith), and one for the horizon:

SKY ( zenith  = (.10, .2, .4),

horiz   = (.10, .2, .65),

dither   = 6

)

QRT Ray Tracer               Page 8                   User Manual

This will produce a blue  sky,  with  a  brighter  color near the

horizon.  (The sky need not  be  blue  -  it  could be red or hot

pink, or vary from red to  green).   QRT  will smoothly blend the

colors from the zenith  to  the  horizon.   Since  the  sky is so

large, you may want to specify  a  greater amount of dithering to

compensate for the displays color resolution limit.

The SKY structure will also produce sky colors below the horizon;

any ray that does not strike an  object  will strike the SKY.  To

fix this, define a ground.  There is no dedicated GROUND command,

since you can define a  very  large  parallelogram  with the same

effect.  You can make it  brown  with  patches  of  green using a

PATTERN, or checkered green and yellow in the classic ray-tracing

ground pattern.

BOUNDING BOXES

QRT supports the use of  bounding  boxes to speed the ray tracing

process.  For images  composed  of  only  a few (1 to 3) objects,

bounding boxes will not do much to  increase speed.  However, for

images where there are groups of objects physically close to each

other, they can greatly reduce  execution  times.  A bounding box

is a conceptual structure that encloses a group of objects.  When

the ray tracer is finding  line/object  intersections,  if a line

does not strike a  bounding  box,  it  cannot possibly strike any

objects within that bounding box.   This saves the ray tracer the

trouble of  checking  intersections  with  all objects within the

box.  In the  case  where  the  ray  DOES  enter  the  box,  some

additional overhead  is  incurred;  however,  this cost is easily

justified by reduced times for negative tests.

Bounding boxes can occasionally  be  useful  for complex objects,

such as quadratic  surfaces,  when  the  object  is fairly small.

Since the time to find the intersection  with a quadratic surface

is large, but bounding box intersections are fast, the ray tracer

can save time for all the negative tests.

QRT Ray Tracer               Page 9                   User Manual

Bounding boxes can contain  other  bounding boxes, in a recursive

manner.  This recursive structure defines an "object tree". There

are two keywords that define the beginning  and end of a bounding

box: BEGIN_BBOX and END_BBOX.  Here is an example:

{ ** QRT Code for a chessman ** }

BEGIN_BBOX

QUADRATIC (  { ** quadr defn ** }

)

QUADRATIC (  { ** quadr defn 2 ** }

)

SPHERE    (  { ** sphere defn ** }

)

BEGIN_BBOX

SPHERE ( )    { ** two spheres ** }

SPHERE ( )

END_BBOX

END_BBOX

This structure may be nested to an  arbitrary level; if you had a

closely spaced group of  chessmen,  you could enclose all of them

with a bounding box.

PATTERNS

QRT permits user  defined  patterns  which  can  be mapped to the

surface of any  object.   A  pattern  is  basically  an organized

method for changing  an  object's  COLORINFO  over the surface of

that object.  For example, a checkered surface can be created, or

a surface given the  appearance  of  brick  or tile.  The current

patterning in QRT is  limited,  yet  powerful  enough to describe

many common patterns.

A  pattern  is  composed  of  a  series  of  sub-patterns.   Each

sub-pattern defines a region on  the  surface  of an object which

will contain a  given  COLORINFO.   These  regions  are presently

limited to  rectangles,  but  this  may  be  expanded in a future

implementation of QRT.

QRT Ray Tracer               Page 10                  User Manual

A pattern can be defined once,  and  used  for many objects.  For

example, a brick pattern might be defined:

PATTERN ( name = BRICK,

{ ** other pattern info here ** }

)

SPHERE  ( { ** sphere definition ** }

pattern = BRICK

)

PARALLELOGRAM ( { ** parallelogram definition ** }

pattern = BRICK

)

This example ignores what is  actually  in the pattern definition

to demonstrate how patterns are  attached  to objects.  A pattern

must be defined before it can be used.   It is given a name, such

as BRICK, and any object can specify this pattern with "PATTERN =

BRICK".  There are  no  practical  restrictions  on the length of

pattern names (if your computer has 1 megabyte of ram, you cannot

have a pattern name longer than 1  million characters).  Multiple

patterns may be defined at the top of an input file, and used for

any object in the file.

What exactly makes  up  a  pattern,  you  ask?   A pattern can be

viewed as a rectangle that  is  repeated  over  the surface of an

object.  If the pattern size is  10  x 10, and you have an object

(say a parallelogram) that is 50 x  50, the pattern will repeat 5

times in each direction.   Within  this  repeating rectangle, sub

patterns can be defined.  A sub-pattern  is a rectangle or circle

within the pattern rectangle with a certain COLORINFO. Any number

of  these  sub-patterns   can   be  created  within  one  pattern

definition. To create a  brick  wall,  you  might  define several

sub-patterns, each with  a  slightly  different  color  of red or

brown.  The sub  patterns  need  not  cover  the  entire  pattern

rectangle; if they do not,  the  COLORINFO  you  defined  for the

object is used instead of the pattern  COLORINFO.  If you defined

sub-patterns for bricks, the  area  not  covered would be mortar,

and the objects COLORINFO would  define  the color of the mortar.

An actual brick pattern is  given  in appendix C.  The proper use

of patterns can be very effective  in producing realistic looking

scenes.  However, complicated patterns  will slow image creation,

so it is recommended that you  first  display test scenes with no

patterns, and then add  pattern  information  when  the  scene is

right.

QRT Ray Tracer               Page 11                  User Manual

INSTANCES

Often, several copies  of  a  complex  object  must  be  created.

Specifying  their  component   primitives   each  time  would  be

difficult. There is a solution to both these problems: INSTANCES.

An INSTANCE is a method by which several primitives (actually, an

arbitrary object tree) can be  grouped  and given a name.  Copies

of these objects may be easily created with one command.  Here is

an example of an instance definition.

BEGIN_INSTANCES

NAME = object1

BEGIN_BBOX

{ ** a bunch of primitives here ** }

END_BBOX

NAME = object2

BEGIN_BBOX

{ ** a bunch more primitives  here ** }

NAME = spheres

BEGIN_BBOX

SPHERE ( )  { ** two sphere definitions ** }

SPHERE ( )

END_BBOX

END_BBOX

END_INSTANCES

There can be only one of these instance  definitions, and it must

appear before any  instances  are  used.   However, any arbitrary

object tree can appear within the  begin/end instance statements.

Several instances are now available for use.  For example:

INSTANCE_OF (  name = object1,

loc  = (100, 10, 20),

)

INSTANCE_OF (  name = spheres,

loc  = (12.3, 24.5, 999)

)

This example shows that any  named  portion of the object subtree

can function as an instance  (the  "spheres" object is at a lower

level than the "object1'  object).   The  "offset" parameters are

required.  They specify a new  position  for the instance (offset

from the  origin).   In  the  instance  definition  segment,  all

objects are defined relative to 0,0,0.  They can then be moved in

the INSTANCE_OF statement with the offset command.

QRT Ray Tracer               Page 12                  User Manual

In addition, the instance can be given a new size:

INSTANCE_OF (  name    = object2,

loc     = (200, 100, 50),

scale   = (1.5, 2.0, .25)

)

The scale factors are optional,  and  are given the default of 1.

The instance will be resized by  the  indicated amount in each of

the directions.  Note that  there  are some restrictions to this:

if spheres are given a different scale  factor in each direction,

they will remain spheres (not  change  to  QUADRATIC types).  The

new radius will be the old  radius  scaled by the smallest of the

scale factors.

DEFAULTS

Each object  in  QRT  is  given  certain  default  surface  light

characteristics (see the section on surface characteristics).  If

these defaults are  not  suitable,  they  can be changed with the

DEFAULT command:

DEFAULT ( diff = (1.00, .1, .1)

no_shadow )

This will make all future objects red unless specified otherwise.

More than one default  command  can  be  used in a file: each one

affects all the  objects  created  after  it, but before the next

default  command.   Any  light  characteristics  can  be  changed

(MIRROR, DITHER, etc).

In addition, the keyword "no_shadow"  can be included (as above).

Ordinarily, QRT computes shadow information for all objects. This

takes a lot  of  time,  especially  for  scenes  composed of many

objects and many lamps.  The  "no_shadow"  command  causes QRT to

bypass the shadow routines.   This  will  result in a much faster

image  generation  time,   but  the  picture  will  not  look  as

realistic.

QRT Ray Tracer               Page 13                  User Manual

PROBLEMS WITH QRT

There are several limitations in this version of QRT:

o  Patterning  does  not   work   well  with  some  quadratic

surfaces.  This is  a  problem  with  the 3d to 2d mapping

function used for quadratics.

o  Patterns  do  not   support   circular  or  non-orthogonal

rectangular   sub-patterns.   This   would   slow  pattern

processing        but increase the utility of patterns.

o  User defined texturing is not implemented - all objects in

QRT appear  smooth.   I  have  some  ideas  on  how to add

textures to objects, but  these  have  not been tested and

are not incorporated into this version of QRT.

o  Some rounded objects such as  spheres  appear distorted if

they are near the edge of the image.  This is a bug in the

routine that creates a ray for a given pixel position.

FUTURE ENHANCEMENTS TO QRT

The following are some things I'd like to add to QRT:

o  Enhanced  patterning   capability.    This  could  include

arbitrary polygonal  patterns,  and non-orthogonal shapes.

Also, the ability to use enumerated  (bit-mapped) patterns

as well as analytic patterns would be useful.

o  Anti-dithering   routines.     These  routines   are  very

computationally   expensive   (they   can  increase  image

generation times by a factor of  3 to 5), so they were not

included in this version.

o  Penumbral shadows.   This  is  also  very  computationally

expensive for a minimal utility  level, so I didn't bother

to include this feature.

o  Fractal generation.  Fractals are,  again, computationally

expensive, but some things,  such  as mountains, cannot be

modeled with a ray tracer any other way.

o  Wavy surfaces.  This is useful for modeling water, rippled

mirrors, etc.  I know  how  to  do  it, I just didn't have

time to add it yet.  It  would  be  nice if the waves were

user definable in amplitude and  x and y wavelength.  Wavy

surfaces are very similar in implementation to textures.

QRT Ray Tracer               Page 14                  User Manual

o  Snowy surfaces.  This is an idea from DBW, which adds snow

to a surface depending on its slope and altitude.

o  Interpolated  normal  surfaces.   This  is  a mechanism to

model  arbitrary  curved  surfaces  by  using  a polygonal

approximation to the surface, and interpolating the normal

vector between surfaces to avoid angular looking surfaces.

o  Image  plane  object  lists.    This  is  a  technique for

increasing the speed for positive line/object intersection

test.  (Bounding  boxes   increase   speed   for  negative

line/object intersection tests).

o  Heuristics for  faster  completion  of  images  using area

coherence.

o  An  interactive  editor.   This  is  another  large  scale

project in itself, and I can't forsee having time to do it

for a long time, but it would be useful.  The editor would

display a preliminary  image  of  the  scene, and when the

user had placed objects  to  his  satisfaction,  it  would

write a QRT input file and call  QRT as a background task.

There are certain problems here - for instance, how do you

quickly generate the outline  of  a user defined quadratic

that can take many forms  (cone,  spheroid, etc)?  And how

can the user  easily  specify  the  object  tree structure

(bounding boxes) with an interactive editor?

QRT Ray Tracer               Page 15                  User Manual