Parameter...

#	meaning

--	-------

0	Octave

1	Fine Detune

2	Octave and Note Detune

3	Vibrato Waveform

4	Vibrato Delay Time

5	Vibrato Rate

6	Vibrato Depth

The following parameters are for line 1:

7	DCO Waveform 1

8	Use DCO Wave 2? (0 = No, 1 = Yes)

9	DCO Waveform 2

10	Ring Modulation

11	Noise Modulation

12	DCA Key Follow

13	DCW Key Follow

14	DCA End Step

15	DCA Initial Attack Rate

16	DCA Initial Attack Level

17	DCA Pre-Sustain Rate(s)		[Note: pre- and post-sustain become

18	DCA Pre-Sustain Level(s)		first half / second half of

19	DCA Sustain (if any) Rate		envelope if no sustain step]

20	DCA Sustain (if any) Level

21	DCA Post-Sustain Rate(s)

22	DCA Post-Sustain Level(s)

23	DCA End Rate

24-33	As 14-23 except for DCW

34-43	As 14-23 except for DCO

44-80	As for 7-43 except for line 2