------------------------------------------------

Season7 -- MS-DOS Videocrypt smart card emulator

------------------------------------------------

1994-06-19 -- Markus Kuhn

This software allows you to connect your PC to a Videocrypt pay-TV

decoder. Season7 can both listen to the data traffic between a card and

a decoder and act like a card. This software also implements the hash

function and the secret key that was used in the 07 series card used by

British Sky Broadcasting on the Astra satellites in 1993 and early

1994. In that time, it was possible to watch these channels without a

Sky card using Season7 (someone also called it OMIGOD), but with the

introduction of the 09 card series, the secret key has been changed. So

you can't use this software easily to watch Sky any more, but it still

descrambles recorded program from before 1994-05-18. Season7 can

basically also decrypt the Adult Channel (the same hash algorithm is

used), but for legal reasons, the secret key of the Adult Channel has

been removed from this version, because it is still in use.

In order to use this software, you will need a Videocrypt decoder with

a card slot (costs about 400 DM), an IBM AT compatible PC with at least

a 12 MHz i286 processor, a serial RS-232 port and DOS 3.3 or higher and

you will have to build a small interface circuit as described in the

file ADAPTER.TXT. This interface may be operated in three modes:

a) A PC listens to data traffic between a card an a decoder

b) A PC talks to a decoder without a card and acts like a card

c) A PC talks to a card without a decoder and acts like a decoder

Season7 only supports mode a) and b), but the file ADAPTER.TXT also

describes the additional circuitry (clock generator) necessary for mode

c) operation.

You can use Season7 to record the data traffic between a card and a

decoder for a certain program. If you make this recording available to

someone else, then it is possible for this person to replay the data to

the decoder with Season7 and to descramble the same program later if it

has been recorded in encrypted from on a VCR. The VCL Videocrypt

logfile format supported by Season7 is especially designed for this

purpose, because it only contains the part of the exchanged data needed

to descramble the image. Other exchanged data like the card serial

number is not stored in a VCL file. Technical details of the Videocrypt

card protocol and the VCL file format are explained in the file

DETAILS.TXT.

IMPORTANT: It is illegal to watch pay-TV programs without a subscription

in the countries in which a regular subscription is available

(for the Sky channels, these are currently Great Britain and

Northern Ireland). The legal situation in other countries

is unclear, but as it is currently tolerated that e.g. in

Germany and Sweden so-called pirate or clone cards are sold,

using this software in other countries should be no problem

at the moment. There is perhaps also no problem if you have

already a regular subscription for the channels and want

to use this software only for educational purposes. THE AUTHOR

OF THIS SOFTWARE TAKES NO RESPONSIBILITY FOR THE LEGALITY OF

USING AND DISTRIBUTING THIS SOFTWARE. As far as the author is

concerned, this software may be used and redistributed freely.

How do you use Season7

----------------------

You have to build an electrical interface between the decoder's card

slot and a serial port of your PC. The file ADAPTER.TXT that comes with

this package gives you more technical information about how to do this.

Then start SEASON7.EXE and give as a command line argument the port

number to which you connected your interface (default is COM2 if you

don't give a number).

E.g. enter

SEASON7 1

if your interface is connected to COM1. If the text 'RESET' or

'Decoding ...' doesn't appear after a few seconds or if your decoder

displays 'YOUR CARD IS INVALID', then please switch to an unscrambled

channel for at least 3 seconds in order to cause a card reset in the

decoder. This software has currently only been tested with a PACE VC100

decoder, but it is expected to work with all other decoders, too.

Without further options, Season7 tries to emulate a Sky series 07 card.

You can leave the program by pressing either q, x or ESC and you can

activate and deactivate a debugging mode by pressing d. In the

debugging mode, all data packets exchanged between the card and the

decoder are listed. By pressing l you can write the last (up to 400)

crypto messages to file VCLOG. As the log file time stamps are in

universal time (UTC, formerly known as Greenwich Mean Time), you should

define your local time zone in environment variable TZ. E.g. put 'set

TZ=MET-1DST' in your AUTOEXEC.BAT if you live in the Middle European

Timezone in summer (2 hours ahead of universal time) and use 'MET-1' in

winter (1 hour ahead of Greenwich). On startup, Season7 shows you the

current universal time. Please check whether it is correct, especially

if you want to provide VCL files for other peoples.

On slow computers (e.g. an i286 processor with 12 MHz is slow enough),

the time needed for the debugging output might already be too long and

the decoder will get a timeout and send a RESET signal. As the protocol

is quite time critical, try to use only simple DOS (no DOS

compatibility box in another operating system) and if you still have

problems, remove unnecessary TSR programms that might steal time and

interrupts from Season7 and try the e option.

Season7 understands several command line options:

d               This activates the debugging mode which will

print all data exchanged with the decoder on

screen. Debugging mode can also be (de-)activated

by pressing d while the program runs.

p               In the passive mode, Season 7 doesn't send any

bytes, but listens only to the data traffic

between a decoder and a real card. E.g. use

'season7 pd >logfile' in order to record to

a file what's happening between a card and

a decoder.

o<filename>     Write a VCL file that allows other peoples later

to descramble the same program from a VCR.

The file name is optional (e.g. 'season7 otest.vcl'),

and if you don't specify one, Season7 generates

one using date, time (hour rounded up by 5 minutes),

satellite name and transponder number (VCmmddhh.stt).

Season7 will ask you about the satellite, the

transponder number and what you are recording

and stores this information together with the

time in the VCL file header. The 'o' option

automatically activates the passive mode, so

you'll have to connect a real card to the adapter

(mode a), because Season7 only listens. Make sure

you have set the system clock and environment

variable TZ correctly.

i<filename>     Instead of emulating a Sky 07 card, use the

specified VCL file in order to answer the queries

of the decoder and descramble a program (mode b).

Useful only for video cassette recorded program.

e               In the economy mode, no time consuming screen

output and data log activities take place in the

main loop. This allows to use Season7 on some very

slow systems. You might want to try this option

together with options wb0 and wa0.

m               This option suppresses the on-screen message

with the Season7 version number, the time and the

channel name.

wa<number>      Allows you to specify the number of microseconds

wait time between a reset impulse from the decoder

and Season7's answer to reset (default: 30000 =

30 ms).

wb<number>      Allows you to specify the number of microseconds

wait time between the bytes sent to the decoder

(default: 500). This value can be modified while

Season7 runs by pressing + and -.

The optimum wa and wb values haven't been determined yet for all

possible processors and decoders, so please play around with wa in case

of reset problems and with wb in case of unstable operation after a

successful reset.

While decrypting, Season7 shows you three hexadecimal numbers from the

decoder's data packets. The second one is used to select the secret key

in the card, so if this one changes, perhaps new codes have been

activated. The third number identifies the channel and strange numbers

there have been used by Sky in the past to confuse hacker cards. In VCL

replay mode (option i), you'll only see at which position in the VCL

file the current answer for the decoder query has been found. If the

numbers bore you, better switch off the PC monitor in order to save

power while watching TV.

I wrote Season7 just for fun as a hobby project and I have no

commercial intentions. Many other Videocrypt hobby experts and I

periodically read the USENET group alt.satellite.tv.europe, so please

post there a message if you have problems, questions, etc. regarding

this software. Please don't contact me directly unless you have new

technical information, and please understand that I don't have the time

to give technical support to unexperienced Season7 users. I wish to

thank the members of the tv-crypt mailing list for their valuable help.

Season7 uses a public domain interrupt driven serial port assembler

routine which has been published by David Kessner <dkessner@vigra.com>

some time ago. Ask him if you are interested in the latest version of

this serial code.

This software was primarily written for European Star Trek fans outside

Great Britain who don't have a chance to get a regular Sky subscription

and have no other way of watching the undubbed version of their

favourite SF series. I don't want to cause any harm to Sky and I even

asked them for a regular subscription some time ago, but they refused

to sell one to Germany. So they have to live with the consequences of

attracting the interest of high tech freaks to the technical details of

their access control system. :-)

I developped this software and made it available to you at no cost and

so I expect of course that if you improve, update or port this software

to other systems, you will also make your work freely available to

others.