Disclaimer Notice:

This file is presented as an  INFORMATIONAL file only.

Do not try any of the things mentioned in the text file

as some of them are illegal to do. The bbs operator take

NO responsibilty.

---------------------------------------

How Ma Bell Works

---------------------------------------

In  this  article,  I  will first describe  the  termination,

wiring,  and  terminal  hardware most commonly used in  the  Bell

system, and I will include section on methods of using them.

-------------

LOCAL NETWORK

-------------

The   local   telephone   network   between   the    central

office/exchange  and  the  telephone subscribers can  be  briefly

described as follows:

From  the  central office (or local exchange) of  a  certain

prefix(es), underground area trunks go to each area that has that

prefix  (Usually  more than one prefix per area.)  At  every  few

streets or tract areas, the underground cables surface. They then

go to the telephone pole (or back underground,  depending on  the

area)  and  then  to the subsribers house (or in the case  of  an

apartment building or mutliline business,  to a splitter or  dis-

tribution box/panel).

Now  that  we have the basics,  I'll try and go in-depth  on  the

subject.

------------------

UNDERGROUND CABLES

------------------

These  are sometimes inter-office trunks,  but usually in  a

residential  area they are trunk lines that go to bridging  heads

or  distribution  cases.  The cables are about 2-3  inches  thick

(varies),  and  are  either  in  a metal  or  pvc-type  pipe  (or

similiar).  Rarely (maybe not in some remote rural areas) are the

cables  just 'alone' in the ground.   Instead they are usually in

an  underground cement tunnel (resembles a small sewer or  storm-

drain.)   The manholes are >heavy< and will say 'Bell system'  on

them.  they can be opened with a 1/2 inch wide crowbar (Hookside)

inserted  in the top rectangular hole.  There are ladder rungs to

help you climb down.  You will see  the cable  pipes on the wall,

with the blue and white striped one being  the inter-office trunk

(at  least  in my  area).  The others  are local  lines,  and are

usually marked or color  coded.  There is almost  always a posted

color code  chart on the  wall, not to mention  Telco manuals de-

scribing the cables and terminals, so I need not get into detail.

Also, there is usually  some kind  of  test equipment,  and often

Bell test sets are  left  in there.

--------------

BRIDGING HEADS

--------------

The  innocent-looking  grayish-green boxes.   These  can  be

either trunk bridges or bridging for residences.  The major trunk

bridging  heads  are usually larger,  and they have the  'Western

Electric' logo at the bottom,  whereas the normal bridging  heads

(which  may  be different in some areas-depending on the  company

you are served by.  GTE B.H.'s look slightly different.  Also, do

not  be  fooled by sprinkler boxes!)  They can be found  in  just

about every city.

To open a bridging head: if it is locked (and you're feeling

destructive),  put a hammer or crowbar (the same one you used  on

the  manhole) in the slot above the top hinge of the right  door.

Pull hard, and the door will rip off. Very effective! If it isn't

locked (as usual),  take a 7/8 inch hex socket and with it,  turn

the bolt  about 1/8 of a turn  to the  right  (you  should hear a

spring release inside). Holding the bolt, turn the handle all the

way to the left and pull out.

To Check for a test-set (which are often left by Bell employees),

go inside - First check  for a test-set  (which  are  often  left

by  Bell  employees).   There should be a panel of terminals  and

wires.   Push the panel back about an inch or so,  and rotate the

top  latch  (round with a flat section)  downward.   Release  the

panel  and it will fall all the way forward.  There is usually  a

large  amount  of wire and extra terminals.   The  test-sets  are

often hidden here,  so don$ overlook it (Manuals,  as  well,  are

sometimes placed in the head).   On the right door is a metal box

of alligator clips.   Take a few (Compliments of Bell.).  On each

door is a useful little round metal device. (Says 'insert gently'

or'  clamp  gently - do not overtighten' etc..)  On the front  of

the disc, you should find two terminals.  These are for your test

set.   (If you dont have one, dont despair -I'll show you ways to

make basic test sets later in this article).

Hook the ring (-) wire to the 'r' terminal;  and the tip (+)

wire  to the other.  (By the way,  an easy way to  determine  the

correct polarity is with a 1.5v LED.   Tap it to the term.  pair,

if  it  doesnt light,  switch the poles until it does.   When  it

lights,find the longer of the two LED poles:  This one will be on

the  tip wire (+).   Behind the disc is a coiled  up  cord.  This

should have two alligator clips on it..  Its very useful, because

you dont have to keep connecting and disconnecting the fone (test

set) itself, and the clips work nicely.

On  the  terminal  board,  there should be  about  10  screw

terminals  per  side.   Follow the wires,  and you can see  which

cable pairs are active.  Hook the clips to the terminal pair, and

you're set!   Dial out if you want,  or just listen (If someone's

on theline).  Later,  I'll show you a way to set up a true  'tap'

that  will let the person dial out on his line and receive  calls

as normal, and you can listen in the whole time.  More about this

later...

On  major  prefix-area bridging heads,  you can  see  'local

loops' ,which are two cable pairs (cable pair = ring+tip,  a fone

line)  that are directly connected to each other on the  terminal

board.   These  'cheap  loops' as they are called,  do  not  work

nearLy  as  well  as the existing ones set up  in  the  switching

hardware  at the exchange office.   (Try scanning your  prefixes'

00xx  to 99xx #'s.)  The tone sides will announce themselves with

the 1008 hz loop tone,  and the hang side will give no  response.

The  first  person  should dial the 'hang' side,  and  the  other

person dial the tone side,  and the tone should stop if you  have

got the right loop.)

If  you want to find the number of the line that you're  on,

you  can either try to decipher the 'bridging log' (or whatever),

which is on the left door.  If that doesnt work,  you can use the

follwing:

---------------------------

ANI # (Automatic Number ID)

---------------------------

This  is a Telco test number that reports to you the  number

that youre calling from (It's the same, choppy 'Bell bitch' voice

that you get when you reach a disconnected #)

For the   213 NPA - Dial 1223

408 NPA - Dial 760

914 NPA - Dial 990

These  are  extremely useful when messing with any  kind  of

line terminals, house boxes, etc.

Now that we have bridging heads wired, we can go on... (dont

forget to close and latch the box after all... Wouldnt want to GE

and Telco people mad, now,would we?)

Continued in: Ma Bell part II