SB-039

Alternator Whine, TR-7600/7625                7-9-79

In some mobile installations, alternator whine may be heard during

transmit.  External B+ filtering does not have sufficient effect.

In some instances, using the cigarette lighter will cause this problem.

Running off the fuse block, and even directly from the battery may still

not eliminate the noise.

First, check power line routing in the engine compartment.  It should be

kept clear of all secondary HV harness wiring, and clear of the alternator,

regulator, and ignition primary circuits.  Careful routing should reduce or

eliminate noise coming in through the power line.

Check for clean ground connections at the antenna and radio.

Check for clean and tight battery connections.

If all of these measures fail, replace L22 on the VCO unit X50-1580-10 from

the original green coil, to either an orange or black coil.  No adjustments

should be necessary.

Old part number L40-1021-03 1mH

New part number L40-1021-43 1mH

Please make these changes in your service manual.

JEB/yn

(c) 71079TKC