SB-833

TR-7800/TR-9000 Optional Ceramic Filters                       10-10-80

FM Adjacent channel rejection in either unit may be improved by installing

an optional narrow ceramic filter.

Filter    Part No.       -6db Bandwidth -70db Bandwidth Retail

CFK-455F  L72-0304-05    +/- 6 kHz      +/- 12 kHz     $ 33.50

CFK-455G  (CFK-455G)*    +/- 4 kHz      +/- 10 kHz     $ 33.50

* Not original part.  No computer part number.

Location: TR-7800, RX unit X55-1270-10 part, L10

TR-9000, RX unit X55-1260-11 part, CF1

To install, desolder the original filter and clear the second set of holes

for the optional filter.  Solder the new filter in place. Be sure all pins

are actually soldered, and that there are no solder bridges between pins or

across the printed circuit board.  Clip the pins flush to the board, and

reinstall the circuit board.

Note:  Squelch operation will vary from unit to unit with an optional

narrow filter.  Tighter coupling from the detector  to the squelch circuit

may be tried to restore "original feel".

JEB/yn

(c) 101080TKC