SB-020

Service notes on TS-700A                             10-18-77

POWER SUPPLY

1.   9 volts down:  The MFC4060 Q11 and 2SA495 Q9 are bad.

2.   No 21V:  Replace Q3 (2SC733Y (X43=1120-00) Power Supply Unit.

3.   Dial light is VERY bright:  Replace shorted Q2 2SA671 on rear panel.

4.   Intermittant +9 volt supply:  The legs of  Q12 2SD235 are too short at

the PC board.  The leads were pulled away from the board.   Replace Q12.

5.   Power supply intermittant when tapped:  Check for short leads on pass

transistors, pulled out of circuit board.

6.   No RX - No TX:  Check diode D6 Zener in power supply.  -6V supply.  If

shorted, replace.

7.   No 13.8 volts at power supply:  The wire on bridge D1 is not soldered.

8.   +9 volts is okay, but 21 volt onl 16 volts:  Q1 is bad in the power

supply 2SC733Y.

TRANSMITTER

1.   No modulation:  Replace bad diode D15 Varactor on X52-1080-21.

2.   USB or LSB is too high frequency:  Recheck alignment of carrier board

according to TKC service Manual procedures.

3.   No RF out AM CW SSB:  Replace D13 on GEN unit X52-108021.

4.   FM RX and TX different frequency:  Readjust all HET unit frequencies.

Also check VFO linearity and adjust TX frequenc L3.

5.   TX wrong frequency:  Adjust L3 at X52-1080-21.  10.7 oscillator for

final TX frequency.

6.   No power out FM:  Check for loose connection at L3, the 10.7 coil on

GEN unit X52-1080-21.

7.   No TX or RX:  The fixed channel switch is in the wrong position.

8.   Low poer out only 8W.  21 volt supply down to 16V:  The protection

diode D7 in the power supply is shorted.

9.   VFO drift:  Q1 3SK22Y in VFO is bad.

10.  No TX FM:  D15 on X52-1080-21 GEN is bad.

11.  Drive control does not peak:  This is normal - leave at center 500

position.

12.  No TX:  TX B terminal touch to ground at X52-1080-21 GEN unit.

13.  No TX FM:  Check diode D16 on GEN unit.

14.  Only 3 watts out:  Bad Q2 2N5642 in final amplifier.

15.  The RF meter doesn't Work, but TX output okay:  Bad diode D1 on BPF,

the 1N60 is open.

16.  No TX:  Bad Q3 in mixer unit (X48-1130-21) 3SK35 (BL).

17.  No VFO output:  Cold solder joint of R2, 100 ohm resistor inside VFO

on circuit board.

18.  Intermittant TX:  Bad ALC pot on mixer board VR2 (X48-1130-21)

19.  No TX 147 Simplex:  Loose resistor not soldered on rear circuit board

of band switch.

RECEIVER

1.   Spurrious signals on receive; if the output level or VFO is too high

or mixer Q3 on HET unit:  SN76514 is bad.

2.   IF the noise blanker does not work:  Check Q6 2SK19GR on X55-1120-00.

3.   There is no audio from receiver-no 21 volts:  Q1 2SC733(Y) in 21 volt

supply is bad.

4.   FM receiver sensitivity is down 30 - 40 db:  Check 10.245 xtal at TP-1

on FM IF unit.  If oscillator level is down, replace crystal.

5.   Receiver sensitivity is intermittant and TX intermittant:  Check

contacts RT relay.  Replace if bent or miss-contact.

6.   If you cannot adjust the center meter:  Maker sure Smeter start, VR4

is set at zero.  If you cannot get enough adjustment with center meter,

adjust VR1, replace Q4 (2SK19GR) on X55-1120-00.

7.   No receive:  Check for solder touch at 133 terminal on Het unit, if

ther is no 133 het output.

8.   No RX or TX and no VFO light:  Check diode D2, 1N60 located on VFO

circuit board.

9.   VFO drifts or moves wit shock:  Wire is not soldered VFO main coil L1.

10.  No repeater operation:  Check for wire that is touching at repeater

switch.

11.  No receive:  Resistor R7 is touching Q1, 3SK41 on X55-1120-00 board.

12.  No RX on 144, or any single band:  Check HET xtals-realign the Het

unit.

13.  Center meter drifts for the 300000series and before:  Send for the TKC

bullent on Center Meter Drift.

14.  VFO is intermittant when turned:  There is loose tension on VFO rotor

and Stator plate contact.  Rebend tension spring.

15.  RX audio output distorted:  Chec Q4 on X49-1060-00.

16.  No SSB or CW receive:  Resistor R73 is touching to the can of T6 on

X52-1080-21.

17.  FM receive okay but SSB sensitivity is down:  Check alignment of SSB

IF transformers T4,5,6.

18.  FM receive is bad:  Bad T1 on the X48-1140-20.

19.  Scratchy audio with internal speaker:  Check for bad speaker.

20.  No 146 band: Replace bad Het Xtal on Het Board.

(c) 101877TKC