SB-055

TS-180S Low 40M ALC               9-14-79

Please annotate your service manual.

Some operators may experience intermittent reduction or loss of ALC,

especially after several minutes transmit time.

On the LPF unit X51-1180-00, change C14, C17 from 470 pf to 390 pF.  Check

ALC level and adjust if necessary, per section 406 ALC meter adjustmet, TS-

180S service Manual.

New part:  CM93D2H390J 390pF +/- 5% 500v mica or ceramic

Please annotate your Service Manual.

JEB/yn

(c) 91479TKC