SB-037

TS-120S VOX operation pick-up time                7-10-79

High speed CW ops may inquire about partial loss of the first character

when operating VOX-CW.  The transceiver will operate up to about 20WPM

before this is apparent.

Improvement may be had by changing three components on the RF Unit X44-

1260-01.

R33  100K ohm, change to 150K ohm

R52   33K ohm, change to 4.7K ohm

C62  .022 uF,  Change to 1 uF @ 50v

Production later than serial # 940701 has this change.

Please make these changes in your service manual.

JEB/yn

(c) 71079TKC