SB-060

TS-120 Repair Notes                12-18-79

Symptom                  Repair

No CW, No Audio          Sidetone oscillator Q25 on AF GEN unit 2SC1815

Receiver Sens Down       T1, L34--696-35 on RF unit Shorted

Analog Dial Cal OFF      Mechanical Adjustment

Carrier in SSB           RF unit D2 and D3 shorted see TKC Service Bulletin

Loss of Digital          Check P5/J5 on Carrier Unit

Smoke at Final Unit      Check for long eds of wire shorted to case inside

final unit also burned L10 and L11, L40-1011-04

(100uH)

SSB Audio bad            IF shift control turned max CW

Power on Meter Max       The RF gain control was turned max CCW

VFO freq drift           Replace the TEMP comp CAP C17

TX Audio BAD             Check the Carrier Point adjust and make sure final

bias is set to 150 Ma

PLL digital NG           Check D15 PLL unit

High IC in RX            Check the Final transistors

15 Meters only           The BCD rotary band switch has a broken wafer.

RX TX only 10M           The BCD divider Q16, 74LS163 on PLL unit bad.

Check the BCD inputs against the pin 2 frequency.

28.5 HF Osc              Check ground screws and tighten at Relay Unit X41-

1250

No CW Filter             Solder short across pins 1-2 J29 IF Unit

No RX                    After the CW filter was installed the pins were

not cut off shorting the signal to ground.

Analog dial off          Perform VFO tracking adjustment at 50 and 450

TX out 40M only          40M 50W and 10M 10W bad finals

No 15M                   Core on the VCO transformer for 15M broken

3.5 and 7 Band same      Shorted trace at Band switch under IF unit

Low RF out               All bands, Q12 SN16913 on AF gen unit, 2SK19GR bad

No TX all bands          L25 open on the RF unit Small gree inductor

No digital no TX         Q12 mixer on the PLL unit bad

No RX no digital         Q26 mixer on PLL unit bad

INtermitt SSB AUdio      Pin 5 of J5 on AF GEN unit loose

No key or TX             D26 and Q24 2SA562 on AF gen unit bad

No TX                    R8 220 ohm at key jack burned open.  Make sure

customer uses shielded line on CW KEY.

15M osc.                 On 15M band only with antenna HF oscillation when

first speaking.  Install Ferrite bead on base of

first mic amp Q18 AF GEN UNIT.

No TX                    Loss of AVB Voltage at RF unit.  Check Q9 2SA473

on the filter unit, also check 6 volt Zener diode.

JEB/yn

(c) 8979TKC