SB-818

TR-2400 Scratchy TX                        3-20-80

Complaints of scratch audio, noise when flexing the antenna, or noisy audio

when handling the unit during transmit may be remedied by tightening the

Printed Circuit Board screws.

To disassemble the unit remove the back cover and battery compartment

screws.  INspec t the VCO shield solder points along the edge and bottom of

the PLL board.  Resolder if you find a broken solder bond.

Flex the metal frame several times to reseat the board, and retighten the

board mounting screws.  Be certain the flat metal spring has good tension

when installed between the RX TX unit a fram (below the PTT switch).

JEB/yn

(c) 32080TKC