SB-036

TS-180S DFC Hum with PS-30                7-10-79

Some TS-180S users may report erratic DFC and audio hum.  You will find

they have placed the PS-30 to the left of the transceiver.  Hum is induced

from the PS-30 into the DFC.

We are awaiting modification and production changes.  Meanwhile, please be

aware of this problem.  Temporarily moving the PS-30 to the right hand side

of the transceiver will cure the problem.  Spacing the two units with the

SP-180, or just seperating at least two inches will also cure the hum.

When full information is available, we will follow up.

Please advise you service ans sales personal.

JEB/yn

(c) 71079TKC