440-1.EXE 	COM1 version

440-2.EXE	COM2 version

440.doc		this file

440-x.EXE is a stand alone compiled basic program designed to control

the KENWOOD 440s (NOT 940) from the keyboard of an IBM-PC computer through

serial ports COM1 or COM2. It is designed to run from a single command

line or through the menus provided in the program itself. By entering all

440-x commands from the command line the program can be used with the

new "!" DOS shell command in version 4.2 of W0RLI CPBBS.

EXAMPLE of a typical command used in the FWD.MB file:

!b			<DOS shell command for port B of the CPBBS

440-2 20 5		<440-2.EXE command string (COM2:)

*** EOF			<END of dos shell

GB0023F W0RLI		<Beginning of next forward list

W0RLI

*** EOF

In the above example the "440-2" runs the 440-2.EXE program then

picks option "20" (SELECT MEMORY CHANNEL) and finally "5" which

selects channel 5. This same kind of command sequence could be used

in the autoexec file to initialize the radio on startup.

This program was written to allow a handicapped fellow ham control his

radio from the keyboard of his PC. My original intent was to write the

thing in C but I could not figure out how to control the PORTS and make

use of the COMBIOS etc.. I knew I could write the program in a few

hours in BASICA but I also knew the result would limit it's operation

to COM1 and COM2 only. To make a long story short I am making the program

available now in compiled basic form while I await additional information

on the control of the communication ports using the COMBIOS programs.

I DO NOT have an interface to control my 440s yet but it is on order so

if any of you have any comments or suggested additions to the program

please leave a message so I can add the features to it.

COMMENT:

One of the considerations for purchasing the 440 was its ability

to be controlled from the keyboard. Since it has a built in auto tuner,

I am many others assumed we could activate the tuner form the computer,

NOT TRUE for the 440. It seems to me that the ability to change frequencies

with the tuner is nearly useless. I have contacted Kenwood and they indicated

that I was the first to complain. If they were to receive "enough" complaints

they would consider a modification to offer tuning from the keyboard. In

my particular case I use the 440 on our Navy MARS packet net. This requires

several off the wall frequencies and tuning is a must on several of them.

For a handicapped operator this capability might mean the difference between

total enjoyment of the ham experience and added frustration. If you agree

that auto tuning is a much need 440 capability, please call or write Kenwood

in California and ask them to design a modifications for us. Thanks!

de KB3MY, CIS 72605,1465 Buster Light, Sliver Spring, Maryland

KB3MY @ KA3OGG

###########################################################################################