IRON.COM User's Guide

Version 1.0

Copyright (c) 1986 by Scott R. Grabowski

All Rights Reserved

Program Overview

This  program  package  consists  of  the  following  files:  IRON.DOC,

IRON.COM,   and   CHART1.TXT.   The  IRON.COM  program  calculates  the

inductance   of  windings  placed  on  an  iron-powder  toroidal  core.

Inductance  versus  turns  data  is  presented in tabular format on the

screen  and  may  be  printed, if the printing option is selected.  The

files  IRON.DOC  and  CHART1.TXT  are  text  files that provide on-line

documentation  and  a  reference  chart of maximum windings that can be

wound on the core.

The   IRON.COM   program   package  is  distributed  as  User-Supported

Software;  consequently,  even  though  it  is  copyrighted,  it may be

copied  and  distributed  without  restriction providing that: both the

program  and documentation files are distributed together as a package,

neither is modified, and no fee is charged for the package.

System  requirements  for  IRON.COM  are  an  IBM  Personal Computer or

compatible  computer  system using either a monochrome or color display

and operating in the PC-DOS 2.0, or higher, environment.

Page 1 of 4

Operation

At  the DOS prompt, type IRON <return> to invoke the program.  A series

of  screen  prompts will appear to assist you in entering the necessary

parameters.

The  part  number field may be used to identify the size and mixture or

type  of  core;  for  example,  a  typical  entry  might  be  "T-50-2",

indicating a toriodal core one-half inch in diameter of mixture 2.

The  wire  size  field  may be used to identify the size (American Wire

Gauge)  of  the  wire  used  for winding.  This data is not used in the

calculation  of  inductance;  however,  keep  in  mind that the maximum

number  of  turns that can be physically wound on a core is affected by

the  wire  diameter.   If  a  general  table  of  inductance  is  being

prepared,  the  response  "Any"  or  simply a <return> might be used in

this  field.   Refer  to  the file CHART1.TXT for the maximum number of

turns  that can be accommodated on different size forms for the various

AWG wire sizes.

For  convenience,  a printed copy of the chart can be obtained by using

the DOS commands; for example, TYPE CHART1.TXT >PRN.

The  uH/100-T  field  is  the  permeability  of  the  core expressed in

microhenries  per  hundred  turns.  This parameter is obtained from the

data  sheet  supplied  by  the core manufacturer.  All calculations are

based upon this parameter.

The  minimum  and  maximum number of turns fields are used to specify a

range  of  turns.   The  default values are 1 and 60 respectively.  The

screen  and  printout  always have a range of 60 turns.  The inductance

of  coils  having  between 61 and 120 turns may be found by entering 61

as  the  minimum  number  and  120  as  the  maximum number.  This same

technique can be used to extend the range even further if necessary.

Once  all  parameters  have  been entered, a table of inductance versus

turns  is  calculated  and  displayed on the screen.  Also, the printer

prompt  will appear on the screen.  If a printed copy is desired, enter

Y <return>; otherwise, enter N <return>.

Then,  the  prompt  "do  another"  will  appear  on  the  screen.  When

Y <return>  is  selected,  the  program will return to the beginning so

that  you  may  enter  parameters.   When  N  <return> is selected, the

program  reverts  to  the  registration  screen.   Pressing <return> in

response to the prompt will return the system to DOS.

The  program  may be aborted at any time by pressing the CTRL and BREAK

keys simultaneously.

Page 2 of 4

Disclaimer

The  Author  reserves the right to make changes to this program package

in  order  to  improve the reliability, function and/or design and does

not  assume  any  liability  from either the application or use of this

material.   Under no circumstance shall the Author be liable to you for

any  damages,  including but not limited to lost profits, lost savings,

or  other  incidental  or consequential damages arising from the use of

or  inability  to use this material.  Further, although the information

presented  herein  has  been checked and is believed to be reliable, no

responsibility is assumed for inaccuracies.

Registration

The  IRON.COM  program  package  is being distributed as User-Supported

Software.   This  gives  you the opportunity to evaluate the program at

your  convenience,  on your own system in order to determine whether or

not  it  serves  your  application  requirements.   In other words, the

user-supported software concept let's you try before you buy.

In  return,  the Author requests that you become a registered user once

you  determine  that  the  program  is  useful  to  you.   To  become a

registered  user,  a  contribution of $5.00 for an individual or $50.00

for  a  corporation  would be appreciated.  Your contribution will help

offset  development  costs  incurred  by  the  Author and assure future

updates   as   well  as  new  product  offerings.   In  addition,  each

registered user is placed on a mailing list for future notification.

Regardless  of  whether  or  not  you  become a registered user, please

continue to copy and share this program with others.

The  Author  welcomes  ideas and suggestions that might be incorporated

into  future  versions  of  this  program package.  Please address your

contribution and/or correspondence to:

Scott R. Grabowski

P.O. Box 471

Newbury, OH  44065

For  those  individuals requiring an invoice for tax recording purposes

or  for  initiating payment through your company accounting system, the

following form is provided.

Page 3 of 4

INVOICE

From: Scott R. Grabowski

P.O. Box 471

Newbury, OH  44065

Date:

Sold To:

ITEM                                                            AMOUNT

Registration Fee For: IRON.COM Program Package

[ ] Individual Registration               $5.00

[ ]  Corporate Registration              $50.00

Amount Enclosed

Sales Tax*

TOTAL AMOUNT

*Ohio residents please include 5% state sales tax.

Thank You

Page 4 of 4