AX.25 Version 2

Multi-channel

TNC  FIRMWARE

(version 1.3)

Copyright 1987, Ronald E. Raikes (WA8DED)

This firmware supports the full AX.25  link-layer  protocol,

version 2.0 as described in the ARRL specification dated  October

1984,  as  well  as   the   pre-existing   version   1.x.    This

implementation supports multiple  simultaneous  link  connections

with either version protocol.  This release  has  been  assembled

for a maximum of four connections, although any reasonable number

of connections is possible by changing one MAXLNK symbol  in  the

source equate file.

The firmware  is  contained  in  two  2764  EPROMs,  and  is

intended to be installed in a TAPR TNC-1 (or equivalent, such  as

the AEA PKT-1 or Heath HD-4040) in sockets  U11  (C000)  and  U12

(E000).  This assumes that a 6264 RAM  is  already  installed  in

socket U7, with additional  6264  RAMs  optionally  installed  in

sockets U8, U9, and U10 in any combination (verify that pin 26 is

connected to +5v).  Additional RAM is highly desirable if  multi-

channel or unattended operation is anticipated.  When  installing

the EPROMs (including newer versions), it will  be  necessary  to

start the tnc with the PARAMETER SOURCE switch set  to  the  PROM

position.  This will cause default parameters to be written  into

NOVRAM  memory  but  not  PERMed.   The  RS-232  port   will   be

initialized to 300  baud  with  xon/xoff  handshaking.   The  tnc

source call sign will be all blanks, and should be set  with  the

'I' command, as well as any other default parameters  that  might

need changing, before issuing a 'PERM' command.  At  this  point,

the PARAMETER SOURCE switch may be set to NOVRAM  for  subsequent

operation.  If the terminal baud rate has been changed,  the  new

value will take effect following a reset.  If the tnc source call

sign is left blank, the tnc will not activate the transmitter PTT

line.

Commands and information are sent to the tnc in the form  of

lines.  Lines may be up to 256  characters  long,  including  the

terminating CARRIAGE RETURN.  If the 256th character  entered  is

not a CARRIAGE RETURN, it will be discarded and a BELL  character

will be output to the terminal.  BACKSPACE and DELETE may be used

to remove single characters from the line.  The entire  line  may

be permanently backspaced out by entering a CONTROL-U or CONTROL-

X.  A CONTROL-R will temporarily backspace out any  partial  line

to allow incoming frames to be  displayed.   A  second  CONTROL-R

will then restore  the  line  to  allow  continuation  of  entry.

During the time a partial line is saved, only  another  CONTROL-R

will be  accepted  from  the  keyboard  (with  the  exception  of

xon/xoff, of course).  BELL characters are echoed to the terminal

when entered or  removed.   Lines  which  begin  with  an  ESCAPE

character (echoed as '* ') are interpreted  as  commands.   If  a

command is issued with no parameter, the current  value  of  that

commands parameter is displayed.  An embedded NULL character in a

command will cause the remainder  of  the  line  to  be  ignored.

Lines without a leading ESCAPE character are sent as information.

The firmware provides the operator  with  five  virtual  tnc

channels, numbered 0 to 4.  The terminal is logically attached to

only one of these  channels  at  a  time,  selected  by  the  'S'

command.  Information sent on channel 0 is always  unproto.   The

unproto path may be set by issuing a 'C' command when  channel  0

is selected.  Channels 1 - 4 are also unproto  if  they  are  not

currently connected.  Outgoing connect requests may be issued  on

any unconnected channel, while incoming connect requests will use

the first available  channel  (provided  the  maximum  number  of

connections set  by  the  'Y'  command  will  not  be  exceeded).

Information received on a connected channel that is not currently

selected will remain queued there until that channel is  selected

(the 'L' command is useful in determining if there is information

waiting on other channels).  Information for transmission is sent

only to the currently selected channel.   When  a  connection  is

ended,  any  information  that  has  not  been   transmitted   or

acknowledged will be discarded.  Received information will remain

queued,  however,  until  it  has  been  displayed.   If  a   new

digipeater  path  is  desired  while  a   connection   is   being

established or is in progress, it is not necessary to  disconnect

first.  Simply re-issuing the 'C' command will  re-establish  the

connection via the new path  without  any  loss  of  information.

Attempting to connect to  the  same  station  on  more  than  one

channel is not permitted.

Which protocol version is used to initiate a  connection  is

controlled by the 'V' command, but the version  will  be  changed

automatically, if necessary, to conform to the version of the tnc

responding.  Version 2 protocol is more  effecient  in  terms  of

network  throughput  and   loading,   especially   under   severe

conditions.  Version 2 protocol is the default and should be used

whenever possible.  (NOTE:  The  existing  firmware  supplied  by

TAPR for the TNC-1 (v3.x) will NOT digipeat version  2  protocol.

This is expected to  remedied  in  v4.0.   All  other  tnc's  and

digipeaters encountered to date  digipeat  version  2  properly.)

When version 2 protocol is used, a  watch-dog  timer  is  started

whenever information  is  not  being  transmitted.   If  the  tnc

remains idle for three minutes, it will poll  the  other  tnc  to

determine if the link is still established.  If  no  response  is

received after the number of tries set by the  'N'  command,  the

tnc will attempt to re-establish the connection.  This  procedure

will also detect the case where someone connects and then  leaves

without disconnecting.  Changing the protocol  version  during  a

connection is not permitted.

The 'F', 'N', 'O',  and  'V'  commands  maintain  individual

parameters for each channel.  The value stored in  channel  0  is

the value that is  saved  in  NOVRAM.   This  value  is  used  to

initialize channels 1 - 4 after a  reset,  and  to  re-initialize

channels 1 - 4 after a disconnect.  This allows the values to  be

changed independently on each channel,  prior  to  and  during  a

connection, and then automatically revert back  to  the  standard

values when the connection is ended.  A 'D' command issued  on  a

disconnected channel 1 - 4 will also re-initialize that channel.

Frame monitoring is controlled  by  the  'M'  command.   The

command parameter determines the types of frames  monitored,  and

is a list of desired  frames  chosen  from  the  letters  in  the

following table:

LTR       FRAME

---       -----

N        None

I        I frames

U        UI frames

S        Supervisory frames

C        Monitor while connected

R        Monitor received frames (addressed to)

T        Monitor transmitted frames (addressed from)

+        Call signs to be included (maximum of 8)

-        Call signs to be excluded (maximum of 8)

The '+' and '-' parameters may not be used together.   If  either

is used, it must be the last parameter (followed by one to  eight

call signs,  if  applicable).   If  no  list  of  call  signs  is

specified to be included or excluded,  all  call  signs  will  be

candidates for monitoring.  Entering a '+' or '-'  with  no  call

signs will empty the list.

An asterisk displayed after a call sign in  the  digipeater  list

indicates the frame was transmitted by that station.  The control

field displayed will be one of the following:

NAME   DESCRIPTION

----   -----------

RRa  - Receive Ready

RNRa - Receive Not Ready

REJa - Reject

UI   - Unnumbered Information

DM   - Disconnected Mode

SABM - Connect Request

DISC - Disconnect Request

UA   - Unnumbered Acknowledge

FRMR - Frame Reject

Iab  - Information

?ccH - Unknown

a  = Next expected frame number (0 - 7)

b  = Frame number of this frame (0 - 7)

cc = Hexadecimal value

In addition, one of the following characters will  be  displayed,

reflecting the protocol version, command/response bits,  and  the

poll/final bit:

(blank) = version 1 frame without poll/final bit

! = version 1 frame with poll/final bit

^ = version 2 command frame without poll bit

+ = version 2 command frame with poll bit

- = version 2 response frame with final bit

v = version 2 response frame without final bit

The protocol identifier field is displayed in hexadecimal,  which

is currently  always  an  F0,  signifying  that  no  layer  3  is

implemented.

An unattended mode, controlled by the 'U' command,  provides

for sending user supplied text to a connecting station, and  then

allows that station to leave a  brief  message.   This  mode  can

operate on all channels simultaneously, but in no way limits  the

operators ability to interact with one of the connected  channels

or  the  ability  to  make  outgoing  connect   requests.    When

unattended mode is enabled, link status messages  are  queued  to

the associated channel and not output to the terminal unless that

channel  is  currently  selected.   Link  status  messages   will

therefore  be  displayed  in   chronological   order   with   the

information from that channel.  In addition, text supplied by the

user with the 'U' command  will  be  sent  to  any  station  that

connects.  If channel 0  is  left  selected,  stations  may  then

connect and leave messages on channels 1 - 4 (limited by the  'Y'

parameter, of course).  The 'L' command may be used to  determine

if messages have been left on any channel.  Selecting  a  channel

containing messages will cause all link  status  and  information

from that channel to be displayed.  If  xon/xoff  handshaking  is

enabled, CONTROL-S and CONTROL-Q may  be  used  to  regulate  the

output to  the  terminal  to  allow  comfortable  reading.   Also

provided,  but  intended  to  be  used  very  sparingly,  is   an

unattended QST facility.  In this mode, user supplied text may be

automatically transmitted unproto to the path set on  channel  0,

at periodic intervals spaced  1  -  65,535  minutes  apart.   The

interval and text must be entered each time QST mode is  enabled.

Use of unattended QST messages is generally degrading to  network

performance and is considered by many to  be  an  annoyance,  and

should only be used when absolutely necessary.

The tnc parallel port supplies link status  information  for

channel 1 on lines PA0, PA1, and PA2 for  use  with  an  external

status indicator.  The following link states are possible:

1 = Disconnected

2 = Link Setup

3 = Frame Reject

4 = Disconnect Request

5 = Information Transfer

The firmware source is written in  6809  assembly  language.

Emphasis was  placed  on  using  a  state  table  driven  design,

utilizing the state tables provided in  the  ARRL  specification.

The source was assembled using a 6809 cross assembler  from  2500

AD Software running on a Z-80/8086  TurboDOS  development  system

(CP/M & MS-DOS compatable).

NOTE:   A  very  small  number  of  Western  Digital  1933   HDLC

Controller ICs manufactured in 1981 and 1982 have been  found  to

be defective.  The symptoms are erratic and unreliable  operation

of the tnc, due to the hdlc controller  occasionally  failing  to

interrupt  upon  successful  completion  of  frame  transmission.

These defective parts can be identified by  inspecting  the  mask

number on the underneath side of  the  IC.   The  defective  mask

number begins with the letters 'BA', and should be replaced.  The

current replacement part is called 1935 and requires no changes.

COMMAND SUMMARY

===============

COMMAND        PARAMETER               DESCRIPTION

-------        ---------               -----------

A (1)            0             Auto linefeed disabled

1             Auto linefeed enabled

B (6)           1-15           Terminal baud rate

C        Cs1 [Cs2 ... Cs9]     Connect path (0=unproto path)

D                              Disconnect

E (1)            0             Echo input disabled

1             Echo input enabled

* F (3)           1-15           Frame acknowledge (seconds)

G               [0]            Get information (host mode)

[1]            Get link status (host mode)

H (1200)       50-9600         HDLC baud rate

I                Cs            Tnc source call sign

JHOST (0)        0             Terminal mode enabled

1             Host mode enabled

K               1-2            Calibrate

L              [0-4]           Display channel status

M (IU)       NIUSCRT+-         Monitor mode

* N (10)          0-255          Number of tries (0=forever)

* O (4)           1-7            Number of outstanding I frames

PERM                           Save parameters to NOVRAM

QRES                           Re-start firmware

R (1)            0             Repeater disabled

1             Repeater enabled

S (1)           0-4            Select channel (0=unproto)

T (30)          0-127          Transmitter delay (10ms)

U (0)   0      [text]          Unattended mode disabled

1      [text]          Unattended mode enabled

2    interval  text    QST mode enabled

* V (2)            1             Version 1 protocol initiated

2             Version 2 protocol initiated

W (16)          0-127          Repeater wait (10ms)

X (1)            0             Transmitter PTT disabled

1             Transmitter PTT enabled

Y (1)           0-4            Maximum connections

Z (3)            0             Flow disabled, xon/off disabled

1             Flow enabled, xon/off disabled

2             Flow disabled, xon/off enabled

3             Flow enabled, xon/off enabled

@   B                          Display number of free buffers

C  (0)       0             Fast clock disabled

1             Fast clock enabled

D  (0)       0             Full duplex disabled

1             Full duplex enabled

P  (F0)     0-FF           Protocol id (hex)

S                          Display current link state

T2 (100)    0-255          Timer T2 interval (10ms)

T3 (18000)  0-32767        Timer T3 interval (10ms)

Default values are shown in parenthesis

*  These commands are applicable to each connection channel

(Values set on channel 0 are used after a reset and

disconnect to initialize each connection channel)

COMMAND DESCRIPTION

===================

The 'A' command is used to enable or disable  the  automatic

insertion of LINEFEED characters after CARRIAGE RETURN characters

to the terminal.  This parameter is stored in NOVRAM.

The 'B' command is used to set the terminal baud rate, using

one of the following parameters:

Parameter     Baud rate

---------     ---------

1            50

2            75

3            110

4            135

5            150

6            300

7            600

8            1200

9            1800

10            2400

11            3600

12            4800

13            7200

14            9600

15            19200

After setting the baud rate with the 'B' command, the value  must

be PERMed, and will then take effect  following  a  reset.   Baud

rates above  4800  may  not  be  reliable  with  sustained  input

(commands or information) from a host computer due  to  the  slow

cpu clock rate and non-vectored interrupt architecture.

The 'C' command is used to initiate a link connection.  Note

that NO 'v' or 'via' is used between the  destination  call  sign

and the digipeater call signs.  A 'C' command may be issued on  a

channel already in use to change the digipeater call  signs,  but

not the destination call sign.  A 'C' command issued when channel

0 is selected sets the unproto path.  Attempting  to  connect  to

the same station on more than one channel is not permitted.

The 'D' command is used to initiate a link disconnection.  A

'D' command issued during the establishment or  dis-establishment

of a link will cause an  immediate  return  to  the  disconnected

state.  A 'D' command issued on a disconnected channel  will  re-

initialize the connection  dependent  parameters  to  the  values

stored in channel 0.

The 'E' command is used to enable or disable the echoing  of

input (commands and information) to the terminal.  This parameter

is stored in NOVRAM.

The 'F' command is used to  set  the  frame  acknowledgement

interval.  This interval is used to compute the timeout  interval

before a packet is retransmitted, using the formula:

time (seconds) = frame ack * (2 * number of digipeaters + 1)

A separate frame acknowlegement interval value is maintained  for

each connection channel.  The value stored in channel 0  is  used

to  initialize  each  connection  channel  after   a   reset   or

disconnection, and is the value that is stored in NOVRAM.

The 'G' command is used to interrogate virtual tnc  channels

when host mode is enabled.  If no  parameter  is  specified,  the

next chronological item (information  or  link  status)  will  be

returned, provided there is one.   This  command  is  invalid  in

terminal  mode.   A  later  section  is  devoted  to  host   mode

operation.

The 'H' command is used to set the  HDLC  baud  rate.   Non-

standard values are rounded down to the next possible baud  rate.

In this case, use an 'H' command with no paramter to display  the

value actually set.  This parameter is stored  in  NOVRAM.   HDLC

rates up to 9600 baud are supported if the  fast  clock  mode  is

enabled.

The 'I' command is used to set the  tnc  source  call  sign.

The initial value is all blanks.  After the source call sign  has

been entered, the 'PERM' command should be used to  save  it  for

use during subsequent operation.  Changing the  tnc  source  call

sign while connected is not permitted.  If the  tnc  source  call

sign is left blank, the tnc will not activate the transmitter PTT

line.

The 'JHOST' command is used to select between  terminal  and

host modes.  A later section is devoted to host mode operation.

The 'K' command is used to calibrate  the  tnc  modem.   The

following parameters set the calibration modes listed:

Parameter         Calibration

---------         -----------

1          FREQUENCY COUNTER

2          TONE LEVEL & NULL

The frequency counter mode will display the measured  frequencies

on the terminal.  When each mode is  selected,  jumpers  settings

are displayed as well as the appropriate ajustment  points.   The

values shown in parenthesis are the normal targets for 1200  baud

operation.

The 'L' command is used to display the link status of one or

all channels.   Information  displayed  includes  the  connection

path, number of receive frames not yet displayed, number of  send

frames not yet transmitted, number of transmitted frames not  yet

acknowledged, and the  current  retry  count.   A  '+'  character

preceeding the channel number indicates  the  currently  selected

channel.  Operation of this command when host mode is enabled  is

somewhat different, and is described in a later section.

The 'M' command is used to set the  frame  monitoring  mode.

The command parameter determines the types of  frames  monitored,

and is a list of desired frames chosen from the  letters  in  the

following table:

LTR       FRAME

---       -----

N        None

I        I frames

U        UI frames

S        Supervisory frames

C        Monitor while connected

R        Monitor received frames (addressed to)

T        Monitor transmitted frames (addressed from)

+        Call signs to be included (maximum of 8)

-        Call signs to be excluded (maximum of 8)

The '+' and '-' parameters may not be used together.   If  either

is used, it must be the last parameter (followed by one to  eight

call signs,  if  applicable).   If  no  list  of  call  signs  is

specified to be included or excluded,  all  call  signs  will  be

candidates for monitoring.  Entering a '+' or '-'  with  no  call

signs will empty the list.  This parameter is stored in NOVRAM.

The 'N' command is used to set the maximum number of times a

frame  will  be  transmitted  without  receiving  an  appropriate

acknowledgement, before a link failure is  assumed.   A  separate

maximum number of tries value is maintained for  each  connection

channel.  The value stored in channel 0  is  used  to  initialize

each connection channel after a reset or  disconnection,  and  is

the value that is stored in NOVRAM.

The 'O' command  is  used  to  set  the  maximum  number  of

unacknowledged I frames that may be outstanding at any one  time.

A separate maximum number of unacknowledged  I  frames  value  is

maintained for each connection  channel.   The  value  stored  in

channel 0 is used to initialize each connection channel  after  a

reset or disconnection, and  is  the  value  that  is  stored  in

NOVRAM.

The 'PERM' command is used to change  the  parameter  values

stored in NOVRAM to those that are currently set.

The 'QRES' command is used to restart the firmware as  if  a

hardware reset had been issued.

The 'R' command is used to enable or disable the digipeating

of frames.  This parameter is stored in NOVRAM.

The 'S' command  is  used  to  select  the  current  channel

number.  This parameter is stored in NOVRAM.

The 'T' command is used to set the transmitter  keyup  delay

interval.  The parameter is specified in 10ms  increments.   This

parameter is stored in NOVRAM.

The 'U' command is used  to  enable  or  disable  unattended

modes.  If QST mode is not used, previously entered text will  be

retained if new text is not provided.  If QST mode  is  selected,

the interval and text must be  supplied.   The  QST  interval  is

specified in minutes from 1 - 65,535.

The 'V' command is used to select whether  version  1  or  2

protocol will be used to initiate a link connection.  A  separate

protocol version value is maintained for each connection channel.

The value  stored  in  channel  0  is  used  to  initialize  each

connection channel after a reset or  disconnection,  and  is  the

value that is stored in  NOVRAM.   Interrogating  this  parameter

during a connection will reflect the protocol  version  currently

being used on that channel.  Changing the protocol version during

a connection is not permitted.

The 'W' command is used to set the digipeater wait interval.

The parameter is specified in 10ms increments.  This parameter is

stored in NOVRAM.

The 'X' command is used to enable or disable the transmitter

PTT line.  This parameter is stored in NOVRAM.

The 'Y' command  is  used  to  set  the  maximum  number  of

connections that may  established  by  incoming  requests.   This

command has no  effect  on  the  operators  ability  to  initiate

outgoing  connection  requests.   This  parameter  is  stored  in

NOVRAM.

The 'Z' command is used to enable or  disable  flow  control

and xon/xoff handshaking to the terminal.   If  flow  control  is

enabled, output to the terminal will be inhibited while  entering

commands or information.  If flow control is disabled, output  to

the terminal will not be restricted.  Flow control  and  xon/xoff

handshaking should be disabled during periods in which the tnc is

operated without a terminal, to  avoid  suspending  output  which

will consume buffers.  If xon/xoff handshaking  is  enabled,  crt

scrolling may be stopped and started using CONTROL-S and CONTROL-

Q characters.  Flow control  and  xon/xoff  handshaking  are  not

performed when host mode is enabled.  This parameter is stored in

NOVRAM.

The '@'  command  is  a  software  maintenance  command.   A

parameter of 'B' will display the  number  of  free  buffers.   A

parameter of 'C0' will select timing parameters  for  the  slower

cpu  clock  rate.   A  parameter  of  'C1'  will  select   timing

parameters for the faster cpu clock rate.  A  parameter  of  'D0'

will select simplex operation for the HDLC port.  A parameter  of

'D1' will select full duplex operation for the HDLC  port.   Both

the 'C' and 'D' parameters are  stored  in  NOVRAM  and  must  be

PERMed if the change is to stay in effect following a reset.  The

'P' parameter is used to  set  the  protocol  id  byte  for  each

channel  and  is  supplied  and  displayed  in  hexadecimal.    A

parameter of 'S' will display the current link state.   The  'T2'

parameter is used to set the timer T2 interval, just as the  'T3'

parameter is used to  set  the  timer  T3  interval.   The  timer

intervals are specified in 10ms increments, and both  are  stored

in NOVRAM.  Timer T2 controls the amount  of  delay  between  the

time an information frame is received and the time the  resulting

response frame is sent.  This delay allows multiple frames to  be

acknowledged with a single response.  Timer T3 is  used  maintain

link integrity.  If there is no activity during the T3  interval,

the tnc  will  poll  to  verify  the  distant  station  is  still

connected.

HOST MODE OPERATION

===================

Host mode is intended to provide a user  interface  suitable

for operation under control of a host  processor.   Commands  and

information to the tnc, as well as status  and  information  from

the tnc, are clearly identified to allow orderly and  unambiguous

communication.  To alleviate any need for  hardware  or  software

handshaking,  the  tnc  will  not  send  to  the  host  processor

unsolicited,  and  all  exchanges  are  limited  to  256   bytes.

Information transfers are fully transparent.

When host mode is enabled, the first byte sent  to  the  tnc

must be a channel number.  If  information  is  being  sent,  the

second byte must be a binary 0.  If a command is being sent,  the

second byte must be a binary 1.   The  third  byte  must  be  the

binary length of the actual information or  command,  decremented

by 1 (vacuous information or commands are  not  permitted).   The

actual  information  or   command   bytes   must   follow   last.

Information sent to channel 0 will be sent unproto.   Information

sent to an unconnected channel 1 - 4 will be discarded.  The  tnc

will respond to both information  and  commands  with  a  channel

number first, followed by a binary code of 0, 1, or 2, signalling

success or failure.  Codes of 1 or 2 will be followed by  a  null

terminated message.  Channels may be  interrogated  for  incoming

information or link status by using  the  'G'  command.   Monitor

headers and monitor information will always be sent to channel 0,

along with connect request link status messages.  All other  link

status messages will be sent to the  appropriate  channel,  along

with that channels connected information.  In response to  a  'G'

command, the tnc  will  respond  with  a  channel  number  first,

followed by a binary code of 0 if  nothing  is  available,  or  a

binary code of 3 - 7, identifying the bytes that follow.  A  code

of  4  indicates  the  monitored  frame  does  not   contain   an

information field.  A code of 5  indicates  the  monitored  frame

does contain an information field, and the next  'G'  command  on

channel 0 will return that information field, preceeded by a code

of 6.

Host to Tnc

-----------

CHANNEL   CODE           DESCRIPTION

-------   ----           -----------

n       0        Information (preceeded by length-1)

n       1        Command     (preceeded by length-1)

Tnc to Host

-----------

CHANNEL   CODE           DESCRIPTION

-------   ----           -----------

n       0        Success (nothing follows)

n       1        Success (message follows, null terminated)

n       2        Failure (message follows, null terminated)

n       3        Link Status (null terminated)

n       4        Monitor Header (null terminated)

n       5        Monitor Header (null terminated)

n       6        Monitor Information (preceeded by length-1)

n       7        Connect Information (preceeded by length-1)

Success messages

----------------

{channel status}

{parameter value}

CHANNEL NOT CONNECTED

Failure messages

----------------

INVALID COMMAND

TNC BUSY - LINE IGNORED

CHANNEL ALREADY CONNECTED

STATION ALREADY CONNECTED

Link Status messages

--------------------

BUSY fm {call} via {digipeaters}

CONNECTED to {call} via {digipeaters}

LINK RESET fm {call} via {digipeaters}

LINK RESET to {call} via {digipeaters}

DISCONNECTED fm {call} via {digipeaters}

LINK FAILURE with {call} via {digipeaters}

CONNECT REQUEST fm {call} via {digipeaters}

FRAME REJECT (x y z) fm {call} via {digipeaters}

FRAME REJECT (x y z) to {call} via {digipeaters}

x y z = FRMR information bytes

Monitor Header format

---------------------

fm {call} to {call} via {digipeaters} ctl {name} pid {hex}

Channel Status format

---------------------

a b c d e f

a = Number of link status messages not yet displayed

b = Number of receive frames not yet displayed

c = Number of send frames not yet transmitted

d = Number of transmitted frames not yet acknowledged

e = Number of tries on current operation

f = Link state

Possible link states are:

0 = Disconnected

1 = Link Setup

2 = Frame Reject

3 = Disconnect Request

4 = Information Transfer

5 = Reject Frame Sent

6 = Waiting Acknowledgement

7 = Device Busy

8 = Remote Device Busy

9 = Both Devices Busy

10 = Waiting Acknowledgement and Device Busy

11 = Waiting Acknowledgement and Remote Busy

12 = Waiting Acknowledgement and Both Devices Busy

13 = Reject Frame Sent and Device Busy

14 = Reject Frame Sent and Remote Busy

15 = Reject Frame Sent and Both Devices Busy

NOTE 1:  Only items a and b are displayed for channel 0.

NOTE 2:  Only states 0 - 4 are possible if version 1 is in use.

DEFAULT PARAMETERS

==================

In some instances, it  may  be  desirable  to  have  default

parameters which differ from the standard values.  To allow  easy

access, all default parameters have been placed at the  beginning

of the EPROM.  The following listing defines the layout  of  this

area:

TYPE     VALUE          DESCRIPTION

----     -----          -----------

BYTE      1BH           COMMAND CHARACTER

BYTE    '      ',60H    SOURCE CALL SIGN (SEE NOTE 1)

WORD     000AH          HDLC BAUD RATE DIVISOR (SEE NOTE 2)

BYTE      06H           TERMINAL BAUD RATE

BYTE      01H           MAXIMUM CONNECTIONS

BYTE      03H           MONITOR MODE (SEE NOTE 3)

BYTE      01H           REPEATER DISABLE/ENABLE

BYTE      10H           REPEATER WAIT (10ms)

BYTE      1EH           TRANSMITTER DELAY (10ms)

BYTE      03H           FLOW CONTROL MODE

BYTE      01H           TRANSMITTER PTT DISABLE/ENABLE

BYTE      01H           CHANNEL NUMBER

BYTE      01H           AUTO LINEFEED DISABLE/ENABLE

BYTE      01H           ECHO COMMAND LINE DISABLE/ENABLE

BYTE      01H           VERSION 2 INITIATED DISABLE/ENABLE

BYTE      04H           MAXIMUM UNACKNOWLEDGED FRAMES

BYTE      0AH           MAXIMUM TRY COUNT

BYTE      03H           FRAME ACKNOWLEDGE INTERVAL

BYTE      00H           FAST CLOCK DISABLE/ENABLE

BYTE      00H           FULL DUPLEX DISABLE/ENABLE

BYTE      64H           TIMER T2 INTERVAL (10ms)

WORD     4650H          TIMER T3 INTERVAL (10ms)

BYTE      F0H           PROTOCOL ID

BYTE    'CQ    ',60H    UNPROTO CALL SIGN (SEE NOTE 1)

BYTE      01H           VALIDATE CALL SIGN ENABLE/DISABLE

BYTE      00H           8-BIT CHARACTER ENABLE/DISABLE

BYTE      00H           STOP BIT/WORD LENGTH (SEE NOTE 4)

BYTE      00H           PARITY (SEE NOTE 5)

DISABLE = 00H / ENABLE = 01H

NOTE 1:  The secondary station id must be shifted  left  one  bit

and or'ed with 60H.

NOTE 2:  The  HDLC  baud  rate  divisor  is  computed  using  the

formula:

SLOW CLOCK:     divisor = (14400 / buad rate) - 2

FAST CLOCK:     divisor = (28800 / buad rate) - 2

The HDLC baud rate divisor is stored most significant byte first.

NOTE 3:  The monitor mode is composed from the following bits:

BIT       FRAME

---       -----

0        I frames

1        UI frames

2        Supervisory frames

3        Monitor while connected

4        Monitor received frames (addressed to)

5        Monitor transmitted frames (addressed from)

NOTE 4:  Stop bit and word length is controlled by the  following

bit assignments:

BIT 7     STOP BIT

-----     --------

0       1 stop bit

1       2 stop bits

1       1.5 stop bits (word length=5/parity disabled)

1       1 stop bit (word length=8/parity enabled)

BIT 6-5   WORD LENGTH

-------   -----------

0 0     8 bits

0 1     7 bits

1 0     6 bits

1 1     5 bits

Bits 4-0 MUST be set to 0.

NOTE 5:  Parity is controlled by the following bit assignments:

BIT 7-6   PARITY CONTROL

-------   --------------

0 0     Odd parity transmitted/received

0 1     Even parity transmitted/received

1 0     Mark parity transmitted/parity check disabled

1 1     Space parity transmitted/parity check disabled

BIT 5     PARITY ENABLE

-----     -------------

0       Parity disabled

1       Parity enabled

Bits 4-0 MUST be set to 0.

######################################################