Batch-YMODEM for HP48 S/SX/G/GX

Version 2.1

(C)1996  Laurent Bernard

Sorry: I'm a french student, english is not my native language, and so

my english is not very good.  But I hope that you'll understand the simple

words and sentences used.

1 INTRODUCTION

2 INFO ABOUT YMODEM LIB

3 GETTING STARTED

4 USING THE HP48 YMODEM LIBRARY

5 USING Y.EXE ON MS-DOS COMPUTERS

6 YMODEM PACKET #0 FORMAT

7 REGISTRATION PROCEDURE

8 DISCLAIMER

9 LIST OF FILES

1 INTRODUCTION

--------------

The story of YMODEM started a few years ago, when I needed to backup my

128k RAM card regularly because of some programming.  Using Kermit was not

acceptable, because it spent really too much time, even though the

transfers were reliable.

After having written a 3D vector demo that was beautiful, but was not

very useful, I wanted to write a program that would be worth using.  At

that period, I had a HP48sx, and I bought a magazine in which they

described the YMODEM Batch file transfer protocol.  I then poked in the

internals of the HP48, and wrote the lib LMODEM, that provided fast

transfers, CRC protected, and was very similar to YMODEM, except for the

coding of the first packet.

This lib just worked fine, I never had a problem with it, and some

friends used LMODEM and did not report any bug to me.

And then , I gained access to the Internet through my school, and the

first thing that I did was to dowload XYMODEM.ZIP, the file containing

the protocol description ( the article I had was not very precise and took

liberties with the original protocol ).  And I wrote what you can use now,

that is YMODEM Batch file transfer protocol for HP48 S/SX/G/GX.

It is fast ( about 1k per second ), quite secure ( using CRC16 to detect

errors ), and it even works with the infrared port, between two HP48.

I spent some time on this product, and so I'd like to have some

feedback.  So I decided to make YMODEM.LIB a shareware, with a modest

contribution ( FF 50 ).  For registration, see the corresponding

section.

2 INFO ABOUT YMODEM LIB

-----------------------

Lib number: 996  ( I hope this doesn't conflict with others...)

Lib version: 2.1

BYTES Output:  Size 5211 bytes

CRC  #F5ABh

3 GETTING STARTED

-----------------

First, upload YMODEM.LIB to your HP48 using Kermit ( if you don't know

how to do, there should be a faq, or just read the manual ).

Then put the lib in any port you like.( even ports 2 or higher on GX!)

Set your IO parameters to:

Speed 9600 bauds

8 data bits

none parity ( 0 )

Connect the HP48 and the PC compatible  with your serial cable.

On the PC, echo

Y S FileName

where FileName is the name of the file you want to send.

( Y.EXE is the name of the program that handles YMODEM transfers)

On the HP48, type in

YRECV

The transfer should then initiate, and finish.

For uploading from the HP48,

on the HP48, put in the stack the name of the var you want to send,

then type

YSEND

on the PC, type

Y R

Those are the most common uses of the YMODEM lib and the Y.EXE program.

You can use the same syntax for transferring between two HPs...

( select the IR port, then

HP #1: VARS YSEND

HP #2: YRECV )

4 USING THE HP48 YMODEM LIBRARY

-------------------------------

Here are the commands available in the YMODEM library.

RC2

This program does the same thing as RCL, and takes the same arguments.

But it has a big advantage: it can RCL in ROM or RAM Cards, so you don't

have to change your card switch when sending objects that are in

protected ports.  This is quite simple to achieve, and I suggest you to

take a look at the code to see how simply it is done.

EXTR

Sometimes when you use YMODEM, you will get strings beginning with the

"HPHP48-?" header instead of the object you expected to receive ( I

explain the reasons of this effect in YCONF item ).  When you want to get

the object, just run EXTR on the string, and, if the object is not too

corrupted, you should readily see it on the first level of the stack.

( cf ASCLOAD & others )

YSEND

This command allows you to send files using the YMODEM ftp.  Basically,

the YMODEM protocol consists of 128 or 1024 bytes packets, transmitted

with CRC or checksum, and the Batch informations are in packet #0 ( the

filename and the size in bytes ).

syntax

There are four possible syntaxes:

Sending a variable ( either global or local ):

Put in stack level 1 the global/local name then execute YSEND.

Sending a variable stored in a port:

Put in stack level 1 the tagged name of the object, then YSEND.

If the object is a backup, then YSEND will send it using its name.

If the object is a library, then there are two possible cases:  if the

author respected HP specifications, i.e. seventh character of the title

string is ":" , then YSEND will send the lib under the name

"Title(1-6).LIB" , where Title(1-6) are the first six chars in the title

of the lib.  Otherwise, then the name will be "LxxID.LIB", where xxID is

the number of the lib, as a 4 characters decimal value.

Notes:  You can even send objects that are in a RAM-Protected or ROM

card.  The error you can sometime encounter on HP48GX occurs when RCLing

a var that is in port 2-up to 31, when there is not enough memory to

copy it to the ram.  In this case, you'll get an "Insufficient Memory"

error.

You can't send nullnamed objects ( the YMODEM protocol is

confused because null names are used to indicate the end of a batch ).

If you try to do so, you'll have an error ("Null name not allowed").

Sending an arbitrary object under an arbitrary name

Put in level two the object you want to send, and in level the name

under which it should be sent, as a string, then run YSEND.

Batch operations

Put a list in level 1, then run YSEND.

The list should be made of either local names, global names, or tagged

objects.

EXAMPLES

This section illustrates common uses of YSEND:

Backing up RAM

Simply execute << HOME PVARS YSEND >>

Backing up a port

<< NbPort PVARS DROP YSEND >>

Using arbitrary names/objects

<< 1  "REAL1"  YSEND >>

PARAMETERS

There are several parameters to select the behaviour of the YSEND

program.  See YCONF for more details.

YRECV

Just run YRECV, and you should readily see the var appended to your

current directory.

YRECV uses the standard IO params, plus additional settings described

in YCONF.

YCONF

This program was designed to allow easy modification of the YMODEM

parameters.  Running YCONF, you should see a menu and the current

settings.  Just press the appropriate key to switch the setting.

Port

Select IR/Wire Port

Speed

Choose between 1200,2400,4800, and 9600 bps.  Just press the SPEED

menu item to walk through the available speeds.

Note:  When using IR transfers, this setting is not taken into

account, and the program automatically selects the highest possible

speed for IR transfers.

YSEND packet size

YMODEM protocol allows sender to send 128 or 1024 bytes packets.

By default, 1024 bytes packets are choosen, but you can switch to

force 128 bytes packets.  You can also execute 33 SF to use 128

bytes packets.  This changes only the behaviour of YSEND.( YRECV

automatically adapts to 128 or 1024 bytes packets. )

YRECV replaces

This is the same option as the one used in kermit ( system flag

-36 ).  I tried to use the same format as the one used by HP for

renaming variables, except that I start using value 0 ( for example,

you can get a 'NAME.0' name) instead of 1.

YRECV prefers

YMODEM transfers can check the integrity of packets using two

different methods: checksum or CRC.  I recommend to use the CRC

option ( that is the default value ), but an old YMODEM sender

could still use Checksum checking, so I put it for compatibility.

If you know that the sender will use checksum, then select

checksum.  Otherwise, if you don't, then select CRC, and YRECV will

try to establish the link using CRC, and if it fails after 5 trials,

will step down to checksum to establish the link.  When user flag 34

is set, checksum will be preferred.

YRECV always Ob->

Some objects that get damaged when stored in the hard disk won't

be well transformed on your HP.  But sometimes there are only a few

bytes added to it ( for example, garbage added at the end of the file

by XMODEM...). By default,  when there is a problem of this kind with

the size of the file, YRECV will put a string (beginning with the

"HPHP48-?" header) in the variable.  When user flag 35 is set, then

YRECV will always try to extract the obj, regardless of the size of

the file transferred.

ABOUT996

Displays information on the lib, and reminds the syntax and arguments

of the progs of YMODEM.LIB.

Important:  You can abort YSEND/YRECV at any time by pressing the ON key.

At the end of transfers, wether successful or not, the IO port is

closed.

5 USING Y.EXE ON MS-DOS COMPUTERS

---------------------------------

Y.EXE is an example of a program implementing the YMODEM ftp on

computers running MS-DOS.

Y.EXE was written using Turbo Pascal 7.0, and I leave Y.EXE in the

public domain, with its sources.

In fact, I am furnishing the source to show you how to interface with

YMODEM.LIB.  I hope you will be able to write compatible software for

your hardware.

Y.EXE is a command line oriented program, but the default parameters

are compatible with the default parameters of the HP48, so you should

not have to do any configuration (except maybe the number of the port).

To send a file to the HP48, just type

Y  S  parameters  filespec

To receive a file, use

Y  R

Here are some explanations:

parameters:(xxx is a numeric value)

-sxxx:          choose the speed. xxx may be 110,150,300,600,1200,2400,

4800,9600; by default,speed is 9600 bps.

-dxxx:          data bits.  xxx may be 7 or 8 (default 8)

-txxx           stop bits.  xxx may be 1 or 2 (default 1)

-pxxx           parity.  xxx may be 0 (none), 1 (odd), 2 (even)

(default none)

-#xxx           select the COM port.   xxx may be 0 or 1.

-r              when receiving, using the -r switch allows Y.EXE to

overwrite existing file (by default, Y.EXE renames

the file).

filespec: are just the normal MSDOS file specifications, with ?,* and so on...

To remind you of the main switches, just type

Y H or Y

Important:  You can abort Y.EXE at any time by pressing the ESC key.

I tried to follow as much as possible the YMODEM protocol (as described

in Chuck Forsberg's XYMODEM.ZIP description).  However, I'm not perfect, and

there are maybe some bugs, or some misinterpretations of the protocol.

The YMODEM lib and the Y.EXE are equivalent: if there is a protocol

error in Y.EXE, then the same error is in the lib.  That's is one of the

reasons why I'm furnishing the source for Y.EXE:  if you find an error in it,

then please email me so I can fix it as soon as possible, both in Y.EXE and

in YMODEM.LIB.

Thanks in advance.

( By the way, to compile Y.EXE, if you have Turbo Pascal installed, use

make -f y.mak

)

6 YMODEM PACKET #0 FORMAT

-------------------------

I'll descrive here the format used for the first packet, because it is

usually the cause of many incompatibilities between ftp programs.

I use only the name and size fields, and no pathname can be given in

the name.  On the HP48, all the sensible information should be in the

first 128 bytes, otherwise you'll get an error.

Here is a figure:

Byte #0:        First char of name

Byte #1:        Second char of name

...

Byte #n:        Last char of name

Byte #(n+1):    should be a null char

Byte #(n+2):    First char of size

...

Byte #m:        Last char of size

Byte #(m+1):    Either a null char or a space

...

The following bytes are not taken into account.

Note:  the size is converted in ascii, and sent as a sequence of characters.

For an implementation example, see the file UYMODEM.PAS.

7 REGISTRATION PROCEDURE

------------------------

I decided to distribute YMODEM.LIB as a shareware.  ( Note that Y.EXE

and the associated files are in the public domain, you can do anything

you want with them, as long as you do give credit where credit is due...)

This is not in my habits to limit the library functions:  the library

coming with this package is fully operational.

But, if you use it regularly, then you have to register.  I fixed the

registration fee to  FF 50 , I think that most people can afford it.

Remember that I'm a student, and that I spent precious time writing

this software, so I'd like you to realize that I did my best to write a

good transfer protocol, and I think it is a program worth using, so if

you like YMODEM.LIB, then go ahead and register.  With users' support,

I can improve the lib, correct bugs and maybe write a ZMODEM.LIB.

So how can you register ?

Just send a FF 50 (check/cash) or USD 10 (cash) to me

Laurent BERNARD-GUTH

37, rue de Lille

75007 PARIS

FRANCE

You can reach me using E-mail (comments, bugs, suggestions):

laurent.bernard@iota-eso.u-psud.fr

(valid up to November 1997)

What about me ?

I'm currently studying in a top level french engineering school

highly specialized in optics (it depends on the Institut d'Optique, in Orsay).

I've been programming since my early years, and I'm interested in neural

networks, communication (with optics fibers), optical computers, image

synthesis (I'm currently implementing a radiosity method) and processing.

By the way, coming to the end of my scholarship, I have to find

an internship, and I'd like to do it in a foreign country, so the question is:

would there be someone out there who could propose me such an internship ?

The subject of the internship should have a link with optics (otherwise

my professors won't accept...), the duration should be >= 3 months, and it

could start in May 1997.

If you've got something for me, then contact me please.

I think that I have to end this request with something like:

'' I look forward to hearing from you... ''

8 DISCLAIMER

------------

These routines are provided "as is" with no expressed or implied

warrantees. Any damage or losses caused by the use of these programs

is solely the responsibility of the user.

(standard disclaimer...)

About the HP48 library, I used ONLY SUPPORTED ENTRY POINTS (except in RC2),

so the program should run well on all versions ( I tested it on Rev P and

Rev C).  However, I can't give you any guarantee of any kind, so if you have

essential data in your HP, then use the built-in protocols.  For the others,

back up, just in case...

You can distribute the HPYMODEM package as long as you don't alter any file,

and that you don't take a charge for distributing the HPYMODEM package

(except for shipping/handling ).

9 LIST OF FILES

---------------

YMODEM.LIB ( 5211 bytes ): The HP48 S/G library

Y.EXE ( 22256 bytes ) : The MSDOS main program

HPYMODEM.DOC ( 16379 bytes ) : The file you're reading now

The sources of Y.EXE:

COMPDIR.PAS  (   276 bytes )

Y.PAS        (  8634 bytes )

UYMODEM.PAS  ( 23525 bytes )

RS232.PAS    (  2288 bytes )

RS232.ASM    (  7050 bytes )

TPZCRC.PAS   ( 10725 bytes, written by J.R. Louvau )

UEZCOMM.PAS  (  3253 bytes )

UJUMP.PAS    (  1710 bytes )