HPRASP 1.0

Robert Brunner

May 5, 1996

rbrunner@uiuc.edu

HPRASP is a model rocket altitude simulation program for the HP48

calculator.  It is based on the program RASP-93 from the Handbook of

Model Rocketry by Harry Stine, but it incorporates some ideas from

RASP version 3.20 available from the rec.models.rockets ftp site at

sunsite.unc.edu/pub/archives/rec.models.rockets/.

HPRASP includes the following features:

- Uses motor thrust curve format compatible with RASP 3.20 format

- Variable integration step size

- Trajectory output at selected time interval

- Pretty user interface (HP48G/GX only)

Unlike the sunsite program, variable air density is not used in this

program.  It also does not handle multiple stages.

The program is written in UserRPL.  It takes about a minute to do a 10

second simulation with a .02 second integration step on a GX.

To use, download the file HPRASP to the calculator.  'HPRASP' BYTES

should return checksum # 43979d, 6024.5 bytes.  After a run,

considerable data may be generated which will probably require 1-3

Kbytes more free memory.  To free this memory, purge the statistics

data variable with the CL\GS (CLear Sigma) command.

G/GX Users:

The main program to run is RASP.  This will produce an input form to

input rocket dimensions and simulation parameters.  After pressing OK,

the program will prompt for what motor to use.  Then the program will

start calculating, which will probably take at least a minute.  Upon

completion, some summary statistics are displayed.

S/SX Users:

The programs can be run on an S/SX, but the procedure is somewhat more

complicated.  I should probably write a program to simplify this, and

would be happy to include any contributed program.

1. Recall a motor description to the stack from the MOTORS directory.

2. Move back to the HPRASP directory

3. press OBJ\-> DROP

4. 'THRUSTCURVE' STO

5. 'M.FUEL' STO

6. 'M.MOTOR' STO

7. Store rocket mass w/o motor in M.EMPTY

8. Store rocket radius in R

9. Store launch rod length in L.ROD

10. Store drag coefficient in C.D

11. Store simulation duration in TMAX.

12. Store desired integration step size in DT

13. Store number of integration steps per output in IOUT.

14. Run RASP.CALC

15. View summary statistics with VIEWSUM

All Users:

Several variable will be updated to contain information about the last

simulation.  These variables will each contain a vector containing [

time(seconds), mass(kg), a(m/s^2), v(m/s), x (m) ].

TRAJ: Contains `\GSDAT', so pressing this button will return a matrix

containing the entire trajectory to the stack.

X.MAX: The time, mass, acceleration, etc. at apogee.

V.MAX: The time, mass, acceleration, etc. at maximum velocity.

A.MAX: The time, mass, acceleration, etc. at maximum acceleration.

BURNOUT: The time, mass, acceleration, etc. at motor burnout.

ROD: The time, mass, acceleration, etc. at the top of the launch rod.

The summary motor statistics can be recalled by pressing VIEWS (the

program VIEWSUM).

A plot of the trajectory can be created using the built-in plotter.

Press right-shift PLOT, choose a scatter plot, and select COLS 1 and 5

for altitude vs. time, 1 and 4 for velocity, etc.  Select autoscale,

if desired, and press OK.  Altitude comparisons are easily done by

plotting succesive simulations without erasing the plots in between.

Thrust Curves:

This version contains thrust curves for several small motors.  More

thrust curves can be entered from data obtained from the rasp.eng file

available at sunsite.unc.edu.  Each motor variable contains a list

with three items as follows:

{ motor-mass fuel-mass thrust-matrix}

The values motor-mass and fuel-mass are both in kilograms.  These

values are in the opposite order from how they appear in rasp.eng (for

no good reason, I must confess).  The thrust-matrix is an nx2 matrix

containing the thrust curve values from rasp.eng.  Column 1 is the

time in seconds, and column 2 is the thrust at that time.  Linear

interpolation is used to obtain thrust values between data points.

Conclusion:

I'd like to hear from you if you use this program.  I'd also

appreciate if you let me know if you have any problems or suggestions

about it.  Send email to rbrunner@uiuc.edu.