SPECIFICATION FOR MS-DOS KERMIT SYSTEM-DEPENDENT MODULES

by Jeff Damens, Columbia University

This document applies to Verson 2.28 of MS-DOS Kermit, and it has some

updates at the end for Version 2.29.  It has not been updated for

version 2.30.

All the system-independent global data structures used in Kermit-MS are defined

in the file MSDEFS.H.

The routine MSXxxx.ASM contains system-dependent support for system xxx, except

for terminal emulation, which is in MSYxxx.ASM, described below.

The  routines  in the MSX module may change any registers but the stack pointer

and segment registers, unless otherwise noted.  A routine that  returns  via  a

RET  instruction is said to return normally; a routine that skip returns is one

that returns to three bytes past the normal return address.

Global variables that must be defined in the system-dependent module:

XOFSNT          byte.  This should be set to a non-zero value if we  are  doing

flow  control  and  have  sent  an XOFF character to the remote

host, zero otherwise.

MACHNAM         byte.  A $-terminated string identifying the machine this  ver-

sion of Kermit is for; it is printed when Kermit starts up.

SETKTAB         byte.  A keyword table associating terminal key names to 16-bit

scan code values, used in the set key command.  If  the  kermit

version  can accept arbitrary decimal values as scan codes, the

word "SCAN" should appear in the table with a scan value of -1.

If  key  redefinition is not implemented, the first byte of the

table should be a zero.

SETKHLP         byte.  A $-terminated string to be printed when ? is  typed  in

the  SET KEY command.  This is usually simply a list of the key

names in  SETKTAB.    SETKHLP  must  be  defined  even  if  key

redefinition  is not implemented, to satisfy the linker; if key

redefinition is not implemented, SETKHLP  will  never  be  dis-

played.

COUNT           word.   The number of characters in the serial input buffer, if

known.  This is how Kermit knows to send an XON if  the  serial

handler  has  sent an XOFF.  If the number of characters in the

buffer isn't known, COUNT should be 0.

These are the required entry points for the system dependent  dependent  module

MSXxxx.ASM.

SERINI

Parameters      None.

Returns         Normally, no return value.

Description     Perform  any initialization that must be done before the serial

port can be used, including setting baud rate,  interrupt  vec-

tors, etc.  Parity and baud rate should be set according to the

values in  the  PORTINFO  structure.    The  external  variable

PORTVAL  points to the PORTINFO structure for the current port.

Calling SERINI more than once without an  intervening  call  to

SERRST should have no effect.

SERRST

Parameters      None.

Returns         Normally, no return value.

Description     Undoes  any  initialization done by SERINI, including resetting

the serial port, restoring any  interrupt  vectors  changed  by

SERINI,  etc.  Calling this more than once without an interven-

ing call to SERINI should be harmless.

CLRBUF

Parameters      None.

Returns         Normally, no return value.

Description     Remove and discard from the  serial  port's  input  buffer  any

characters  sent by the remote host that have not yet been read

by Kermit, and set COUNT to 0.  This  is  used  before  a  file

transfer  to flush NAK's that accumulate in the buffer when the

remote host is in server mode.

OUTCHR

Parameters      A character in AH.

Returns         Skip returns if the character  has  been  transmitted;  returns

normally  if  the character can not be transmitted because of a

hardware error.

Description     Sends the character in AH out  the  currently  selected  serial

port.    OUTCHR  can  assume  that SERINI will have been called

previously. OUTCHR should call the external  routine  DOPAR  to

set  the parity of the character if the communications hardware

doesn't automatically set  parity.    Flow  control  should  be

honored;  the external variable PORTVAL contains a pointer to a

PORTINFO structure (as defined in MSDEFS.H) containing the cur-

rent flow control definitions.

COMS

Parameters      None.

Returns         Normally  if  a parse error is encountered, skip returns other-

wise.

Description     Called by the SET PORT command.  On  a  machine  with  multiple

serial  ports,  COMS  should  parse for the name or number of a

serial port and make that the port used by succeeding calls  to

SERINI, PRTCHR, OUTCHR, and SERRST.  It should set the external

variable PORTVAL to point to one of the  external  port  struc-

tures  PORT1 or PORT2, and set COMFLG in the FLAGS structure to

1 for port one, 0 for port 2.   For  implementations  that  use

only  one  serial port, COMS should print a message to that ef-

fect and skip return.

VTS

Parameters      None.

Returns         Normally if a parse error is encountered, skip  returns  other-

wise.

Description     Parses  for an ON or OFF, sets HEATH-19 emulation while in ter-

minal emulation appropriately.  The VTFLG field  of  the  FLAGS

structure  should  be set non-zero if HEATH-29 emulation is on,

zero otherwise.  If HEATH-19 emulation is not done, VTS  should

print a message and skip return.

DODEL

Parameters      None.

Returns         Normally, no return value.

Description     Erases the character immediately to the left of the cursor from

the screen, then backs up the cursor.

CTLU

Parameters      None.

Returns         Normally, no return value.

Description     Move the cursor to the left margin, then clear the line.

CMBLNK

Parameters      None.

Returns         Normally, no return value.

Description     Clears the screen and homes the cursor.

LOCATE

Parameters      None.

Returns         Normally, no return value.

Description     Homes the cursor.

LCLINI

Parameters      None.

Returns         Normally, no return value.

Description     Performs any system-dependent initialization required  by  this

implementation.

PRTCHR

Parameters      None.

Returns         Normally,  with  the next character from the currently selected

serial port in AL.  Skip returns if no character is available.

Description     Reads the next character from the current serial port.   PRTCHR

can assume SERINI has been called previously, and should handle

flow control correctly.

DOBAUD

Parameters      None.

Returns         Normally, no return value.

Description     Sets the baud rate for the current port.  The baud rate  should

be  obtained  from  the  BAUD  field of the PORTINFO structure,

pointed to by the external variable PORTVAL.

CLEARL

Parameters      None.

Returns         Normally, no return value.

Description     Clears from the cursor to the end of the current line.

DODISK

Parameters      None.

Returns         Normally, no return value.

Description     Sets the external variable DRIVES to the number of disk  drives

attached to the machine.

GETBAUD

Parameters      None.

Returns         Normally, no return value.

Description     Store  current  baud rate of the currently selected port in the

BAUD field of the current PORTINFO structure, which is  pointed

to  by PORTVAL.  If the baud rate is to default to a particular

value, this routine can store that value into  the  BAUD  field

instead.

BEEP

Parameters      None.

Returns         Normally, no return value.

Description     Rings the terminal bell.

PUTHLP

Parameters      A pointer to a string in AX.

Returns         Normally, no return value.

Description     Writes  the null-terminated string given in AX to the terminal.

This is used to display help and status messages.  The IBM  and

Rainbow versions write the string in a reverse video box.

PUTMOD

Parameters      A pointer to a string in AX.

Returns         Normally, no return value.

Description     Writes  the null-terminated string given in AX to the last line

of the screen, in inverse video if possible.

CLRMOD

Parameters      None.

Returns         Normally, no return value.

Description     Clears the line written by PUTMOD.

POSCUR

Parameters      Row in DH, column in DL.

Returns         Normally, no return value.

Description     Positions the cursor to the row and column given in DX.    Rows

and columns both originate at 0 (not 1!).

SENDBR

Parameters      None.

Returns         Normally, no return value.

Description     Send a break to the current serial port.

SHOWKEY

Parameters      Pointer to a terminal argument block in AX (see TERM below).

Returns         Normally,  with  a  string  pointer in AX and the length of the

string in CX.

Description     Called by the SHOW KEY command.  Reads a key from the  terminal

and returns a string containing implementation-dependent infor-

mation about the key. In the usual case,  the  string  contains

the  key's (machine-dependent) scan code, and the key's defini-

tion (if any) from the terminal argument block.  The length  of

the  returned  string should be returned in CX.  The string may

contain any characters; unprintable characters will  be  quoted

when  the  string  is  printed.  If the implementation does not

support key redefinition, SHOWKEY may return  a  static  string

saying so.

TERM

Parameters      Pointer to terminal argument block in AX.

Returns         Normally, no return value.

Description     Do  terminal  emulation,  based  on  argument  block  described

below...

The following entry points were added for version 2.29 by  Joe  R.  Doupnik  of

Utah State University (May 1986):

DTRLOW

Parameters      None.

Returns         Return-skip normally or just Return if command not confirmed.

Description     Parses HANGUP command and calls local procedure SERHNG to lower

modem control lines DTR and RTS.  Most MS Kermits have not  im-

plemented  SERHNG; a "not implemented" message results in these

cases.

DUMPSCR

Parameters      None.

Returns         Normally.

Description     Copies text from the screen to a file. Word DMPNAME  holds  the

address (offset in data segment) of a zero terminated string of

the filename or  device;  the  default  string  is  KERMIT.SCN;

MSSSET  stores this string. Method of copying is to capture the

screen to a buffer and read out the text to a file, with  lines

trimmed of trailing blanks and terminated by CR/LF. A Form Feed

ends the file. The filename is checked for collision with  sys-

tem,  readonly,  subdirectory types etc and a complaint message

is displayed if such a collision occurs.  The  file  is  opened

and  closed  for each screen dump operation.  Current implemen-

tations are on IBM and Rainbow machines, and  they  append  new

material  to  a  pre-existing  file.  On other systems a simple

Return is done.

IHOSTR

Parameters      None.

Returns         Normally.

Description     Initializes the remote host for our  reception  of  a  file  by

sending  the  flow-on  character  (XON normally) to release any

held data. Called by receive-file code just after  initing  the

serial port.

IHOSTS

Parameters      None.

Returns         Normally.

Description     Initializes  the  host for our sending of a file by sending the

flow-on character (XON normally) to release any held data, such

as NAKs, flushes the serial port input buffer, waits one second

(by calling SLEEP), and exits if the buffer is empty.    Other-

wise, the cycle of clear buffer, sleep one second, check buffer

is repeated. Called by send-file routine just after  initializ-

ing the serial port.

VTS

Parameters      None.

Returns         Skip-return normally or just Return if parser error.

Description     Parses  SET  TERM  commands.  The IBM version has many keywords

which are further parsed and cause  jumps  to  action  routines

within  this  procedure.  Non-IBM  Kermits  usually  parse  the

keywords ON and OFF, for Heath-19 emulation or not.  Common  to

all  MS  Kermits  is the setting of global byte FLAGS.VTFLG in-

dicating the terminal emulation type (types are defined in  the

header file MSSDEF.H). For the VT102 emulator in the IBM Kermit

VTS also writes into structure VTEMU to pass information to the

emulator  about colors, tabstops, line-wrap, and other Set Term

parameters and controls global byte REFRESH  passed  to  module

MSYIBM to govern screen refresh methodology.  Small local pars-

ing support procedures are included here.

Structure VTEMU is defined in header file MSSDEF.H and contains

the following fields:

VTFLGST         Byte.  Holds  VT102 setup flag bits, which when

set mean -

VSNEWLINE (01H)

ANSI Newline mode active

VSWRAP (02H)

Line wrap is active

VSSHIFT3 (04H)

Sharp sign (#) replaced by Pound Sterling

VSKEYCLICK (08)

Keyclick on

VSMARGINBELL (10H)

Sound bell when cursor passes column 72

VSCURSOR (20H)

Cursor is a block (vs underline)

VSSCREEN (40H)

Screen is to be reverse video

A  constant VSDEFAULTS is defined in the header

file as the OR of bits above which is  used  by

the  emulator at Kermit startup time in lieu of

Set Term.

VTTBS           Word. Holds address (offset in data segment) to

default  tabstop  byte array. This array is lo-

cated in file MSZIBM.

VTTBST          Word. Holds address (offset in data segment) to

active  tabstop  byte  array. This array is lo-

cated in file MSZIBM.

ATT_PTR         Word. Pointer (offest in data segment) to video

attributes  byte  pair (which is stored in file

MSZIBM).

Further, global byte FLAGS.VTFLG holds the kind  of  terminal  being  emulated.

Kinds are defined in the header file as constants:

TTGENERIC       0       no emulation

TTHEATH         1       Heath-19

TTVT52          2       VT52 submode of VT102

TTVT102         3       VT102

TTTYPES         4       number of types defined above.

VTSTAT

Parameters      Enter with register DI pointing to status buffer STTBUF.

Returns         Register  CX counting the right most column used in status dis-

play and register DI 40 bytes larger that at entry.  Uses  nor-

mal Return, not Skip-return.

Description     Displays  Status information about terminal emulator. Called by

procedure STAT0 in file MSSSET. This procedure  is  similar  to

normal  Status  ones  except  the data items are from structure

VTEMU. Command macros for VTSTAT are defined  in  file  MSXxxx.

VTSTAT writes the status information directly to the screen and

returns to STAT0 with character in line pointer register DI set

40  bytes  larger  than when entered (even though several lines

may have been displayed) and register CX holding the number  of

chars  shown on the last line. Non-IBM Kermits just do a simple

Return. Normally VTSTAT is  the  last  Status  item  called  by

STAT0.

Terminal  emulation  is  supplied in the system dependent files MSXxxx, MSYxxx,

and possibly MSZxxx, depending on the machine. The  terminal  "argument  block"

passed  to procedure TERM is listed below; but note that the VT102 emulator for

IBM machines uses an additional global structure VTEMU.  The terminal  argument

block passed to the terminal emulator has the following fields:

FLGS            Byte containing flags.  Flags are:

SCRSAM (80H)    If   on,   the   terminal   emulator  shouldn't

re-display the screen when entered.

CAPT (40H)      Capture output.  If on, the routine  passed  in

field  CAPTR is called with each character sent

to the screen.

EMHEATH (20H)   On if terminal emulation is active.   Obsolete.

The   preferred   method  is  to  examine  byte

FLAGS.VTFLG for the terminal type.

HAVTT (10H)     A key redefinition table is present.

TRNCTL (08H)    Print control character X  as  ^X  (useful  for

debugging).

MODOFF (04H)    Do not display emulator mode line if on.

LCLECHO (01H)   Echo  keyboard  characters on the screen in ad-

dition to sending them to the port.

PRT             Port to use for terminal emulation, used  only  in  mode  line.

This is just a copy of COMFLG in FLAGS.

COLS            Number of columns on screen.

ROWS            Number of rows on screen.

CAPTR           Routine  to  call  to with each character sent to the screen if

CAPT flag is on.  Characters are passed in AL.

BELLD           Bell divisor (used only on IBM).

KLEN            Number of keys in key redefinition table, if HAVTT flag is on.

KTAB            Address of key redefinition table.  The key redefinition  table

is a table of KLEN 16-bit scan codes.  Each (machine dependent)

scan code represents a key that is redefined.

KRPL            Address of key replacement table.  The  key  replacement  table

parallels the key redefinition table given in KTAB.  Entries in

the replacement table are  16-bit  pointers  to  redefinitions.

Each  redefinition  has  a  one-byte  length,  followed  by the

definition.

ESCC            Escape character (single byte).  When this character  is  typed

to the emulator, it should return.

BAUDB           byte.    Bits  describing the baud rate so it can be printed on

the mode line.  This is a copy of the BAUD field  in  the  POR-

TINFO structure.  Currently used only on the IBM.  See MSDEFS.H

for possible values.

PARITY          byte.  Current parity to print on the mode line.    This  is  a

copy  of PARFLG in the PORTINFO structure.  Currently used only

on the IBM.  See MSDEFS.H for possible values.