PCbridge Installation Guide

Vance Morrison

What you need to install PCbridge:

The PCbridge distribution comes in two parts, the binary-documentation

archive, as well as the source code archive.  If you are using PCbridge

in a 'standard' configuration (for example, with 2 WD8003E ethernet

cards), then you will only need the binary-documentation archive.  If

you wish to modify the code, or use non-standard configurations

(a three interface bridge for example), you will also need to recompile

the code.  See compile.doc for details on how to do this.

In addition to the software you will also need

1) An IBM XT or AT compatatable.   PCbridge uses very little memory

itself (about 80K) so a machine with 256K of memory is sufficient.

Additional memory simply will not be used.

2) The proper number and type of interface cards.  As stated

in the readme.doc WD8003E cards are NOT recommended unless

you have no file serving traffic crossing the bridge.

WD8003EBT or WD80013EBT are the recommended cards.  Standard

COM boards can be used for the serial line if a remote bridge

is being configured.

-------------------------------------------------------------------------

If you are buying new hardware:

Because of the 16 bit data path, it is STRONGLY recommended that

if you are buying new hardware for you PCbridge that you get a

AT clone and use 3c507 or WD8013EBT cards.   This PC need not have

much memory (256K suffices), and does not need a monitor.  PCbridge

also does not used the keyboard in any way, but some BIOS boot

roms REQUIRE the keyboard to be connected for the machine to

boot.  I have found that many of these BIOS roms will NOT check

for a keyboard if the DIP switches inside the PC are set for

no-monitor (or sometimes monitor = EGA).   Machines with this

type of boot rom are convenient because you need not buy the

keyboard and bridge is more compact without it.

Such stripped down ATs cost about $400-$500 at the time of this

writing.  WD8013EBT cards cost $250, and 3c507 about $250-$300

so a VERY nice bridge could be put together for under $1000.

With 386 machines coming down in price, people have asked if

it is worth buying one of these for a high end bridge.  Although

the cache and higher clock rate may help, I suspect that a

fast 286 machine will provide much better cost/performance.

In short, a 386 machine is overkill.

If you are buying COM boards for a remote bridge, buy the

ones with the 16550AF chip on them.  These boards have a built

in FIFO buffer on them that will reduce interrupts by a factor

of 10, and thus leave more time for the CPU to filter/forward

packets.

-------------------------------------------------------------------------

Installing the standard WD ethernet-ethernet bridge:

The PCbridge distribution comes with a two interface ethernet bridge

for the WD8013EBT, WD8003EBT and WD8003E, cards.   The file for each

of these versions is simply the name (truncated because of DOS), with

a .EXE extension.   To use it simply

1) Rename the proper file to 'BRIDGE.EXE'.

2) Set the I/O address of the first to WD80XX card  to 280H (the

default) and the second to 2A0H.  PCbridge does not use interrupts

so you can leave the IRQ line on the default of 3.  Also jumper

each card for the type of ethernet you will use (thick or thin)

3) Install these cards in the PC that will become the bridge.  This

PC needs 256K of memory.

3) Create a bootable Floppy and put BRIDGE.EXE on it.  Create

and AUTOEXEC.BAT file that executes BRIDGE.EXE.

4) Hook up the cables and turn on the PC.

That's it. PCbridge does the rest.   If the bridge does not work, see

trouble.doc.

-------------------------------------------------------------------------

Installing the standard 3Com ethernet-ethernet bridge:

The PCbridge distribution comes with a two interface ethernet bridge

for the 3c507 cards.  This file is named 3c73c7.exe. To use it simply

1) Rename it to 'BRIDGE.EXE'.

2) Use the 3c507.exe file that came with the 3c507 cards to set

the  first card to

I/O address to 280H,

Shared memory address to D0000H

Shared memory length to 64K

Interupt to some unused IRQ (say 3)

ROM disabled (length = 0)

0-wait state to enable if you PC can handle it

(most fast clones can, you may want to disable

it at first if you are not sure)

and the second card to

I/O address to 2A0H,

Shared memory address to C0000H

Shared memory length to 64K

Interupt to some unused IRQ (say 3, (it can be the same))

ROM disabled (length = 0)

0-wait state to enable if you PC can handle it

(most fast clones can, you may want to disable

it at first if you are not sure)

Note because of the way the 3c507.exe program works, you can

only have one 3c507 card in the PC when you are configuring it.

Thus you have to configure one card, replace it with the other,

configure that one, and only then place both cards back in the

PC.

3) Install these cards in the PC that will become the bridge.  This

PC needs 256K of memory.

3) Create a bootable Floppy and put BRIDGE.EXE on it.  Create

and AUTOEXEC.BAT file that executes BRIDGE.EXE.

4) Hook up the cables and turn on the PC.

That's it. PCbridge does the rest.   If the bridge does not work, see

trouble.doc.

-------------------------------------------------------------------------

Installing the Starlan cards with the standard distribution:

The standard WD8003E program can be used for a starlan-starlan or

starlan-ethernet bridge (if WD8003E cards are used for the ethernet

side).   Simply treat a starlan card like an ethernet card in the

instructions above.  Note, however, that it is really recommended that

a WD8003EBT is used for the Ethernet side (if a starlan-ethernet

bridge is desired).  This configuration, however is not supported by the

standard distribution, PCbridge will have to be recompile.  See

compile.doc for details.

-------------------------------------------------------------------------

Installing the standard ethernet-serial-ethernet bridge:

PCbridge also comes with a file called REMOTE.EXE which is designed

to do remote bridging.   The  expected configuration is to have

two PCs with an ethernet and a COM port on each one.  The PCs are

then connected via a serial line.  The PCs will then bridge packets

between the far ethernets.  The standard distribution is designed

to work with the WD8003E card.  Although the other cards (WD8013EBT

and WD8003EBT) will work too, the extra buffer space of these cards

is not used.  Thus it is recommended that you configure and recompile

a special version if you use these cards in the remote bridge.

1) Rename the proper file to 'BRIDGE.EXE'.

2) Set the I/O address of both to WD8003E cards to 280H (the default).

You can leave the IRQ line on the default of 3 (but make sure it

is NOT 4, since that is what the COM1 board uses).

3) Set each of the serial boards to be COM1.  (I/O = 3F8H, IRQ=4)

4) Install one serial board and one WD8003E board in each PC.

5) Create two  bootable Floppies and put BRIDGE.EXE on each.  Create

on each floppy an AUTOEXEC.BAT file that executes the command

BRIDGE <baud rate>

where <baud rate> is the baud rate of the serial line (9600, 19200 etc).

If no baud rate is given, 19200 is the default.

6) Hook up the cables and turn on both PCs.

That's it. PCbridge does the rest.   Note that since EVERY broadcast

and multicast packet on BOTH networks has to be forwarded through the

serial line, the remote bridge will only work well for networks that

have little broadcast traffic (certainly less than 1 broadcast/sec).

Normally PCbridge assumes that it may always send a character down the

serial line (no flow control).  If you are using a compressing modem,

this assumption is no longer correct.  PCbridge can be configured to

understand hardware flow control (that is only send characters when

the RS232 signal CTS is asserted).  To enable this feature, however

the sofware must be recompiled.  See compile.doc for details on how

to do this.