SMTPSERV

A Simple SMTP Server

SMTPSERV is a program which runs on your pc and allows it to accept SMTP

based mail from other sources, even from several sources at one time.

Introduction:

On the Internet, mail is usually transferred using SMTP (Simple Mail

Transfer Protocol).  The mail sender's computer simply calls up the

receiver's computer (where something like SMTPSERV is running) and

asks it to accept the mail on behalf of the receiver.

With PC networks, SMTP service is not as easy since the receiver's

personal computer is not usually running a program like SMTPSERV

or it may even be turned off.  This makes it pretty hard to send

mail to the pc user!

There are several workarounds, such as sending all the person's Email

to their UNIX account, but many sites simply wish or need to have their

mail handled by pcs.

The classic way of doing this is to have a mail gateway, a machine

which is always running a program which waits for incoming mail.  This

machine (or another) will then convert the mail into a format usable

for on it's local network.

Brad Clemments' CHARON gateway does an excellent job of this for Novell

Netware users.  Check it out via anonymous ftp to omnigate.clarkson.edu

if you are running Netware.

Most other mail gateways are more generic, i.e. they don't know how to

send mail on the local network.  Instead, they simply accept mail and

place it into a subdirectory repository from which users or another pc

(or another task) must sort out for whom the mail is destined.

SMTPSERV fits in this latter class along with Phil Karn's NOS and the

commercial SMTPSRV (FTP Software, approx $400).

Phil's program is free and incredibly flexible but is not usable by

anyone other than educational institutions or for the private use of

amateur radio operators.

We originally used NOS but found it unable to handle our high message

load reliably (avg. about 2000 inbound per day).  With that sort of

average load, you can immagine that several hosts were typically SMTPing

in similtaneously.  As FTP's product only allows a single incomming

session at a time, it offerred no hope.  We could have developed our

own using the FTP toolkit, but that would require other sites to also

purchase FTP's kernal to use our product.

SMTPSERV was designed as a direct replacement for NOS (in this SMTPD

mode), so it uses similar file naming conventions.  BTW, the similarity

is only in the function, the internals are radically different.

So, in summary, SMTPSERV simply accepts inbound mail and places it into

a single subdirectory.

It does not:

forward mail            sort mail by userid         notify users

notify users            fail if invalid userid      transmit mail (yet)

- - -

Details:

SMTPSERV works by allocating several tcp sockets so it can accept multiple

connections.  Each socket is managed independantly and one slow connection

will not affect the performance of other zippy connections.  Currently

the system supports a maximum of about 15 sessions - this is a DOS

file limit (actually, just an annoyance, I will be increasing that number).

Inbound messages are assigned a unique 32 bit message number.  The numbers

will not necessarily be consequetive integers though they will be ascending

until you hit the 2^32 messages wraparound.  When a message is being

received, SMTPSERV will initially attempt to create two files:

xxx.CMD     containing From: and To: directives

xxx.TMP     containing the body of the message

Eg. Message 5321 would generate the following intermediate files:

+-5321.CMD---------+  +--5321.TMP----------------------------------+

|From: erick@maggot|  |Received: from maggot by gnat with SMTP     |

|To: bruce@gnat    |  |       id A5321 ; Fri, 20 Dec 91 23:03:48   |

+------------------+  |Date: Fri, 20 Dec 91 15:05:49 -0800         |

|From: erick@maggot                          |

|To: bruce@gnat                              |

|                                            |

|Beers?                                      |

+--------------------------------------------+

If the message is successfully received, the files are renamed:

xxx.CMD  ------>   xxx.WRK

xxx.TMP  ------>   xxx.TXT

With the CMD file being renamed last - ie. if you see a WRK file, you can

process it delete it, or do whatever you wish.

If the message was not successfully received, SMTPSERV deletes all the

files related to that message.

When parsing the .WRK file, ensure you do the following things:

1) Check for multiple To: lines, this is called mail exploding,

you must send a copy to each intended recipient

2) Return an error message to the sender if any one of the

intended recipients will not receive the mail

3) Send mail destined to the local postmaster account actually

go to someone.

- - -

Installation:

To use SMTPSERV you must create a valid WATTCP.CFG file as described

in the documentation available in the standard WATTCP installation notes

(location given at end of this document).

There are some parameters which are peculiar to SMTPSERV or perhaps just a

little more important when SMTPSERV is in use.

smtp.sessions=numsessions

The default value is 8, but it should work fine up to about 15.  I

currently use 12.  The limit of 15 comes from 20 dos file handles

minus standard io, standard error, standard prn, etc.  This limit will

probably be changed at a later date.

eg. smtp.sessions=12

NOTE: YOU SHOULD HAVE THE FOLLOWING LINES IN YOUR CONFIG.SYS

(you can use larger values if you wish)

FILES=20

BUFFERS=20

smtp.subdir=pathname

The newly received files must be stored somewhere.  Select a subdirectory

where they will not be inconvienenced.  You really should give a

full path specifier including drive: and a path from the root directory.

eg. smtp.subdir=n:\spool\rqueue

inactive=seconds

The inactive value tells WATTCP to give up if its partner does not

do anything within a certain period of time.  This is not a keepalive,

it is an inactivity timeout, WATTCP makes no attempt to correspond

with the partner except to hangup.  A good value for inactive is

1200 seconds or 20 minutes

eg. inactive=1200   ; 20 minutes

mss=bytes

The maximum segment size should be made small enough that ip packets

will not become fragmented.  A good value is 500 bytes.

eg. mss=500

hostname=name

The mail server really should be given a name and told its name.

eg. hostname=snail

domain=name

This too should be set.

eg. domain=uwaterloo.ca

You must also load a packet driver for your netword card.  If you don't

know what I mean by that, ftp the notes described at the end of this doc.

- - -

Sample WATTCP.CFG file, you will have to configure one for your machine.

my_ip=129.97.176.99         # set ip address

netmask=255.255.254.0       # 23 bit subnet mask

nameserver=129.97.129.140   # primary and

nameserver=129.97.128.196   #    secondary nameservers

gateway=129.97.176.1        # my gateway to the world

# for advanced subnetting details see the complete

# wattcp installation guide ftpable from

# sunee.uwaterloo.ca in pub/wattcp/apps.zip

domainslist="watstar.uwaterloo.ca"

inactive=1200

hostname=maggot

smtp.subdir=n:\spool\rqueue

smtp.sessions=12

- - -

More Information:

SMTPSERV was written using the WATTCP TCP/IP communications library

in about 500 lines of C code.  WATTCP handles most of the complexities

of network communciations so the programmer can concentrate on the

data rather than the connection.

If you have any suggestions on how SMTPSERV can be improved, please

let me know.  The next planned addition is an outbound mail service

(like Karn's embedded one).  Once that's in place I will be calling up

the Paegus mail folks to see how we can get our two programs to work

together.

For some other free and neat examples of WATTCP based network

applications anonymous ftp to sunee.uwaterloo.ca and read

pub/wattcp/README try out the new MSKermit 3.11 with TELNET capabilities.

Details on packet drivers and indepth configuration notices are

available via anonymous ftp to sunee.uwaterloo.ca in pub/wattcp/apps.zip

Erick