The

Alad_IRQ (TM)

User Guide

Computer Telecommunication Systems, Inc

Copyright 1993, Computer Telecommunication Systems, Inc.

(c) Copyright 1993 Computer Telecommunication Systems, Inc. All

Rights Reserved.

No part of this User Guide may be reproduced, transmitted,

transcribed, stored in a retrieval system, or translated into any

language (natural or computer), in any form or by any means,

without the prior written permission of Computer

Telecommunication Systems, Inc. See the accompanying files

VENDOR.DOC, LICENSE.DOC, and SYSOP.DOC for specific information

pertaining to distribution of this shareware version of the User

Guide.

Information in this User Guide is subject to change without

notice and does not represent a commitment on the part of

Computer Telecommunication Systems, Inc.

Computer Telecommunication Systems, Inc. may make improvements

and/or changes in the User Guide and/or in Alad_IRQ at any time.

Trademarks and Copyrights

IBM(R) and is a registered trademark of International Business

Machines Corporation.

The CTS Serial Port Utilities(TM), SwapIRQ(TM), Alad_IRQ(TM) and

AT Command Test(TM) are trademarks of Computer Telecommunication

Systems, Inc.

GEnie and GEnie PC Aladdin are trademarks of General Electric

Company, USA.

Other names listed in this User Guide are the proprietary

trademarks of their respective companies.

Computer Telecommunication Systems, Inc.

3847 Foxwood Road, Suite 1000

Duluth, Georgia 30136-6100 USA

404-263-8623 (voice)

404-263-0124 (fax)

CompuServe ID:76662,2315

GEnie: J.Jerrim

ii

Copyright 1993, Computer Telecommunication Systems, Inc.

TABLE OF CONTENTS

1.0 Registering Alad_IRQ  . . . . . . . . . . . . . . . . . .   1

2.0 Association Of Shareware Professionals Member . . . . . .   4

3.0 Introducing Alad_IRQ(TM)  . . . . . . . . . . . . . . . .   5

4.0 Installing Alad_IRQ . . . . . . . . . . . . . . . . . . .   6

5.0 Before Using Alad_IRQ . . . . . . . . . . . . . . . . . .   7

6.0 Using Alad_IRQ  . . . . . . . . . . . . . . . . . . . . .   8

6.1 Help Screen  . . . . . . . . . . . . . . . . . . . .   8

6.2 Running Aladdin with Alad_IRQ  . . . . . . . . . . .   8

6.3 Starting Aladdin With Alad_IRQ . . . . . . . . . . .   9

7.0 Using Alad_IRQ with Windows . . . . . . . . . . . . . . .  10

8.0 Examples  . . . . . . . . . . . . . . . . . . . . . . . .  11

8.1 Example 1: Your modem is on Com3 using IRQ 5 . . . .  11

8.2 Example 2: Your serial ports support high IRQs . . .  11

8.3 Example 3: Sending Special Commands to Aladdin with

Alad_IRQ  . . . . . . . . . . . . . . . . . . . . .  11

9.0 Customer Support Policy . . . . . . . . . . . . . . . . .  12

Appendix A: Conventional IRQ and Address Assignments for

Serial Ports . . . . . . . . . . . . . . . . . . . . . .  13

ISA (IBM AT and Compatible) Computers  . . . . . . . . .  13

Microchannel Architecture (IBM PS/2) Computers . . . . .  13

Appendix B: About Computer Telecommunication Systems  . . . .  14

The CTS Serial Port Utilities  . . . . . . . . . . . . .  14

AT Command Test  . . . . . . . . . . . . . . . . . . . .  16

SwapIRQ  . . . . . . . . . . . . . . . . . . . . . . . .  16

MouseOFF and MouseON . . . . . . . . . . . . . . . . . .  16

Serial Cards (Comm Ports)  . . . . . . . . . . . . . . .  16

Custom Programming Services  . . . . . . . . . . . . . .  17

New Products . . . . . . . . . . . . . . . . . . . . . .  17

iii

Copyright 1993, Computer Telecommunication Systems, Inc.

1.0 Registering Alad_IRQ

When you register to use the Alad_IRQ software, you receive:

*  The most current version of Alad_IRQ on the diskette size of

your choice.

*  You will receive a FREE copy of SwapIRQ, a DOS terminate and

stay resident utility which allows any DOS communication or

game software to use any IRQ. SwapIRQ also comes with a

printed User Guide. SwapIRQ uses less than 800 bytes of RAM,

and can be loaded and unloaded as needed.

*  The most current shareware version of the CTS Serial Port

Utilities (SPU). The SPU includes PortInfo, a port and

interrupt conflict diagnostic, plus control utilities for

all port parameters. Registered users of the SPU also

receive the 75 page 8 1/2 x 11 printed User Guide, including

30 pages of tutorial information on serial ports, UARTs,

sharing interrupts..and more. See Appendix B for details

about the SPU.

*  FREE minor version updates.

*  Announcements about major updates. You will receive a FREE

upgrade to any major update within 12 months of your

registration.

Registration of Alad_IRQ costs $20 for each computer it is used

on. Multiple copy discounts are available. Contact CTS for more

information.

1

Alad_IRQ V1.0 LICENSE REGISTRATION FORM - 112

Name: _________________________________________________________

Daytime telephone: __________________  Evening: _______________

Fax Number: _________________    GEnie ID: ____________________

Company Name: _________________________________________________

Address: ______________________________________________________

______________________________________________________

ALL ORDERS - Choose format: 5.25" ____ 3.5" ____

QTY   Description                      Unit Price      Subtotal

===   ===========                      ==========      ========

___   Alad_IRQ V1.0 License                $20.00     $________

Sales Tax:     Georgia residents add 5% ($1/copy)     $________

Shipping / Handling (All countries):        $3.00     $________

TOTAL (US dollars, please)   $________

PAYMENT METHOD:    Cash __           Check__      Money Order__

Visa __      MasterCard__         Discover__

=================================================================

CREDIT CARD INFORMATION

=================================================================

Name as it appears on the card: _________________________________

Card No. ________  ________  ________  ________  Expires  ___/___

Cardholder signature: ___________________________________________

=================================================================

SEND TO

=================================================================

Computer Telecommunication Systems, Inc.

3847 Foxwood Road, Suite 1000

Duluth, Georgia  30136-6100  USA

404-263-8623 (Voice)

404-263-0124 (FAX)

GEnie: J.Jerrim

CompuServe: 76662,2315

NOTE: Prices are subject to change without notice.

Copyright 1993, Computer Telecommunication Systems, Inc.

DISCLAIMER OF WARRANTY

USERS OF THIS SOFTWARE MUST ACCEPT THIS DISCLAIMER OF WARRANTY:

THIS SOFTWARE IS SUPPLIED AS IS. COMPUTER TELECOMMUNICATION

SYSTEMS INC. DISCLAIMS ALL WARRANTIES, EITHER EXPRESS OR IMPLIED,

INCLUDING BUT NOT LIMITED TO IMPLIED WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE, WITH

RESPECT TO THE SOFTWARE AND THE ACCOMPANYING WRITTEN MATERIALS.

NO LIABILITY FOR CONSEQUENTIAL DAMAGES. IN NO EVENT SHALL

COMPUTER TELECOMMUNICATION SYSTEMS INC. OR ITS SUPPLIERS BE

LIABLE FOR ANY DAMAGES WHATSOEVER (INCLUDING, WITHOUT LIMITATION,

DAMAGES FOR LOSS OF BUSINESS PROFITS, BUSINESS INTERRUPTION, LOSS

OF BUSINESS INFORMATION, OR OTHER PECUNIARY LOSS) ARISING OUT OF

THE USE OF OR INABILITY TO USE THIS COMPUTER TELECOMMUNICATION

SYSTEMS INC. PRODUCT, EVEN IF COMPUTER TELECOMMUNICATION SYSTEMS

INC. HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES. BECAUSE

SOME STATES DO NOT ALLOW THE EXCLUSION OR LIMITATION OF LIABILITY

FOR CONSEQUENTIAL OR INCIDENTAL DAMAGES, THE ABOVE LIMITATION MAY

NOT APPLY TO YOU.

3

Copyright 1993, Computer Telecommunication Systems, Inc.

2.0 Association Of Shareware Professionals Member

_______

____|__     |                (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

This program is produced by a member of the Association of

Shareware Professionals (ASP). ASP wants to make sure that the

shareware principle works for you. If you are unable to resolve a

shareware-related problem with an ASP member by contacting the

member directly, ASP may be able to help. The ASP Ombudsman can

help you resolve a dispute or problem with an ASP member, but

does not provide technical support for members' products.  Please

write to the ASP Ombudsman at 545 Grover Road, Muskegon, MI

49442-9427 USA, FAX 616-788-2765 or send a CompuServe message via

CompuServe Mail to ASP Ombudsman 70007,3536.

4

Copyright 1993, Computer Telecommunication Systems, Inc.

3.0 Introducing Alad_IRQ(TM)

Alad_IRQ is a powerful DOS utility which allows your existing

GEnie PC Aladdin (TM) communications software to use any

interrupt (IRQ) supported by your serial port or internal modem.

You can also use it in a DOS session under Windows to provide

access to an IRQ not supported by Aladdin. Alad_IRQ works with

DOS versions 2.0 and higher, and uses 8K of memory (RAM).

Serial cards and board modems in your computer system are

installed at a particular address and use an IRQ (an Interrupt

ReQuest line) to signal the controlling software that they

require attention. Some cards only allow selection of a Com port:

Com1, Com2, etc., and use the industry standard addresses and

IRQs as described in Appendix A. Other serial ports let you

independently select a port address and IRQ which do not follow

any particular standards.

Communication software requires two pieces of information to work

with a serial card or a board modem. First, software must know

the address used by the board so that commands can be issued.

Secondly, the software must know which IRQ is used by the board

to request attention. Finally, you must be able to configure the

software to use the address and IRQ used by the serial card.

Otherwise, the board or modem will not work with the software.

The GE PC Aladdin software (referred to as Aladdin from here on)

allows you to select the serial port to use by number: Com1,

Com2, Com3 or Com4. Current versions of Aladdin use the industry

standard IRQs 3 & 4 for all serial ports. However, using the

Alad_IRQ utility enables the Aladdin software to operate with any

IRQ supported by your internal modem or serial card.

Alad_IRQ does this by redirecting the IRQ used by your serial

port or modem over to the IRQ expected by the Aladdin software.

Alad_IRQ detects when your hardware generates the IRQ, and then

signals Aladdin to process it. There is more to it than this, of

course, but this should give you the idea.

5

Copyright 1993, Computer Telecommunication Systems, Inc.

4.0 Installing Alad_IRQ

Alad_IRQ is normally placed in a subdirectory on your hard disk.

The following instructions assume you will put Alad_IRQ in a

directory named "ALADIRQ":

1. Create an "ALADIRQ" subdirectory on your hard disk using

the DOS "Make Directory" (MD) command:

Example: C>MD ALADIRQ <Enter>

NOTE: <Enter> means to press the Enter (Return) key.

2. Change the default directory to the directory you created

using the DOS Change Directory (CD) command:

Example: C>CD ALADIRQ <Enter>

3. Copy the distribution files to the ALADIRQ directory. If

the distribution files are on a floppy diskette in drive A,

you would enter:

Example: C:\ALADIRQ>COPY A:\*.* <Enter>

If your directory contains a README.1ST file, then check

this file for additional information not included in this

User Guide.

4. Copy the Alad_IRQ.EXE file to the directory containing

your Aladdin software. You must run Alad_IRQ from the

directory containing Aladdin.

That's it! Alad_IRQ is ready to run!

6

Copyright 1993, Computer Telecommunication Systems, Inc.

5.0 Before Using Alad_IRQ

Before you use Alad_IRQ with your serial ports, you should

verify that your system has no serious serial port or

interrupt conflicts. If you have been using your current

system for some time without problems, then you can probably

assume that your system is configured properly. If you have

changed your system setup recently, or if you have

occasional difficulties, then a little additional testing is

worth the effort. If you have the CTS Serial Port Utilities,

run "PortInfo /0", paying careful attention to the error and

warning messages regarding port and interrupt conflicts. Any

serial port or IRQ conflicts detected by PortInfo must be

resolved before using Alad_IRQ.

If you do not have the CTS Serial Port Utilities (SPU), then

we suggest you contact us to obtain a copy, particularly if

you observe any problems with Alad_IRQ. The shareware

version of the SPU is included for all registered users of

Alad_IRQ. The shareware version of the SPU is also on many

online systems under the filename CTSSPUxx.yyy, where xx is

the current version number (1.2), and yyy is the compression

type used on the system.

After you have verified that your system is in good working

order, then you need to determine which interrupt your

hardware is generating, and which interrupts your software

can use. You will use this information when starting

Alad_IRQ. The PortInfo summary screen shows you which IRQ

your serial port or internal modem is using - save this

value for later reference. The next section discusses how to

determine which IRQ Aladdin wants to use.

7

Copyright 1993, Computer Telecommunication Systems, Inc.

6.0 Using Alad_IRQ

You use Alad_IRQ as a replacement command line for the Aladdin

software. You will tell Alad_IRQ which IRQ is being used by the

serial port or internal modem, and which IRQ Aladdin expects it

to use. You can also tell Alad_IRQ to pass on any "Special

Commands" to the Aladdin software.

6.1 Help Screen

You can display the help screen from the DOS prompt by

entering either of the following commands from any directory

containing Alad_IRQ:

Alad_IRQ <Enter>

-OR-

Alad_IRQ /? <Enter>

6.2 Running Aladdin with Alad_IRQ

Before you run Aladdin with Alad_IRQ, you need to do three

things:

1. Copy the ALAD_IRQ.EXE program into the same

directory or sub-directory where your Aladdin software

exists.

2. You need to determine which IRQ your serial card or

modem is actually using. Write down this IRQ number

down as the "Hardware IRQ".

If you are not sure which IRQ is used, then please use

the PortInfo program in the SPU package described

previously to find out. DO NOT use MSD, as it does not

accurately report the IRQs used by your serial ports.

3. You need to determine which IRQ the Aladdin software

will be using. If you select Com1 or Com3 in the

Aladdin setup screen, then Aladdin uses IRQ 4.

Otherwise, it will use IRQ 3. Write this value down as

the "Software IRQ".

8

Copyright 1993, Computer Telecommunication Systems, Inc.

6.3 Starting Aladdin With Alad_IRQ

Whenever you use Aladdin with a non-standard IRQ, then you

will need to start Aladdin by running Alad_IRQ. The command

format to start Alad_IRQ is:

Alad_IRQ /ih /is <Enter>

where "h" is the "Hardware IRQ" you listed in step 2, and

"s" is the "Software IRQ" you listed in step 3. You MUST put

the IRQ values in the order shown.

The valid 'h' and 's' values are decimal 2-7 for an IBM

PC/XT computer, and 3-7, 9-12, 14 and 15 for an IBM AT or

compatible machine. If your serial card or internal modem

board has a setting for IRQ 2, it is actually using IRQ 9 in

an IBM AT or compatible computer. You should specify IRQ 9

as the hardware IRQ for Alad_IRQ.

6.4 Example: Using Aladdin on Com4 with IRQ 9

Alad_IRQ /i9 /i3 <Enter>

This command tells Alad_IRQ that the hardware is using IRQ

9, and Aladdin is using IRQ 3. Alad_IRQ will install itself

to use these values, and then start up Aladdin.

IMPORTANT NOTE: There is one important limitation when using

Alad_IRQ. The "software interrupt" cannot be used by a

hardware device at the same time that Alad_IRQ is in use. In

the example above, the IRQ 3 line must be dedicated to

Alad_IRQ. The Com2 serial port, which normally uses IRQ 3,

must be inactive while Alad_IRQ (and Aladdin) is loaded.

9

Copyright 1993, Computer Telecommunication Systems, Inc.

7.0 Using Alad_IRQ with Windows

You can use Alad_IRQ in Windows just like you do in DOS. Before

using Alad_IRQ with Windows, it is important to have Windows

properly configured for the serial ports you have in your system.

You can do this either through your Windows control panel, or by

editing the WIN.INI file with a text editor. See your Microsoft

Windows documentation for additional information.

You can use Alad_IRQ under Windows in a DOS box to provide access

for DOS-based communication applications to IRQs not supported

directly by the application. Load Alad_IRQ as a described for DOS

in a Windows DOS box before running your DOS communications

program. Then run the application just as you would under DOS.

10

Copyright 1993, Computer Telecommunication Systems, Inc.

8.0 Examples

8.1 Example 1: Your modem is on Com3 using IRQ 5

Problem: Aladdin only uses Com3 with IRQ 4.

Solution: Use Alad_IRQ to "redirect" IRQ 5

generated by the modem to IRQ 4 (the conventional

IRQ for Com3) when using Aladdin.

Alad_IRQ /i5 /i4

^   ^

|   |

|   +-------- Aladdin uses IRQ 4

|

+------------ The modem uses IRQ 5

8.2 Example 2: Your serial ports support high IRQs

Problem: Aladdin doesn't - it only uses IRQs 3 & 4

Solution: Use Alad_IRQ to redirect the high IRQs

to the conventional IRQs supported by Aladdin. For

example, if you need to use Com4, IRQ 11, Aladdin

that expects Com4 to use IRQ 3. To use Aladdin

with this port, you run Alad_IRQ with the command:

Alad_IRQ /i11 /i3 <Enter>

8.3 Example 3: Sending Special Commands to Aladdin with

Alad_IRQ

Problem: You want to use special commands with Aladdin

while using Alad_IRQ to provide access to non-standard

IRQs

Solution: Alad_IRQ will pass all commands on the

command line on to the Aladdin software. Just

append them to the Alad_IRQ command line:

Alad_IRQ /i5 /i4 /NOEMS /CLOCK <Enter>

Will allow Aladdin to use Com1 or Com3 on IRQ 5

(depending on the Aladdin setup screen) with the

NOEMS and CLOCK special commands.

11

Copyright 1993, Computer Telecommunication Systems, Inc.

9.0 Customer Support Policy

Customer support is provided via telephone, fax, and electronic

mail. We provide for our products for any registered user within

the following guidelines:

1. We are in the office during normal US east coast business

hours, and we answer calls that come anytime if we are in the

office. If you don't reach us during business hours, please leave

a detailed message and we will reply via fax, electronic mail (E-

mail), or regular mail.

2. Messages sent to us on CompuServe or GEnie E-mail will usually

receive a reply within 24 hours.

3. We will be happy to assist with Alad_IRQ installation problems

before you register. However, we cannot provide technical

assistance until we've actually received your registration. We

can register you over the phone via credit card if you need

immediate assistance.

Be prepared to answer the following questions before you call for

assistance:

*  The version number of Alad_IRQ. You can view this

information in the program title by starting Alad_IRQ.

*  The brand name and model number of your computer.

*  What equipment you have in your computer. A general purpose

diagnostic program such as Microsoft's MSD can be helpful in

determining what other equipment is installed.

*  The version of DOS you are using (type "VER" at the DOS

prompt).

*  A complete description of the problem.

*  Can you duplicate the series of steps that results in the

problem?

*  Did an error message appear? What did it say?

You may contact CTS via voice phone at 404-263-8623 during normal

business hours (Eastern Time Zone), or 24 hours / day via FAX at

404-263-0124, and on CompuServe at 76662,2315 or GEnie at

J.Jerrim.

12

Copyright 1993, Computer Telecommunication Systems, Inc.

Appendix A: Conventional IRQ and Address Assignments for Serial

Ports

ISA (IBM AT and Compatible) Computers

Serial Port    IRQ

-----------    ---

Com1          4   (standard)

Com2          3   (standard)

Com3          4   (conventional)

Com4          3   (conventional)

NOTE: there are no standards or conventions for Com5 and

higher

Microchannel Architecture (IBM PS/2) Computers

Serial Port    IRQ

-----------    ---

Com1          4

Com2          3

Com3          3

Com4          3

Com5          3

Com6          3

Com7          3

Com8          3

NOTE: These are standards as defined by IBM.

13

Copyright 1993, Computer Telecommunication Systems, Inc.

Appendix B: About Computer Telecommunication Systems

Computer Telecommunication Systems, Inc. was formed in 1990 to

provide new communication software products, custom programming

services, and consulting on personal computer asynchronous

communications. We currently offer (as of December, 1993) the

following products and services:

The CTS Serial Port Utilities (TM)

----------------------------------

The CTS Serial Port Utilities (SPU) is a collection of DOS

software utilities that allows you to diagnose and control

the serial ports in your computer. Most serial port problems

can be resolved by using these utilities. Each of the

utilities support all standard and any user-defined serial

ports.

PortInfo is a comprehensive utility which tests your system

to determine everything possible about your serial ports,

including interrupt and addressing conflicts. PortInfo

provides you with FAILURE and WARNING messages about actual

and possible conflicts.

PortInfo provides details on the following:

*    Detects multiple serial ports at the same address (eg,

two ports installed as Com1!)

*    Interrupt conflicts between a bus mouse and a serial

port

*    Shared interrupts (2 serial ports, or a serial port and

a serial mouse sharing an IRQ)

*    Conflicts between COM4 and 8514/A compatible monitors

*    Unusual BIOS and DOS serial port setups

*    Serial ports that cannot generate interrupts

*    Helps to determine when interrupts can be shared

14

Copyright 1993, Computer Telecommunication Systems, Inc.

Other information provided:

*    Actual IRQ used by each port and the current IRQ status

*    DOS port assignments (com1-com4, mouse)

*    Port (UART) Identification (8250, 8250A/16450, 16550,

16550A), the T/Port, and some "emulated UARTs"

*    FIFO status: If it exists, on / off, else 'None'

*    If the FIFO is ON, the current FIFO receiver trigger

level is displayed.

*    All programmable port parameters: speed, format,

parity, current RTS and DTR settings

*    Current status of modem lines: Ring Indicator, Carrier

Detect, Clear to Send, Data Set Ready, with indication

of when the lines are tied together (looped back).

Other features:

*    Ports are COMPLETELY restored to their original

settings

*    Supports standard serial ports 1-4 on PC/AT, 1-8 on

PS/2

*    Supports up to 8 user definable serial ports (port

address and IRQ)

*    Displays detailed messages about conflicts with

reference numbers to assist looking up the detailed

assistance in the User Guide.

Other Utilities:

*    Control the 16550A FIFO trigger level

*    Set port speed and format

*    Add a serial port to the BIOS Comm Port assignment

table

*    Swap serial ports in the BIOS Comm Port assignment

table

*    Control the RTS, DTR, and Out2 (IRQ enable) lines

*    Reset a serial port to initial settings

While this is a great help to an individual user who is

experiencing a serial port problem, the SPU is a "must have"

tool for consultants and PC support personnel who support a

number of computers and users. One of the license options

for the SPU is specifically designed for consultants.

The CTS Serial Port Utilites is being distributed via

shareware. Please refer to the included file ORDER.DOC for

information about the different license options and what is

provided with each one.

15

Copyright 1993, Computer Telecommunication Systems, Inc.

AT Command Test (TM)

--------------------

AT Command Test (ATCT) is a software application that

performs a comprehensive test of the AT command set of any

modem claiming to be "Hayes Compatible". ATCT tests most of

the standard modem commands, all result codes, and many

other characteristics. ATCT provides three different reports

of command set compatibility with nine different Hayes modem

product lines.

ATCT is used today both as a qualification tool for

purchasing modems by major corporations, and as a diagnostic

tool for testing modems returned to corporate support

services prior to obtaining warranty service.

Contact CTS for more information.

SwapIRQ (TM)

------------

A small utility that allows software to use IRQs that are

not supported directly by the software. For example, a

communication application that supports COM3 on IRQ 4 can

now use COM3 on any IRQ. Or, a game that requires a sound

card on IRQ 7 can now use a sound card that generates

interrupts on IRQ 5. Contact CTS for more information.

MouseOFF and MouseON

--------------------

MouseOff disables a serial mouse, allowing a modem or other

device using the same interrupt (IRQ) to operate without

interference. MouseOn re-enables the mouse after you have

finished using the other device. MouseOff and MouseOn are

especially useful in sharing an interrupt between a mouse

and a modem when the communications software detects and

tries to use the mouse.

Serial Cards (Comm Ports)

-------------------------

We carry a variety of serial cards that contain 16550A or

compatible UARTs with bi-directional parallel ports.

16

Copyright 1993, Computer Telecommunication Systems, Inc.

Custom Programming Services

---------------------------

Call us to discuss your special communications software

requirements.

New Products

------------

This manual may be several years old by the time you read

this! Call us to see what's new!

17