A Quick Reference

CONEX VERSION 6.2

07-11-1991

Copyright (C) 1991 Erhard Hilbig

CONEX (latin connection) is used to communicate between IBM personal

computer and a mainframe or between 2 IBM PC's.

These are the highlights of the program:

-  The IBM PC can be used as terminal via COM1 - COM4, via Interrupt 14,

DECNET LAT, FTP TCP/IP and B&W TCP/IP at a mainframe.

-  Parameters like baudrate (50-115200), parity, full/half duplex,

xon/xoff or hardware handshake can be set in CONEX.

-  Files can be transfered without protocol, e.g. to save a session to

disk.

-  Several DOS commands can be used inside CONEX, such as "dir", "cd",

"del" or a DOS shell can be entered.

-  All incoming characters can be sent to a printer.

-  Files can be transfered to and from the mainframe via KERMIT

protocol.

-  Tektronix 4010/4014 and DEC VT102 can be emulated. The crosshair in

GIN mode can be moved by a mouse.

-  Graphical output can be displayed on a HPGL plotter or transfered to

other programs.

-  Up to 10 pages of screen output can be scrolled back to screen and

edited.

-  All keys including the numerical keypad can be programmed.

-  Incoming characters from the port can be remapped by the user.

-  The configuration of CONEX can be saved to an ASCII file, which can

be protected by a password or CONEX can be patched by itself.

-  Several sessions, especially net session, can be opened concurrently.

-  CONEX is completely written in assembler and therefore very quick

and small. The only file necessary to run CONEX is "conex.exe".

*======================*

|  How to start CONEX  |

*======================*

Simply start CONEX version 6.1 with

conex [host name|number] [options]

parameters

----------

host name   - connect to name of host in a network

(must have been set with Alt-I before!)

host number - enter session 0 - 9 (default: 0)

options

-------

/i<filename> - load different ini file (default: conex.ini)

*================*

|  DOS Commands  |

*================*

DOS commands can be executed nearly at any time using the alternate keys.

=> Alt-D(irectory) - list directory

When asked for "Dir mask:" you can enter filenames using wild cards ?

and *, e.g. you get all files ending with .asm if you enter "*.asm".

You can scroll the filenames using the cursor keys.

If you are to enter a filename e.g. with "Alt-E" or "S" in kermit

mode, you can switch to this screen with "page up/down". Multiple

files may then be selected with the keys "INS" or "DEL" or a single

file simply with "Return".

=> Alt-C(hange directory) - Change to another drive/directory

You can change to a new drive by adding "<drive>:" to your pathname.

=> Alt-E(rase)  - Erase file

Wild cards cannot be used when entering the filename, but you can

switch to a selection menu with "Page up/down".

=> Alt-X (eXecute) - execute DOS commands

Any valid DOS commands can be executed. But if you want to enter the

DOS shell completely give "Return". You return to CONEX with "exit".

*===================*

|  Port Parameters  |

*===================*

=> Alt-I - Initialize connection

The following parameters can be modified

Host:       name of the host to which you are connected. It is only

important if you are working on a network with LAT,

PC/TCP or BW/TCP.

Port:       Com, interrupt 14, LAT, PC/TCP or BW/TCP

Portnumber: 1, 2, 3, 4 (only COM/Int 14)

Baudrate:   50, 75, 150, 200, 300, 600, 1200, 1800, 2400, 3600, 4800,

9600, 19200, 38400, 57600, 115200 (only COM/Int 14)

Bits/parity:7 space, 7 even, 7 odd, 7 mark, 8 odd, 8 even, 8 none

Stopbits:   1, 2 (only COM/Int 14)

Duplex:     half, full

Handshake:  none, hardware, xon/xoff (only COM/Int 14)

Prompt:     You can enter the prompt of the connected machine here.

When you enter "return" in "edit mode" all characters

following this prompt are transmitted.

NOVELL offers interrupt 14 as a link to its programs, so you can use

it to connect in a NOVELL net.

LAT (Local Area Transport) is used to connect PC and mainframe via

DECNET. A resident program called "lat" must be loaded before using

this option. LAT is contained in DECnet-DOS which is available from

DEC (Digital Equipment Corporation). This software has been tested

with DECnet-DOS version 2.1.

With PC/TCP you can make a telnet connection on ethernet. The PC/TCP

kernel ("ethdrv") from FTP Software Inc. must be loaded before, it is

not public domain. The program has been tested with version 2.04/2.05.

PC/TCP is a registered trademark of

FTP software, Inc.		   anonymous file server

26 Princess Street		   vax.ftp.com (128.127.2.100)

Wakefield, MA 01880

(617) 246-0900

BW/TCP has the same features as PC/TCP. The internet number can be

entered directly as hostname or the hostname can be resolved with a

hosts file. Enter its location in Alt-Y. Hostname resolution with a

name server is not supported.

BW/TCP is a product from Beame & Whiteside Software Ltd.

Beame & Whiteside Software Ltd	   Tel + Fax:

P.O. BOX 8130			   (416) 648 6556

Dundos, Ontario

CANADA

L9H 5E7

=> Alt-S - Save all current parameters or Patch conex

The parameters are normally written to the file "conex.ini" and are

automatically read again when you start CONEX. If you select a

different name as save file you have to activate it with the command

line option "/i<filename>".

The following parameters are saved: port parameters (Alt-I), packet

parameters (kermit mode C-command), vt100-emulation (Alt-M), graphics

enabled (Alt-G), plot parameters (Alt-P) and status line (Alt-L) in

the different sessions 0 ... 9 and the keyboard macros. The save file

is an ASCII file which can be altered by a normal editor, see

"tech.ref" for more details. It can also be encrypted with a

password especially to protect the keyboard macros.

Replacing the ini-file by the exe-filename (normally "conex.exe") will

save parameters to conex.exe itself. Afterwards the INI-file is not

automatically read, lest you force CONEX to read it with the /i-option.

Password protection is not supported then.

=> Alt-0..9 - change to another session

Up to 10 different network session can be opened concurrently

depending on the limit of the resident network software. Port and

kermit parameter can be set differently in each session, the screen

is saved and restored, too. These sessions can be directly selected

with Alt-0 to Alt-9.

=> Alt-N(ame) - change to another session with host name

All sessions are listed and can be selected with the cursor keys.

Open sessions in a network are marked with an arrow.

=> Alt-B(reak) - send break

Sends a break over the serial line (short break), if Alt-B ist

pressed twice, the signals RTS and DTR are dropped for a short time

(long break). These commands are normally used to interrupt the

connection between mainframe and PC. In the networks NOVELL, DECNET

or TELNET a break is sent according to network standards.

*======================*

|  Terminal Emulation  |

*======================*

The PC can be used as a normal terminal. The folling modes can be

selected with function keys:

=> F7 - normal mode

All input characters are immediately sent to the mainframe.

=> F8 - edit mode, scroll back

In this mode the screen can be scrolled back and modified with the

cursor keys (home, end, left, right, up, down, page up/down, del,

ins, ctrl-end = delete to end of line). You can scroll back up to 10

screen pages. You can enter this mode with shift+cursor keys, too.

With "return" the cursor line is sent to the mainframe and CONEX is

switched back to normal mode. If a prompt has been defined with Alt-I

for this session it is stripped off. With ESC or F7 you return to

normal mode without transmitting anything.

=> F9 - kermit mode

Kermit must be started on mainframe to transfer files. (see below)

=> Alt-M - change emulation, character set

The PC behaves like a DEC VT102 terminal when it receives ESC

commands. Some keys send different code: function keys F1 - F4, cursor

keys, + key = return key on DEC terminal, the numerical keypad. The

keys "page up/down", "ins", "del", "home" and "end" send codes of an

VT202 terminal.

Character set 1 (characters 33-127) can be selected from ASCII, UK,

graphic, german and user. Character set 2 (characters 128-255) can be

selected from multinational and user. User character set is defined

in CONEX.INI.

=> Alt-L - change status line

You can select from the following items: 1. normal status line, 2.

empty status line, 3. status line with column numbers, 4. no status

line. The selection can be saved with Alt-S.

=> Alt-H - Help

This gives a list of all possible commands in CONEX, of modified

keys and last not least my address. Keys which have been replaced by

a single key are marked with an arrow.

=> F10 - quit CONEX

DTR and RTS can be dropped when you leave the program. If you changed

any parameters you are asked to save them. Network connections are

always closed.

=> Ctrl-backspace = DEL, Ctrl-return = LF

One of the unknown facts is that DEL (127) and LF (10) can be

produced this way.

=> Ctrl-NUM LOCK or PAUSE

Stop output to screen, any other key resumes output to screen again.

This command is executed by the normal keyboard device driver.

*==================*

|  Graphic Screen  |

*==================*

The program can switch automatically into graphic mode when it

receives a graphic commands and behave as a tectronix 4010 or 4014. The

following graphic cards are supported: hercules, CGA, EGA and VGA.

The graphic card is automatically detected on entry to CONEX.

=> F5 - enter graphic mode

The screen is switched into graphic mode. Some keys like Alt

combinations will not work any more in this mode.

=> Alt-G - enable graphic mode

When a graphics command GS, FS, US or ESC ENQ, ESC FF, ESC SUB is

received CONEX will automatically switch into graphic mode. You can

switch back to normal by pressing F7.

In GIN mode the crosshair can be moved using the cursor keys; the

shift key speeds up this movement. If a Microsoft compatible mouse is

connected and the mouse driver loaded you can move the crosshair with

the mouse as well.

=> Alt-P - Write plot file

The graphics can be written to a file in HPGL format and plotted on a

HPGL compatible plotter later on. Pagewidth, margins and velocity of

the pen can be selected as well.

*======================*

|  Writing Key Macros  |

*======================*

=> Alt-K - Write key macro

All keys can be programmed, if they are detected by BIOS (Int 11h)

calls. Characters in a macro are sent out at a rate of 10

character/sec.

The following keys have special meanings in programming:

F1	  begin command character, here displayed as "<"

F2	  end command character, here displayed as ">"

F3	  seperation character in commands, here displayed as "|".

F4	  enter function, alt, control keys literally

press F4 followed by special key

�IN0   virtual keys which must be entered using F4,I,N,#.

...	  When a new connection is opened the corresponding macro

�IN9   IN# is automatically started. On com ports a new connection

is determined from the presence of DTR and CTS.

The following combinations have special meanings in macros:

<W#>

Wait for # seconds. # = 1...9

<R<string1> | <string2> | ...>

Wait until the mainframe sends <string1> or <string2> or ... The

strings must be seperated with key F3 "|".

<D#1 | #2 | ...>

Dial telephone number #1, if it is occupied try next number #2

... The special string sent before or after the phone number

must be set with Alt-Y, especially prefix (string before

number), suffix (string after number) and connect|fail (answer

of mainframe if connected or connection failed).

- Labels are ignored if you replace a key by a single key.

- A memory region of 10000 bytes is reserved for keyboard macros.

- During the execution of a macro inputs from the keyboard can be

done, only ESC stops the macro.

=> Alt-Y - configuration of CONEX

You can enter strings to dial a telephone number and page length

of printer which is activated by Ctrl + Print Screen. The dial

sequence is preset to Hayes modem.

*==========================================*

|  Transfer of Files without any Protocol  |

*==========================================*

=> Alt-W - write file

Use this command to save all incoming characters to a file either to

remember what you were doing or to transfer files. The transfer may

be interrupted with the Alt-W.

=> Alt-R - read file

The file is transfered to the mainframe as it is, especially CR and

LF at end of line are sent. The transfer can be interrupted with

Alt-R.

=> Control + Print Screen - print incoming characters

All incoming characters are transfered to the printer. If a page

length is defined with Alt-Y, the printed pages will be numbered.

Pressing Control+Print Screen twice produces a form feed and page

numbering begins anew.

*=================================*

|  Transfer Files in Kermit Mode  |

*=================================*

You must start kermit on the mainframe and enter "server", "receive" or

"send", then press F9 to enter local kermit mode to start the file

transfer.

=> I(nit parameter) P(acket)

These parameters determine how the data packet will be build and

normally need not be altered.

packet length - length of a packet including packet number, type and

checksum. (default 94)

padding character - characters sent before any packets to synchronize

transmission. (normally not necessary)

quote control character - control character are quoted with this

character, Control-A is coded as '#A'. (default #)

quote 8-bit character - characters whose 8th bit is set are quoted

with this character, hex A8 is coded as &H. (default &)

repeat character - 8 characters 'HHHHHHHH' is shortened to 3

characters ~'H, 8 is coded as '. (default ~)

type of checksum - CONEX supports 3 kinds of checksums, which consist

of 1, 2 or 3 bytes. (default 1)

packet terminator - Every packet is terminated with this character.

(default = CR)

=> I(nit parameter) C(ommon)

ASCII/binary file type - Binary files (files with characters over 127)

should be transfered with file type binary. The mainframe must be

set to this mode, too, normally with "set file_type binary".

Sometimes binary files can also be transfered with "parity none",

which is quicker.

Repeating - repeated characters are compressed in a packet. (see

repeat character)

Max number of retries - number of trials to resend a packet, which

has been refused by the mainframe.

Time out in secs - If the PC did not answer for the specified time

the last packet is resend.

=> S(end) - send files

Multiple files seperated with blanks and "wild cards" can be entered.

If you want to select from a file list, press "page up/down" and

select with "ins" or "del".

=> R(eceive) - receive files

This command is used to get files from the mainframe. If this file

already exists on the PC the old one is renamed to ".bak".

If the sender has already been told the name of the file ("send

foo"), simply press return when asked for "receive file". It is also

possible to enter "wild cards" which are understood by kermit at the

mainframe.

=> T(race) - handles output of packets to screen

trace errors - display all error packets on screen

trace all	- display all packets on screen

=> F(inish) - terminate server on the mainframe

Kermit on the mainframe is terminated and CONEX changes back to

normal mode.

=> L(ogout) - terminate session on the mainframe

Like "finish" but terminate session, too.

=> cursor keys

Scroll the messages at the bottom of the screen.

=> C(hange directory) - change directory on the mainframe

This and the following commands only work, if kermit on the

mainframe supports generic commands.

=> D(irectory) - list directory on the mainframe

=> E(rase) - erases file on the mainframe

=> N(ame) - rename file on the mainframe

=> U(sage) - query disk usage

=> M(essage) - send a short message

=> Q(uery server) - query server status

=> During a file transfer the following commands can be used.

ESC - interrupts transmission completely. This should be the last

resort if nothing else works.

^T  - change trace mode (= "T" in main menu)

^F  - interrupts the transfer of current file, the transmission

proceeds with the next file.

^G  - interrupts file transfer completely.

@   - enter debug mode: trace is set on; packets are only transmitted

if a key is pressed

CR  - resend last packet, if transmission stopped

*====================*

|  error characters  |

*====================*

Some characters, which are output to screen, have special meanings:

Blinking character  - Parity or Framing error: A bad character was received.

Alter bits/parity, if all characters are blinking.

rubout (dec 178)    - Overrun error at COM port. Too many characters have

been received at the port.

up arrow (dec 24)   - Sender time out: Characters could not be sent out.

In a network environment the connection is

interrupted.

down arrow (dec 25) - Receiver time out: Characters could not be received

from remote host. The connection is interrupted.

*============*

|	Problems  |

*============*

1. Interrupt numbers >7 on COM have not been tested.

2. Characters in Tektronix mode are not transmitted to HPGL file.

*===============*

|  Past history |

*===============*

6.0   1st Public Domain Version in comp.binaries.ibm.pc

6.1   default COM3/4 Interrupt = 4 and 3

COM3/4 Interrupt + IO address configurable

up to 4 LAT sessions

ESC FF, ESC SUB, ESC ENQ automatically switch into graphic mode (Alt-G)

support extended keyboard (esp. F11, F12)

rotate interrupts configurable in conex.ini

format of CONEX.INI changed!!!

dial sequences are saved in CONEX.INI

start initial macro at 1st opening of a session (Alt-K, �IN#)

characters from port can be remapped (Alt-M, character set: user)

conex can be patched (Alt-S, file name: conex.exe)

support B&W TCP/IP (Alt-P)

ESC stops macros only in normal or graphics mode

alphabetically sorted output in directory listing

alphabetically sorted output of macros in help screen

multinational supplement character set implemented

ESC ENQ supported in graphics mode

hercules graphic screen increased to 720 X 360 pixels

6.2   default setting changed to bits/parity: 8 none, emulation: VT100

single key replacement shown with Alt-H

default kermit packet size = 2000 bytes (long packet)

*================*

|  Future plans  |

*================*

- more rows, columns on EGA/VGA card

- FTP file transfer

- direct access to packet drivers

- emulate tektronix 41XX (I am looking for a description!! Send me a Mail.)

- support kermit larger packet size

- support other file transfer protocols

*==========*

|  Author  |

*==========*

Erhard Hilbig

Hochschulrechenzentrum

Universitaet-Gesamthochschule Paderborn

Postfach 1621

W-4790 Paderborn (Germany)

e-mail: zahilb@pbhrzt.uni-paderborn.de

phone: 05251/60-2433	       room: N2-316

FAX:   05251/60-3207

This program has been developed by me and can be distributed

freely. There is also a more detailed documentation in german

available, which is written in LaTeX. You can have it as LaTeX

DVI file or as a print file, which must be output using

"copy conex.lst prn/b".

If you find any bugs or have an idea how to improve the program,

send me a message!