The Promidi File Format.

Systems Design Associates, Inc.

Version 1.0

March 1, 1989

This document describes the format of the data in playable Promidi files.

These MSDOS files are identified via a file extension of "Vnn", where n

is a digit from 0 through 9.

There are three types of data in the Promidi File Format:

The Fence Byte,

Timestamped Midi Messages, and

Mark Messages.

A Promidi File is a series of 1 or more Midi beats.  Each beat

may be considered to consist of 192 clock tics.

Between each beat is a fence byte.  The value of the fence byte

is 192 (C0 hex).  Each Midi message has a leading timestamp byte

with a value of 0 - 191.  This timestamp represents the clock tic

in which the Midi message is to be sent, relative to the beginning

of that beat.

Mark messages are not timestamped.  They are distinguished from

timestamped Midi messages and fences by the fact that their first

byte is greater than 192.  There are single, 2-byte, and 8-byte

mark messages.	The length of a mark message is determined by

its first byte as follows:

193 - 223			Single byte mark messages.

224 - 238			2-byte mark messages.

239 			8-byte mark message.

240 - 255			Single byte mark messages.

Mark messages (if any) are the first messages in a given beat,

leading any timestamped Midi messages.

Running status is used in Promidi files, primarily to increase

throughput (the Midicard plays and records running status).

The Fence byte is between beats, therefore there is no fence byte

at the beginning of the file; ie, if a Fence is the first byte

in a Promidi file, it means that there is at least 1 empty beat

at the beginning of the file.  3 consecutive Fences would mean

at least 3 empty beats (each with 192 tics), etc.

Mark Messages of Interest.

Tempo

E0

<tempo data>

The Tempo Data consists of a byte in the range 10 - 250 (decimal),

where 10 is the value for the fastest tempo and 250 the slowest.

The formula for determining the value given beats per minute is:

X = 7622 / Beats Per Minute

Like all 'mark' messages, these should appear at the beginning of

a Midi beat, that is to say, before any timestamped Midi messages

in that beat.

Relative Tempo

E2

<rel. tempo data>

Relative tempo data is a signed byte that is added to the

Midicard's current Tempo Data.  A -1 (255) increases the tempo,

and a 1 decreases it.

As a special case the values 128, and 129 (decimal) are used

to half and double time the Midicard (respectively).

128 will multiply the Midicard's current Tempo Data by 2,

thereby half-timing the tempo.

129 will divide the Midicard's current Tempo Data by 2,

thereby doubling the tempo.

Time Signature

E4

<time signature data>

Time signature data is a byte that determines the time signature

for the Midi Data following the mark message.

7   6   5     4	3   2	1   0

+-------------+---------------------+

| Beat Equals |   Beats Per Meas.   |

+-------------+---------------------+

Where:

Beat Equals =  "beat equals" indicator

Beats Per Meas. =   The number of beats per measure 1 - 20.

"Beat Equals" reflects what kind of note represents a beat:

Beat Equals value:		 Note representing a beat:

0				16th

1				8th

2				dotted 8th

3				qtr.

4				dotted qtr.

5				half

6				dotted half

The numerator and denominator of the time signature may be

calculated as follows:

Beat Equals value:		    Denominator value:

0				   16

1				   8

2				   16

3				   4

4				   8

5				   2

6				   4

Beat Equals value:		    Numerator value:

0			    Beats Per Meas.

1			    Beats Per Meas.

2			    Beats Per Meas. * 3

3			    Beats Per Meas.

4			    Beats Per Meas. * 3

5			    Beats Per Meas.

6			    Beats Per Meas. * 3

Key Signature

E7

<key signature data>

The key signature data determines the number and kind of any accidentals.

7   6   5   4	3     2   1   0

+-----------------+-----+-------------+

|		     | Kind| Accidentals |

+-----------------+-----+-------------+

Where:

Accidentals =	    0 - 7

Kind =		    0	Sharps

1	Flats