It is recommended that you print this out and then run the program

and experiment as you read.

==========

Audiotool was written to provide direct access to the Amiga's sound

generating hardware.  The Amiga has four Direct Memory Access (DMA)

channels and four Digital to Analog (D/A) Converters (also called DACs).

The DACs feed two audio line outputs (the left and right jacks on the back).

In the normal operating mode, a DMA produces a stream of numbers

(representing an audio waveform) to a DAC which converts it (with help from

a filter stage and other hardware) to an analog signal which is then sent

to a line output (represented by a speaker).  The line output provides

a continuos analog signal varying in strength within the limits of

+/- 0.4 Volts.  If the line is connected to an amplifier and speaker,

the speaker vibrates, generating air vibrations we call sound.

The timbre of the sound (the quality of a sound which differentiates

it from others of the same frequency and amplitude) is controlled by the

stream of numbers feeding the DMA.  These are represented by four waveform

representations in boxes (sine, square, ascending and descending sawtooth

respectively).  You may select one by clicking on a box with the left mouse

button.  The pitch or frequency of the sound is determined by two factors,

the number of samples in the waveform representation, and the sampling

frequency being used.  Each number in the waveform representation is called

a sample.  The sequence of samples (in this program) represents one waveform

cycle.  One waveform cycle is one up and down movement of a wave (as

represented by the waveform boxes).  The top row of numbered boxes

represent the number of samples in the waveform.  A different number may be

selected by clicking in a box with the left mouse button.  The more samples,

the lower the output frequency.

The sampling frequency is the number of samples that are output from

the DMA every second.  This is controlled by the top slide pot.

This pot may be thought of as a speed control for the DMA.  The number

selected by this pot is displayed in the box just above the slide pot

on the left.  This box is also a numeric input gadget.  By clicking the

left mouse button in this box and then using the delete, backspace, and

numeric keys, you may enter a new number.  But its easier to just hold

the left mouse button down on the slide pot and move it back and forth.

As the number selected increases, the output frequency decreases.

The output frequency is displayed above the slide gadget to the right

in Hertz.  One Hertz is one cycle of sound per second.

The DAC also has a slide pot, it controls the volume or amplitude of

the sound (how loud it is).  It operates similar to the other slide gadget.

The bigger the number, the louder the sound.

The outputs of the DACS are hardwired to the line outputs.  In other

words, there is no way to move a particular DAC's output to the other line

(left or right) instead of the one it is connected to.

The DMA inputs and output are much more versatile however.  Each DMA

has an on/off switch.  It is the crude knife switch image just under the

DMA box.  Each knife switch may be toggled on/off or up/down by clicking

the left mouse button over the image.  (Yes, yes, always the left button,

the right button is useless with this program, there aren't even any menus

for Pete's sake.)  The speed of a DMA controller may be switched from the

manual pot to the output of the DMA on its left.  This means that the

frequency of the right DMA's output will vary by the waveform selected as

input by the left DMA unit.  Try this example, turn on the second DMA and

select a nice audible tone.  Now turn on the first DMA and select a

frequency of about 2 Hz.  To do this, select the 128 samples box and then

move the frequency slide to the right.  As you get below 30 Hz, the sound

will become inaudible except for a clicking sound if you have selected a

waveform with a sharp edge.  Now click the second DMA's top toggle switch

(the one that disconnects the DMA from the frequency slide gadget and

connects it to the first DMA's output).  Now select different waveforms.

Fun, no?

In the same way, the volume of a DAC may go from a steady level under

manual control to modulation by the DMA channel on it's left.  You can

even modulate both frequency and volume at the same time.

Features:

There is no way to quickly switch the waveform input of a DMA

channel.  One must either turn off the DMA channel for more than two

samples or let the hardware automatically reload its input address

register after it finishes its current cycle.  This program uses the

latter.  It becomes apparent when small frequencies are being used.

Waveforms will not switch immediately when alternate waveforms are

selected.  When switches are thrown changing the routing of DMA output,

some strange sounds are produced until the DMA resets itself with the

new waveform.  Sometimes, the DMA does not recover by itself.  In these

cases, the former method may be manually used by clicking off the DMA,

and then clicking it back on.

Equations:

The frequency slide pot produces a number which is output to the

Amiga's period register.  It is a clock divider value.  It divides the

master clock rate of 3,579,545 ticks per second to produce the sampling

frequency used by the DMA unit.

Letting         P = period register value

N = number of samples per cycle in waveform representation

F = output frequency

cycles             ticks               ticks                samples

F ------ = 3,579,545 ------ divided by P ------- divided by N -------

second             second              samples              cycle

The volume level is represented in decibels.  This is the ratio of the

output level to the maximum output level.  The volume register can take on

values from 1 to 64 (0 is off) yielding dB values according to the following

formula:

Letting         V = volume register value

D = decibel rating

D = 20 * common log ( V / 64 )

Technical notes:

I have tried to produce a program that graphically interacts with

the Amiga hardware that is simple yet not limiting.

The DMA on/off switches each represent one bit in the DMA control

register (DMACON).  The remaining three switches per channel represent

two bits in the audio control register (ADKCON).  Hence not all switch

combinations are legal.  Opening up the switch that controls the feed

into the top of a DAC for instance, must force the closure of at least

one of the pot control switches for the channel on the DAC's right.

The program arbitrarily closes the period control switch in this

case because the author feels this is the most interesting case of the

three.  Here is a chart showing the bit values and the corresponding

switch positions.

feed     feed       "DAC feed"   "period feed"   "volume feed"

period   volume        switch        switch          switch

0        0           closed         open            open

1        0            open         closed           open

0        1            open          open           closed

1        1            open         closed          closed

To allow a full range of frequencies, I had to split the operation

into six octaves.  The middle four allow the frequency pot to select

period values from 200 to 400.  This is to allow reasonable operation of

the slide gadget.  However, any valid period value may be input with

the numeric keys.  Valid values are 124 to 65,535 inclusive.  The outer

two octave ranges lift the normal value restrictions to allow for more

than a full octave.  Here is a chart that shows what I mean.

"samples"    Pot selectable       Frequency

number      period values          range

4          124 to   400    7.2 kHz to 2.2 kHz

8          200 to   400    2.2 kHz to 1.1 kHz

16          200 to   400    1.1 kHz to 560  Hz

32          200 to   400    560  Hz to 280  Hz

64          200 to   400    280  Hz to 140  Hz

128          200 to 65535    140  Hz to 0.43 Hz

The Amiga 1000 has a low pass filter that operates from 4 kHz to 7 kHz.

It is possible to hear antialising distortion above 5 kHz with this program

for this reason.

Each octave range has its own set of samples for each of the four

waveforms.  This means that there are 6 * 4 or 24 sets of samples, not

just four.  But wait, its still more complicated yet.  The DAC's take

values ranging from -127 to 128, so that when a DMA is directly feeding

a DAC, the waveform samples must have values ranging from -127 to 128.

But when a DMA is switched to modulate the next channel's period register,

it must produce valid period register values.  These values range from 124

to 65,535 as we have seen.  This means another 24 sets of waveform sample

values.  When modulating periods, the samples take on values from 200 to

400, thus modulating frequencies by exactly one octave.  When modulating

the next channel's volume register, values from 10 to 64 are used, thus

modulating volume by 16 decibels.  When modulating both volume and period,

twice as many samples are required.  One for volume, then one for period,

etc.  Therefore there are:

4 (waveforms) * 6 (octave ranges) * 4 (operating modes) = 96 (sets of samples)

The program automatically switches between these as required to make

operation seem simple.

The source code is available to answer other questions.

This program expects to run in a 640 x 200 sized workbench window.

A custom window may be opened instead by making a quick change to

window.c.

Comments, ideas, code improvements, and questions may be sent

to the author.  No reply guaranteed.

David Reynolds

32 Greenwood Rd.

Burlington, Ma  01803