|

|                         TALKER

|

|

|

|

|             Direct Numerical Control Software

|

|                        Written by

|               Computers in Action. (c) 1992

|

Computers in Action                          TALKER User Manual

This documentation has been written for the disk based manual of

TALKER and therefore can not show the screen dumps that are

littered trough out the paper manual.

This program has been designed for users of CNC machines to store

and retrieve their programs in a time efficient way.

Keyboard input is simplified throughout TALKER, by using a user

friendly method of pull down menus.

Using the benefits of your Personal Computer and the way TALKER

has been written, CNC machine programs can be kept together so

that one type of program files are kept separate from another

machines program files, or the program files for one customer can

be kept separate from the program files of another customer.

Computers in Action                          TALKER User Manual

>>>>>>>>>>>>>>>>>>>> Hardware Required <<<<<<<<<<<<<<<<<<<<

TALKER requires a PC with at least 512k of ram and a hard disk.

TALKER makes no special demands on your computers display,

any display can be used.  TALKER works in text mode on your

computer and does not need colour any display type can be used

but a minimum of a colour CGA screen is advisable).

>>>>>>>>>>>>>>>>>>>> Acknowledgments <<<<<<<<<<<<<<<<<<<<

TALKER was written using Turbo Pascal from Borland,

TechnoJock's Turbo Toolkit from TechnoJock Software and Turbo

Asynch Plus from Blaise Computing.

This user manual was entirely written using Wordstar 5.5 from

Wordstar International Corporation and the screen dumps were

reproduced using Inset from Inset Systems.

All trademarks acknowledged.

First Edition, 3 rd Print  ( Disk Version )

January, 1992.

Computers in Action.

29 Branwell Close.

Christchurch.

Dorset

BH23 2NP.

Computers in Action                          TALKER User Manual

>>>>>>>>>>>>>>>>>>>> Table of Contents <<<<<<<<<<<<<<<<<<<<

Page

Chapter 1

How to use this Manual ........1.1

Conventions in this Manual ....1.2

What you Type .................1.2

File name Input ...............1.2

Chapter 2

Installing Talker .............2.1

Backup of Talker ..............2.1

Chapter 3

Quick Install .................3.1

Input Method ..................3.1

Current Machine ...............3.1

Current FilePath ..............3.1

Heidenhain Control ............3.2

Setup .........................3.2

Blockwise .....................3.2

Language ......................3.2

Jump to Change ................3.2

Communication Port ............3.2

Baud Rate .....................3.2

Parity ........................3.2

Databits ......................3.2

Stopbits ......................3.3

Chapter 4

How to use the Menu's .........4.1

Main Menu Options .............4.1

Main Options ..................4.1

Sub-Menu Options ..............4.2

Getting Help ..................4.2

Chapter 5

Communication Menu Options ....5.1

Send a Program ................5.1

Receive a Program .............5.2

Terminal ......................5.2

Change ........................5.2

Computers in Action                          TALKER User Manual

Chapter 6

Programming Menu Options ......6.1

Edit a program ................6.1

Print a Program ...............6.1

Renumber a Program ............6.2

View a Program ................6.2

Change ........................6.2

Chapter 7

Tools Menu Options ............7.1

Edit a Tooling File ...........7.1

Print a Tooling File ..........7.2

Change ........................7.2

Chapter 8

Install Menu Options ..........8.1

Add a Machine Parameters ......8.1

Current Machine .............8.2

FilePath ....................8.2

Heidenhain Controller .......8.4

Setup Number ................8.4

BlockWise ...................8.5

Language ....................8.5

Jump to Tool ................8.5

Port ........................8.6

Baud Rate ...................8.7

Parity ......................8.7

Stopbits ....................8.8

Databits ....................8.8

Delay .......................8.8

Timeout .....................8.9

Echo ........................8.9

File Extension ..............8.9

Qty of EOL Terminators ......8.10

EOL Terminators .............8.10

Starting Number .............8.11

Step ........................8.11

Line Prefix .................8.11

Good Characters .............8.12

Program Start ...............8.12

Program End .................8.12

Leading Zero's ..............8.12

Number of Zero's ............8.13

Modify Machine Setup ..........8.13

View the Machine Setup's ......8.14

Delete a Machine Parameter ....8.14

Save Machine Parameters .......8.14

Change ........................8.14

Computers in Action                          TALKER User Manual

Chapter 9

System Menu Options ...........9.1

Printer Setup .................9.1

Changing Printer Codes ......9.2

Paper Setup ...................9.3

Length of Paper .............9.3

Page Num Line Number ........9.3

Number of Print Lines .......9.4

Form Feed at Page End .......9.4

Page Number Label ...........9.4

Page Paper Width ............9.4

Printed Page Offset .........9.4

Heading each Page ...........9.5

System Setup ..................9.5

Software Licensed to ........9.6

System Date .................9.6

Main Password ...............9.6

Printer File Name ...........9.6

Editor File Name ............9.6

Directory Manager .............9.7

Colour Setup ..................9.8

Import a Program ..............9.9

Support .......................9.9

Chapter 10

The End Menu Options ..........10.1

Quit and Save .................10.1

Abort (No Save) ...............10.1

Appendix A

The Editor ....................A.1

The Editing Screen ..........A.1

Movement Commands ...........A.2

Insert & Delete Commands ....A.4

Block Commands ..............A.5

Search Commands .............A.6

Exit Commands ...............A.7

Appendix B

Directory Manager .............B.1

Help ........................B.1

Reset Drive .................B.1

Make a Subdirectory .........B.2

Rename a Subdirectory .......B.2

Delete a Subdirectory .......B.2

View File Attributes ........B.2

Delete a File ...............B.2

Rename a File ...............B.3

Computers in Action                          TALKER User Manual

Change Directory ............B.3

Information .................B.3

Log to Another Drive ........B.3

Information .................B.3

Home key ....................B.3

End Key .....................B.3

Cursor Movement .............B.4

Choose a File ...............B.4

Exit Key ....................B.5

Appendix C

Help System ...................C.1

Next Page ...................C.2

Previous Page ...............C.2

Index Page ..................C.2

Find Text ...................C.2

Appendix D

Technical Details .............D.1

Serial Communication ........D.1

Databits ....................D.1

Parity ......................D.1

Baud Rate ...................D.2

Handshaking .................D.3

Program Transfer ............D.3

Load Memory .................D.3

Drip Feed ...................D.3

Screened Cable ..............D.4

Parity Check ................D.4

Transfer Blockwise ..........D.5

Warning ( Parameters ) ......D.5

Appendix E

Examples of Cabling ...........E.1

Standard ....................E.1

Analam Crusader .............E.1

Heidenhain ..................E.2

Emcotronic ..................E.2

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  1 <<<<<<<<<<<<<<<<<<<<

How to use this Manual

Read this manual in conjunction with using TALKER.  You are

urged to read all up to and including chapter three and refer

to the other chapters and appendixes as required.

Chapter 1 explains how to install TALKER on your hard disk.

Chapter 2 is for the man/woman in a hurry.  The quick install.

Chapter 3 describes how to use the menu system.

Chapter 4 explains the menu options available from the

Communication Menu.

Chapter 5 explains the menu options available from the

Programming Menu.

Chapter 6 explains the options available from the Tools Menu.

Chapter 7 explains the options available from the Install Menu.

Chapter 8 explains the options available from the System Menu.

Chapter 9 explains the options available from the End Menu.

Appendix A gives the complete instructions on how to use the

multi feature editor.

Appendix B instructions on the use of the Directory Manager.

Appendix C explains the use of the Context sensitive help.

Appendix D Communication Tecnical details.

Appendix E examples of RS232 cabling connections.

Computers in Action         Page 2           TALKER User Manual

Conventions used in this Manual

What you type

In this manual anything that you have to type will shown

between < > these two characters, and in upper case.

eg.  <TALKER>

Key names will be in shown between [ ].

eg.  [Enter]

or

[Pg Up]

The Enter key is sometimes called Return or New-Line, However

in this manual it will always be called Enter.

When two keys are to be pressed together, they are seperated by

a hyphen.  The first key will be held down and then the other

key pressed ( see (a) below ). If more than two keys are to

pressed then follow the above rule, release both keys and press

the next key ( see (b) below ).

eg.  a. [Ctrl-K]

Press and hold the [Ctrl] key then press and release the [K]

key, then release the [Ctrl] key.

b. [Ctrl-Q F]

Press and hold the [Ctrl] key then press and release the [Q]

key, then release the [Ctrl] key.  Then press and release the

[F] key.

File Name Input is achieved throughout TALKER by a pop up box.

The first line reminds you what you are entering a file name

for ( eg. send a file ).  The centre line is where you actually

type in the file name.  The  bottom line of the box displays

the default.  This default displayed is usually the last file

name used, if the default is the required name then just press

[Enter].

If the file name you require is not the default and you do not

know the name then enter a wildcard character ([*] and/or [?])

and press [Enter].  On doing this you will invoke what we have

called the Directory Manager ( see appendix B ).

The Directory Manager now shows all the files that match the

wildcard format.  You may now move around the list of files

using the cursor keys and select the file require by pressing

[Enter].

Computers in Action         Page 3           TALKER User Manual

Wildcard's are special characters used by TALKER (and are the

standard one's used in DOS).  These two characters are * and ?.

The * can be substituted for one or more of the file name while

the ? takes the place of only one character.  The wildcard

characters can be used in the file name or extension.

eg.  *.BA? will match all files in the selected

directory that have three letters in the extension

the first two which are B and A.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  2 <<<<<<<<<<<<<<<<<<<<

Installing TALKER

Set up your PC as shown in the owner's manual. Switch on your

machine and when DOS has loaded the DOS prompt will show.

The DOS prompt is usually a alphabetical character and a

flashing cursor.  If you have more than one hard drive then the

next letter in the alphabet is used.

eg.  Single Floppy Drive, One Hard Drive Computers.

Floppy Drive is usually ...........A

Hard Drive is usually .............C

Double Floppy Drive, Two Hard Dive Computers.

1 st Floppy Drive is usually ......A

2 nd Floppy Drive is usually ......B

Boot Hard Drive is usually ........C

2 nd Hard Drive is usually ........D

Insert the TALKER distribution disc in a floppy disc drive.

Now run the install program by entering.

<A:Install>   or  <B:Install>.

followed by [Enter].

The screen will now ask for you to confirm the Source drive

(this is the drive that contains the distribution disk).

Only A or B will be accepted.

Enter the destination of the TALKER files.  Enter hard disk and

subdirectory location.

Default = c:\Talker\

If the subdirectory is not present on the hard drive you will

be warned, and asked if it OK to make it.  Answer <N> and you

can retype the destination again.

Next enter your name or the name of your company.  This name is

used in printouts.  This name can not be altered once you have

installed TALKER.

Computers in Action         Page 2           TALKER User Manual

Once you have entered all the information you will be prompted

to continue with the installation or to start again and re-enter

the information.  Once you are satisfied all files will be

copied to the destination location.

At any time through out the installation you may press the

[Esc] key to quit.

A batch file called TALKER.BAT is created in your Root

directory of your hard disk. When finished the message 'Ok you

are now ready to run TALKER' is displayed on your screen and

the DOS prompt will reappear.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  3 <<<<<<<<<<<<<<<<<<<<

Quick Install

This chapter is for the user who has not the time to read this

manual completely but wants to get up and running as soon as

possible.

Once you have installed TALKER on to your hard drive, (the

details of which are in Chapter Two).  Type <TALKER> followed

by [Enter].

Press the [I] key.

Press the [A] key.

You are now in the Add a machine parameter option.

Throughout this option you will be required to answer the

questions in one of two methods:-

1:   Type in the information asked for and press

[Enter] when finished.

2:   Use the [Left Arrow], or [D] keys to go down

and the [Right Arrow], or [U] keys to go up to

the number or string displayed, press [Enter] to

confirm your choice.

The help message at the bottom of the screen is there to

remind you of which method is to be used.

Current Machine Selected

Type in a name of the machine.

FilePath

Type in the location of the CNC programs.

Heidenhain Control

Set this to 'No' if the machine tool is not a

Heidenhain controller, otherwise select the type

of controller.

Computers in Action         Page 2           TALKER User Manual

Setup

Set this to the same number as your User

Parameters for your Heidenhain controller.

Usually setup to be 168.  This option will not

become available if you have selected 'No' to the

prompt 'Heidenhain Control'.

Blk

Which is short for BlockWise.  Set to 'No' if you

are not going to use BlockWise, otherwise set to

'Yes'.  This option will not become available if

you have selected 'No' to the prompt 'Heidenhain

Control'.

Lang

Which is short for language.  Set this to the

language type of 'ISO' (G codes) or 'DIALOG'

(Heidenhain language).  This option will not

become available if you have selected 'No' to the

prompt 'Blk'.

Jump

Set to 'Yes' if you want to be asked if you wish

to jump to a tool change position in the program.

This option will not become available if you have

selected 'No' to the prompt 'Blk'.

Port

Select the port to which the CNC machine is connected.

Baud

Select the current Baud rate set on the CNC machine.

Parity

Set to the same as the CNC Machine. Parity not

selectable if a Heidenhain controller is being

used.

Databits

Set to the same as the CNC Machine. Databits not

selectable if a Heidenhain controller is being

used.

Computers in Action         Page 3           TALKER User Manual

Stopbits

Set to the same as the CNC Machine. Stopbits not

selectable if a Heidenhain controller is being

used.

F10

Use this key once you have finished the

installation or at any time throughout the

installation.  You will be prompted to confirm

you have finished.

Now you are returned to the main menu, press the [S] key

or select Save from the sub-menu to save this parameter.

You are now ready to start sending or receiving programs.

See chapter 8 for more details of the Installation of

machine parameters.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  4 <<<<<<<<<<<<<<<<<<<<

How to use the Menu's

At the DOS prompt, change to the root directory with the

command <CHDIR .> followed by [Enter] then type <TALKER>

followed by [Enter], the Main Screen will be displayed.

Main Options

The top of the screen shows the Main Menu Options.

The Main menu consists of six options with the first,

Communication, being Highlighted when you first run

TALKER.

When you select another menu option the Highlighting

bar will then move to that option.  To move the

Highlighted bar across the menu options use the left

or right arrow keys.  On reaching the option required

press the [Enter] key or down arrow key and the

sub-menu options will appear.

The other method is to use the 'HOT KEY', just press

the first letter of the option required on the

keyboard and the sub-menu options will appear.  The

'HOT KEY' letter is Highlighted.

Sub-Menu Options

The Sub-Menu options are the options that are pulled

down from the Main options on the top line.  Use the

up and down arrow keys to move the cursor bar through

the options available and press [Enter] when you have

reached the option required.

A faster method of using the menu, is to use the

#'HOT KEY'# method.  Again, this is the first letter

of the option name, by pressing the key on the

keyboard will select the option without you having to

press the [Enter] key.

Once a pull down menu is displayed selection of the

other pull down menus can be displayed by using the

left and right arrow keys.

Help

Help is available at any time.  If you are in any

doubt when you are being asked for a keyboard input,

pressing the [F1] key will bring into action the

context sensitive help. (see appendix C for details).

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  5 <<<<<<<<<<<<<<<<<<<<

Communication Menu Options

Send.

There are two methods of sending a program file to

the CNC machine.

1.   Make the CNC machine ready to receive the

program, using the normal methods for

receiving a program.  Then move to the

computer and select this option.  You will

be prompted to input the name of the file

you wish to send to the CNC machine ( see

Input File Name ). Choose the program file

by pressing [Enter]. A box will now appear

and show the file line by line as it is sent

to the CNC machine.

2.   Once you have selected this option you will

be prompted to input the name of the file

you wish to send to the CNC machine ( see

Input File Name ). Choose the program file

by pressing [Enter].  A box will now appear

but TALKER waits for you to walk to the CNC

machine.  Once at the CNC machine, select

the receive a program option. The CNC

machine now informs TALKER that it is ready

to Receive.  The file is now sent to the CNC

machine and file can be displayed line by

line as it is sent.

The protocol used to send this program file will be

the machine parameters you have setup in the Install

menu option. The Current Machine and Current Filepath

parameters are displayed on the main screen.

Computers in Action         Page 2           TALKER User Manual

Receive.

Use this option to receive a program from the CNC

machine and at the same time save the information to

disk.

Once selected you will be prompted to input a file

name for the program you wish to store ( see Input

File Name ).

If the file name already exists, you will be prompted

to confirm you wish to Overwrite.

If the file name is new then you will be prompted to

confirm that it is a new file name.

After you have selected or input a program file name

and answered 'Y' to one of the above questions

TALKER is ready to receive the program.  Now send the

program to your computer from the CNC machine.

As the program is received by TALKER it is displayed

on the screen line by line. TALKER saves the program

using the name you have supplied and in the location

specified by you ( which is shown in the Current

Filepath box ).

Note:   If you do not want to see the file line

by line coming from the CNC machine then

set the 'ECHO' to 'NO' in the install

option.

Terminal.

On selecting this option you can then 'Talk' directly

to the CNC machine.  This is similar to the old

teletype machines.  Not all CNC machines can be

addressed in this way.

Change.

This option allows you to change from one machine

setup to another.  The new setup is shown immediately

in the two boxes on the screen.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  6 <<<<<<<<<<<<<<<<<<<<

Programming Menu Options

Edit

This option allows entry into the multi feature

editor.  Once selected you will be prompted to input

the name of the file you wish to edit ( see Input

File Name ).

If the file name is new then you will be prompted to

confirm that it is a new file name.

All the edit features are documented in Appendix A.

Note:    If you do not want to use the editor

supplied with TALKER but prefer to use an

editor that you are more familiar with,

then this is possible by entering the

full filepath and name in the System

Setup.

Print

The print out of your program can be made by

selecting this option.  Once selected you will be

prompted to input the name of the file you wish to

print ( see Input File Name ).

Printing of a program uses the conventional parallel

printer port ( LPT1 ) and uses the Printer codes

setup in the Printer Setup and Paper Setup.

Renumber

This option uses the defaults you have setup in the

Install option to renumber your program.

You will first be asked to input the file name (see

Input File Name).  The screen will then show two

boxes, the one on the left will show the program you

choose unmodified, the other box shows the program

with the new line number. The renumbering can be

halted temporarily by pressing the [SpaceBar] and

restarted by pressing the [Spacebar] again.

Alternatively pressing the [Esc] key to quit the

renumbering.

Once the renumbering process has been completed you

will be asked to confirm that the changes made are

OK.

Computers in Action         Page 2           TALKER User Manual

Press [Esc] and your original program will remain as

it was before you called the renumber option.

Press [Spacebar] and the new renumbered program will

be saved with all the new line numbers.

Hint:   When you are writing a new program,

miss out the line numbers and select

this option when you have finished and

all the line numbers will be put in for

you.

View

This option allows you to view a program, knowing you

can not alter the program.  Any modifications that

are made will be lost has you can not save the

program to disk.

Once selected you will be prompted to input the name

of the file you wish to edit (see Input File Name).

All the edit features are documented in Appendix A.

Note:  If you are not using the editor supplied

with TALKER then this option will not be

available to you

Change

This option allows you to scroll from one machine

setup to another.  The new setup is shown immediately

in the two boxes on the screen.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  7 <<<<<<<<<<<<<<<<<<<<

The Tools Menu Options

Edit

This option allows the user to keep #tooling

information# about a program in a text file and can

be printed ( if required ) when you next Send the

program to the CNC machine.

This information can be linked to the program and so

be printed automatically when the program is next

used if the rules are followed.

Rules

1.  All tooling information must have a file

extension of TOL.

2.  The tooling file must have the same name

as the program file ( except for the

extension ).

eg.  Main program  P89823.AAA

Tooling file  P89823.TOL

In this way when you download the program

file to the CNC machine, TALKER checks to

see if there is a tooling file.  If the

tooling file exists you will be asked if you

wish to print it.

All the edit features are documented in Appendix A.

Print

Tooling information can also be printed out at any

time by using this option.  Once selected you will be

prompted to input the name of the file you wish to

print ( see Input File Name ).  Printing of tooling

information uses the Printer codes setup in the

Printer Setup and Paper Setup.

Change

This option allows you to scroll from one machine

setup to another.  The new setup is shown immediately

in the two boxes on the screen.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  8 <<<<<<<<<<<<<<<<<<<<

The Install Menu Options

Add

This option allows you to add a machine parameter to

the list.  The maximum number of setups TALKER allows

is twelve.

Advance through the parameters by pressing the

[Enter] key or use the cursor keys. You may end at

any time by pressing [F10] and any parameter that you

have not setup will be set to the default ( shown is

this chapter by the character = ).

The input screen for Add is shown above and all the

installation options are listed below.

Current Machine Selected#

The information required here is for your use

only.  Type in a name that will appear in the

Main Screen Current Machine box.

eg.

1.   All work done for a customer called

'J.R.Smith' could be labelled.

J.R.Smith, using Bridgeport 1.

2.   Programs that are to be sent to one

machine called Miller Maximum

19500-234 could be labelled.

Miller Max

[Enter] to finish.

FilePath

There are two methods to input the file path.

The first, is by the use of the Directory Manager

( see appendix B ), start by pressing the [Tab]

key to create your new subdirectories.

The second method is to type in the name of the

subdirectory where you have or are going to put

your programs for this machine.  If this method

is to be used then the subdirectory must exist.

Computers in Action         Page 2           TALKER User Manual

The FilePath is one of the most important parts

of TALKER.

DOS allows the use of subdirectories

(subdirectories can be thought of as folders

holding information on a similar subject).

Shown in the figure below is a representation of

subdirectories setup for three different machines

and one machine that has two different customers.

Using this method of subdirectories for customers

and/or machine tools, you can keep your programs

separate but located together in a place that is

easily accessible.

-----------

|   ROOT  |

-----------

|

|

-----------

|  TALKER |

-----------

|

|

--------------------------------------------

|                     |                    |

----------            ----------           ----------

| B_PORT |            |  EMCO  |           |  MILL  |

----------            ----------           ----------

|

----------------

|              |

---------      ---------

| SMITH |      | JONES |

---------      ---------

Computers in Action         Page 3           TALKER User Manual

Heidenhain Control

This switch is used to say if the machine tool

has a Heidenhain controller.  If the Machine Tool

is not controlled by a Heidenhain then move the

switch to read 'No' otherwise move the switch

through the list of controller types until the

controller type displayed matches the one fitted

to your machine.

Available Options:-

No =

TNC125

TNC131

TNC135

TNC145

TNC150

TNC155

TNC351

TNC355

TNC2500

Use cursor keys to select

[Enter] to choose.

Setup

This is the same number in your User Parameters

for your Heidenhain controller. Usually setup to

be 168.  This number sets the Parity, Stopbits

and Databits.  This option will not become

available if you have selected 'No' to the prompt

'Heidenhain Control'.

Available Options:-

1 to 255

Use cursor keys to select

[Enter] to choose.

Computers in Action         Page 4           TALKER User Manual

Blk

Which is short for BlockWise.  If you are going

to use the Heidenhain method of program transfer

(Blockwise) then set this switch to 'Yes' (see

appendix D).  This option will not become

available if you have selected 'No' to the prompt

'Heidenhain Control'.

Available Options:-

No

Yes

Use cursor keys to select

[Enter] to choose.

We highly recommended that if you have a

Heidenhain controller you use the Blockwise

method of transfer because of the excellent

method of checking for data errors.

Lang

This prompt is short for language.  Use this

switch to select the language you will be working

with.  The 'ISO' (G codes) or 'DIALOG'

(Heidenhain language). This option will not

become available if you have selected 'No' to the

prompt 'Blk'.

Available Options:-

ISO

DIALOG

Use cursor keys to select

[Enter] to choose.

Computers in Action         Page 5           TALKER User Manual

Jump

This switch is used to allow entry into the

program at a tool change point instead of

starting at the beginning.  By setting this

switch to 'Yes', whenever you start to Send a

program file to the CNC machine you will be asked

if you wish to jump to a Tool Change point.  This

is useful if a tool breaks on a long run, the

blocks already done can be skipped over after

replacing the tool and re-starting the Send

option. This option will not become available if

you have selected 'No' to the prompt 'Blk'.

Available Options:-

No

Yes

Use cursor keys to select

[Enter] to choose.

Recommended for the experienced user.  Care must

be taken to ensure that the tool does not damage

the work piece.  Computers in Action can not be

held responsible for any damage to your work

piece or machine by the use of this option.

Port

The port you select will be dependent on your

hardware.  In the example above we would ideally

want a computer with 3 communication ports (COM1,

COM2 and COM3).  Alternatively we can get around

this by using a simple data switch box available

from Computers in Action or from any good

hardware dealer) connected to a communication

port to switch from one machine to another.  More

communication ports can be added as required by

purchasing a multi communication card ( also

available from Computers in Action).

Available Options:-

COM1 =

COM2

COM3

COM4

Use cursor keys to select

[Enter] to choose.

Computers in Action         Page 6           TALKER User Manual

Baud

This is the rate at which bits will be

transmitted per second to the machine tool. The

higher the number, the faster the bits are sent.

So at 1200 baud, up to 120 bytes of your program

will be sent every second.  Select the fastest

the machine tool will accept.  Both TALKER and

the machine tool must have the same Baud rate

selected.

Available Options:-

110

150

300

600

1200 =

2400

4800

9600

19200

Use cursor keys to select

[Enter] to choose.

Parity

This is used as a means of checking each byte of

the data being sent or received. Both TALKER and

the machine tool must have the same Parity

selected.  Parity not selectable if a Heidenhain

controller is being used.

Available Options:-

MARK

SPACE

NONE

EVEN =

ODD

Use cursor keys to select

[Enter] to choose.

Computers in Action         Page 7           TALKER User Manual

Stopbits

TALKER and the machine tool must be set to the

same stopbits.  Stopbits not selectable if a

Heidenhain controller is being used.

Available Options:-

1 =

2

Use cursor keys to select

[Enter] to choose.

Databits

TALKER and the machine tool must have the same

Databits selected.  Databits not selectable if a

Heidenhain controller is being used.

Available Options:-

7 =

8

Use cursor keys to select

[Enter] to choose.

Delay

Some CNC machine tool controllers need a delay

between each character that is sent to it,

otherwise they loose characters.  We have set the

default to 8 milliseconds because in our

experience this is enough for most controllers,

if you are still loosing characters then increase

the number.

If you are using a Heidenhain controller in

Blockwise mode set this to 0. 1 = 1 millisecond.

Available Options:-

0 to 100

Default = 8

Use cursor keys to select

[Enter] to choose.

Computers in Action         Page 8           TALKER User Manual

Timeout

This is the time in milliseconds that TALKER will

wait with no action (characters being received or

sent) in the input or output queue when in the

Send or Receive options.

If you are using a Heidenhain controller in

Blockwise mode set this to 0.  In this way TALKER

will wait until the CNC controller request

action. 1 = 1 millisecond.

Available Options:-

0 to 100

Default = 0

Use cursor keys to select

[Enter] to choose.

Echo

Use this switch to enable or disable the

displaying of the program being received from or

sent to the machine tool.

Available Options:-

Yes =

No

Use cursor keys to select

[Enter] to choose.

Computers in Action         Page 9           TALKER User Manual

Ext

If you have not setup different directories for

different machines or customers, then this mask

can be used to sub-divide the subdirectory.

The program file name is made up of a file name

using a maximum of 8 characters and an extension

of 3 characters preceded by a period.

eg.       PROGRAM.EXT

File name extension are commonly used to indicate

the type of file or in TALKER's case can be used

to indicate job sequence, drawing issue etc.

Enter the file name extension and this will be

used to show all files that match this when the

Directory Manager is called.  Normal wild card

symbols can be used (*,?).

Default = *

[Enter] to finish.

Note  TALKER uses the extension of $K$ to

indicate a file that has been altered.

Do not use this extension.

Qty

TALKER stores all programs with CR & LF, If

however, your machine tool needs different end of

line terminators then change this number to the

number of terminators it requires.

Available Options:-

0 to 3

Default = 0

Use cursor keys to select

[Enter] to choose.

Terminators

After you have entered the quantity of

terminators you can now enter the hexadecimal

values to each one.

[Enter] to finish.

Computers in Action         Page 10          TALKER User Manual

Starting Number

This number will be used as the starting number

when the renumber option is called.

Available Options:-

0

1

10 =

100

1000

10000

Use cursor keys to select

[Enter] to choose.

Step

This step is the number between each line of your

program.

Available Options:-

1

10 =

100

1000

Use cursor keys to select

[Enter] to choose.

Line Prefix

Line prefix is the character that precceds the

line number.  Some controllers do not use line

prefix, if you require no line prefix then select

the space.

Available Options:-

All characters of the alphabet

and the Space.

Default = N

Use cursor keys to select

[Enter] to choose.

Computers in Action         Page 11          TALKER User Manual

Good Characters

TALKER needs to know what characters could be

used on each line of your program.

Note

Do not include the line prefix

letter in this list of characters.

Default = BCEFGLMRSTXYZ

[Enter] to finish.

Program Start

This line of text will be placed at the begining

of your program.

[Enter] to finish.

Program End

This line of text will be placed at the end of

your program.

[Enter] to finish.

Leading Zero's

Set this option to Yes if you want your program

line numbers to padded out to the left with

zero's.

Available Options:-

No

Yes

Use cursor keys to select

[Enter] to choose.

Computers in Action         Page 12          TALKER User Manual

Number of Zero's

Set this option to the length of line number.

This option will not become available if you have

selected 'No' to the prompt 'Leading Zero's'.

e.g.

4 = 0001

5 = 00001

etc.

Available Options:-

1 to 9

Use cursor keys to select

[Enter] to choose.

Line Num Spaces

Set this option to the number of spaces you

require after the line number.

e.g.

Set to 2 = N0001  X20.45Y30.6

Set to 4 = N0001    X20.45y30.6

Available Options:-

0 to 4

Use cursor keys to select

[Enter] to choose.

F10     Use this key once you have finished the

installation or at any time throughout the

installation. You will be prompted to confirm you

have finished.

Esc     The escape key can be used at any time.  You will

be prompted to ensure you really wish to quit.

Computers in Action         Page 13          TALKER User Manual

Modify

This option allows you to Modify a machine parameter

already in the list.  The procedure is the same as

the Add option.

Modification of the parameters is done by using the

cursor keys to move the highlighted bar to the

parameter you wish to change.  The original contents

of the parameter is highlighted for you to change.

Once all changes are made press [F10] to exit.

See the Add option for details on the parameters.

View

Use this option to scroll through all your setups.

Once selected you use the left arrow or right arrow

keys to move back and forwards through the list of

setups.

Press [E] or [Esc] to quit.  The setup that is being

shown when you quit is the setup that is active.

Delete

On selecting this option you will be prompted to

confirm deletion of the currently active setup.

TALKER must have one setup, so TALKER does not allow

you to delete the very last setup.

Save

All the machine parameters in the list are saved to

disk.  Once you have Added a setup to the list

TALKER expects you to ask for the setup to be Saved,

in this way you can modify a setup temporarily and as

long as you do not select this option, when you run

TALKER again the setup is back to the original.

As you leave TALKER (see End option) you are

given the chance to save the machine parameter setup

file.

Change

This option allows you to scroll from one machine

setup to another.  The new setup is shown immediately

in the two boxes on the screen.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  9 <<<<<<<<<<<<<<<<<<<<

The System Menu Options

Printer Setup

This option allows you to change the printer codes

which are sent to the printer.  Throughout TALKER

there are various reports that can be printed on to

paper.  These reports often use special effects, such

as Enlarged, Bold etc., to clarify reports, making

them easier to read. Some printers have a different

language to others and therefore some of the codes

for special effects also differ.

Changing Printer Codes

When you received your printer you should have had a

Reference Manual.  In the manual there is a section

on `Print Style Commands' or `Printer Control

Commands'.  This section will contain a list of

special effects together with a set of numbers:-

e.g.    Emphasised On  : 1B 45

Emphasised Off : 1B 46

The above example is shown as hexadecimal, but

can be also shown as Ascii code.

Enter the correct code for each control command, each

individual number must begin with a # symbol.

e.g.      Emphasised On  : #27#69

End each individual command with [Enter].

F10     Use this key once you have finished entering all

the printer codes.  You will be prompted to

confirm you have finished.

Esc     The escape key can be used at any time.  You will

be prompted to ensure you really wish to quit.

Computers in Action         Page 2           TALKER User Manual

Paper Setup

Select this option to change the paper size and the

formats used throughout TALKER.

Length of Paper.

This is the actual length of the paper in

lines per page. Change this number when you

use longer or shorter paper lengths.

Standard paper A4 = 66 lines.

[Enter] to finish.

Page Num Line Number.

This number is the line number of each page on

which you want the Page Number Label (see

below) and the page number to be printed.

[Enter] to finish.

Number of Print Lines.

This number is the actual number of lines that

are printed on.

[Enter] to finish.

Form Feed at Page End.

Option to allow for sheet feeder use.  With a

sheet feeder fitted set this to 'Y'.  This

then sends to the printer a Form Feed.

[Y] or [N] required

[Enter] to finish.

Page Number Label.

This text, is the text that is printed next to

the page number at the bottom of each page, on

the Page Num Line Number (see above).

[Enter] to finish.

Computers in Action         Page 3           TALKER User Manual

Page Paper Width.

Set this to the width of your paper.  8"

printers set to 80, and 15" printers to 130.

[Enter] to finish.

Printed Page Offset.

This number is the number of characters from

the left hand side of the page that are left

blank before starting printing.  Use this

offset to move the printed output to the

position you require.  No checking is made to

ensure that you have enough paper to the right

of the offset.

[Enter] to finish.

Heading each Page.

Set this to Y if you want the text displayed

in the System Setup in the prompt 'Software

Licensed to' to be printed at the top of each

page of print out.

[Y] or [N] required

[Enter] to finish.

F10     Use this key once you have finished the setting

up the paper and printing formats. You will be

prompted to confirm you have finished.

Esc     The escape key can be used at any time.  You will

be prompted to ensure you really wish to quit.

System Setup

TALKER needs to now about the system and the way you

want to run it.

Software Licensed to.

This name is printed at the top of each page

of print out if you have responded Y in the

Paper Setup to Heading Each Page.

Computers in Action         Page 4           TALKER User Manual

System Date.

The battery backed clock within your

computer gives the date.  If your date is

incorrect then inputting the correct date will

change the battery backed clock.

[Enter] to finish.

Main Password.

In any area where programs can be Received,

Edited and altered in any way, TALKER can be

made to ask for a password first before

continuing.  If you do not want any password

protection then leave this field empty,

otherwise enter here your password.  The

password is case sensitive.

[Enter] to finish.

Printer File Name.

This file name is the name of the file that

contains all the special codes for the listing

printer (see Printer Setup).  If the file name

entered does not exist then you will be

warned, asked to accept your choice or

re-enter the file name.

[Enter] to finish.

Editor File Name.

TALKER comes with a fully functioning editor,

with all the commands you are likely to want

(see appendix A).  If you still want to use

your own editor, whose commands you know, then

this is possible by entering here the full

filepath and name.

Then when the Edit option is selected from the

Programming menu you will be transported into

your own editor.  The biggest disadvantage to

using your own editor is that the editor does

not know where your programs are, so you will

have to find them yourself.  On exiting the

editor you will be returned to TALKER. The

Edit option from the Tools menu option

continues to use the editor supplied with

TALKER.

[Enter] to finish.

Computers in Action         Page 5           TALKER User Manual

F10     Use this key once you have finished setting the

system setup.  You will be prompted to confirm

you have finished.

Esc     The escape key can be used at any time.  You will

be prompted to ensure you really wish to quit.

Dir Manager

is multi-feature disk manager.  Using this option

you can:-

a.  Make, Rename or Delete a subdirectory.

b.  Rename or Delete a file.

c.  Move around the disk structure.

d.  Change to another disk drive.

e.  View the Attributes of the file.

Appendix B covers all the available functions in

detail.

Colour Setup

If you don't like the colours we have chosen then

use this option to change the 'Look and Feel' of

the program.

Use [Left Arrow] and [Right Arrow] keys to select

between the Foreground and Background.

Use [Up Arrow] and [Down Arrow] keys to select

an item on the shown list.

Use [PageUp] and [PageDown] keys to scroll

through all available pages of selectable

colours.

To set the colour for each foreground use the keys

between 0 to 9 and A to F, the background colours can

only be 0 to 7.

Computers in Action         Page 6           TALKER User Manual

Press the [S] key to save your modification to disk

before you leave the colour setup.  Your

modifications will take effect immediately you exit

from the colour setup.

The [R] key will reset all colours to the colours

that we have decided we like when we wrote TALKER.

Don't forget to press [S] to save before you exit.

The [L] loads your last saved set of colours.

Import a Program

This is really copy a program. From a user input

location to the current filepath.

On selecting this option a box will pop up, which

by using the left and right arrow keys allows

selection of the source disk drive (this is the

drive that has the program you wish to copy).

Press [F10] when you have selected the drive and

the Directory Manager (see Appendix B) will pop

up to help you find the program.

Once you have found the program press [Enter] to

copy this program to the Current Filepath shown

in the current filepath box on the main screen.

Support

This page informs you where you can get support

for this system if you need it.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> C h a p t e r  10 <<<<<<<<<<<<<<<<<<<<

The End Menu Options

Quit and Save

Use this option to leave TALKER and save the

machine parameters.

From anywhere in the menu system, Two presses of

the Esc key will take you to this option.

Press [Enter] to leave.

Abort (No Save)

End TALKER without saving the machine parameters.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> A p p e n d i x  A <<<<<<<<<<<<<<<<<<<<

The Editor

The editor is a part of TALKER that allows you to enter

and modify your CNC machine programs, then store it in a

file for later retrieval to be sent to the CNC machine or

more modifying of the CNC machine program file.

We have chosen a command structure that has been well and

truly tested on many word processors, so to the new

comer, you will soon pick up the commands and to the

experienced user, the command structure is probably the

same as you have used before.

The Editing Screen

The top line of the window is the status line providing

the following information:-

Command echo

When the first key of a command assigned to two

keystrokes is pressed, or when the Insert control

character command is given, the keystroke is echoed

at the left edge of the status line.

eg.   if the first keystroke is [Ctrl-K], then

the ^K will appear until the second keystroke.

Line x

Shows the line number containing the cursor.

Col x

Shows the column number of the cursor.

Toggle insert mode                   [Ctrl-V] or [Insert]

Toggles between insert and overwrite. The word Insert

is displayed on the status line when in the insert

mode.

Toggle autoindent                              [Ctrl-Q I]

When autoindent the word indent appears on the status

line. The Enter command takes the cursor to the next

line and to the column of the first non-blank

character on the previous line.

Computers in Action         Page 2           TALKER User Manual

Toggle fixed tabs                              [Ctrl-O T]

When the word Tab is in the status line and the

cursor at column 1, pressing the [Tab] key will move

the cursor to column 9 and further pressing of the

[Tab] key will move the cursor on 8 columns each

time. When the word Tab is not on show then the tabs

are taken from the non-blank text on the previous

line.

K:ANYFILE.EXT

The drive, name, and extension of the file being

edited. The path will not be shown.

Movement Commands

Character left                     [Ctrl-S] or Left Arrow

Moves the cursor one character to the left.

Character right                   [Ctrl-D] or Right Arrow

Moves the cursor one character to the right.

Word left                   [Ctrl-A] or [Ctrl]-Left Arrow

Moves the cursor to the beginning of the word to the

left.

Word right                  [Ctrl-F] or [Ctrl]-Left Arrow

Moves the cursor to the beginning of the word to the

right.

Line up                              [Ctrl-E] or Up Arrow

Moves the cursor to the line above. If the cursor is

on the top line then window scrolls down one line.

Line down                          [Ctrl-X] or Down Arrow

Moves the cursor down one line.  If at the bottom of

the screen then window scrolls up one line.

Computers in Action         Page 3           TALKER User Manual

Scroll up                                        [Ctrl-W]

Scrolls the text up the screen one line at a time but

the cursor stays on the same line until it reaches

the bottom of the screen.

Scroll Down                                      [Ctrl-Z]

Scrolls the text down the screen one line at a time

but the cursor stays on the same line until it

reaches the top of the screen.

Page up                               [Ctrl-R] or [Pg Up]

Moves the cursor one page up with an overlap of one

line.

Page down                             [Ctrl-C] or [Pg Dn]

Moves the cursor one page down with an overlap of one

line.

Beginning of program           [Ctrl-Q R] or [Ctrl-Pg Up]

Moves the cursor to the first character in your

program.

End of program                 [Ctrl-Q C] or [Ctrl-Pg Dn]

Moves the cursor to the last character in your

program.

Beginning of line                    [Ctrl-Q S] or [Home]

Moves the cursor to column 1 in the current line.

End of line                           [Ctrl-Q D] or [End]

Moves the cursor to the end of the current line.

Top of screen                   [Ctrl-Q E] or [Ctrl-Home]

Moves the cursor to the top line of the window.

Bottom of screen                 [Ctrl-Q X] or [Ctrl-End]

Moves the cursor to the bottom of the window.

Computers in Action         Page 4           TALKER User Manual

Top of block                                   [Ctrl-Q B]

Moves the cursor to the position of the block-begin

marker set with [Ctrl-K B].

Bottom of block                                [Ctrl-Q K]

Moves the cursor to the position of the block-end

marker set with [Ctrl-K K].

Previous cursor position                       [Ctrl-Q P]

Moves to the last cursor position. This command is

particularly useful to move back to the previous

position after a Find or a Find-and-replace operation.

Jump to marker 0..3                [Ctrl-Q 0]..[Ctrl-Q 3]

Moves the cursor to one of the four invisible text

markers created with the Set marker command.

Set marker 0..3                    [Ctrl-K 0]..[Ctrl-K 3]

Sets one of the four invisible text markers at the

current position.

Insert and Delete Commands

The following commands allow you to insert and delete

characters, words, and lines.

Enter                                            [Ctrl-M]

In the Insert mode, this command inserts a line break

at the cursor position. If Autoindent mode is in

effect, the cursor moves to the next line and to the

same column as the non-blank character in the

previous line; otherwise, it moves to column 1 of the

new line.

In Overwrite mode, this command moves the cursor to

column 1 of the next line without a new line.

Insert line                                      [Ctrl-N]

Inserts a line at the cursor position.

Computers in Action         Page 5           TALKER User Manual

Insert control character                         [Ctrl-P]

Allows control characters to be entered into your

program.

Tab                                     [Ctrl-I] or [Tab]

Moves the cursor to the next tab stop if in the

Insert mode the text to the right of the character

also moves. In the Autoindent mode only the cursor

moves.

Delete current character             [Ctrl-G] or [Delete]

Deletes the character at the cursor position and

moves all the character to the right of the text to

the left by one character.

Delete character left             [Ctrl-H] or [Backspace]

Deletes the character to the left of the cursor and

moves the cursor and any text to the right of cursor

position one character to the left.

Delete word                                      [Ctrl-T]

Deletes the word to the right of the cursor.

Delete to end of line                          [Ctrl-Q Y]

Deletes all the text from the right of the cursor to

the end of the line.

Delete line                                      [Ctrl-Y]

Deletes complete cursor line moving text up one line

and positioning cursor at column 1.

Restore line                                   [Ctrl-Q L]

On pressing this command all alterations to current

line will be restored to how they were before entry

into the line.

Computers in Action         Page 6           TALKER User Manual

Block Commands

Start block                            [Ctrl-K B] or [F7]

Marks the beginning of a block.

End block                              [Ctrl-K K] or [F8]

Marks the end of the block.

Copy block                                     [Ctrl-K C]

Makes a copy of the marked block to start from the

current cursor position.

Move block                                     [Ctrl-K V]

Moves the marked block to start from the current

cursor position.

Delete block                                   [Ctrl-K Y]

Deletes the marked block.

Hide block                                     [Ctrl-K H]

Toggles the marked block on or off.

Read block                                     [Ctrl-K R]

This command reads in a specified file from disc into

the program at the current cursor position. On

entering this command a box will appear and ask for a

file name. Entering [Tab] or [*] will take you into

Directory Manager.

Write block                                   [Ctrl-K  W]

This command writes a marked block to a specified

file. On entering this command a box will appear and

ask for a file  name. Entering [Tab] or [*] will take

you into Directory Manager.

Print                                          [Ctrl-K P]

Prints a marked block if any, otherwise prints

complete program.

Computers in Action         Page 7           TALKER User Manual

Search Commands

Find                                           [Ctrl-Q T]

Will search in your program for a string of up to 30

characters. The last used search string will be shown

for you to accept, re-enter or edit by using the

following commands:-

[Ctrl-D]  shows the character to right.

[Ctrl-R]  restores previous string.

[Ctrl-S]  moves cursor to left.

[Ctrl-U]  cancel search command.

[Esc]     cancel search command.

[Enter]   accept last or entered string.

[Ctrl-P]  allows control characters.

After the entry of the search string the last options

used will be shown for you to accept or re-enter

using the following options:-

B    searches your program from the cursor position

backwards.

L    Searches only the block marked.

n    finds the n occurrence of the search string from the

current cursor position.

U    ignores case.

W    searches whole words only.

Find and Replace                               [Ctrl-Q F]

This command works in the same manner as the Find

command, except that you can replace the searched

string with a an other string of up to 30 characters.

After entering the search string the last replace

will be displayed for you to accept, re- enter or

edit. The options are then displayed. The options are

the same as for the Find command with to additions:-

G    global search. the whole program is searched

regardless of cursor position.

N    replaces the found string with the replace without

asking. Without this option being set, when

the search string is found, you will be

prompted to 'Replace Y/N'.

To speed up global search and replace with the N

option being set, press a non- printing character and

then the screen will not be refreshed after every

replace.

Computers in Action         Page 8           TALKER User Manual

Find Next                                        [Ctrl-L]

Repeats the last Find, and Find and replace using the

same strings and options.

Exit commands

Save and Continue                      [Ctrl-K S] or [F9]

Saves the program and returns to carry on editing.

Save and Exit                         [Ctrl-K K] or [F10]

Saves the program and returns you to the Main Screen.

Quit                                  [Ctrl-K Q] or [Esc]

Quits the program but if any modifications have been

made since the last Save then you will be prompted to

confirm exit.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> A p p e n d i x  B <<<<<<<<<<<<<<<<<<<<

The Directory Manager

Directory Manager can be called when ever you use a

wildcard symbol in the file name entry system or directly

from the System menu.

Once you input a file name that contains wildcards, a box

will appear and show all the files that match the

wildcard mask.  The display will show the files that

match in the current filepath.

Filenames are shown as the name they are stored,

subdirectories are shown with the subdirectory name

preceded by /.

Too move around the hard disk there is two special

entries on the directory listing.

\.    When the cursor is on this item and the [Enter] key

is pressed you will be moved back to the Root

directory of the disk.

\..   This directory entry when highlighted and the

[Enter] key is pressed you will be moved back one

level in the directory chain.

The various functions of the Directory Manager can be

accessed by using one of the function keys listed below.

Help

[F1]    This key either turns on the help, or turns it

off.  The [H] key also can be used.

Reset Drive

[F2]    This key resets the current drive to the root

directory. The [S] key also can be used.

Make Subdirectory

[F3]    On selecting this option you will be presented

with a box for you to enter the name of the new

subdirectory.  When you have finished entering

the name the subdirectory will be created and

shown in the normal way.  The [M] key also can be

used.

Computers in Action         Page 2           TALKER User Manual

Rename Subdirectory

[F4]    To select this option the cursor must be on the

subdirectory you wish to rename.  Enter the new

name and press [Enter].  You will be asked to

confirm that the new name is correct.  If correct

the screen will change to show the new name. The

[N] key also can be used.

Delete Subdirectory

[F5]    To select this option the cursor must be on the

subdirectory you wish to delete.  You will be

asked to confirm that you wish to delete this

subdirectory.  If you answer Y and there is no

files or subdirectories in that subdirectory then

the screen will change and show the subdirectory

directory.  If the deletion can not be

accomplished you will be warned.  The [D] key

also can be used.

View File Attributes

[F6]    This option allows you see the Attributes of the

file. The attributes are shown as letters if the

letter is uppercase then this means it is set,

and lowercase shows not set.

R or r    Readonly File

H or h    Hidden File

S or s    System File

V or v    Volume ID

D or d    Directory

A or a    Archive

F or f    AnyFile

Delete a File

[F7]    To select this option the cursor must be on the

file you wish to delete.  You will be asked to

confirm that you wish to delete this file.  If

you answer Y then this file is deleted  If the

deletion can not be accomplished you will be

warned.  The [E] key also can be used.

Rename a File

[F8]    To select this option the cursor must be on the

file you wish to rename.  Enter the new name and

press [Enter].  You will be asked to confirm that

the new name is correct.  If the new name is

correct the screen will change to show the new

name. The [R] key also can be used.

Computers in Action         Page 3           TALKER User Manual

Log on to Another Drive

[F9]    This command enables you to move from on disk

drive to another.  On selecting this option a box

will appear for you to insert the drive letter.

If drive exists and there is a disk there the

screen will show a directory of the Root.  If any

errors occur you will be warned. The [L] key can

also be used.

Change Directories

[F10]   To use this command the cursor must be on a

subdirectory.  Once selected the screen will

change and show a directory listing of the chosen

subdirectory. The [C] key can also be used.

Information of the Disk Drive

[F11]   If your keyboard does not have the [F11] key, or

does not respond when pressed, then you can use

the [I] key or press the [Shift] and [F1] keys

simultaneously. The information shown when this

option is selected is:-

Disk Volume Name.

Available Disk Space.

DOS Version.

Home Position

[Home]  This key will take the cursor back to the

beginning of the directory listing.  If the

cursor is already home then no action is taken.

End Position

[End]   Pressing this key will take the cursor to the end

of the of the directory listing.  If the cursor

is already at the end then no action is taken.

Cursor Movement

[Up Arrow]

Pressing this key once you will move the cursor

up one line.  If the cursor is at the top of the

listing then the cursor will not move.

Computers in Action         Page 4           TALKER User Manual

[Down Arrow]

Pressing this key once will move the cursor down

one line.  If the cursor is at the bottom of the

listing then the cursor will not move.

[Left Arrow]

By pressing this key once you will move the

cursor one column to the left.  If the cursor is

at the far left column and there is another row

above then the cursor will move to the far right

column of the row above.

[Right Arrow]

By pressing this key once you will move the

cursor one column to the right.  If the cursor is

at the far right column and there is another row

below then the cursor will move to the far left

column of the row below.

[Page Up]

This key will move the complete display up one

page. If there not another page above the page

shown then nothing will happen.

[Page Down]

This key will move the complete display down one

page.  If there not another page below the page

shown then nothing will happen.

Choose a File

[Enter] This key is also know as return on some

keyboards. Once you have moved the cursor to the

file you wish to select press this key.

Exit Key

[Esc]   As the word 'escape' means, use this key to

breakout of the Directory Manager and return to

'whence you came'.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> A p p e n d i x  C <<<<<<<<<<<<<<<<<<<<

The Help System

The help available within TALKER is what is knows as

Context Sensitive Help. This means when ever you ask for

help, TALKER knows where you are and gives the help

available for that position in TALKER.

The Help is accessed by pressing the [F1] key.

The first page of help is displayed.  The top right hand

side displays the number of pages of help on this

subject.  The top left displays the page number.

The bottom of the help page is five menu items.  The menu

items can be accessed by the following methods.

1.   Use the left and right arrow keys.  Select

by pressing [Enter].

2.   Pressing the [Tab] key advances the

highlighted bar one menu item to the right.

Select by pressing [Enter].

3.   Press the corresponding key on the keyboard

to the highlighted letter in the menu item.

Next

Used to view the next page.  If there is no next page

then nothing happens.

Prev

This option brings into view the previous page.  If there

is no previous page then nothing happens.

Indx

This option brings up the list of topics available in the

help system.  Use the following keys to access the

information.

Up and Down arrow keys are used for single line movement.

The [Home] and [End] keys moves the cursor bar to the

beginning or end of the list. The [Pg Up] and [Pg Dn] key

moves the cursor bar up or down one page of the list.

Computers in Action         Page 2           TALKER User Manual

The [Esc] key exits from the help.

To choose a subject press the [Enter] key on the topic

you require.  This then shows the first page of the help

available on that subject.

Find

This option searches the complete TALKER help file, on a

subject ( or word ) you have typed in.

To enter search text or to edit the text that is already

present press the [E] key. When finished press [Enter]

and the search will start.

If the search is successful then that page of help is

displayed. To search again press the [F] key as usual to

get the box displayed in Figure 17. The options work in

the way you would expect.

First:          finds the first occurrence of the entered

text.

Next:           finds the next occurrence in the

file.

Previous:       moves backward through the file.

Quit or [Esc]: exits out of the find option and back to

the help page

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> A p p e n d i x  D <<<<<<<<<<<<<<<<<<<<

Communication Technical Details

Serial Communication

Programs or data files transferred from the machine

tool controller by the use of a serial interface

fitted to the controller housing, through a cable to

the serial port of your computer.

In order for TALKER to work correctly, an RS232

serial interface must be fitted to your machine tool

controller. Usually this is provided as a 25 way 'D'

type female connector - with the corresponding

connections on your computer of a 25 way 'D' type

male connector or a 9 way 'D' type male connector.

In order to achieve successful serial communications,

various parameters at both ends of the serial link

must be configured.

Databits

Each character on you computer keyboard is

represented by an 8 bit number, where 8 bits make a

byte and the available range of numbers extends from

0 up to and including 255.  These unique numbers are

allocated to the corresponding keyboard characters

(letters of the alphabet and number digits) in

accordance with the American Standard Code for

Information Interchange (ASCII).  A table of the

ASCII values is shown in Figure 18.

Parity

Each bit of data sent is represented in two ways,

either a logic '0' or logic '1' and every character

can be represented as a series of 1's and 0's.  As a

means of ensuring the same bits are received as were

actually transmitted an extra parity bit is added as

a '1' if the number of logic '1's is odd (Odd Parity)

or if the number of logic '1's is even (Even Parity)

nothing is added.

As data is transmitted this extra bit is calculated

and added to the series of bits sent from the

computer and removed and checked by the receiving

machine tool.  When the in-coming data does not

verify correctly with the parity bit a 'Parity Error'

will be flagged by the machine tool controller.

Computers in Action         Page 2           TALKER User Manual

Baud Rate

The speed of communication of a serial interface is

known as baud rate and represents the maximum number

of bits per second that can be sent.  Therefore a

baud rate of 1200 equals 120 characters per second.

It is critical for correct data transfer that both

the computer and the machine tool controller

communicate at the same baud rate.

Dependent on the actual baud rate used, each data bit

must be maintained in a set condition for a specific

amount of time.  For 1200 baud rate this time is

0.833 ms (1/1200), during this period the receiving

station (computer or machine tool) can be assured

that the input signal condition will not change and a

single sample half way through the bit taken to

establish the type of bit sent.

Figure a) above shows the typical construction of a

byte of serial data and includes a start bit and stop

bit.  These are used by the receiving station to

synchronise it's operation with the transmitting

station.  Figure b) shows an example of 'Even Parity'

where the total number of logic '1's is made an even

number by making bit 8 (the parity bit in this

example) a logic 1.  Bits 2,4,6 & 8 are all '1' and

an even number 4.

Handshaking

The two types of handshaking are Software and

Hardware and may be used on their own, together or

not at all.  Software handshaking use a number of

agreed character values to achieve it's operation.

These values, according to ASCII standard, provide

the means for a receiving station to start (X-On or

DC1) transmitting, stop (X- Off or DC3) transmitting,

acknowledge (Ack) good data, negative acknowledge

Nak) or cancel (Can) transmitting - amongst others. A

transmitting station can also use the codes to

indicate the start of data follows (Stx), the end of

a single data block (Etb), the end of a program (Etx)

or the end of transmission (Eot).

Hardware handshaking is a physical method of starting

and stopping data transfer and indicating the

condition of a device.

Computers in Action         Page 3           TALKER User Manual

In the two types of handshaking there would also be

two types of cabling (although the actual wiring

details may vary considerably).  The first would have

the data lines connected, transmit to receive each

way and the output handshaking lines on one device

connected to the inputs of the other device and vice

versa.  Alternatively the data lines would be

connected as before but the output handshaking lines

at each station connected to it's own input

handshaking lines - thereby each station enables or

disables itself to communicate.

Program Transfer

Transfer of programs to the Heidenhain series of

machine tool controllers can be accomplished in two

ways, for reference purposes we will refer to these

two modes of operation as 'Load Memory' and 'Transfer

Blockwise'.

Data Transfer - Load Memory

Using this method of operation a program would be

sent to the machine tool controller (whilst the

machine tool is idle) and stored for latter execution

in the controllers memory.  This would limit the size

of program that can be processed to the size or

capacity of the machine tools memory. The advantage

this gives the user is the ability (if supported by

the controller) to be able to edit any part of the

program before execution.  A single program file

could be shared between a number of different

machines (but with similar specification) and be

edited on each machine tool for that machines

differences.

Data Transfer - Drip Feed.

Using this method of operation very large programs

can be sent to the machine tool for processing.  The

transfer of data is carried out at the same time as

program blocks are processed by the machine tool -

executed - thereby allowing very complex profiles to

be machined from program files many mega bytes long.

The main disadvantage to using the drip feed method

are, the source file cannot be edited on the machine

tool so must be correctly produced by the CAD/CAM

system, and the whole file has to be re-sent

following the replacement of a broken tool (some

programs may take many hours to complete).

Computers in Action         Page 4           TALKER User Manual

Screened Cable - Safe Program Transfer.

Applications operating in environments that switch

large currents - such as machine tools with powerful

electric motors - can induce signals in low power

data cables such as RS232 interfaces.  These

'Switching Transients' may corrupt any data being

sent along the cable with potentially disastrous

results.  Two methods for combating this problem are

using the 'Transfer Blockwise' method of data

transfer and linking the machine tool controller with

a screened serial cable.

Programs sent in either 'Load Memory' or 'Drip Feed'

methods may be sent using a simple 'Parity check'

procedure or using the secure Heidenhain 'Transfer

Blockwise' method.  Whichever method is chosen the

use of a screened cable can provide additional

security for serial data transfer.

The cable wiring details included with this manual

recommend the use of serial cables with a full metal

screen over the whole length of the cable which is

earthed at each end.  This provides a high degree of

security for serial data transfer but the equipment

should be periodically inspected to ensure cable

integrity.

Parity Check

Each individual character sent to the machine tool

controller will be sent with a parity bit so that the

receiving station can perform a simple check for

possible corruption occurring during the transfer.

This method does not provide for any error correction

so that should a parity error arise during data

transfer the whole transfer will be aborted.  The

advantage of using the parity method is it is very

simple to implement and is usually carried out

automatically at a 'Hardware Level' by the serial

interface components.

Computers in Action         Page 5           TALKER User Manual

Transfer Blockwise.

Data blocks are sent to the machine tool controller

using a block check character (BCC) system that

allows for the re-transmission of suspect data blocks

from the computer to the machine tool controller.

Each program block or line is processed by the

computer to add block definition characters and a

security character (based on the data block contents)

before being sent to the machine tool controller.

When the block has been received by the controller it

re-calculates the BCC character and compares it with

the one received, if they do not match the block is

requested to be re-transmitted from the computer.

Data transfer is performed on a block basis such that

following a single block being sent by the computer

no further data is sent until the machine tool

acknowledges that the transfer was good.  Good block

transfers and ac#knowledged with the 'ACK' character

and bad block transfers are negative acknowledged

with the 'NAK' character.

Program blocks 'Drip Feed' using 'Transfer Blockwise'

will be stored in a buffer area of the controllers

memory for later execution.  Following the execution

of the program blocks they are discarded thereby

freeing up area in the buffer to receive more data

blocks.  Because of this method of operation programs

cannot be edited in the machine tool controller.  The

'Transfer Blockwise' method can be implemented for

program transfer in both 'Load Memory' and 'Drip

Feed' modes of operation, and its use is highly

recommended if your machine tool controller supports

it.

Warning

Heidenhain Machine Parameters

The correct operation of the Heidenhain machine

controller and TALKER may require certain 'Machine

Parameters' to be viewed and optionally modified.  A

number of machine critical parameters are contained

within the 'Machine Parameter' list and the user must

be very careful of the accidental modification of the

'Machine Parameters' that are not indicated in this

manual. To prevent accidental modification of the

'Machine Parameters' a user code has to be entered by

the user on the control panel before their contents

can be accessed. Users wishing to access the 'Machine

Parameters' should contact the machine tool

manufacturer or Heidenhain for the appropriate code

numbers.

Computers in Action         Page 1           TALKER User Manual

>>>>>>>>>>>>>>>>>>>> A p p e n d i x  E <<<<<<<<<<<<<<<<<<

Cabling Examples

This appendix gives details of known working cable

configurations that are up and working using TALKER

and various CNC controllers.

Please send us your cabling details, so they can be

included in the next re-print of this manual.

Cabling can sometimes be difficult hence this

appendix.  Always use a screen cable no more than 25

metres in length.

The following examples give the pin outs (each pin on

the connector has a number and this number is shown

below) for the 25 way connector and the 9 way

connector (shown in () brackets) fitted to Personal

Computers.

eg.

1 ------------ 1  this means that pin 1 on both

connectors are connected to

each other.

4  ---|   this means that these two cables are

20 ---|   joined together on the same

connector.

Standard 3 wire link

Computer                         CNC

3    (2) ------------------------ 2

2    (3) ------------------------ 3

7    (5) ------------------------ 7

8    (1) ----|

20   (4) ----|

6    (6) ----|

4    (7) -------|

5    (8) -------|

Anilam Crusader II and EmcoTronic M1

Computer                      Crusader

3    (2) ------------------------ 2

2    (3) ------------------------ 3

7    (5) ------------------------ 7

4    (7) ------------------------ 5

5    (8) ------------------------ 4

Computers in Action         Page 2           TALKER User Manual

Heidenhain Controllers

TNC125, TNC131, TNC135, TNC145, TNC150, TNC155,

TNC351, TNC355 and TNC2500

This cable is suitable for both Memory Load and

Blockwise Transfer.

Computer                         TNC

3    (2) ------------------------ 2

2    (3) ------------------------ 3

4    (7) ------------------------ 5

5    (8) ------------------------ 4

7    (5) ------------------------ 7

6    (6) ----|------------------- 20

8    (1) ----|              |---- 8

20   (4) -------------------|---- 6