SIMCGA - Simulate CGA with Hercules Monochrome Card

Written in September 1986 by

Chuck Guzis

153 North Murphy Ave.

Sunnyvale, CA  94086

This memory-resident utility allows you to "fool" most software requiring

a Color Graphics Adapter into using your Hercules (or compatible) mono-

chrome adapter in the graphics mode.  Graphics images are reproduced

in normal aspect ratio, using as much of the available screen area as

is possible.

The trick used here is to program the HGC to display more lines of 3

lines per character time instead of 4 (The CGA displays 2).  A service

routine hooked into the hardware timer interrupt (int 8) copies one line

to the third displayed line to give a filled-out image.

The method used here works and results in some useful color gray-scaling,

but it does have a few minor drawbacks -

1.  Since the copy operation occurs in real-time concurrent with the

display, some "ghosting" on moving images is inevitable.

2.  Since the image is expanded vertically by 1/2, small line details

will be affected by the line doubling technique.  This is mostly

an aesthetic annoyance.

3.  Those utilities which "read" the CGA registers at 03d4 et seq.

are going to find no registers to read.  This does not seem to

affect many programs.

4.  40x25 text mode is not supported - the hardware just isn't there.

This mode is mapped into 80x25 and again, doesn't seem to bother

many utilities.

5.  When in text mode (modes 0, 1, 2 and 3), the normal monochrome

character attributes are not available - this is a problem with

the way the HGC handles the attribute memory map.

Quite a few programs have been tested, including Flight Simulator, PC-CAD,

BASICA and some games with no hitches.

This package contains three programs - SIMCGA, SETCGA, SETMONO.  SIMCGA

is the memory-resident part and must be run before the CGA simulation

feature is available.  SETMONO will restore the monochrome-only mode;

SETCGA will reset CGA compatibility after a SETMONO.  If this feature

is used routinely, you might want to put this in your AUTOEXEC.BAT file -

SIMCGA

SETMONO

and then run a SETCGA when you need the CGA feature.

--------------------