6-7

WARNING: Instruction forms marked with "*" by the mnemonic are

part of the extended 186/286/NEC instruction set.

Instructions marked with "#" are unique to the NEC

processors.  These instructions will NOT work on the 8088 of

the IBM-PC; nor will they work on the 8086; nor will the NEC

instructions work on the 186 or 286. If you wish your

programs to run on all PC's, do not use these instructions!

Opcodes    Instruction    Description

-------    -----------    -----------

37          AAA           ASCII adjust AL (carry into AH) after addition

D5 0A       AAD           ASCII adjust before division (AX = 10*AH + AL)

D4 0A       AAM           ASCII adjust after multiply (AL/10: AH=Quo AL=Rem)

3F          AAS           ASCII adjust AL (borrow from AH) after subtraction

14 ib       ADC AL,ib     Add with carry immediate byte into AL

15 iw       ADC AX,iw     Add with carry immediate word into AX

80 /2 ib    ADC eb,ib     Add with carry immediate byte into EA byte

10 /r       ADC eb,rb     Add with carry byte register into EA byte

83 /2 ib    ADC ew,ib     Add with carry immediate byte into EA word

81 /2 iw    ADC ew,iw     Add with carry immediate word into EA word

11 /r       ADC ew,rw     Add with carry word register into EA word

12 /r       ADC rb,eb     Add with carry EA byte into byte register

13 /r       ADC rw,ew     Add with carry EA word into word register

04 ib       ADD AL,ib     Add immediate byte into AL

05 iw       ADD AX,iw     Add immediate word into AX

80 /0 ib    ADD eb,ib     Add immediate byte into EA byte

00 /r       ADD eb,rb     Add byte register into EA byte

83 /0 ib    ADD ew,ib     Add immediate byte into EA word

81 /0 iw    ADD ew,iw     Add immediate word into EA word

01 /r       ADD ew,rw     Add word register into EA word

02 /r       ADD rb,eb     Add EA byte into byte register

03 /r       ADD rw,ew     Add EA word into word register

0F 20      #ADD4S         Add CL nibbles BCD from DS:SI into ES:DI (CL even,NZ)

24 ib       AND AL,ib     Logical-AND immediate byte into AL

25 iw       AND AX,iw     Logical-AND immediate word into AX

80 /4 ib    AND eb,ib     Logical-AND immediate byte into EA byte

20 /r       AND eb,rb     Logical-AND byte register into EA byte

81 /4 iw    AND ew,iw     Logical-AND immediate word into EA word

21 /r       AND ew,rw     Logical-AND word register into EA word

22 /r       AND rb,eb     Logical-AND EA byte into byte register

23 /r       AND rw,ew     Logical-AND EA word into word register

63 /r      *ARPL ew,rw    Adjust RPL of EA word not smaller than RPL of rw

62 /r      *BOUND rw,md   INT 5 if rw not between [md] and [md+2] inclusive

9A cd       CALL cd       Call far segment, immediate 4-byte address

E8 cw       CALL cw       Call near, offset relative to next instruction

FF /3       CALL ed       Call far segment, address at EA doubleword

FF /2       CALL ew       Call near, offset absolute at EA word

0F FF ib   #CALL80 ib     Call 8080-emulation code at INT number ib

98          CBW           Convert byte into word (AH = top bit of AL)

F8          CLC           Clear carry flag

FC          CLD           Clear direction flag so SI and DI will increment

FA          CLI           Clear interrupt enable flag; interrupts disabled

0F 12/0    #CLRBIT eb,CL  Clear bit CL of eb

0F 13/0    #CLRBIT ew,CL  Clear bit CL of ew

0F 1A/0 ib #CLRBIT eb,ib  Clear bit ib of eb

0F 1B/0 ib #CLRBIT ew,ib  Clear bit ib of ew

6-8

0F 06      *CLTS          Clear task switched flag

F5          CMC           Complement carry flag

3C ib       CMP AL,ib     Subtract immediate byte from AL for flags only

3D iw       CMP AX,iw     Subtract immediate word from AX for flags only

80 /7 ib    CMP eb,ib     Subtract immediate byte from EA byte for flags only

38 /r       CMP eb,rb     Subtract byte register from EA byte for flags only

83 /7 ib    CMP ew,ib     Subtract immediate byte from EA word for flags only

81 /7 iw    CMP ew,iw     Subtract immediate word from EA word for flags only

39 /r       CMP ew,rw     Subtract word register from EA word for flags only

3A /r       CMP rb,eb     Subtract EA byte from byte register for flags only

3B /r       CMP rw,ew     Subtract EA word from word register for flags only

0F 26      #CMP4S         Compare CL nibbles CD at DS:SI from ES:DI (CL even,NZ)

A6          CMPS mb,mb    Compare bytes ES:[DI] from [SI], advance SI and DI

A7          CMPS mw,mw    Compare words ES:[DI] from [SI], advance SI and DI

A6          CMPSB         Compare bytes ES:[DI] from DS:[SI], advance SI and DI

A7          CMPSW         Compare words ES:[DI] from DS:[SI], advance SI and DI

99          CWD           Convert word to doubleword (DX = top bit of AX)

27          DAA           Decimal adjust AL after addition

2F          DAS           Decimal adjust AL after subtraction

FE /1       DEC eb        Decrement EA byte by 1

FF /1       DEC ew        Decrement EA word by 1

48+rw       DEC rw        Decrement word register by 1

F6 /6       DIV eb        Unsigned divide AX by EA byte (AL=Quo AH=Rem)

F7 /6       DIV ew        Unsigned divide DXAX by EA word (AX=Quo DX=Rem)

C8 iw 00   *ENTER iw,0    Make stack frame, iw bytes local storage, 0 levels

C8 iw 01   *ENTER iw,1    Make stack frame, iw bytes local storage, 1 level

C8 iw ib   *ENTER iw,ib   Make stack frame, iw bytes local storage, ib levels

Fany          Floating point set is in Chapter 7

F4          HLT           Halt

F6 /7       IDIV eb       Signed divide AX by EA byte (AL=Quo AH=Rem)

F7 /7       IDIV ew       Signed divide DXAX by EA word (AX=Quo DX=Rem)

F6 /5       IMUL eb       Signed multiply (AX = AL * EA byte)

F7 /5       IMUL ew       Signed multiply (DXAX = AX * EA word)

6B /r ib   *IMUL rw,ib    Signed multiply immediate byte into word register

69 /r iw   *IMUL rw,iw    Signed multiply immediate word into word register

69 /r iw   *IMUL rw,ew,iw Signed multiply (rw = EA word * immediate word)

6B /r ib   *IMUL rw,ew,ib Signed multiply (rw = EA word * immediate byte)

E4 ib       IN AL,ib      Input byte from immediate port into AL

EC          IN AL,DX      Input byte from port DX into AL

E5 ib       IN AX,ib      Input word from immediate port into AX

ED          IN AX,DX      Input word from port DX into AX

FE /0       INC eb        Increment EA byte by 1

FF /0       INC ew        Increment EA word by 1

40+rw       INC rw        Increment word register by 1

6C         *INS eb,DX     Input byte from port DX into [DI]

6D         *INS ew,DX     Input word from port DX into [DI]

6C         *INSB          Input byte from port DX into ES:[DI]

6D         *INSW          Input word from port DX into ES:[DI]

CC          INT 3         Interrupt 3 (trap to debugger) (far call, with flags

CD ib       INT ib        Interrupt numbered by immediate byte     pushed first)

CE          INTO          Interrupt 4 if overflow flag is 1

CF          IRET          Interrupt return (far return and pop flags)

6-9

77 cb       JA cb         Jump short if above (CF=0 and ZF=0)    above=UNSIGNED

73 cb       JAE cb        Jump short if above or equal (CF=0)

72 cb       JB cb         Jump short if below (CF=1)             below=UNSIGNED

76 cb       JBE cb        Jump short if below or equal (CF=1 or ZF=1)

72 cb       JC cb         Jump short if carry (CF=1)

E3 cb       JCXZ cb       Jump short if CX register is zero

74 cb       JE cb         Jump short if equal (ZF=1)

7F cb       JG cb         Jump short if greater (ZF=0 and SF=OF)  greater=SIGNED

7D cb       JGE cb        Jump short if greater or equal (SF=OF)

7C cb       JL cb         Jump short if less (SF/=OF)                less=SIGNED

7E cb       JLE cb        Jump short if less or equal (ZF=1 or SF/=OF)

EB cb       JMP cb        Jump short (signed byte relative to next instruction)

EA cd       JMP cd        Jump far (4-byte immediate address)

E9 cw       JMP cw        Jump near (word offset relative to next instruction)

FF /4       JMP ew        Jump near to EA word (absolute offset)

FF /5       JMP md        Jump far (4-byte address in memory doubleword)

76 cb       JNA cb        Jump short if not above (CF=1 or ZF=1)

72 cb       JNAE cb       Jump short if not above or equal (CF=1)

73 cb       JNB cb        Jump short if not below (CF=0)

77 cb       JNBE cb       Jump short if not below or equal (CF=0 and ZF=0)

73 cb       JNC cb        Jump short if not carry (CF=0)

75 cb       JNE cb        Jump short if not equal (ZF=0)

7E cb       JNG cb        Jump short if not greater (ZF=1 or SF/=OF)

7C cb       JNGE cb       Jump short if not greater or equal (SF/=OF)

7D cb       JNL cb        Jump short if not less (SF=OF)

7F cb       JNLE cb       Jump short if not less or equal (ZF=0 and SF=OF)

71 cb       JNO cb        Jump short if not overflow (OF=0)

7B cb       JNP cb        Jump short if not parity (PF=0)

79 cb       JNS cb        Jump short if not sign (SF=0)

75 cb       JNZ cb        Jump short if not zero (ZF=0)

70 cb       JO cb         Jump short if overflow (OF=1)

7A cb       JP cb         Jump short if parity (PF=1)

7A cb       JPE cb        Jump short if parity even (PF=1)

7B cb       JPO cb        Jump short if parity odd (PF=0)

78 cb       JS cb         Jump short if sign (SF=1)

74 cb       JZ cb         Jump short if zero (ZF=1)

9F          LAHF          Load: AH = flags  SF ZF xx AF xx PF xx CF

0F 02 /r   *LAR rw,ew     Load: high(rw) = Access Rights byte, selector ew

C5 /r       LDS rw,ed     Load EA doubleword into DS and word register

8D /r       LEA rw,m      Calculate EA offset given by M, place in rw

C9         *LEAVE         Set SP to BP, then POP BP (reverses previous ENTER)

C4 /r       LES rw,ed     Load EA doubleword into ES and word register

0F 01 /2   *LGDT m        Load 6 bytes at m into Global Descriptor Table reg

0F 01 /3   *LIDT m        Load 6 bytes at m into Interrupt Descriptor Table reg

0F 00 /2   *LLDT ew       Load selector ew into Local Descriptor Table reg

0F 01 /6   *LMSW ew       Load EA word into Machine Status Word

F0          LOCK (prefix) Assert BUSLOCK signal for the next instruction

0F 33/r    #LODBITS rb,rb Load AX with DS:SI,bit rb (incr. SI,rb), rb+1 bits

0F 3B/0 ib #LODBITS rb,ib Load AX with DS:SI,bit rb (incr. SI,rb), ib+1 bits

AC          LODS mb       Load byte [SI] into AL, advance SI

AD          LODS mw       Load byte [SI] into AL, advance SI

AC          LODSB         Load byte [SI] into AL, advance SI

AD          LODSW         Load byte [SI] into AL, advance SI

6-10

E2 cb       LOOP cb       noflags DEC CX; jump short if CX/=0

E1 cb       LOOPE cb      noflags DEC CX; jump short if CX/=0 and equal (ZF=1)

E0 cb       LOOPNE cb     noflags DEC CX; jump short if CX/=0 and not equal

E0 cb       LOOPNZ cb     noflags DEC CX; jump short if CX/=0 and ZF=0

E1 cb       LOOPZ cb      noflags DEC CX; jump short if CX/=0 and zero (ZF=1)

0F 03 /r   *LSL rw,ew     Load: rw = Segment Limit, selector ew

0F 00 /3   *LTR ew        Load EA word into Task Register

A0 iw       MOV AL,xb     Move byte variable (offset iw) into AL

A1 iw       MOV AX,xw     Move word variable (offset iw) into AX

8E /3       MOV DS,mw     Move memory word into DS

8E /3       MOV DS,rw     Move word register into DS

C6 /0 ib    MOV eb,ib     Move immediate byte into EA byte

88 /r       MOV eb,rb     Move byte register into EA byte

8E /0       MOV ES,mw     Move memory word into ES

8E /0       MOV ES,rw     Move word register into ES

8C /1       MOV ew,CS     Move CS into EA word

8C /3       MOV ew,DS     Move DS into EA word

C7 /0 iw    MOV ew,iw     Move immediate word into EA word

8C /0       MOV ew,ES     Move ES into EA word

89 /r       MOV ew,rw     Move word register into EA word

8C /2       MOV ew,SS     Move SS into EA word

B0+rb ib    MOV rb,ib     Move immediate byte into byte register

8A /r       MOV rb,eb     Move EA byte into byte register

B8+rw iw    MOV rw,iw     Move immediate word into word register

8B /r       MOV rw,ew     Move EA word into word register

8E /2       MOV SS,mw     Move memory word into SS

8E /2       MOV SS,rw     Move word register into SS

A2 iw       MOV xb,AL     Move AL into byte variable (offset iw)

A3 iw       MOV xw,AX     Move AX into word register (offset iw)

A4          MOVS mb,mb    Move byte [SI] to ES:[DI], advance SI and DI

A5          MOVS mw,mw    Move word [SI] to ES:[DI], advance SI and DI

A4          MOVSB         Move byte DS:[SI] to ES:[DI], advance SI and DI

A5          MOVSW         Move word DS:[SI] to ES:[DI], advance SI and DI

F6 /4       MUL eb        Unsigned multiply (AX = AL * EA byte)

F7 /4       MUL ew        Unsigned multiply (DXAX = AX * EA word)

F6 /3       NEG eb        Two's complement negate EA byte

F7 /3       NEG ew        Two's complement negate EA word

NIL (prefix)  Special "do-nothing" opcode assembles no code

90          NOP           No Operation

F6 /2       NOT eb        Reverse each bit of EA byte

F7 /2       NOT ew        Reverse each bit of EA word

0F 16/0    #NOTBIT eb,CL  Complement bit CL of eb

0F 17/0    #NOTBIT ew,CL  Complement bit CL of ew

0F 1E/0 ib #NOTBIT eb,ib  Complement bit ib of eb

0F 1F/0 ib #NOTBIT ew,ib  Complement bit ib of ew

0C ib       OR AL,ib      Logical-OR immediate byte into AL

0D iw       OR AX,iw      Logical-OR immediate word into AX

80 /1 ib    OR eb,ib      Logical-OR immediate byte into EA byte

08 /r       OR eb,rb      Logical-OR byte register into EA byte

81 /1 iw    OR ew,iw      Logical-OR immediate word into EA word

09 /r       OR ew,rw      Logical-OR word register into EA word

0A /r       OR rb,eb      Logical-OR EA byte into byte register

0B /r       OR rw,ew      Logical-OR EA word into word register

6-11

E6 ib       OUT ib,AL     Output byte AL to immediate port number ib

E7 ib       OUT ib,AX     Output word AX to immediate port number ib

EE          OUT DX,AL     Output byte AL to port number DX

EF          OUT DX,AX     Output word AX to port number DX

6E         *OUTS DX,eb    Output byte [SI] to port number DX, advance SI

6F         *OUTS DX,ew    Output word [SI] to port number DX, advance SI

6E         *OUTSB         Output byte DS:[SI] to port number DX, advance SI

6F         *OUTSW         Output word DS:[SI] to port number DX, advance SI

1F          POP DS        Set DS to top of stack, increment SP by 2

07          POP ES        Set ES to top of stack, increment SP by 2

8F /0       POP mw        Set memory word to top of stack, increment SP by 2

58+rw       POP rw        Set word register to top of stack, increment SP by 2

17          POP SS        Set SS to top of stack, increment SP by 2

61         *POPA          Pop DI,SI,BP,SP,BX,DX,CX,AX (SP value is ignored)

9D          POPF          Set flags register to top of stack, increment SP by 2

0E          PUSH CS       Set [SP-2] to CS, then decrement SP by 2

1E          PUSH DS       Set [SP-2] to DS, then decrement SP by 2

06          PUSH ES       Set [SP-2] to ES, then decrement SP by 2

6A ib      *PUSH ib       Push sign-extended immediate byte

68 iw      *PUSH iw       Set [SP-2] to immediate word, then decrement SP by 2

FF /6       PUSH mw       Set [SP-2] to memory word, then decrement SP by 2

50+rw       PUSH rw       Set [SP-2] to word register, then decrement SP by 2

16          PUSH SS       Set [SP-2] to SS, then decrement SP by 2

60         *PUSHA         Push AX,CX,DX,BX,original SP,BP,SI,DI

9C          PUSHF         Set [SP-2] to flags register, then decrement SP by 2

D0 /2       RCL eb,1      Rotate 9-bit quantity (CF, EA byte) left once

D2 /2       RCL eb,CL     Rotate 9-bit quantity (CF, EA byte) left CL times

C0 /2 ib   *RCL eb,ib     Rotate 9-bit quantity (CF, EA byte) left ib times

D1 /2       RCL ew,1      Rotate 17-bit quantity (CF, EA word) left once

D3 /2       RCL ew,CL     Rotate 17-bit quantity (CF, EA word) left CL times

C1 /2 ib   *RCL ew,ib     Rotate 17-bit quantity (CF, EA word) left ib times

D0 /3       RCR eb,1      Rotate 9-bit quantity (CF, EA byte) right once

D2 /3       RCR eb,CL     Rotate 9-bit quantity (CF, EA byte) right CL times

C0 /3 ib   *RCR eb,ib     Rotate 9-bit quantity (CF, EA byte) right ib times

D1 /3       RCR ew,1      Rotate 17-bit quantity (CF, EA word) right once

D3 /3       RCR ew,CL     Rotate 17-bit quantity (CF, EA word) right CL times

C1 /3 ib   *RCR ew,ib     Rotate 17-bit quantity (CF, EA word) right ib times

F3          REP (prefix)  Repeat following MOVS,LODS,STOS,INS, or OUTS CX times

65         #REPC (prefix) Repeat following CMPS or SCAS CX times or until CF=0

F3          REPE (prefix) Repeat following CMPS or SCAS CX times or until ZF=0

64         #REPNC (prefix)Repeat following CMPS or SCAS CX times or until CF=1

F2          REPNE (prefix)Repeat following CMPS or SCAS CX times or until ZF=1

F2          REPNZ (prefix)Repeat following CMPS or SCAS CX times or until ZF=0

F3          REPZ (prefix) Repeat following CMPS or SCAS CX times or until ZF=1

CB          RETF          Return to far caller (pop offset, then seg)

C3          RET           Return to near caller (pop offset only)

CA iw       RETF iw       RET (far), pop offset, seg, iw bytes

C2 iw       RET iw        RET (near), pop offset, iw bytes pushed before Call

6-12

D0 /0       ROL eb,1      Rotate 8-bit EA byte left once

D2 /0       ROL eb,CL     Rotate 8-bit EA byte left CL times

C0 /0 ib   *ROL eb,ib     Rotate 8-bit EA byte left ib times

D1 /0       ROL ew,1      Rotate 16-bit EA word left once

D3 /0       ROL ew,CL     Rotate 16-bit EA word left CL times

C1 /0 ib   *ROL ew,ib     Rotate 16-bit EA word left ib times

0F 28/0    #ROL4 eb       Rotate nibbles: Heb=Leb   HAL,Leb=LAL  LAL=Heb

D0 /1       ROR eb,1      Rotate 8-bit EA byte right once

D2 /1       ROR eb,CL     Rotate 8-bit EA byte right CL times

C0 /1 ib   *ROR eb,ib     Rotate 8-bit EA byte right ib times

D1 /1       ROR ew,1      Rotate 16-bit EA word right once

D3 /1       ROR ew,CL     Rotate 16-bit EA word right CL times

C1 /1 ib   *ROR ew,ib     Rotate 16-bit EA word right ib times

0F 2A/0    #ROR4 eb       Rotate nibbles: Leb=Heb   Heb=LAL  AL=eb

9E          SAHF          Store AH into flags  SF ZF xx AF xx PF xx CF

D0 /4       SAL eb,1      Multiply EA byte by 2, once

D2 /4       SAL eb,CL     Multiply EA byte by 2, CL times

C0 /4 ib   *SAL eb,ib     Multiply EA byte by 2, ib times

D1 /4       SAL ew,1      Multiply EA word by 2, once

D3 /4       SAL ew,CL     Multiply EA word by 2, CL times

C1 /4 ib   *SAL ew,ib     Multiply EA word by 2, ib times

D0 /7       SAR eb,1      Signed divide EA byte by 2, once

D2 /7       SAR eb,CL     Signed divide EA byte by 2, CL times

C0 /7 ib   *SAR eb,ib     Signed divide EA byte by 2, ib times

D1 /7       SAR ew,1      Signed divide EA word by 2, once

D3 /7       SAR ew,CL     Signed divide EA word by 2, CL times

C1 /7 ib   *SAR ew,ib     Signed divide EA word by 2, ib times

1C ib       SBB AL,ib     Subtract with borrow immediate byte from AL

1D iw       SBB AX,iw     Subtract with borrow immediate word from AX

80 /3 ib    SBB eb,ib     Subtract with borrow immediate byte from EA byte

18 /r       SBB eb,rb     Subtract with borrow byte register from EA byte

83 /3 ib    SBB ew,ib     Subtract with borrow immediate byte from EA word

81 /3 iw    SBB ew,iw     Subtract with borrow immediate word from EA word

19 /r       SBB ew,rw     Subtract with borrow word register from EA word

1A /r       SBB rb,eb     Subtract with borrow EA byte from byte register

1B /r       SBB rw,ew     Subtract with borrow EA word from word register

AE          SCAS mb       Compare bytes AL - ES:[DI], advance DI

AF          SCAS mw       Compare words AL - ES:[DI], advance DI

AE          SCASB         Compare bytes AX - ES:[DI], advance DI

AF          SCASW         Compare words AX - ES:[DI], advance DI

0F 14/0    #SETBIT eb,CL  Set bit CL of eb

0F 15/0    #SETBIT ew,CL  Set bit CL of ew

0F 1C/0 ib #SETBIT eb,ib  Set bit ib of eb

0F 1D/0 ib #SETBIT ew,ib  Set bit ib of ew

0F 01 /0   *SGDT m        Store 6-byte Global Descriptor Table register to M

D0 /4       SHL eb,1      Multiply EA byte by 2, once

D2 /4       SHL eb,CL     Multiply EA byte by 2, CL times

C0 /4 ib   *SHL eb,ib     Multiply EA byte by 2, ib times

D1 /4       SHL ew,1      Multiply EA word by 2, once

D3 /4       SHL ew,CL     Multiply EA word by 2, CL times

C1 /4 ib   *SHL ew,ib     Multiply EA word by 2, ib times

6-13

D0 /5       SHR eb,1      Unsigned divide EA byte by 2, once

D2 /5       SHR eb,CL     Unsigned divide EA byte by 2, CL times

C0 /5 ib   *SHR eb,ib     Unsigned divide EA byte by 2, ib times

D1 /5       SHR ew,1      Unsigned divide EA word by 2, once

D3 /5       SHR ew,CL     Unsigned divide EA word by 2, CL times

C1 /5 ib   *SHR ew,ib     Unsigned divide EA word by 2, ib times

0F 01 /1   *SIDT m        Store 6-byte Interrupt Descriptor Table register to M

0F 00 /0   *SLDT ew       Store Local Descriptor Table register to EA word

0F 01 /4   *SMSW ew       Store Machine Status Word to EA word

F9          STC           Set carry flag

FD          STD           Set direction flag so SI and DI will decrement

FB          STI           Set interrupt enable flag, interrupts enabled

0F 31/r    #STOBITS rb,rb Store AX to ES:DI,bit rb (incr. DI,rb), rb+1 bits

0F 39/0 ib #STOBITS rb,ib Store AX to ES:DI,bit rb (incr. DI,rb), ib+1 bits

AA          STOS mb       Store AL to byte [DI], advance DI

AB          STOS mw       Store AX to word [DI], advance DI

AA          STOSB         Store AL to byte ES:[DI], advance DI

AB          STOSW         Store AX to word ES:[DI], advance DI

0F 00 /1   *STR ew        Store Task Register to EA word

2C ib       SUB AL,ib     Subtract immediate byte from AL

2D iw       SUB AX,iw     Subtract immediate word from AX

80 /5 ib    SUB eb,ib     Subtract immediate byte from EA byte

28 /r       SUB eb,rb     Subtract byte register from EA byte

83 /5 ib    SUB ew,ib     Subtract immediate byte from EA word

81 /5 iw    SUB ew,iw     Subtract immediate word from EA word

29 /r       SUB ew,rw     Subtract word register from EA word

2A /r       SUB rb,eb     Subtract EA byte from byte register

2B /r       SUB rw,ew     Subtract EA word from word register

0F 22      #SUB4S         Sub CL nibbles BCD at DS:SI from ES:DI (CL even,NZ)

A8 ib       TEST AL,ib    AND immediate byte into AL for flags only

A9 iw       TEST AX,iw    AND immediate word into AX for flags only

F6 /0 ib    TEST eb,ib    AND immediate byte into EA byte for flags only

84 /r       TEST eb,rb    AND byte register into EA byte for flags only

F7 /0 iw    TEST ew,iw    AND immediate word into EA word for flags only

85 /r       TEST ew,rw    AND word register into EA word for flags only

84 /r       TEST rb,eb    AND EA byte into byte register for flags only

85 /r       TEST rw,ew    AND EA word into word register for flags only

0F 10/0    #TESTBIT eb,CL Test bit CL of eb, set Z flag

0F 11/0    #TESTBIT ew,CL Test bit CL of ew, set Z flag

0F 18/0 ib #TESTBIT eb,ib Test bit ib of eb, set Z flag

0F 19/0 ib #TESTBIT ew,ib Test bit ib of ew, set Z flag

9B          WAIT          Wait until BUSY pin is inactive (HIGH)

0F 00 /4   *VERR ew       Set ZF=1 if segment can be read, selector ew

0F 00 /5   *VERW ew       Set ZF=1 if segment can be written to, selector ew

6-14

9r          XCHG AX,rw    Exchange word register with AX

86 /r       XCHG eb,rb    Exchange byte register with EA byte

87 /r       XCHG ew,rw    Exchange word register with EA word

86 /r       XCHG rb,eb    Exchange EA byte with byte register

9r          XCHG rw,AX    Exchange  with word register

87 /r       XCHG rw,ew    Exchange EA word with word register

D7          XLAT mb       Set AL to memory byte [BX + unsigned AL]

D7          XLATB         Set AL to memory byte DS:[BX + unsigned AL]

34 ib       XOR AL,ib     Exclusive-OR immediate byte into AL

35 iw       XOR AX,iw     Exclusive-OR immediate word into AX

80 /6 ib    XOR eb,ib     Exclusive-OR immediate byte into EA byte

30 /r       XOR eb,rb     Exclusive-OR byte register into EA byte

81 /6 iw    XOR ew,iw     Exclusive-OR immediate word into EA word

31 /r       XOR ew,rw     Exclusive-OR word register into EA word

32 /r       XOR rb,eb     Exclusive-OR EA byte into byte register

33 /r       XOR rw,ew     Exclusive-OR EA word into word register

* Starred forms will not execute on 8086/8088!  See note at top of chart.

# These instructions work only on NEC chips!  See note at top of chart.