Sicko's Celerity 2.xx HST Diagnostics Utility

Version 1.10

Released Feb, 1994

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

DESCRIPTION

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

CEL-HST is a program which, in conjunction with U.S. Robotics HST Modems, can

monitor line noise, keep a history of line noise problems, and even inform

your callers when they experience line noise.

CEL-HST is compatible with Celerity 2.xx.  It is placed in your main.bat

file so that it is run after every caller logs off.  When run, it

issues an ATI6 command to your U.S. Robotics HST modem.  The ATI6 command asks

the modem to report session statistics for the most recent connection.  These

statistics hold the clues as to what kind of line noise a caller experienced

while online.

By running CEL-HST you can AUTOMATICALLY inform your users when they experience

excessive line noise.  This helps them to understand why their file transfers

were slower than expected, or why the keyboard response time was sluggish.

What many people seem to have forgotten is that, in spite of the advent of MNP

and v.42 modems, line noise STILL EXISTS today!  These fancy modems do not

magically make line noise disappear.  Rather, these error correcting modems do

exactly as their name implies:  they CORRECT errors that occur due to line

noise.  Line noise still exists even though the caller no longer sees it, as

garbage while typing, or as errors in file transfers.  Instead, the caller sees

line noise as a "slow down" in file transfer speeds or as "sluggishness" when

typing at the keyboard.

Even sysops, the ones that are more likely to understand these new modems,

often jump to the erroneous conclusion that when a transfer is slower than

expected that it must be the fault of:  a) the software, b) the network, c)

the multitasker, or d) the modem.  What seldom crosses their minds is that the

true cause of the slowdown is probably nothing more than what people have

experienced ever since modems first came into existence:  line noise.

CEL-HST is here to help educate callers.  To let them know when they have

experienced excessive line noise.  To show them how it has been determined that

line noise was, in fact, experienced.  And to let them know what they can do to

improve the situation.

After a caller is disconnected, CEL-HST will be run.  It will issue the ATI6

command and then ANALYZE the results to determine if excessive line noise was

experienced.  If it was, then the program will automatically generate a message

for the caller, placing it in their NOTE file and is displayed the next time

they login. The customized message will indicate what was

experienced and what the caller can do to improve his connections.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

1) This program works ONLY with USR HST modems.  That includes both the regular

HST as well as the HST Dual Standard.  The reason for this restriction is

that the only modems that have the ATI6 command, that we are aware of, are

the HST line of modems.

2) I've only tested this with COM1, I havent looked at the code lately but

there is a command line for which com port to use. I doubt it will work

with anything BUT 1 & 2.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

INSTALLATION

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

To install CEL-HST all you need to do is copy the following files into your

utilities directory:

CEL-HST.EXE

CEL-HST.TXT

CEL-HST relies on CALLER.LST that is created by Celerity in each of the

node directories as a user logs-on. This is a NODE SPECIFIC UTILITY.

Modify you MAIN.BAT so that after celerity exits and its about to recycle

add the following lines:

The sample looks like this

f:\celerity\utils\cel-hst.exe 1 f:\node2 f:\celerity\filemail f:\celerity\utils\cel-hst.txt /dbg

del f:\node2\caller.lst

You SHOULD delete the caller.lst after you run CEl-HST, so the same caller

doesn't get duplicate messages if the board cycles for some reason.

A breakdown of the command line follows:

f:\celerity\utils\cel-hst.exe 1 f:\node2 f:\celerity\filemail f:\celerity\utils\cel-hst.txt /dbg

^^^^^^^^^^^^^^^^^^^^^^^^|^^^^ | ^^^^|^^^ ^^^^^^^^^|^^^^^^^^^^ ^^^^^^^^^^^^^^|^^^^^^^^^^^^^^ ^^|^

|     |     |             |                         |                 |

Path to CEL-HSTS.EXE ---|     |     |             |                         |                 |

|     |             |                         |                 |

Com Port----------------------|     |             |                         |                 |

|             |                         |                 |

Path to Specific Node---------------|             |                         |                 |

|                         |                 |

Path to mail directory----------------------------|                         |                 |

|                 |

Path and filename of text file to read message from-------------------------|                 |                                                            |

|

Debug if you want it (It will write out a dup to 0.NOT)---------------------------------------|

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

CUSTOMIZATION

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The CEL-HST.TXT file is a text file which can be translated or modified with a

few small guidelines:

1) The first three lines will be written verbatim to the users' note file.

The only parameter passed is the Node #. As long as the "@1" is not removed, the

NODE NUMBER will be put in its place.  If you are not running multiple

nodes or don't wish to have the node number recorded in the message you

can remove the entire "(NODE @1)" portion of the subject.

2) There are 7 more sections in the file:  TOPTEXT, RESENT, RETRAINS,

MODEMERRORS, FINALSPEED, DISCONNECT

Each section begins with ** and the name of the section.

The last line of each section is just ** (or the end of the file).  The

ORDER of the file is VERY IMPORTANT.  Do not change the order of the

sections. And make sure there is an ** after each section.

3) The content of each section can be rewritten, translated, modified in

anyway you see fit with just one exception:

The "@1" and "@2" and "@3"parameters in each section MUST REMAIN in the same

order and in the same section in which they are found.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

WHAT CAN YOU DO ABOUT LINE NOISE

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Unfortunately, the only course of action *may* be to simply hang up and try

your call again - hoping for a cleaner connection the next time around.

The phone company can route your call MANY different ways - even when calling

across town but more so when calling long distance.

Some things you can do are:

1) Use only high quality modems with v.32 and v.32bis modulation with mnp or

v.42 error correction.

2) Call your local phone company and have them inspect your phone lines.

3) Try a different long distance carrier.

If you are calling long distance then switching to a different long distance

carrier can often result in cleaner phone lines.  In our experience, and in

the experience of several of our customers, the most reliable long distance

carrier has been AT&T.  (though in some areas even AT&T may not be the best

carrier to use)

EVEN IF AT&T is not your default long distance carrier you can TRY THEM OUT

as an experiment to see if AT&T can give you a better connection.  All you

have to do is dial 10288 in front of the number you want to dial.

For example, to reach the Celerity Support BBS via AT&T dial:

10288,1-310-693-9405.

If AT&T *is* your long distance carrier then trying a *different* carrier

may still be the solution!  Here are a few more common codes:

10022 - MCI

10033 - SPRINT

10700 - Tel-America

10759 - Telenet

There are many more.  Contact your local phone company for other long

distance carrier "equal access" codes.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

ANALYSIS

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

So just what DOES the analysis entail?  The following are the current rules

used by CEL-HST to determine whether or not a message should be generated.  If

one or more of the following is true the a message is written to the caller:

- 1000 or more blocks were sent AND 25% or more blocks had to be RE-sent

- Retrains Requested or Granted is greater than 20

- The number of Blers, Link Naks or Link Timeouts, individually, is

greater than 150

Note that values less than those looked for in the above can STILL indicate

that line noise was experienced but that it simply was not sufficient to

warrant a message being generated.  If a caller asks why the transfers were

slow but did not receive an automated message, you can look up in the capture

file to get a copy of the ATI6 report.

The number of blocks RE-sent is a very clear indication of line noise.  Any

amount of time spent RE-sending a block of data that has already been sent has

a direct impact on the throughput and is only necessary if the receiving modem

was unable to receive the block properly the first time around.

Retrains requested and granted is another very clear indication of line noise.

The modems attempt to shift to a lower (or higher) bps rate as line conditions

change in search of a speed which can be more effecient.  In other words, it is

possible for a 4800 bps connection to be FASTER than a 9600 bps connection if

it means that fewer errors will occur!

Modem errors such as Blers, Link Naks and Link Timeouts also generally reflect

line noise and are often seen in combination with the above items.  However,

they can also be the result of INCOMPATIBLE MODEMS talking to each other.  It

is highly recommended that the highest quality modems be used.  In the world of

modems you really do "get what you pay for".

The final speed of the connection is also reported if it is SLOWER than the

original connection speed.  Obviously if retrains have caused the connection

speed to drop from 14400 bps down to 7200 bps the caller should *expect* to see

slower file transfers.  CEL-HST will let the caller know that this occured even

though his very own modem likely will not!

The disconnect reason is provided so that the caller can see why the connection

was dropped.  Many, for some reason, tend to believe that the SOFTWARE HUNG UP

on them when, in fact, it was the modem that decided to hang up due to the

excessive line noise.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

DISCLAIMER

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

I'm not responsible for anything, anywhere, anytime, that you may think

that this program has done.

Use at your own risk.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

REVISION HISTORY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Version 1.0

-----------

I wrote this about 4-5 months ago (Oct, 1993) and have been using it on my

board, my users seem to like it. I also have found it useful, for tracking

down problems with modems, bad phone wiring, and most importantly I don't

receive messages from users stating that the board hung-up on them anymore.

Got the idea while trying to fix these 21.6 USR modems. Then saw the authors

of PCBoard already beat me to the idea. So I borrowed their text-file (what

you see here almost verbatim) and wrote all the code myself.

PCBATI6 is copyrighted by CDC. I modified their docs, if they wish to pursue

this they can track me down.

I've released this freeware since I borrowed their docs.

I'd like to thank all those that have supported me up to this point; The Byter,

(Lenon, Baby Bear : My co-sysops), Zygote and Hobby1, and Wacky Rabbit for a

registration trade.

Sadly, for me atleast, for I enjoy programming for Celerity, this will be the

last utility I release for Celerity, there is no interest from the other

Celerity sysops whatsoever.

Version 1.10

------------

I did not say I would not continue to program. Just that I will not start

any new endeavoures (sp?) but will continue to support the utilities I've

already started.

From time to time CEL-HST would not read the modem. I played and played with

it and it just seems flakey. It randomly gets a timeout error while waiting

for the modem to respond, even when the modem is pumping info to the comport.

I added a bunch of new messages. It will now write out an error message to

the 0.NOT while when things go wrong. I added a bunch more error reporting.

I am now checking for a 0 baud connection and ignoring this. Added colors

to the CEL-HST.TXT file (you can do your own).

2/13/94

Added more error stuff today