* ��������������������������������������������� *                  /

/                                                //

/                                                 / /

/                                                  / /

/                                                   / /

/                                                   /  /

/                   MGCOMTP v0.9b                    /  /

/           Copyright 1994 by Mark Gauthier.         /   /

/                                                    /    /

/                                                     /    /

/                                                     /    /

/                                                     /     /

������������������������������������������������������Ŀ     /

�                                                      �    /

�       HS    AA    CD   OH   RD   SD   TR   MR        �  /

�                                                      �/

��������������������������������������������������������

C O M U N I C A T I O N  F U N C T I O N S  &  P R O C E D U R E S

WHAT IS MGCOMTP

���������������

It a unit for TURBO PASCAL that content many procedures and

functions to access comport.  It is possible to access up to

eight ports simultaneously and these ports can be open via a

fossil driver or an interrupt that use 8250 registers.

IMPORTANT

���������

This software is SHAREWARE so, i don't guaranty anything

except it will take space on your disk.  You are responsible

of any damage that could, append during execution of this

program.

REQUIREMENT

�����������

TURBO PASCAL VERSION 5.5,  6,  7, or BORLAND PASCAL 7.0

You must rename coresponding file to .TPU.

MGCOMTP0.TP5 -> MGCOMTP0.TPU  ( TP 5.5                   )

MGCOMTP0.TP6 -> MGCOMTP0.TPU  ( TP 6.0                   )

MGCOMTP0.TP7 -> MGCOMTP0.TPU  ( TP 7.0, BP 7.0 Real Mode )

MGCOMTP0.TPP ->               ( BP 7.0 Protected mode    )

REGISTERED

����������

If you plan to use this software, please complete

the information sheet at the bottom of this document.

HOW TO CONTACT AUTHOR

���������������������

By electronics mail on any system:

FIDONET

Mark Gauthier �> 1:242/818.5

FRANCOMEDIA

Mark Gauthier �> 101:160/305.5

or by mail.

MGCOM / MARK GAUTHIER

58  �milien�FR�NETTE, Sainte�TH�RESE.

QU�BEC, CANADA

J7E 5K3

FUNCTION        OPENCOM

��������������������������������������������������������������������������

SYNTAX:   OpenCom(ComPort:Byte; C: Connect_Type): Boolean;

DESCRIPTION:

Open a comport whit or whitout a FOSSIL driver.

Case type of connection is PORTS, then this function

reserve buffers of transmission and install an interrupt.

PARAMETERS:

comport; a number from 1�8, port to open.

C; How to open port via fossil = FOSSIL, whitout = PORTS.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt;

{  Open whit fossil COM1  }

END.

BEGIN

IF NOT OpenCom(1, Ports) THEN Halt;

{  Open whitout fossil COM1  }

END.

PROCEDURE       CLOSECOM

��������������������������������������������������������������������������

SYNTAX:   CloseCom(ComPort:Byte);

DESCRIPTION:

Close a comport that was previously open via function

OpenCom.  If com was open whitout fossil, interrupt driver is

uninstall and buffers are clears.  DTR is leave high.

PARAMETERS:

comport; a number from 1�8, port number to close.

EXAMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1  }

CloseCom(1);                         { CLOSE COM1 }

END.

PROCEDURE       CLOSEALLCOMS

��������������������������������������������������������������������������

SYNTAX:   CloseAllComs;

DESCRIPTION:

Procedure that made exactly the same thing than CloseCom,

except this one close all com currently open via function

OpenCom.

PARAMETERS:

Nothing;

EXAMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { Open COM1   }

IF NOT OpenCom(2, Fossil) THEN Halt; { Open COM2   }

CloseAllComs;                        { Close COM's }

END.

PROCEDURE       HANGUP

��������������������������������������������������������������������������

SYNTAX:   HangUp;

DESCRIPTION:

Procedure that drop line using DTR.  It leave DTR low for

some 1/1000sec, and return whit DTR high.  Another way to do

this is to send "+++" wait a bit, and send ATH+#13.

PARAMETERS:

Nothing;

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) Then;  { OPEN COM1        }

{   }                            { Any code.        }

HangUp;                          { Dropping the line }

CloseAllComs;                    { Close all com's. }

END.

FUNCTION        CHECKCD

��������������������������������������������������������������������������

SYNTAX:   CheckCD(ComPort:Byte): Boolean;

DESCRIPTION:

Function that verify the status of CARRIER DETECT that

itself specify if line is up or down.  If CARRIER DETECT is

on this function will return TRUE else it return FALSE.

PARAMETERS:

comport; a number from 1�8, com number to check.

EXEMPLE:

CONST ESC = 27;

VAR Ch:Char;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { Open COM1        }

REPEAT

Ch:=ReadKey;                   { Enter character }

Write(Ch);                     { Writing          }

IF NOT CheckCD(1) THEN         { Check CD value.  }

Write('line cut.  ');          { CD is OFF        }

UNTIL Ch=Esc;                        { While <> ESC     }

END.

PROCEDURE       SETDTR

��������������������������������������������������������������������������

SYNTAX:   SetDTR(ComPort:Byte; S:Boolean);

DESCRIPTION:

Procedure that set the status of DTR.  If S = TRUE then

DTR is set high, else dtr is set to low.

PARAMETERS:

comport; a number from 1�8, comport number to set dtr.

S; specify if a DTR must be low or high.

EXEMPLE:

USES Crt;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1       }

SetDTR(1, TRUE);                     { DTR (COM1) High }

SetDTR(1, FALSE);                    { DTR (COM1) Low  }

END.

PROCEDURE       PURGEINPUT

��������������������������������������������������������������������������

SYNTAX:   PurgeInput(ComPort:Byte);

DESCRIPTION:

Procedure use to destroyed all waiting data in the input

Buffer.

PARAMETERS:

comport; a number from 1�8, com number where you drop data.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1          }    }

PurgeInput(1);                       { PURGE INPUT BUFFER }

END.

PROCEDURE       PURGEOUTPUT

��������������������������������������������������������������������������

SYNTAX:   PurgeOutput(ComPort:Byte);

DESCRIPTION:

This procedure is similar to Purge Input, except that

output buffer is clear.

PARAMETERS:

comport; a number from 1�8, com number to clear OutBuf.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1           }

PurgeOutput(1);                      { PURGE OUTPUT BUFFER }

END.

PROCEDURE       FLUSHOUTPUT

��������������������������������������������������������������������������

SYNTAX:   FlushOutPut(ComPort:Byte);

DESCRIPTION:

This procedure wait until all characters in output buffer

are send before returning.

EXEMPLE:

BEGIN

IF NOT OpenCom(1 Fossil) THEN Halt; { OUVERTURE cause COM      }

FlushOutput(1);                     { FLUSH "OUTPUT" (COM1) }

END.

FUNCTION        READCH

��������������������������������������������������������������������������

SYNTAX:    ReadCh(ComPort:Byte): Char;

DESCRIPTION:

Function that verify if a character is waiting in the

input buffer.  If no character is available, the result return

is #0.  If a characters is available then the result is the

character number.

PARAMETERS:

comport; a number from 1�8, com number to scan.

EXEMPLE:

VAR Ch:Char;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1          }

REPEAT

Ch:=ReadCh(1);                 { SCAN COM1          }

UNTIL CH <> #0;                      { if a character is }

END.                                      { received then EXIT.}   }

FUNCTION        READCHW

��������������������������������������������������������������������������

SYNTAX:    ReadChW(ComPort:Byte): Char;

DESCRIPTION:

Function similar to ReacCh, except this one return only

when character is available.

PARAMETERS:

comport; a number from 1�8, com number to scan.

EXEMPLE:

VAR Ch:Char;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1 }

Ch:=ReadChW(1);                      { SCAN COM1 }

END.

FUNCTION        MULTIRECVCH

��������������������������������������������������������������������������

SYNTAX:    MultiRecvCh(VAR ComPorts: Byte): Char;

DESCRIPTION:

Function that return a character from one of the

specify COM in parameter Comports.  If all port specify by

Comports as been scan and no character is available then

result is #0.  If one character as been pick�up, then

result contain the character number and comports, contain

the com number where the character as been pick�up.

PARAMETERS:

comports;  hold port number's to be scan.

7 6 5 4 3 2 1 0       (Bits)

� � � � � � � �> COM1

� � � � � � ���> COM2

� � � � � �����> COM3

� � � � �������> COM4

� � � ���������> COM5

� � �����������> COM6

� �������������> COM7

���������������> COM8

EXEMPLE:

VAR ComPorts:Byte;

Ch:Char;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { Open COM1        }

IF NOT OpenCom(3, Fossil) THEN Halt; { Open COM3        }

ComPorts:=1+4;                       { Com1+Com3        }

REPEAT

Ch:=MultiRecvCh(ComPorts);     { SCAN COM1 & COM3 }

UNTIL CH <> #0;                      { while than <> #0 }

END.

FUNCTION        MULTIRECVCHW

��������������������������������������������������������������������������

SYNTAX:    MultiRecvChW(VAR ComPorts: Byte): Char;

DESCRIPTION:

Function similar to MultiRecvCh, except, this one will

not return until a character is available.

PARAMETERS:

comports;  hold port number's to be scan.

7 6 5 4 3 2 1 0       (Bits)

� � � � � � � �> COM1

� � � � � � ���> COM2

� � � � � �����> COM3

� � � � �������> COM4

� � � ���������> COM5

� � �����������> COM6

� �������������> COM7

���������������> COM8

EXEMPLE:

VAR ComPorts:Byte;

Ch:Char;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1        }

IF NOT OpenCom(3, Fossil) THEN Halt; { OPEN COM3        }

ComPorts:=1+4;                       { Com1+Com3        }

Ch:=MultiRecvChW(ComPorts);          { SCAN COM1 & COM3 }

END.

FUNCTION        READBLOCK

��������������������������������������������������������������������������

SYNTAX:    ReadBlock(ComPort:Byte; BufPtr: Pointer;

MaxRead:Word): Word;

DESCRIPTION:

Function that transfer a specify number of bytes from input

buffer to a user buffer.  The result of the function indicate

the number of bytes transfer.

PARAMETERS:

comport; a number from 1�8, com buffer number to be scan.

BufPtr;  Pointer, that point your buffer (user buffer).

MaxRead; Maximum number of characters to transfert usualy this

is your buffer size.

EXEMPLE:

VAR BytesRead:Word;

BUF : Array[1  10] OF Char;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt;            { Open COM1 }

BytesRead:=ReadBlock(1,Addr(Buf[1]),Size(Buf)); { READ Buf  }

END.

FUNCTION        WRITECH

��������������������������������������������������������������������������

SYNTAX:    WriteCh(ComPort:Byte; Ch:Char): Boolean;

DESCRIPTION:

Function that send a character, return true is the

character as been accept and put in output buffer else

return false.

PARAMETERS:

comport; a number from 1�8, com number to send ch.

Ch;      character to send.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1        }

IF NOT WriteCh(1, '!') THEN          { SEND character  }

Write('a "buffer" is full.');

END.

PROCEDURE       WRITECHW

��������������������������������������������������������������������������

SYNTAX:   WriteChW(ComPort:Byte; Ch:Char);

DESCRIPTION:

Procedure similar to WriteCh, except this one will not

return until it find a place in output buffer for your

character.

PARAMETERS:

ComPort; a number from 1�8, com number to send character.

Ch;      character to send.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1        }

WriteChW(1, '!')                     { SEND character  }

END.

PROCEDURE       WRITESTRW

��������������������������������������������������������������������������

SYNTAX:   WriteStrW(ComPort:Byte; S:String);

DESCRIPTION:

Procedure that send a string.  Procedure will not return

until all character of the string as been send.

PARAMETERS:

ComPort; a number from 1�8, com number to send string.

S;       String of character.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1   }

WriteStrW(1, 'Hello!!!')             { SEND STRING }

END.

PROCEDURE       WRITESTRLNW

��������������������������������������������������������������������������

SYNTAX:   WriteStrLnW(ComPort:Byte; S:String);

DESCRIPTION:

Procedure similar to WriteStrW, except this one

add linefeed and Carriage Return code to the end of your

string.

PARAMETERS:

comport; a number from 1�8, com number to send string.

S;       a string of character.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1   }

WriteStrLnW(1, 'Hello!!!')           { SEND STRING }

END.

PROCEDURE       WRITESTRDELAY

��������������������������������������������������������������������������

SYNTAX:   WriteStrDelay(ComPort:Byte; S:String; x: Word);

DESCRIPTION:

Procedure that send a string of character, and take a pause

of X 1/1000sec between each character.  This procedure

do not return until all character of the string as been send.

PARAMETERS:

comport; a number from 1�8, com number to send string.

S;       a string of character.

X;       number of 1/1000sec between each sends.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1   }

WriteStrDelay(1, 'Hello!!!', 55)     { SEND STRING }

END.

PROCEDURE       WRITESTRLNDELAY

��������������������������������������������������������������������������

SYNTAX:   WriteStrLnDelay(ComPort: Byte; S:String; x: Word);

DESCRIPTION:

Procedure similar to WriteStrDelay, except this one

add linefeed and Carriage Return to your string.

PARAMETERS:

comport; a number from 1�8, com number to send string.

S;       a string of character.

X;       number of 1/1000sec to wait between each sends.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1   }

WriteStrLnDelay(1, 'Hello!!!', 55)   { SEND STRING }

END.

FUNCTION        WRITEBLOCK

��������������������������������������������������������������������������

SYNTAX:    WriteBlock(ComPort:Byte; BufPtr:Pointer;

MaxSend:Word): Word;

DESCRIPTION:

Function that transfer data from user buffer, to output

buffer.  The function result content the number of byte

transfer.

PARAMETERS:

comport; a number from 1�8, com buffer number to transfer.

BufPtr;  Pointer, that point on user buffer.

MaxSend; Maximum number of characters to transfer usually,

the size of user buffer.

EXEMPLE:

CONST Buf:Array[1  5] OF CHAR = ('1' '2' '3' '4' '5');

x:Word;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt;    { OPEN COM1  }

x:=WriteBlock(1, Addr(Buf[1]), SizeOf(Buf));

{ TRANSFERE BUFFER.                                  }

{ X = a number of characters accept.                 }

END.

PROCEDURE       MULTISENDCHW

��������������������������������������������������������������������������

SYNTAX:   MultiSendChW(var ComPorts: Byte; ch:Char);

DESCRIPTION:

Procedure that send a character in all com specify in

parameter Comports.  It will not return until all com

accept the character.

PARAMETERS:

comports;  hold port's number to use.

7 6 5 4 3 2 1 0       (Bits)

� � � � � � � �> COM1

� � � � � � ���> COM2

� � � � � �����> COM3

� � � � �������> COM4

� � � ���������> COM5

� � �����������> COM6

� �������������> COM7

���������������> COM8

Ch;        Character to be send.

EXEMPLE:

VAR ComPorts:Byte;

Ch:Char;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { Open a COM1         }

IF NOT OpenCom(3, Fossil) THEN Halt; { Open a COM3         }

ComPorts:=1+4;                       { Com1+Com3           }

Ch:=#27;

MultiSendChW(ComPorts,Ch);           { Send in COM1 & COM3 }

END.

FUNCTION        MULTISENDCH

��������������������������������������������������������������������������

SYNTAX:    MultiSendCh(ComPorts: Byte; ch:Char): Byte;

DESCRIPTION:

Function similar a la procedure MultiSendChW, except

this one will not wait to send the character in all com.

The function result hold which com have accept the character.

PARAMETERS:

comports;  hold port's number to use.

7 6 5 4 3 2 1 0       (Bits)

� � � � � � � �> COM1

� � � � � � ���> COM2

� � � � � �����> COM3

� � � � �������> COM4

� � � ���������> COM5

� � �����������> COM6

� �������������> COM7

���������������> COM8

Ch;        Character to be send.

EXEMPLE:

VAR ComPorts:Byte;

Ch:Char;

Accept:Byte;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { Open COM1       }

IF NOT OpenCom(3, Fossil) THEN Halt; { Open COM3       }

ComPorts:=1+4;                       { Com1+Com3       }

Ch:=#27;

Accept:=MultiSendCh(ComPorts,Ch);    { Send Ch all COM }

If Accept and 1 = 1 Then Write('Com1 Accept ch');

If Accept and 4 = 4 Then Write('Com3 Accept ch');

END.

PROCEDURE       SETHANDSHAKE

��������������������������������������������������������������������������

SYNTAX:   SetHandShake(ComPort:Byte; HARD, Soft: Boolean);

DESCRIPTION:

Procedure that change the verification of transmit mode.

if HARD is TRUE, then RTS/CTS mode is active.  If

SOFT is TRUE then Xon/Xoff mode is active.  If they are

FALSE they are desactived.

PARAMETERS:

comport; a number from 1�8, com number to set.

Hard;    Status of RTS/CTS mode.

Soft;    Status of XON/XOFF mode.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1           }

SetHandShake(1,TRUE,TRUE);           { COM1 <�RTS/CTS ON   }

{ COM1 <�Xon/XOFF ON  }

END.

PROCEDURE       SETPARAMS

��������������������������������������������������������������������������

SYNTAX:  SetParams(ComPort:Byte; Baud:LongInt; WordSize:Byte;

Parity:Char; StopBits:Byte);

DESCRIPTION:

Modify communication parameters for a comport.

PARAMETERS:

comport; a number from 1�8, com number to modify.

Baud;    Transmission speed.

if : PORT �> (300�115200) : FOSSIL �> (300�38400)

Word Size; from 5�8.

Parity;   'N' �> NONE,

'O' �> ODD,

'E' �> EVEN.

EXEMPLE:

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1   }

SetParams(1,2400,8,'N',1);           { MODIFY COM1 }

END.

PROCEDURE       SETCOMHARDWARE

��������������������������������������������������������������������������

SYNTAX:   SetComHardWare(ComPort:Byte; COMADDR, ComIntr: Word);

DESCRIPTION:

Modify hardware setting for a comport.  This procedure

do nothing on comport open whit fossil.  You will need to

use type ports to open special comport address or interrupt.

PARAMETERS:

comport; a number from 1�8, com number to set.

ComAddr; HARDWARE address of this port.

ComIntr; INTERRUPT to use for this port.

EXEMPLE:

{ STANDART for COM1 is ADDR = $3F8 and INTR = 4 }

BEGIN

{ Exemple of NON STANDARD �> COM1 = $3F8 and INTR = 3 }

IF NOT OpenCom(1, Ports) THEN Halt; { OPEN COM1       }

SetComHardWare(1,$3F8,3);           { MODIFY COM1     }

END.

FUNCTION        GETFOSSILNAME

��������������������������������������������������������������������������

SYNTAX:    GETFOSSILNAME (ComPort:Byte; Var ID: String): Boolean;

DESCRIPTION:

Function that return the fossil driver name for a specify

port.  If this function return false the port specify is not

open whit a fossil driver so ID is unpredictable.  If the return

value is TRUE ID contain the fossil driver name.

PARAMETERS:

comport; a number from 1�8, com number to get info.

ID;      Contain the fossil driver name.

EXEMPLE:

VAR Name:String;

BEGIN

IF NOT OpenCom(1, Fossil) THEN Halt; { OPEN COM1     }

GetFossilName(1, Name);              { Recover name. }

END.

FUNCTION        GETSTATUS

��������������������������������������������������������������������������

SYNTAX:    GetStatus(ComPort:Byte): Word;

DESCRIPTION:

Function that return status for an open port.  Status

is different between Fossil and ports.

if COMPORT was open whit FOSSIL:

14 13 12 11 10 9 8 7 6 5 4 3 2 1 0 (Bits)

�  �          � � �

�  �          � � ����������������> "CARRIER Detect"

�  �          � ������������������> character available.

�  �          ��������������������> input buffer overflow

�  �

�  �

�  �������������������������������> space in output buffer

����������������������������������> output buffer empty

if COMPORT was open whit PORTS:

7 6 5 4 3 2 1 0 (Bits)

� � � � � � � ���> input buffer empty.

� � � � � � �����> input buffer full.

� � � � � �������> output buffer empty.

� � � � ���������> output buffer full.

� � � �����������> input buffer overflow.

� � �������������> output buffer overflow.

� ���������������> RTS/CTS.

�����������������> Xon/XOFF.

PARAMETERS:

comport; a number from 1�8, com number to get info.

EXEMPLE:

VAR Info:Word;

BEGIN

IF NOT OpenCom(1 Fossil) THEN Halt;  { OPEN COM1       }

Info:=GetStatus(1);                  { Recover STATUS  }

IF INFO AND 256 = 256 Then;  { character available ?  }

END.

REGISTERED INFO for MGCOM_TP v0 9

�������������������������������

Last name     :  ______________________________

First name    :  ______________________________

Address       :  ____________________________________________

City          :  ________________________

State         :  ____________________

Country       :  _________________

ZIP           :  _________________

DATE          :  _________________

Where did you get MGCOMTP ? ____________________________________________

***********************************************************************

A payment of 25$CAN or 20$US must be made to MARK GAUTHIER.

DON'T send cash, if you do so, you take the risk that your money

can be intercept.

***********************************************************************

Comments:

: ___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

MGCOM / MARK GAUTHIER

58 Emilien_FRENETTE, Sainte-Th�r�se

PQ, CANADA

J7E 5K3