Welcome

The PktGOLD Multimode test drive disk contains a self-extracting program,

TDSFX.EXE that creates a run-time version of the  commercial program

along with minimal documentation, a quick reference guide, and some

other useful files. A complete online help system is also provided.

Both are easily installed by typing the following:

INSTALL A: C:   (to install FROM A: to drive C:)

The installer puts the programs in a subdirectory called TESTDRIV

which you can erase later if you wish.

If you get this program from a BBS, it may be in a self-extracting

GOLDTD.exe file.

As a functional demo program, it can be used several times. You can

make multiple connects, Shell to DOS, Cut/Paste, and do many experiments

with the program. The program will run for 30 minutes at a time, which

is plenty of time to work through each of the following suggested

exercises.

If you like the program, please consider purchasing a copy. That way

we can provide you with updates, technical support, and you can provide

us with some incentive to keep at it, improving things to make your

operation easier, simpler, faster, and above all, more fun.

SOUND DEMO

The program starts by introducing you to the "Sound Demo" which makes

HF "sounds" on your PC speaker, and explains what you are hearing.

To skip the sound demo, answer "N" (no) to the first question. You

will be given a chance to run the "live" demo after that.

Things to Try

The program has many features and settings, but operating is simple

and efficient. To start with, on your first 30 minute tour, just

make a couple of connects to see what's involved.

If you're an experienced packet user, have a couple of callsigns ready

that you can connect to. You'll be doing a multiple connect right off

the bat.

Get the program going. Enter the right COM port and Baud rate, your

callsign and name. You'll do this each time you run the program.

The program will establish a link to your TNC. It may take a minute

or so the first time you run the program, but once you've got things

working, the program "comes up" very quickly. It tries NOT to reset

your TNC, that is why it takes some time to figure out how to avoid

resetting.  For your own convenience, LEAVE THE BATTERIES IN the tnc.

You'll want to leave batteries in your TNC, so that messages are saved,

and configuration information is retained even if you turn off your

TNC or it loses power.

LESSON #1: Single Connect.

1.  Type in just a callsign, e.g. WA4EGT or type in a path, such

as WA4EGT  or  WA4EGT VIA DANAPT if you need to digipeat.

Note: If you don't know who you can connect to, use the

Mheard function, [Alt-F2]. Notice that when you depress the

[Alt] key, the "keyline" below changes to show you what each

key does. [Alt-F2] is shown as MHEARD.

2.  Hit the [F7] key (connect key) when you've typed in a callsign.

Don't type C CALLSIGN, just CALLSIGN then [F7].

3.  After connecting, use [Alt-F7] to disconnect. Choose NORMAL

disconnect. Notice that if frames are still pending, the

disconnect is suspended until the frames are sent and

acknowledged.

If there is still time in the countdown sequence, hit [Alt-F7]

again, and select to "cancel the disconnect" request.  Then

you will have to do step #3 again to initiate another disconnect

request.

4.  Go to the NETWORK VIEW screen using [F2] if you are not there

already.

5.  Go to the PREVIOUS SESSIONS screen using [F3]. Notice that

the text of your previous session is there.

6.  Return to the connected sessions using the [F4] key.

LESSON #2: Multiple Connect

1.  Type in a callsign or path (callsign VIA ...) and hit [F7].

2.  Do step 1 at least once more, so that you have two (or more)

sessions going.

3.  Switch between sessions using the [F4] "NEXT" key.

4.  Switch to the NETWORK VIEW screen using the [F2] key. Notice

the full-screen.

5.  See a list of stations to which you are connected by depressing

the Ctrl key (by itself).

6.  Go to each session using the [F4] key. Enter the operator's name

using the [Alt-n] key sequence.  Do this for each session.

The names are saved permanently in Packet-GOLD, but are only

saved temporarily in the Test Drive version.

7.  Go back to the NETWORK VIEW screen using [F2].

8.  If text "comes in" from a session, and you are on another session

screen, or the Previous Sessions or Network View screen, you will

see a message "Pending From xxxx" in the upper right corner of

the status box.  Depressing the "next" key [F4] will take you

to that session immediately.

9.  Go to each session using the [F4] key, and start typing a line

but do not hit the [Enter] key to send it quite yet.  Switch

to each session and put in a partial line.  Notice that each

session has it's own "text entry/command entry" line. You can

move the cursor using the "arrow" keys, overwrite or insert

text.  The program saves the cursor "state" for each session

as you switch from session to session.

10.  Initiate a disconnect on each session. When all are disconnected,

go to the previous sessions screen using [F3] and review the

sessions.

LESSON #3: NET/ROM Node

1.  Identify a node, or several nodes that you use and can hit

easily.  Determine the node ALIAS. There are a number of

advantages to using node ALIAS names, rather than callsigns,

such as:

a.  You can establish multiple connects to the ALIAS that

are not possible using the node callsign.

b.  When a routed node disappears from the nodelist, you

will be informed with "invalid callsign" if aliases are

used. If just the callsign is used, and the routed node has

been dropped or disappeared, the node attempting the

route will drop back to an ordinary connect attempt and

eventually retry out.  Any callsign used and not found

in the nodelist is assumed to be a direct connect from

the node, and not a routed connect. Using ALIASES assures

that if the node has been dropped from the routes, you

will be immediately informed.

2.  Assume the node is called  NRNDE.  You want to connect to W1AW

after connecting to NRNDE. Type:

NRNDE|W1AW

on the command/text entry line and hit [F7]. The program will

establish a connect to NRNDE-n and then issue a connect to

W1AW automatically.

3.  Establish a few connects using the NRNDE node. You will notice

a couple of things:

a.  Each connect goes to a different SSID of NRNDE. For example

you might see NRNDE-10, and NRNDE-13 etc. The program

assures that if you use the "or bar (|)" technology, that

multiple connects to the are established automatically.

b.  The connect progress is shown to you on the session screen.

c.  As you make progress, you will notice the callsign in the

status banner indicates the current connected station.

4.  If you have a two hop connect, like NRNDE and then to SECNDE,

you can type:

NRNDE|SECNDE|W1AW  [F7]

The program will establish a connect to NRNDE-n, then request

a connect to SECNDE, and finally ask for a connect to W1AW.

Note: BPQ nodes may require using a "port" number in the

connect request (except when specifying a routed node in its

internal node list).  If W1AW is on port 1 of a BPQ node in

the example above, you would type:

NRNDE|SECNDE|1 W1AW  [F7]

5.  If you need to Digipeat into the first node, and the digipeater

is W2JUP, try:

NRNDE VIA W2JUP|SECNDE|W1AW [F7]

LESSON #4: Quick Connects

1.   Using the connect paths from prior examples, you can now

setup a few "quick connects" or "point and shoot" connects.

Once in the program, hit [Alt-S] for setup. Then highlight

QUICK CONNECTS, then choose ADD.

2.   Enter the name of the remote station, hit [Enter], then

type in the path.  For example:

Name:   ARRL HdQrtrs

Path:   NRNDE VIA W2JUP|SECNDE|W1AW

3.   Do this for each callsign. Some "paths" may simply be the callsign

of a station that is nearby, and doesn't require any special

digipeating or node usage.

When done, use the [ESC] key to bring you back to the program

window area.

4.   Press the [F7] key with a blank entry on the command line. This

causes the program to present you with a "pick list" of your

quick connects.  Highlight one, and hit [Enter] to start the

connect process.  Do this for all of the entries in your Quick-

Connects list.

5.   Note: For emergency use, when "Tactical" callsigns are contemplated,

enter the tactical callsign under as the NAME, and the FCC call

in the PATH. Also enter the Tactical callsign as the "NAME" of

the station in your NAMES list (created by [Alt-n] or in the

setup area under Names).

LESSON #5: Changing Parameters

1.   Enter a parameter name, such as MYALIAS and hit [F10], the

command key.  The program will display the contents of the

parameter.

2.   Type a parameter and a value, such as MYALIAS FIRESTN followed

by [F10]. This will enter the parameter value.

3.   Type DISPLAY [F10] to see a list of parameter groups. Highlight

one and hit [ENTER].  You may now move the cursor to a parameter

and alter it's value.  Typing DISPLAY M [F10] jumps immediately

to the Monitor parameters.

4.   Hit [F10] by itself (no parameters on the command entry line).

You will see (at a minimum) STARTUP and SHUTDOWN.  These are

the names of special parameter files with the extension .TNC.

Highlight STARTUP, hit [Enter] and notice that all of the

startup parameters are loaded into your TNC.

5.   Use [Alt-S] to get to the setup menu. Select TNC files.

Highlight STARTUP and hit [Enter].  This is a .TNC file. You

can edit it (we suggest not editing it unless you know what

you wish to change).  Use [F1] help to display your options.

6.   In the setup area, under TNC files, you will notice <new> and

<current>.  Selecting <new> allows you to create a parameter

file with settings you may wish to try.  Experiment.

LESSON #6: Sending CQ

1.   On the Network View screen, hit [Alt-U] to enter the CQ/UNPROTO

mode.  Now anything you type on the screen followed by the

[Enter] key is sent out as "unproto" or unnumbered information

frames.

2.   Look at the value of your UNPROTO by typing UNPROTO [F10].

If it is not CQ, then set unproto to CQ by typing:

UNPROTO CQ [F10]

3.   Each time you leave the Network View screen, the CQ/Unproto

switch is turned off. This is done to avoid sending frames

that are by accident. If you stay on the Network View screen,

and are in the CQ mode, all text you type is sent out as

[UI] frames.  You might set your UNPROTO to CQ VIA (some call).

Then you can see your CQ frames.

4.   If you have newer ROMS, set MXMIT (monitor Xmit) to on, by

typing MXMIT ON [F10]. This allows you to see when your frames

are transmitted.

LESSON #7: Changing Program Options

1.   Use [Alt-S] to bring up the setup menu. Select Setup from

the menu.  Explore the various menu areas.

2.   Look at the Alt-n messages. Notice that ?CALL and ?NAME are

in one or more messages. These are "macros" that are filled

in by the program with the current CALL and NAME of the

connected station.  To use an ALT-N message, when connected

to some station, hit [Alt-0] to [Alt-9] and the text will

be placed on the text entry line.  To erase or clear the

text entry line, use [Alt-X].  Using [Alt-n] followed by

[Alt-x] is a convenient way to find a message if you've

forgotten what it is. Or just type [Alt-S], S (for setup)

A (for Alt-n), look at the messages, and use [ESC] to

return to the program.

3.   Look at OTHER. Here you can change the morse code speed, and

several other interesting parameters.

4.   If you have EGA or VGA, try changing the monitor settings. You

can also adjust the size of the NETWORK view portion that

appears on each session screen.

LESSON #8: Maildrop Access

1.   If nobody is connected to your maildrop, and you are not

connected to other stations, use [Alt-F8] to access your

maildrop. Those with newer roms (August 1991 or later) and

our Enhanced Edition can access the maildrop even if connected.

2.   Use the L)list, R)ead, K)ill etc. commands. To leave the

maildrop, use B)ye.  Notice that text read and maildrop

commands appear in the Previous Sessions screen.

LESSON #9: Cut/Paste text

1.   Go to some screen (any one of Network View, Session, Previous

Session, Maildrop). Hit [F5]. Highlight "Select Lines" and

use the [End] key to signal the start and end of some text

lines.

2.   When the second [End] key is hit, you will be given options

again. You can save text, or edit it, or quit.  Quitting

leaves this menu, but saves the text in a CUT/PASTE clipboard.

3.   Go to some session where you'd like to Paste the text. You

read a message from your Maildrop or a session, and send it

to another session.  Hit [F5], choose "Copy text to session"

and voila, the text is "pasted" into that session. You can

paste the same text into other session, or even into the

maildrop.

LESSON #10: Exploration

1.   Scan through the Help list, [F1].  You will find lots of

interesting features there. You can turn on and off the

Binary filter, remove and insert your own text lines using

[Alt-L], search for text using [F9] and [Alt-F9], etc.

2.   The release version has many more features, including

binary file transfer, conferencing etc.

LESSON #10a: For PK232 owners

1.   Try the other modes. Use the enclosed "TNC" files if you

wish. Hit [F10] (with no parameters on the command line)

and highlight some mode, then hit ENTER.

2.   Use MYSEL [F10] to check your Amtor SELCAL.

Use [Alt-S] to enter the setup area, then TNC file editor.

Edit any TNC files the way you like them for the various modes.

You may find that simply typing the mode name is all that is

necessary to get into a non-packet mode.

3.   The [ESC] key is the transmit key down and up key. In non-packet

modes, start typing. When you want to key down and begin

transmitting hit the ESC key. Keep typing away. When you've

finished typing what and want your system to unkey after sending

the text, hit ESC again. Your text will continue being transmitted

until the text is sent, at which time it will unkey.

To leave ARQ AMTOR, just type AMTOR [F10]. To switch modes, type in

the mode name, followed by [F10] or use the TNC files for that mode

by hitting [F10] with a blank command entry line.

4.   AMTOR ARQ: Put the other station's SELCAL on the text entry line

(e.g. WEGT) and push the [F7] or "connect" key. The TNC will begin

Phasing in ARQ.

To end an ARQ session use [Alt-F7] and select a "disconnect" mode.

5.   FEC: Put the TNC in AMTOR mode, it should show Receive:Idle.

Type in text, when you're ready to transmit FEC, just hit [ESC].

6.   BAUDOT: Type BAUDOT [F10]. Set the shift and baud rate. The

TXREV function is not assigned a function key (RXREV is however)

but you can check TXREV using  TXREV [F10] to see it's setting.

Type in your text. When you are ready to transmit, hit [ESC].

To unkey when the buffer empties, hit [ESC] again after the

text begins to transmit. A <<K>> appears while sending, and

the <<U>> indicates that the TNC will Unkey when the buffer

is empty.  You can "Zap" the transmit buffer as one of the

options in the [Alt-F7] menu.

LESSON #11: Why use Packet-GOLD?

1.  The AEA PK232 and PK88 TNCs have a very distinctive advantage

of the other units available today. The quality and completeness of

their computer interface, or "Host Mode" is unique to the

AEA products. For example, the host mode provides information about

the number of retries, and number of unacknowledged frames, which

in our parlance are known as "pending" frames No other host

mode that we know of provides this information. The AEA Host Mode

for their multi-mode TNC also supports non-packet modes, again unique

to AEA.

2.  While we consider the hardware to be excellent, and the

Host Mode computer interface to have no equal, we believe that our

PktGOLD software is the best available. We invite you to compare

PktGOLD to any other program, including the new program from

AEA (a modified version of Pakterm).

3.  PktGOLD was the first to offer binary file transfers,

first to support the new Maildrop features, and again remains the

best software for the users of AEA TNCs. Before you invest any time

or money in other terminal software, you must consider the advanced

features of PktGOLD. Ask any of our users for a comparison.

LESSON #12: Ordering PktGOLD

1.   Get your VISA or MASTERCARD in front of you.

2.   Call (714) 496-6639 and ask for a copy of Packet-GOLD.

or

3.   Send us a check, money order, or your VISA or MasterCard

number to:

InterFlex Systems Design Corp.

Post Office Box 6418

Laguna Niguel, CA  92607-6418

Be sure to include your name, address, and callsign.

VISA or MasterCard orders, please include a telephone

number to call if any problems arise.

Note: Price: $59.95 + $5 shipping. CA residents add 6.25%.

Subject to change.  The Enhanced Edition is available for

$79.95 plus $5 for shipping.

Good luck. We appreciate your business, and look forward to providing

you the best software available.