Box Plot For Windows 3.0

By Rick Carlson

Sparta Sound Co.

!!!!!!!!!!!!!!!  UNREGISTERED SHAREWARE VERSION 1.3 !!!!!!!!!!!!!!!!!

PLEASE REGISTER

****************************************************************************

Boxplot is distributed as Shareware. It is NOT a free Program.

If you use Boxplot then you should register it.

****************************************************************************

Registered Boxplot users will recieve complete documentation, as well as

many more speaker files from various popular manufacturers. Included in the

documentation is a mini tutorial on Loudspeaker enclosure design,

construction, and tuning. If you are new to speaker building then I

strongly recommend the tutorial. Nothing is worse than spending many hours

building a speaker enclosure only to be disapointed because of lack of

proper tuning.

Introduction

Box Plot is a Microsoft Windows based program that calculates the

frequency response and maximum Sound pressure level for a sealed

or vented loudspeaker enclosure.

Boxplot is based on the Sealed or Vented box system.  While there

are many other types of enclosures for loudspeaker systems

(Horns, Infinite baffle, Transmission lines), the Vented enclosure is

currently the most popular type among speaker manufacturers and audiophiles.

It is also one of the easier types to build correctly.

This Version of Box Plot requires Microsoft Windows 3.0 or later

to run.  A math coprocessor is not required but greatly speeds

up the frequency response calculations.

This program can be used by audio hobbyist or professional to

design and modify high quality loudspeaker enclosures.  You

should be familiar with the Thiele/Small loudspeaker parameters

in order to best utilize this program, but all that is required

is a basic understanding of vented loudspeakers and a good data

sheet for your drivers. I strongly recommend the tutorial for

beginners.

Operation:

This Program was designed to allow 'almost' real time update of

the response plot as box parameters are changed.This is usefull

when trying to shoehorn a speaker into a non optimum box.  Many

different combinations of box parameters can be tried in a short

period of time.  If Auto Plot is turned on, then clicking on the

OK button of the BOX Parameters dialog box will replot the

response curves.  Autoplot can be turned off, for slower machines.

Speaker Parameters:

Speaker parameters are entered by bringing up the a dialog box

under Speaker, on the main menu.  When you ar satisfied with your

changes, click on the OK button to save the parameters.  The

parameter names are the Thiele/Small parameters specified on most

good spec sheets.  Parameters can be entered in either metric or

english units.  Selecting the meteric or english checkbox will

change the entire programs units not just the speaker parameters.

Fs      Free air resonance of the speaker.

Vas     Volume of air equivalent to the speakers compliance.

Qms     Mechanical Q of the speaker.

Qes     Electrical Q of the speaker.

Xmax    Maximum linear cone displacement.

Pd      Maximum power dissipation (in watts).

Re      Dc resistance of the voice coil.

Diam    Effective piston diameter of speaker.

Box Parameters:

Box Parameters are entered in a dialog box.   When  the  OK

button  is clicked the Box Parameters dialog box stays on the

screen.  To close the dialog box click on the Close button or

use the system drop down menu (upper left of dialog  box)  to

close  the box.  The reason the dialog box stays on screen is

to make it easier to edit parameters and see  the  effect  on

frequency response.

There are 6 types of boxes that can be modeled with Box Plot.

Closed Box:

Simple sealed Enclosure.

Closed Box with first order equalizer:

Sealed box with first order hi pass filter. The first order

filter usually provides subsonic attenuation.

Closed Box with second order filter:

Sealed box with a second order hi pass filter. The filter

is usually an active filter placed before the power amp.

Adjusting the Q of this filter modifies the low end response

of the system. This is usually used to extend the lower

3dB point.

Vented Box:

Box with a tuned port.

Vented Box with first order eq:

First order Eq is more usefull here than is a sealed box.

Because it will limit cone excursion at subsonic frequencies.

Vented Box with second order eq:

Same as sealed box.

Box Parameters:

Alpha:  System Compliance ratio.

Alpha is the ratio of box volume (Vb) to speaker Vas.

Alpha = Vas / Vb

Alpha tells you how big to make the box.

H:      System tuning ratio.

H is the ratio of the speaker free air resonance to

the system tuning frequency (Fsb).

H = Fs/Fsb

This determines how to design the vent.

Ql: Q from leakage losses.

This is the enclosure Q at Fb resuting from leakage loss.

(better boxes have higher Q's. This parameter must be measured

after the enclosure is built. Start with 7 as a good estimate.

Fe/Fs:

This is the ratio of the Equalizer resonant frequency to

the box frequency.

D

This is the equalizer Damping coiefficent. D = 1/Q

Comments:

The comments field is a text string,  255 chars  long  that

can be used to annotate the graph.

Vent Design:

This  box  will  show  the vent length based on the present

box/ speaker combination. The Air Speed mach number is the

vent air speed relative to the speed of sound. The air speed

should be kept below .2-.3 in order for wind noise from the

vent to be inaudiable.

Auto Plot:

When the Auto Plot menu item is checked,  anytime a speaker

or  box parameter is changed,  the graph will be regenerated.

This keeps the graph up to date with the  latest  parameters.

This  also  may  be  annoying on slower machines so it can be

turned off.

File:

New:

This will reset all parameters to some predefined settings.

The settings are based on nothing at all, just some numbers

that wont do real wierd things. (The speaker parameters are

from a well known manufacturer.)

Save:

Save as:

This will save the speaker, box, and comments in the specified

file.

Open:

Read a file on the disk. The default file extension is .SPK

but has no real significance.

Print:

This will print the response curves as shown on the screen

along with the speaker and box parameters.

Printer Setup:

This will bring up the active printer setup box. Any changes

made in the setup box will be saved as the current printer

setup.

To register Please send $15.00 to

Rick Carlson

62 Fox Trail Rd

Sparta, NJ. 07871

Compuserve 71041,3015

I will send out a disk with the latest version of BOXPLOT,

the complete documentation, and the Speaker tutorial.

Registered users will recieve notification of program updates.

If you have suggestions, comments, bug reports or just feel

like writing please do so.

Let me know what other kinds of programs would be usefull to

the audiophile.