Name:

ROMREL(.COM)

Function:

ROMREL will release the hooks into the system BIOS made by the

Netware Remote Boot ROM to facilitate the use of alternate

device drivers with the network interface card (NIC).

Usage:

ROMREL <int_number>

Version:

2.0 - Release 08/10/89

Description:

In order to support remote booting using Netware on a PC, the

NIC is equipped with a special ROM, usually provided by the NIC

manufacturer.  The ROM will take control of the NIC at cold

boot time, and map a disk image resident on the Netware server

as drive A: on the PC.  In order to do this, the ROM must hook

into various BIOS disk I/O vectors, as well as the card itself.

NET3.COM contains code to, in part, undo these hooks, thereby

returning control of the drive to the local BIOS and the NIC to

the IPX/SPX driver.

The purpose of ROMREL is to emulate this "unhooking" process

normally performed by NET3 without actually loading another

protocol in place of the ROM, as does IPX.COM.  The goal of ROMREL

is to leave the system vectors set as if the system were loaded

from a local hard disk or floppy without the use of the ROM.

ROMREL is not required in the "normal" scheme of things where

IPX and NET3 are used in the AUTOEXEC.BAT file after remote boot

to establish a working connection to a Netware server.  ROMREL

is required when some device driver/protocol other than IPX/SPX

is to be loaded, or when IPX does not actually contain the NIC

device driver.

For example, with ROMREL the user could utilize a Netware server

to boot DOS, but then attach to an NFS server to map a local

drive (say, F:) to a remote filesystem.  Alternately, ROMREL

could be used to release the ROM so a generic network driver

can be loaded, such as a Packet Driver conforming to the FTP, Inc.

specification.  This would allow Netware IPX/SPX to share

the NIC with other protocols, such as TCP/IP.  Some implementations

of the Packet Driver are incorporated into IPX.COM so as to

eliminate the need for ROMREL.  However, most implementations

(especially those in the Public Domain) are designed into a separate

TSR to be loaded before IPX.COM.  Without ROMREL, this TSR would

conflict with the Boot ROM, as both driver and ROM would be trying

to use the NIC at the same time.

One thing to realize at this point is that once ROMREL is executed,

drive A: disappears.  Therefore, ROMREL would have to be used in

conjunction with a RAM Disk so execution could continue.  To

accomplish this, CONFIG.SYS would be configured to create a RAM

Disk upon boot (using VDISK, for example).  The AUTOEXEC.BAT file

would then copy from the A: image to the RAM Drive those files

necessary for continuing execution after ROMREL.  In the previous

example, that would include the packet driver TSR, IPX, and NET3.

The system can then be made to log into the server and continue

with any other initialization as necessary.

ROMREL will release the hooks into the system DOS/BIOS and

optionally mask off the interrupt set on board the NIC via the

8259 bus interrupt controller.  Masking is usually done to prevent

any spurious interrupts which may be generated by the NIC from

jumping to the ROM interrupt handler before the new device driver

can be loaded.  (Also, some device drivers may not properly

install their interrupt handling code until after some internal

initialization takes place.  During this time the NIC could

interrupt the system and jump to the wrong device handling code.)

If you wish to have ROMREL release the interrupt, invoke it as:

romrel <int_number>

where int_number is 2-7 on a PC/XT, or 2-15 on an AT.  For example,

if the network card is set to use IRQ 2, use:

romrel 2

If you do not want ROMREL to release the interrupt, specify an

int_number of 0, as:

romrel 0

It is highly recommended that you specify the actual IRQ number.

Also, be careful to specify the correct interrupt number for the

NIC.  ROMREL does not have any way of determining that the

interrupt you specify actually belongs to the NIC.  Using the

wrong number could, say, disable a COM port or even the floppy

disk driver!  Be precise.

Furthermore, ROMREL is compatible with various network-aware

mark and release programs if invoked before a mark is issued.

Also, if release will be used, you should have ROMREL mask off

the card interrupt.  If you don't, the interrupt may be pointing

back to the Boot ROM after a release is executed, and the system

will most likely crash.

Author:

Glen M. Marianko

Disclaimer:

The user of this software assumes all responsibility for its

use.  The user has been made aware of the potentially volatile

results of incorrect use of this software, which may lead to

the loss of data.  The user is advised to test this software

thoroughly before relying upon it or using it in connection with

any valuable data, and that the user frequently perform data

backups so as to prevent any significant loss of data.

This software is provided AS-IS without any warranty, either

expressed or implied, as to its performance, merchantability,

or fitness for purpose.  The entire risk as to the results

and performance of the software is assumed by the user.

Should the program prove to be defective, the user assumes the

cost of all necessary servicing, correction, and repair.

Copyrights:

Portions of ROMREL are (C) Copyright 1983, 1988 Novell, Inc.

Bugs:

The author is not aware of any bugs as of this writing.  Please

report any bugs directly to the author.

#################################################################################