TURBO BRAILLE 3.6

User's Guide Supplement

Another Sound Alternative

from

KanSys, Inc.

Copyright (c) 1987 - 1997

by

KanSys, Inc.

43 N. Sixth St., Apt. #1

Hudson, NY 12534-2527

Telephone: (518) 828-0687

Turbo Braille 3.6

User's Guide Supplement

The information in this document

supplements the standard Turbo Braille

Version 3 Manual. It describes the features

added to support interpoint printers, block

letters, graphics embossing, and discusses

several new or modified dot commands. For

more complete information on the use of

Turbo Braille, please refer to the Version

3 manual.

Interpoint Braille.

Interpoint braille printers emboss

braille on both sides of the paper. They do

so  by delaying the printing of an odd

numbered page (1, 3, 5, etc.) until the

following even numbered page (2, 4, 6,

etc.) is received from the host computer.

Thus two pages at a time are embossed, one

on the front side of the paper and the

other on the reverse side.

If Turbo Braille knows your printer is

"interpoint" instead of "single sided," it

will always send an even number of pages to

the printer, so that the last page of a

document with an odd number of pages is

correctly embossed. Without this feature,

it is often possible to find the first page

of the next document embossed on the back

of the last page of the first document,

obviously an unwelcome discovery. Printing

multiple copies of an interpoint document

with an odd number of pages is only

possible if your software is aware of the

need for a trailing blank page for the back

side of the last sheet.

Braille Graphics.

Most modern braille embossers permit

the creation of graphics images in a

special mode that provides uniform dot

spacing across each line and down the page.

See Appendix A. for a detailed discussion

of the geometry of braille graphics. Turbo

Braille Version 3.6 supports three types of

graphics, in both single sided and

interpoint modes.

(1) Block Letters. You may include a

line of block letters in a braille document

by using the ".LR" command, described

below, in a BIF file. The number of

characters per line, and the number of

lines of block letters per page, depend on

the dot to dot spacing or "resolution"

which your printer supports. Characters

with ASCII values from 32 to 126 may be

displayed. These include the upper and

lower case English alphabet, the ten

digits, and the normal collection of

punctuation marks and special characters.

(2) Dot Files. You may import a Dot

File into a Turbo Braille document by using

the ".DF" command, described below, in a

BIF file. A Dot File is an ASCII file

consisting of lines of spaces and periods,

each of which produces a corresponding line

of blanks and dots in the graphics output.

Dot Files may be produced from a document

Scanner by the Personal Data Systems PICTAC

program; or from TIFF files, WPG files, or

a numerical specification by the KanSys

Inc. LowRez software; or they may be

created manually with an ASCII editor.

(3) Graphics Files. You may import a

Graphics File into a Turbo Braille document

by using the ".GF" command, described

below, in a BIF file. Graphics Files

contain ASCII characters arranged in lines,

each of which produces three lines of dots

in the graphics output. Graphics Files may

also be produced by the PICTAC program, the

LowRez software, or they may also be

created manually, a much more difficult

task than with dot files.

Escape Sequences

Most modern braille embossers require

that an escape sequence be sent to them to

switch to graphics mode and back to text

mode. Turbo Braille permits you to define

such escape sequences with the "/XG" and

"/XT" interactive mode commands. The

default values for these escape sequences

are those required by the Romeo, Juliet,

BookMaker, and VersaPoint printers.

The reverse side of a page containing

braille graphics cannot contain other

embossed material. Printers that support

both interpoint and graphics embossing will

automatically abandon interpoint when a

graphics escape sequence is received. Turbo

Braille uses two other escape sequences to

switch to single sided mode at the top of a

page containing block letters or graphics,

and back to double sided mode at the top of

a page containing only text. This prevents

split pages and loss of control of page

numbers. You may define these escape

sequences with the new "/XD" and "/XS"

interactive mode commands. The default

values for these escape sequences are those

used by the Juliet and BookMaker printers.

(The Romeo and VersaPoint do not support

interpoint embossing).

New Commands, Switches, and Features

(1) The "/P" command (interactive mode

only) has been enhanced to permit several

new printer characteristics to be defined.

In addition, some default values have been

changed for greater convenience and

generality.

In addition to specifying the maximum

number of cells per line and lines per

page, this command asks whether to use

blank lines or formfeeds to finish short

pages, whether the printer is to run

continuously or pause after every page,

whether it is single sided or interpoint,

and what the spacing is from dot to dot in

graphics mode.

Printers 0 through 9 are all initially

defined as having a physical maximum of 80

cells per line, 27 lines per page, use

formfeeds, run continuously, single sided,

with 80 mils (0.080 inch) dot to dot

spacing in graphics mode. A default logical

page size of 40 cells per line by 25 lines

per page is achieved with the ".RM" and the

".PL" dot commands. Printers 1 to 9 may be

user modified; printer 0 may not.

(2) Default values for the ".RM" and

".PL" dot commands establish 40 cells by 25

lines as the default logical page size.

Note that these values are less than the

maximum values contained in the printer

definitions. To change the right margin or

page length you would normally use these

dot commands rather than change the printer

definition values, which should match or

exceed the capacity of your actual braille

printer. The maximum choice of 80 cells per

line was made to support refreshable

braille displays such as the Alva Braille

Device. The maximum choice of 27 lines per

page was made to support 11" paper height.

Do not change this value unless your paper

is not eleven inches high.

A "short page" is one for which the

".PL" value is less than the maximum lines

per page of which your printer is capable.

Such short pages are "finished" by adding

blank lines, or by issuing a formfeed

character, as specified in the printer

definition.

(3) Separate running headers and

running footers are available with the

".RH" and the ".RF" dot commands. The

default value for the ".RH" (Running

Header) dot command is the name of the file

being processed. The default value of the

".RF" (Running Footer) dot command is the

date of the translation. These headers and

footers will only be printed and positioned

if requested by the .TL (Top Line) and .BL

(Bottom Line) commands. The default values

of the ".TL" and the ".BL" dot commands

place the running header left justified on

the top line of every page, and the running

footer centered on the bottom line of every

page. To change this default page format

you must change the .TL and .BL commands,

and not the .RH and .RF commands.

In addition, when BIF files are

created from WordStar, WordPerfect, or text

files, ".RH" and ".RF" dot commands are

placed at the start of the BIF file,

containing the name of the original file

from which the BIF file was generated and

the date of the translation. No ".BL" or

".TL" dot command is inserted.

(4) The ".LR" dot command is used in a

BIF file to create block letter images of

the text on the next line or lines. Turbo

Braille uses the current printer definition

to determine proper spacing and whether

escape sequences are necessary to switch to

single sided mode as well as graphics mode.

Usage:

.LR

.LR n

.LR n,0

where "n" is the number of subsequent lines

to convert to block letters, and the "0"

suppresses automatic centering of those

lines. The default value for "n" is 1. No

word wrap or line filling occurs with this

command. The next "n" lines are not

examined to determine if they contain other

dot commands.

(5) The former Version 3.0 ".RF" dot

command is renamed ".BF" (from "read file"

to "BIF file") and two new file importing

dot commands are now available: ".DF" and

".GF" for Dot Files and Graphics Files

respectively. Images retrieved by these

commands are, by default,  stripped of

blank space at the top and bottom of the

image, and appear centered from left to

right in the braille document. This default

action can be suppressed by following the

file name with a "0" as with the ".LR" dot

command described above. Turbo Braille uses

the current printer definition to determine

proper spacing and whether escape sequences

are necessary to switch to single sided

mode as well as graphics mode. Usage:

.DF filename

.GF filename

.DF filename,0

.GF filename,0

where "filename" is the name of the file

containing the image, either in dot file or

graphics file format. You may include a

reference to a DOS device or directory with

the filename. If you do so, remember to

enter a pair of backslashes (\\) with

directory names, as required by the BIF

files in which these dot commands are

placed. The trailing "0" suppresses

stripping and centering of the image.

(6) Four new interactive mode commands

are available for handling escape

sequences. They are "/XD" (double sided),

"/XG" (graphics), "/XS" (single sided), and

"/XT" (text). The corresponding dot

commands, ".XD", ".XG", ".XS", and ".XT",

may be used only on the Turbo Braille

command line or at the interactive mode

prompt, and may not be used within a BIF

file. These dot commands also require that

you first direct Turbo Braille to write its

output directly to the printer with the

"/W1" switch. They will then send escape

sequence data directly to your printer. The

primary use of these commands is to make

the corresponding data available to Turbo

Braille for its automatic use when

inserting block letters or graphics images

into a document.

(7) The ".BP" (Begin Page) dot command

helps interpoint printers keep odd numbered

pages on the front side of the page. If the

current printer definition is interpoint

and the ".BP" command requests an odd

numbered page, Turbo Braille will add an

extra blank page if necessary, so that the

new odd numbered page is always printed on

the front side of the paper.

(8) The ".FI" (Fill) and ".NF" (No

Fill) commands are no longer available in

Version 3.6. The filling of lines and the

filling of pages are now independently

controlled with the ".FL" (Fill Lines) and

the ".FP" (Fill Pages) commands. To keep

lines and pages separate, the ".KL" (Keep

Lines) and the ".KP" (Keep Pages) commands

are now available. Keeping lines separate

means that whenever a new line starts in

the original document, a new line of

braille will also start. Keeping pages

separate means that whenever a new page

starts in the original document, a new page

of braille will also start.

(9) Horizontal tabbing is accomplished

with the new ".TB" and ".TD" commands. The

former drops spaces to the specified tab

position while the latter drops guide dots.

The format of these commands is:

.TB n

.TD n

where "n" is the cell position where you

want the next text to appear. If that cell

position is already used on the current

line, these commands are ignored.

Other Enhancements

Turbo Braille Version 3.6 follows the

new conventions concerning the placement

and appearance of the symbol for inches and

for the percent sign following numbers. The

British currency symbol is also recognized.

Two predefined letter substitutions for the

".SB" command exist: "\F" expands to the

current file name, and "\D" expands to the

current date, whenever they appear anywhere

in a BIF file. The external table of

translation rules has been expanded to

correct most of the mistranslations called

to our attention. A greater level of user

feedback is provided in the interactive

mode when entering commands and switches

one at a time following the TBRL prompt.

Appendix B lists the 48 dot commands

available in Turbo Braille BIF files for

format control. Those marked with an

asterisk (*) may also be used on the

command line, or in interactive mode when

processing WordPerfect, WordStar, or ASCII

source documents.

Appendix A: Braille Graphics

Most braille embossers operate in a

"character mode" and in a "graphics mode."

In character mode, standard six dot cells

are placed at 0.25 inch intervals across

each line, and lines of cells are placed at

0.4 inch intervals down each page. A

typical page of braille characters consists

of 25 lines of 40 cells each, a 10 inch

square area centered on an 11.5 by 11 inch

page.

The space between adjacent dots within

a braille cell is 3/32 inch both

horizontally and vertically, and between

two adjacent dots in different cells is

5/32 inch horizontally and 7/32 inch

vertically. (3/32 + 5/32 = 8/32, or 0.25

inch cell to cell horizontally, and 3/32 +

3/32 + 7/32 = 13/32, or about 0.4 inch cell

to cell vertically). These narrow intra-

cell spaces and wide inter-cell spaces mean

that if a page is completely filled with

braille cells in which all six dots are

present, one can easily identify individual

cells because of the extra space between

cells. It also means that in character

mode, smooth unbroken lines cannot be

formed simply by composing them from parts

of braille cells.

In graphics mode on the other hand,

the extra space between cells horizontally

and vertically is eliminated. A page filled

with six dot cells appears as a uniform

field of dots, and individual cells can no

longer be identified within that field. Dot

spacing is the same between adjacent dots

in both directions, regardless of whether

they are in the same cell or in adjacent

cells. Smooth unbroken lines can easily be

formed, and other interesting graphic

effects can also be achieved.

Some braille embossers retain the

standard 3/32 inch dot spacing in graphics

mode, while others reduce this figure to an

even smaller distance of 2/25 inch to

achieve a higher resolution. Graphics mode

on the BookMaker and the Juliet from

Enabling Technologies both use 2/25 or

0.080 inch spacing; Blazie Engineering

reports that the Braille Blazer uses 0.090

inch spacing; the Cranmer Modified Perkins

Brailler, The Romeo from Enabling

Technologies, the VersaPoint from

Telesensory Systems, and the Index family

of embossers, all use the standard

character mode distance of 3/32 inch or

0.094 inch in their graphics modes. When

specifying these distances in Turbo

Braille, you use mils, or thousandths of an

inch, as the standard unit. The previous

resolutions are therefore 80, 90, and 94

mils.

Some braille embossers require a

mechanical adjustment to be placed in their

graphics modes, while others require only

that a software escape sequence accompany

the data to be embossed. A few embossers

require both a mechanical adjustment and a

software escape sequence. Among those

embossers mentioned earlier, the Braille

Blazer is mechanical only, the Romeo is

both mechanical and software, and the

others are software only. Mechanical-only

embossers generally cannot intermix normal

character mode braille and graphics mode

braille on the same page. Software-only or

mechanical-and-software embossers generally

do support mixed formats on the same page.

Most embossers that use software

escape sequences have adopted a standard

method of entering and exiting graphics

mode. The two characters "Escape 1" enter

graphics mode, and the two characters

"Escape ]" (escape right-bracket) exit

graphics and return to text mode. The Index

braille printers use "Escape ^A" and

"Escape ^B" instead, where the caret (^)

indicates that the "A" or "B" is "Control

A" or "Control B". Check your printer

manual to determine what software escape

sequence, if any, is required for your

printer. In Turbo Braille, you may use the

"/XG" and the "/XT" interactive commands to

change these default values if necessary.

Appendix B: BIF File Dot Command List

Those marked with * may also appear on

the command line or at the interactive mode

prompt when processing WordPerfect,

WordStar, or ASCII source documents.

".BF" BIF File import.

* ".BL" Bottom Line format.

* ".BP" Begin Page.

".BR" Break.

".CE" Center.

".DF" Dot File import.

* ".DV" Divider lines.

* ".FL" Fill Lines.

* ".FP" Fill Pages.

".GF" Graphics File import.

* ".GR" Grade of braille translation.

".HL" Horizontal Line.

* ".IN" Indent.

".IT" Italics.

* ".KL" Keep Lines separate.

* ".KP" Keep Pages separate.

* ".LC" Lower Case.

".LR" Letters.

* ".MC" Mixed Case.

".NE" Need lines.

* ".PL" Page Length.

* ".PN" Page Number in original document.

* ".PO" Page Offset.

".PP" Paragraph.

* ".RF" Running Footer.

* ".RH" Running Header.

* ".RM" Right Margin.

".SB" Substitute a string for a letter.

".SK" Skip blank line.

".SP" Skip blank line.

".TB" Tab, dropping blank spaces.

".TD" Tab, dropping guide dots.

".TI" Temporary Indent.

* ".TL" Top Line format.

* ".X0" Escape sequence 0.

* ".X1" Escape sequence 1.

* ".X2" Escape sequence 2.

* ".X3" Escape sequence 3.

* ".X4" Escape sequence 4.

* ".X5" Escape sequence 5.

* ".X6" Escape sequence 6.

* ".X7" Escape sequence 7.

* ".X8" Escape sequence 8.

* ".X9" Escape sequence 9.

* ".XD" Escape sequence for Double sided

mode.

* ".XG" Escape sequence for Graphics mode.

* ".XS" Escape sequence for Single sided

mode.

* ".XT" Escape sequence for Text mode.