USS39 BAR CODE FONTS FOR AUTOCAD

Jon Fleming  CIS 70334,2443

The enclosed font files are for use with any version of AutoCAD,

creating USS39 (nee Code 39) bar codes.  Each file contains the

complete USS39 character set:

Digits 0 through 9

Uppercase letters A through Z

Seven special characters:  - (minus) . (period) space $ / + %

One start/stop character:  * (asterisk)

(each bar code should begin and end with a start/stop character.

If you print the human-readable equivalents of each character

near the bar code, do not print the asterisks that represent the

start/stop characters.)

There is also a non-printing line feed character so AutoCAD can do

multiple lines of text.

See the end of this file for how to use a larger character set

and how to use check characters.

All the files are optimized for a 300 dpi (or higher) imaging

device (e.g. a typical laser printer) with a "pen width" of 0.00333"

[0.085 mm].  Each file is optimized for a different size on the paper:

USS39W10  1.0" [25.4] mm high

(each character is 0.66" [16.8 mm] wide)

USS39N10  1.0" [25.4 mm] high

(each character is 0.33" [8.4 mm] wide)

USS39T10  1.0" [25.4 mm] high

(each character is 0.165" [4.2 mm] wide)

USS39N05  0.5" [12.7 mm] high

(each character is 0.33" [8.4 mm] wide)

USS39T05  0.5" [12.7 mm] high

(each character is 0.165" [4.2 mm] wide)

The USS39T05 and USS39T10 fonts are "iffy" on a 300 dpi printer;

the results may not be just right unless the characters fall

exactly on the grid of the device's dot centers.  They should work

well on a 600, 1200, or 2400 dpi imager with a 0.00333" [0.085 mm]

"pen width".

The USS39W10 and USS39N05 characters may be reduced from their

optimum size to as little as 1/4" [6.35 mm] high and still meet

specifications, provided the imaging or reduction system has the

resolution to maintain the ratio of bar width to space width.  The

best way to do this is to print the characters at the optimum size

and photographically reduce them.

Note that 1/4" is the minimum height for bar code characters

unless you have control over the reader that will be used and can

ensure that it will be set up to allow smaller characters.

Similarly, the USS39N10 characters may be reduced from their

optimum size to as little as 0.5" [12.7 mm] high and still meet

specifications.

The USS39T10 and USS39T05 characters may not be reduced any

significant amount from their optimum size and still meet

specifications.

Any of the fonts may be printed at a larger than optimum size, but

the "pen width" will have to be adjusted to maintain solid bars.

Results will be best if the pen width is a multiple of the imager's

resolution. For example, for a 1.5" high bar code you might use

USS39T05 with a 0.10" [0.25 mm] pen width.  This is three times the

optimum height for USS39T05 and three times the pen width required at

the optimum height, and the pen width is three times the dot size of

a 300 dpi imager.

All the SHP files contain extensive comments.  For AutoCAD 10 and

earlier, you will have to remove these comments with a text editor if

you wish to re-compile the fonts.

USS39.DWG (in AutoCAD 2.5 format) contains a sample of each font

at optimum size and the complete character set.

These files may be used and distributed with no restrictions.

Although it is not required, bar code readers may be programmed

to recognize the entire ASCII 127-character set by representing each

ASCII character as one or two bar code characters according to the

following table:

ASCII  USS-39  ASCII   USS-39  ASCII   USS-39  ASCII   USS-39

NUL    %U      space   space   @       %V      `       %W

SOH    $A      !       /A      A       A       a       +a

STX    $B      "       /B      B       B       b       +B

ETX    $C      #       /C      C       C       c       +c

EOT    $D      $       /D      D       D       d       +D

ENQ    $E      %       /E      E       E       e       +E

ACK    $F      &       /F      F       F       f       +F

BEL    $G      '       /G      G       G       g       +G

BS     $H      (       /H      H       H       h       +H

HT     $I      )       /I      I       I       i       +I

LF     $J      *       /J      J       J       j       +J

VT     $K      +       /K      K       K       k       +K

FF     $L      ,       /L      L       L       l       +L

CR     $M      -       -       M       M       m       +M

SO     $N      .       .       N       N       n       +N

SI     $)      /       /O      O       O       o       +O

DLE    $P      0       0       P       P       p       +P

DC1    $Q      1       1       Q       Q       q       +Q

DC2    $R      2       2       R       R       r       +R

DC3    $S      3       3       S       S       s       +S

DC4    $T      4       4       T       T       t       +T

NAK    $U      5       5       U       U       u       +U

SYN    $V      6       6       V       V       v       +V

ETB    $W      7       7       W       W       w       +W

CAN    $X      8       8       X       X       x       +X

EM     $Y      9       9       Y       Y       y       +Y

SUB    $Z      :       /Z      Z       Z       z       +Z

ESC    %A      ;       %F      [       %K      {       %P

FS     %B      <       %G      \       %L      |       %Q

GS     %C      =       %H      ]       %M      }       %R

RS     %D      >       %I      ^       %N      ~       %S

US     %E      ?       %J      _       %O      DEL     %T,%X,%Y,%Z

Also not required, but allowed, is a check character for enhanced

data security.  The USS39 specification recommends:

1.  Assign each character a numeric value:

CHAR   VALUE   CHAR    VALUE   CHAR    VALUE

0      0       F       15      U       30

1      1       G       16      V       31

2      2       H       17      W       32

3      3       I       18      X       33

4      4       J       19      Y       34

5      5       K       20      Z       35

6      6       L       21      -       36

7      7       M       22      .       37

8      8       N       23      space   38

9      9       O       24      $       39

A      10      P       25      /       40

B      11      Q       26      +       41

C      12      R       27      %       42

D      13      S       28

E      14      T       29

2.  Add the values of all the characters in the message.  Take the

result modulo 43 (the remainder when the total is divied by 43).

Append the character corresponding to the final result to the

message, before the stop character, as the check character.