.

[ ii                                                       QBALL ]

TABLE OF CONTENTS

Dedication, Disclaimer, License, Acknowledgments, Copyright Info i

Table of Contents.............................................. ii

List of Appendixes............................................ iii

List of Figures............................................... iii

List of Tables................................................. iv

Introduction.................................................... v

About Shareware................................................ vi

_____________________

Chapter                                          page

1. Getting Started ............................. 1

1.1 Files Required ........................... 1

1.2 Will It Work ?............................ 1

1.3 Copying the Program....................... 1

1.4 QBALL Installation........................ 1

1.5 General Instructions...................... 1

1.6 Starting QBALL............................ 3

2. Computing Ballistic or Gun Data ............. 7

2.1 Preliminary Data Entry.................... 7

2.2 Compute, Display and Save Range Tables.... 8

2.3 Displaying Graphics...................... 11

2.4 Changing Input Data...................... 17

2.5 Other Ballistics Procedures.............. 19

Point-Blank Range.......................... 19

Wind Vector Corrections.................... 20

Slant Angle of Fire........................ 21

Sectional Density.......................... 22

Free Recoil Velocity and Energy............ 22

2.6 Bullet Lookup Facility................... 23

3. Inferred Ballistics Data.................... 25

3.1 Ballistic Coefficients................... 25

Ballistic Coefficient: r1 = 0.............. 25

Ballistic Coefficient: r1 = 50............. 26

3.2 Inferring a Muzzle Velocity.............. 28

Chronograph Not Available (Bullet Drop).... 29

Chronograph Available (Downrange Velocity.. 32

4. Printed Output ............................. 33

4.1 Printer Warnings......................... 33

4.2 Screen Dumps............................. 33

4.3 Printer Setup............................ 33

4.4 Print Functions.......................... 34

4.5 Typical Printed Output................... 34

5. Configuring Display/Default Starting Values. 37

5.1 The Utilities and Configuration Menu..... 37

5.2 Toggle Active Switches................... 37

5.3 Configuring Display Colors............... 38

5.4 Using the Values You've Set.............. 39

5.5 Invoking a DOS Shell..................... 40

5.6 Reading Range Tables/Translating Graphics 40

6. Recalling Saved Trajectory Data............. 41

7. File Handling............................... 43

[ iv                                                       QBALL ]

LIST OF FIGURES (continued)

Figure                                         page

24.  Ballistic Coefficient Entry 2............ 27

25.  Ballistic Coefficient Display 2.......... 27

26.  Infer Muzzle Velocity Menu............... 28

27.  Vertical Drop Generation................. 29

28.  Inferred Muzzle Velocity

Data Entry and Result......... 29

29.  Fraction Table........................... 30

30.  Measurements to Find a Group Center...... 31

31.  Muzzle Velocity from

One Downrange Velocity........ 32

32.  "Print?" prompt.......................... 34

33.  Typical Range Table Printed Output....... 34

34.  Sample Output, Ballistic Coefficient..... 35

35.  The Utilities Menu....................... 37

36.  The Switch Toggle Menu................... 37

37.  Set Display Colors Screen................ 38

38.  Recall Trajectory Data Display and Menu.. 41

39.  Recall Trajectory Data

Menu and Choice Box........... 41

40.  Saved Trajectory Data Graphic Display.... 42

41.  QBALL's Range Table Reader............... 47

42.  QBALL's Graphics Translator.............. 48

_____________________

LIST OF TABLES

Table                                          page

1.  Free Recoil, Selected Weapons............ 23

B1.  Data Taken From Gun Digest Table......... 57

B2.  QBALL-Calculated Data.................... 57

B3.  Data Taken From Speer Manual............. 58

B4.  QBALL Data, Inferred Muzzle Velocity..... 58

B5.  Difference Table......................... 58

C1.  Saved Trajectory Data File............... 59

D1.  Remaining Velocity With Given Inputs..... 63

D2.  Trajectory Comparisons Under

Specified Conditions..... 64

D3.  Inferred Muzzle Velocity with

Various Bullet Drops..... 65

D4.  Data Comparison, Inferred vs

Nominal Muzzle Velocity.. 65

F1.  Sierra Ballistics........................ 67

F2.  BALCALC.................................. 68

F3.  BALTEC................................... 69

F4.  Barnes Ballistics........................ 70

[ vi                                                       QBALL ]

About Shareware

So you've spent a lot of money--$200, $300, $400, or more--for a

computer program. You've tried it out, and have found that it's

unsuitable for you, (or worse, doesn't work) and you want your money

back. Know what? Usually you're out of luck, that's what.

Enter Shareware.

Shareware allows you to try software before buying it, much as

you might test drive a new car. If you try a Shareware program and

continue using it, you are expected to register, just as you would

have to pay for the car. Individual programs differ on details: you

may get anything from the simple right to continue using the soft-

ware to an updated program with printed manual. Shareware programs

are neither "free" nor are they in the public domain. Copyright law

applies to Shareware software alike. The Shareware copyright holder

--usually the author--retains all rights to the program.

Shareware authors are accomplished programmers, just like commer-

cial authors, and the programs are of comparable quality. (In both

cases, there are good programs and  bad ones!) The difference is the

method of distribution. The copyright holder may authorize you to

copy and distribute the software, either to all and sundry or to a

specific group. For example, some authors require written permission

before a commercial disk vendor may copy their Shareware.

In sum, Shareware is a distribution method, not a type of software.

You should find  software that suits your needs and pocketbook,

whether it's commercial or Shareware. The Shareware system makes

fitting your needs easier: you can try before you buy. And because

the overhead is low, prices are low. Shareware also has a most basic

guarantee: if you don't use it, you don't pay for it.

QBALL is a "Shareware program" and can be provided to you at a

nominal charge for your evaluation. (The author's direct-order fee

of $7.50 for the Shareware Edition covers the cost of materials

and postage.) If you decide to order QBALL for evaluation, please

share it with your friends. Please do not give it away altered or

as part of another system. If you find QBALL useful, and continue

to use it after a 60-day trial period, you must make a registration

payment of $25.00 to the author. The $25.00 registration fee will

license one copy for use on one computer at one time. You must treat

licensed Shareware just as you would a book. Licensed Shareware may

be used by any number of people and may be freely moved from one

computer to another, so long as it cannot be used at one location

while being used at another. A book, after all, cannot be read by

two people at the same time--usually, anyway.

Commercial users must register and pay for their copies of QBALL

within 30 days of first use. Contact the author to make site-license

arrangements. Anyone distributing QBALL for remuneration must first

contact the author for authorization. (Authorization is hereby given

distributors recognized as Associate Members by the Association of

Shareware Professionals (ASP) who may begin offering QBALL at once.

Please inform the author to ensure you have the latest version.)

So there you are: quality software, at a reasonable price, with

an unbeatable guarantee.

Wotta deal!

-- based on original text by Paul Mayer

.

[ 2                                                        QBALL ]

Response to "([Y] or N)". Press upper- or lower-case "Y" or "N".

"Enter" is the default for "Y". If another key is pressed, QBALL

will "squeak" and do nothing further.

[ESC]ape key. "ESC" will "back out" of data entry, and will

"back up" one menu providing a menu is displayed on the screen.

There is one exception: "Quit to DOS" requires that you press "Q"

and confirm with "Y", and respond to one further prompt.

Data entry and error trapping. Type the value or string to be

used, then press [Enter]. If an entry already appears, press

[Enter] to accept it, or overtype to change it. QBALL will not

accept the wrong data type, and out-of-range values will produce

an error message. Use the [backspace] key to edit your entries.

Text entry. When entering descriptive text, the usual editing keys

may be used, i.e., delete, backspace, home, end, and the left/right

arrow keys. You may toggle <insert> to overtype already-entered

text. [Enter] will accept whatever appears in the entry window;

[ESC] aborts text entry.

Special cases of [Enter]. When a value appears in the data entry

window, [Enter] will cause that value to be used in subsequent com-

putations. (Standard values are displayed in [brackets] on the

prompt line.) [Enter] is also the default for [Y]es at all Yes/No

prompts.

Help. Limited help is available during data entry: press [F1] to

invoke help; press any key to return to data entry. (Help messages

automatically clear after an appropriate delay.)

Swap units option. The unit of range measure may be swapped back

and forth between Yards and Meters using the "New Input Data"

menu. As distributed, QBALL defaults to Yards; this may be changed

using the configuration routines.

Noises. QBALL will "squeak" upon erroneous data value entry, and

"chirps" upon completion of a computation or upon invocation of

<S>wap units. QBALL defaults to sounds "on"; sounds may be toggled

"off" for peace and quiet. A "flash" replaces noises in quiet mode.

Graphics. Given a CGA or an EGA card/monitor, graphics are always

enabled. You may suppress display of the graphics grid, but there

is no provision for toggling graphics completely "off."

Print options. Ensure that your printer is on/on-line if you

want hard copy; QBALL will detect whether or not your printer is

on and on-line, but will not detect "paper out"! Display of the

printer prompt may be toggled "on" or "off;" QBALL defaults to

"on."

Redirection of data output. QBALL does not support redirection

of data output, e.g., to a disk file, except as otherwise pro-

vided within the graphics and computations routines.

[ 4                                                        QBALL ]

Assuming that everything is working properly, the screen has

cleared, and the following is displayed:

------------------------------------------------------------

> Ballistic Computations: Typical Firearm <

+=====================================+

|      *** M A I N   M E N U ***      |

|  ---------------------------------  |

|  <C>ompute Ballistic or Gun Data    |

|  <I>nfer C1 or MV from Firing Data  |

|  Use <U>tilities / Configure QBALL  |

|  <R>ecall Saved Graphics Data       |

|  ---------------------------------  |

|  <Q>uit -- Return to DOS            |

+-------------------------------------+

--------------------------------------------------------------

[ MV:      ][ C1:      ][ Wt:     ][ Met: 59d 0' 29.53" 78%  ]|

|

----^---    ----^------  ---^--     ----^------------------- |

Muzzle      Ballistic    Bullet     Metro conditions         |

Velocity    Coefficient  Weight     Temperature, dF          |

in                       in       Gun Altitude, Feet       |

Feet per                 Grains     Barometric Pressure, "Hg |

Second                             Relative Humidity, %     |

|

---------------

[ H: 1.00 ][Yds]

--^---   --^--

Sight    Range

Height   Units

in      as

Inches   Shown

-------------------------------------------------------------

Figure 2. Main Menu. All of QBALL's functions are

accessed from this menu. Key to the Information

(Info) Line, standard metro, 1.00" sight height,

range unit = "yards", the defaults, shown.

The Main Menu is QBALL's "central dispatcher;" all of QBALL's

functions are accessed from the Main Menu. Before examining the

options, however, look at the bottom line of the display. This is

the information, or info line. The info line is displayed at all

times while QBALL is running; information is updated every time it

is changed. There are some blanks on the line right now; we'll be

filling them in presently.

[ 6                                                        QBALL ]

This Page For Notes.

[ 8                                                        QBALL ]

2.2  Compute, Display and Save Range Tables.

============================================

Having entered the required data, the following is displayed.

-------------------------------------------------------------

> Ballistic Computations: Typical Firearm <

+===================================+

|     *** Computations Menu ***     |

|  ------------------------------   |

|  Range <T>able (0 Must Be Set)    |

|  Set/Reset Current <Z>ero Range   |

|  <O>ther Ballistics Procedures    |

|  <N>ew or Changed Input Data      |

|  ------------------------------   |

|  <ESC> to  M A I N   M E N U      |

+-----------------------------------+

--------------------------------------------------------------

[ MV: 2200 ][ C1: .300 ][ Wt: 180 ][ Met: 59d 0' 29.53" 78%  ]

|

---------------

[ H: 1.00 ][Yds]

-------------------------------------------------------------

Figure 3. Computations Menu.

AHA! All the blanks on the information line have been filled in.

As you can see, you can change the Input Data by using option <N>

on this menu; this option is discussed in section 2.4. <ESC> will

return to the Main Menu; try it and see. Press <C> + <Y>/[enter]

to get back here. <O>ther Ballistics Procedures are described in

section 2.5. Let's take a look at the other two options.

Press <T> to develop a Range Table. Since no valid zero range has

been set, QBALL will prompt you for one. Enter 250 at the prompt,

thus:

Zero Range to use................. 250

This zero range will remain valid until changed using Set/Reset

Current <Z>ero Range, or until QBALL internally changes it. This

will occur if the input data is changed, or if a point blank zero

range is developed as explained in section 2.5.

A zero range must be set in order to compute and display a range

table. If no valid zero range is set, QBALL will ask you for one.

You may manually set or reset the zero range by pressing <Z>.

[ 10                                                       QBALL ]

Should you develop a range table using a larger range, or a

smaller range increment, QBALL displays up to eleven lines on the

screen, then prompts you thus:

---> More? ([Y] or N) <C>ontinuous <---

Press <M>, <Y>, or [Enter] to display the next part of the range

table. Press <C> to display all the data lines without stopping.

You may also press <N> or <ESC> to go directly to the next menu. If

you step through the range table or use continuous display, at the

end you will be prompted thus:

===> Press any key <===

The hardcopy print routines recompute all tabular data, so there

is no need to be concerned that information has scrolled off the

screen. Put a marker in this page, and return to it to experiment

later.

Press any key to display this Choose Next Output Menu:

+=================================+

|   *** Choose Next Output ***    |

|  ----------------------------   |

|  <R>edisplay Range Table        |

|  <P>rint Range Table            |

|  <S>ave Range Table to File     |

|  Display Trajectory <G>raphic   |

|  ----------------------------   |

|  <ESC> to Computations Menu     |

+---------------------------------+

Figure 6. Choose Next Output Menu

If you would like to take another look at the range table, then

press <R> to redisplay the latest range table.

If your printer is set up, and <P>rint Range Table is displayed

in regular (not low-intensity) characters, then press <P> to print

a hardcopy of the range table. QBALL displays

+======================+

| Printing Range Table |

+----------------------+

and returns you to the Choose Next

Output menu. Note that <P>rint... has been deactivated, as shown by

its display in low-intensity characters. NOTE: <P>rint... is a one-

shot function. If you want more than one copy, <S>ave the range

table and make copies of the saved file.

[ 12                                                       QBALL ]

> Ballistic Computations: Typical Firearm <

+--------------------------------------------------------------+

| | 8                                                        8 |

|I|                         o   *                              |

| | 6                *                                       6 |

| |                                        *                   |

|N| 4                                                        4 |

| |        *                                                   |

| | 2                                                        2 |

|C|                                                            |

| |-0------------------------------------------------*-------0-|

| *                                                            |

|H|-2  [draw a smooth curve]                                -2 |

| |    [connecting the "*".]                                   |

| |-4                                                       -4 |

|E|                                                            |

| |-6                                                       -6 |

| |                                                            |

|S|-8                         YARDS                         -8 *

| |       50        100        150        200       250        |

+--------------------------------------------------------------+

|     [ --- Remaining Velocity/Energy, Current Round --- ]     |

+--------------------------------------------------------------+

|           >Current<                  +=======================+

|Max Ht :  7.43" @ 135                 | *** Graphics Menu *** |

|Min Ht : -8.47" @ 300                 | --------------------- |

|C1 / Wt: [As Shown]                   |Toggle <G>rid (Now ON) |

|Muz V/E:  2200 / 1934                 |<S>ave Current Data    |

|Ter V/E:  1485 / 881                  |<R>ecall <1>           |

|ZeroRg :  250 Yards                   | --------------------- |

|Metro  : [As Shown]                   |<ESC>ape to Choose Next|

+--------------------------------------------------------------+

|        [ --- info line schematically indicated --- ]         |

+--------------------------------------------------------------+

Figure 8. The Graphics Screen

The upper portion of the screen represents your firing range as

viewed from the side. The firing point is at the left; the maximum

range, 300 yards, at the right. Each range increment is indicated

by a vertical line, numbered across the bottom. The horizontal

lines, indicating distance above or below the line of sight are

numbered at both the left and right. The curved line indicates the

bullet's trajectory relative to the line of sight: the emphasized

line numbered "0-0." The circle on the curved line indicates the

point at which the maximum height is reached. The numbers immedi-

ately below the trajectory plot are the remaining velocity and

energy at the indicated range for the current round.

The data shown at the bottom left are: the maximum and minimum

heights, and the ranges at which they occur; the muzzle and termi-

nal velocities and energies; the zero range; and refers you to the

info line for the ballistic coefficient, bullet weight, and metro.

[ 14                                                       QBALL ]

Now let's play "what if...?" What would happen to the trajec-

tory if, say, the muzzle velocity is increased from 2200 to 2700

FPS?  Let's find out. Press <ESC> to return to the Computations

Menu, and with the Computations Menu displayed, press <N> to

access the "New or Changed Input Data" Menu.

A complete explanation of the New Input Data Menu can wait for a

bit; this time, when it is displayed, press <V> to change the

muzzle velocity. Note that "2200" is displayed in the entry

window; overtype the new velocity, thus:

Muzzle Velocity         (FPS)  2700

Note that is possible to change more than one item of input data

at a time; this is not recommended, because it is difficult to

determine which change has which effect. Now <ESC> back to the

Computations Menu. Note that the zero and maximum ranges must be

reset. Press <T>, and enter zero range = 250 and maximum range = 300.

Press [Enter] to accept the range increment of 50 yards. After the

range table is displayed, press any key. When presented with the

"Choose Next" menu, press <G>. The new graphic representation of

trajectory data looks like Figure 10, page 15.

+---------------------------------------------------------------+

| HINT:                                                         |

|   If you plan to save a lot of data, especially range tables, |

| you need all the disk room you can get! Range tables get big, |

| fast!   If you're running QBALL from the distribution disk or |

| from the distribution disk, then quit QBALL, and copy all the |

| required files to a fresh (empty) disk. Then restart QBALL.   |

|                                                               |

|   Although QBALL will detect "disk full" errors, why take the |

| chance?  Should "disk full" occur, QBALL will disable all the |

| "save" functions for the duration of the current session.     |

+---------------------------------------------------------------+

+---------------------------------------------------------------+

| HINT:                                                         |

|   You may  save QBALL's trajectory screens in .GIF and other  |

| standard graphics formats using "Screen Thief",  a Shareware  |

| third-party utility which is (C) Nildram Software.  You must  |

| have a VGA monitor. Registration is $39.00.                   |

|   You may order and register "Screen Thief" from: The Public  |

| (sofware) Library (PsL). Call 1-800-2424-PsL to order with a  |

| credit card. (PsL adds a $5.00 S&H charge.)                   |

|   This information is provided with the permission of Adrian  |

| Mardlin, "Screen Thief's" author. QBALL's author has no aff-  |

| iliation with Nildram Software or PsL.                        |

+---------------------------------------------------------------+

[ 16                                                        QBALL ]

> Ballistic Computations: Typical Firearm <

+--------------------------------------------------------------+

| | 8                                                        8 |

|I|                         2   x                              |

| | 6                x                                       6 |

| |                         o   *          x                   |

|N| 4                *                                       4 |

| |        x                               *                   |

| | 2      *                                                 2 |

|C|                                                            |

| |-0------------------------------------------------|-------0-|

| *                                                            |

|H|-2  [draw smooth curves]                                 -2 |

| |    [connecting the "*"]                                    |

| |-4  [and the "x".      ]                                 -4 |

|E|                                                            *

| |-6                                                       -6 |

| |                                                            |

|S|-8                         YARDS                         -8 x

| |       50        100        150        200       250        |

+--------------------------------------------------------------+

|     [ --- Remaining Velocity/Energy, Current Round --- ]     |

+--------------------------------------------------------------+

|           >Current<    >[yours ]<    +=======================+

|Max Ht :  4.56" @ 138   7.43 @ 135 Yds| *** Graphics Menu *** |

|Min Ht : -5.31" @ 300  -8.47 @ 300 Yds| --------------------- |

|C1 / Wt: [As Shown]    .300 / 180 gr  |Toggle <G>rid (Now ON) |

|Muz V/E:  2700 / 2913  2200 / 1934    |<S>ave Current Data    |

|Ter V/E:  1877 / 1408  1485 / 881     |<R>ecall <1> thru <2>  |

|ZeroRg :  250 Yards     250 Yards     | --------------------- |

|Metro  : [As Shown]    59d0'29.53"78% |<ESC>ape to Choose Next|

+--------------------------------------------------------------+

|        [ --- info line schematically indicated --- ]         |

+--------------------------------------------------------------+

Figure 11. Two Sets of Data Graphics Screen

The trajectory plot for the recalled data is shown, as you can

see, in a contrasting color. The figure "2" is printed at the

trajectory's maximum height. The information section at the bottom

of the screen now summarizes both sets of trajectory data. Note

that the value of the parameter we changed, the muzzle velocity, is

highlighted. The actual metro used for developing the saved trajec-

tory data is displayed.

You may save up to fifteen (15) trajectory graphics files, num-

bered 1 through 15 in hexadecimal notation, or 1 through F. If

there are enough files saved, you may find yourself pressing what

you think is a letter, but is actually a number.

You may also display the same fifteen trajectories. Should the

display become cluttered with too many plots, you may erase all of

them except that of the current data by toggling the grid. You

retain the ability to recall data from disk.

[ 18                                                       QBALL ]

Muzzle Velocity: may be from 300 FPS to 4500 FPS, inclusive.

Ballistic Coefficient: must be greater than 0.0000 and less than

1.0000; QBALL will automatically add a leading decimal point to

your entry. You may also use bullet [L]ookup.

Bullet Weight: must be greater than 0 and less than 1000 grains.

If the bullet weight is less than 50 grains, 1/10 grain increments

are allowed, e.g., 17.6. You may also use bullet [L]ookup.

Metro. (Meteorological Conditions). You must either enter or

confirm temperature, gun altitude, barometric pressure, and

relative humidity. If you press [Enter] at each data prompt, QBALL

uses whatever is displayed in the entry window; "standard" metro

values are shown in [brackets] on the prompt line. You may save

"custom" values for later start-up use by QBALL; see Chapter 5.

Metro data values must be within the following ranges:

Temperature:           -50d to (plus)125d Fahrenheit;

Gun Altitude:         -1312 feet to 29,028 feet;

Barometric Pressure:    25 inches to 35 inches of mercury;

Relative Humidity:      0% to 100%.

Sight Height. This, the vertical distance between the axis of

the weapon's bore and its sights, may be fine-tuned to a specific

weapon. The supplied value, 1.00 inches, is "generic," and should

be changed. Sight Height is saved in initialization data; see

Chapter 5. QBALL will accept values from -2 inches to +5 inches,

and rounds all sight heights to two decimal places, e.g., 1.375

inches is rounded to 1.38 inches.

Yds/Met Swap. This option chooses the unit of range measure

which QBALL uses in ballistics computations, either yards or

meters. If you wish to use metric measure, please keep in mind

that the results, while accurate, may appear a bit strange. The

range increment may not be displayed quite as expected, because

yards and meters do not take kindly to being converted one to the

other, and QBALL does all computations in English measure (yards,

feet, inches, etc.). Additionally, computations using metric

measure are a bit slower due to the two conversions required. You

may now confidently display saved trajectory data, whichever unit

is in use, and whichever was saved. QBALL detects any conflict,

and converts appropriately. Only range measure is affected.

Heights and deflections will continue to be computed in inches;

velocities, in feet per second.

<ESC>ape returns to the indicated menu.

The info line should now show this data:

Muzzle velocity..................... 2700

Ballistic Coefficient............... .300

Metro.................. 59d 0' 29.53" 78%

Sight height........................ 1.00

[ 20                                                       QBALL ]

When you have "pressed any key," you are returned to the Computa-

tions Menu, where you'll discover that the zero range has been set

to 257. Now step through the process of developing a range table

for this zero, and a maximum of 304 yards. You should use a range

increment which will give you trajectory heights at even multiples

of 25 or 50 yards. Assuming that you choose a range increment of 50

yards, you'll know how high to sight in at, say, 100 yards so as to

be "dead on" at 257, thus assuring a deviation of no more than 5

inches from the line of sight out to 304 yards.

The graphics routines, if invoked, will draw additional horizon-

tal lines at, in this case, 5 inches above and below the displayed

line of sight. Such lines will be displayed until a new zero range

is chosen. If the maximum deviation is less than the sight height,

an "X" is placed on the sight line at the least range for which the

bullet's path is within that maximum deviation.

So what does this do for you?  Well, with this point-blank zero,

you can hold "dead on" out to over 300 yards, and be assured that

the bullet will not strike more than five inches high or low. Beats

estimated holdover, doesn't it?

Wind Vector Corrections.

------------------------

One of the columns in the range table gives the wind deflection,

in inches, for a "pure" 10-MPH crosswind. To compute the deflection

for other windspeeds and directions, press <W> and enter the wind

speed in miles-per-hour, and the wind direction clockwise from 0d to

360d, relative to the line of fire. Pure headwind is 0d. In this ex-

ample, the windspeed is taken as 20 MPH, and the direction is 135d,

or from the right rear.

+=================================================+

|                                                 |

|    Wind velocity      [10 MPH]           20     |

|    Wind direction     [ 270d ]          135     |

|          ============================           |

|        Range wind =  21 feet per second         |

|        Cross wind =  14 miles per hour          |

|                                                 |

|          Crosswind correction :  -1.40          |

|                                                 |

| Use correction factor(s) in tables ? ([Y] or N) |

|                                                 |

+-------------------------------------------------+

Figure 16. Wind Vector Corrections.

Answering [Y]es will cause QBALL to display the 14-MPH crosswind

value in range tables. QBALL takes winds blowing from left to right

as positive, and from right to left as negative. No correction is

applied for the range wind, as the effect is significant only at

long range, with low velocities and a small value for the ballistic

coefficient. (Try changing the muzzle velocity up or down by 21 FPS

and see.) Should you wish to return to using the default crosswind,

you must re-enter the standard values when prompted for the wind

velocity and direction, and reply [Y] to the "use in tables?" prompt.

[ 22                                                       QBALL ]

There are no provisions for printing the Slant Fire Table, nor

are the values integrated into range table computations. A clear

exposition of the geometry of this problem may be found in the

Lyman Manual, listed in the Bibliography.

Sectional Density.

------------------

The sectional densities of two different bullets may be used to

compare their relative penetrating power. Sectional density is

defined as the bullet weight in pounds divided by the square of its

diameter in inches. Press <S>, and enter the diameter of the bullet

in either inches or millimeters. You may use the bore diameter of

the weapon if you do not have a specific bullet diameter; QBALL

assumes that any entered value greater than one (1) is in milli-

meters, and makes the conversion for you. (If you have used QBALL's

Bullet [L]ookup, the bullet diameter is already set; press [Enter]

to display the sectional density.)

The sectional density is shown in printed and saved range table

headers, but there is no specific routine to print just this value,

nor does QBALL at present include any routine which makes use of

the sectional density. Additionally, the sectional density is not

displayed on any screen other than the screen which displays its

computation.

Free Recoil Velocity and Energy.

--------------------------------

We all know that a firearm "kicks." Simple Newtonian physics,

right? But how hard does it kick?  One way to compare absolute num-

bers, and without getting into the mare's nest of "actual" versus

"perceived" recoil, is to compare the recoil velocity and energy

imparted to the weapon upon firing. Press <F>. This example uses a

powder weight of 38 grains, which seems to be a fair average powder

weight from several random examples taken from the Speer Reloading

Manual. You must also enter the weapon's weight, in either pounds

and ounces or in pounds and decimal fractions of a pound. Assume an

8 1/2 pound rifle.

Enter the powder weight, 38, and the weapon weight as either

8<space>8 or 8.5. (Note that space -- it must be included if you're

using pounds-ounces weapon weight entry.)

The following is displayed:

+===============================================+

|                                               |

|         Gun Recoil Velocity:  11.17 FPS       |

|         Free Recoil Energy :  16.48 Ft#       |

|                                               |

+-----------------------------------------------+

Figure 18. Free Recoil Velocity, Energy Display.

[ 24                                                       QBALL ]

You may navigate through the list by using the PgUp, PgDn, Home,

End, and the arrow keys. Press [Enter] to accept the highlighted

bullet, or [ESC] to exit and enter the value manually.

+===================> Bullet Lookup <===================+

| Mak  Dia   Wt    C1    Type      Remarks              |

| ---  ----  ----  ----  --------  -------------------- |

| NOS  .308   165 .410   SPTPT     16330                |

| NOS  .308   165 .428   SPITZ     27585                |

| SRA  .308   165 .453   BTSPITZ   2145                 |

| HDY  .308   165 .459   BTSPIRE   3045                 |

>>>>SPR  .308   165 .459   SPITZ     2035 <<<<<<<<<<<<<<  |

| NOS  .308   165 .475   SPTBLT    39584                |

| SPR  .308   165 .477   BT        2034                 |

| NOS  .308   168 .447   HP M      28473                |

| SRA  .308   168 .475   BTHP M    2200                 |

| HDY  .308   168 .505   BTHP      3050                 |

+-------------------------------------------------------+

PgUp  PgDn  ^  v  Home  End  [ESC]

Figure 19. Typical Bullet Lookup Display

If the bullet indicated >thus< is chosen, both the bullet weight

and the ballistic coefficient will be set to the values shown in

the list. (In this case, the bullet is a 165-grain Speer spitzer

with a diameter of .308 inches and a ballistic coefficient of .459;

2035 in the "Remarks" field is its catalog number.) The bullet dia-

meter is also set for use in the sectional density routine. Bullet

Mak(er) and Type will be displayed on the header line until you tell

QBALL not to so by changing the weapon/bullet designator using the

<N>ew... data menu.

Bullet makers (manufacturers) are abbreviated as follows:

HDY = Hornady          SRA = Sierra

LYM = Lyman            SPR = Speer.

NOS = Nosler

Bullet type abbreviations preceded by a lower-case "c" are cast

bullets. The true bullet weight will depend upon the alloy from

which it is cast. The weight given, however, is a workable first

approximation.

If D.NDX is not available, and M.NDX is, then a listing sorted

by maker, diameter, and weight is displayed. There is no way to

resort the listing unless both index files are present.

The bullet lookup indexes are loaded when QBALL starts. Specific

bullet data is not loaded until called for.

[ 26                                                       QBALL ]

You may enter as many as twenty-five (25) individual range/velo-

city pairs. The average v@1, v@2, and ballistic coefficient are

reported. If r1 is greater than zero, i.e., you are not working

with a muzzle velocity, QBALL reports a calculated muzzle velocity.

Let's work through the process. We'll do this twice: once, with r1

set to 0, and once more with r1 set to 50; r2 will be set at 100 in

both cases. (The data is that found in the table on page 9.)

Ballistic Coefficient: r1 = 0.

------------------------------

When prompted for the "first range," press [enter]; for "second

range," enter 100; press "Y" or [Enter] when prompted for the metro.

The following screen will be displayed. Enter the appropriate velo-

city values as shown. QBALL will quickly return the value of the

ballistic coefficient.

+==============================================================+

|               -----------------------------------            |

|                      Metro: as displayed                     |

| Relative Air Density = 1.000 Relative Sonic Velocity = 1.000 |

|               -----------------------------------            |

|                                                              |

|                       For Round Number 1                     |

|            Muzzle Velocity       (Ft/Sec)      2200          |

|            Second Velocity       (Ft/Sec)      1939          |

|                                                              |

|               For Standard Atmosphere, C1 = .298             |

|                                                              |

|               Calculate another Round? ([Y] or N)            |

|                                                              |

|                                                              |

|          [ Lower & Upper Ranges (yards) Muzzle & 100 ]       |

+--------------------------------------------------------------+

Figure 22. Entry Screen, Infer Ballistic Coefficient

Since we're working from tabular data, another round is super-

fluous; press "N." The center of the screen will clear, and the

following information is displayed:

+============================================+

| -----------------------------------------  |

|       Average C1 for 1 round = .299        |

| Between Mean Velocities 2200 and 1939 FPS  |

|   Calculated Muzzle Velocity = 2200 FPS    |

| -----------------------------------------  |

|                                            |

|          Print C1 Data? ([Y] or N)         |

+--------------------------------------------+

Figure 23. Ballistic Coefficient Display 1.

Press "N". If no bullet weight has been entered, you will be asked

for one; QBALL returns you to the Computations Menu. Let's try the

infer ballistic coefficient routine again, with r1 greater than zero.

<ESC> to the Main Menu, and press <I>, then <B>.

[ 28                                                       QBALL ]

This may have seemed like an exercise in futility: we did, after

all, know what the ballistic coefficient was in the first place.

Suppose, however, you have a ballistics table from, say, Winches-

ter-Western, which includes only velocity data?  Inferring a ball-

istic coefficient based on the published velocity figures will

allow you to make your own "custom" firing table for a particular

factory round. (Most ammunition manufacturers are noticeably reti-

cent when it comes to some ballistics data. Note, too, that manu-

facturers' velocity data may be suspect.) Additionally, should you

be reloading your own ammunition, and have access to two chrono-

graphs, you could make a custom firing table for your handloads.

+-------------------------------------------------------------+

|   If you have only one chronograph, you could fire a string |

| of, say, five rounds at one range, then five more at a dif- |

| ferent range,  thus getting  two different sets of velocity |

| data.  One major disadvantage is that metro or other condi- |

| tions may  change  between the  two strings.  You'd also be |

| playing pretty fast and loose with the statistics involved. |

| (The reliability of data so obtained might be pretty shaky; |

| better some questionable data than no data at all!)         |

+-------------------------------------------------------------+

3.2 Inferring a Muzzle Velocity.

===============================

Assume that you have obtained a supply of ammunition, and that

you know the ballistic coefficient for the bullet in use. QBALL

provides two methods of obtaining a muzzle velocity through live

firing; the first from measured bullet drop at known ranges, the

second from measurement of a single downrange velocity.

At the Infer Data Menu, select "Infer Muzzle <V>elocity" to dis-

play the Infer Muzzle Velocity Menu.

+=======================================+

|  *** Infer Muzzle Velocity Menu ***   |

|  ����������������������������������   |

|  From Bullet <D>rop Measurement       |

|  From One Downrange <V>elocity        |

|  ����������������������������������   |

|  <ESC> to Infer Data Menu             |

+---------------------------------------+

Figure 26. Infer Muzzle Velocity Menu

Use "From Bullet <D>rop Measurement" if you have no chronograph;

use "From One Downrange <V>elocity" if you have access to a chrono-

graph.

[ 30                                                       QBALL ]

In this case, the calculated muzzle velocity, 2695 FPS, is not

the same as that with which we started, 2700 FPS. How much differ-

ence does 5 FPS make? Given, for example, a maximum range of 250

yards, and zeroed at 200 yards, the differences are really incon-

sequential, amounting to a (at most) 0.1 inch trajectory variation,

and .002 second difference in the time of flight. There might well

be this much variation in muzzle velocity--hence trajectories, and

terminal velocity and energy--between successive rounds in a given

string, even if fired under identical conditions with a single lot

of ammunition.

Now for some precautions. The bullet-drop method of inferring a

muzzle velocity is extremely sensitive to errors in drop measure-

ment. To a degree, the shorter the ranges involved, the more criti-

cal the bullet drop measurement becomes. For this reason, QBALL

forces you to use a minimum zero range of 50 yards/meters, and to

measure the drop at a range of at least twice the zero range. (The

maximum allowed range is 400 yards/meters.)

Two or three hundredths of an inch error in drop measurement can

make a big difference in the resulting calculated muzzle velocity.

(See the section in Appendix E, Error Analysis, page 64-65 to see

just how much difference can result.) Even .001 (one one-thousandth)

of an inch difference in the vertical height measurement gives some

variation. You must locate the group center as accurately as possi-

ble -- no "eyeballing" allowed! -- to the nearest .01 inch. This is

relatively simple if you have a scale graduated in fiftieths or

hundredths of an inch; if not, the fraction table in Figure 29 may

be helpful.

+---------------------------------------------------------------+

| 1/64 = .01563  17/64 = .26563  33/64 = .51563  49/64 = .76563 |

| 1/32 = .03125   9/32 = .28125  17/32 = .53125  25/32 = .78125 |

| 3/64 = .04688  19/64 = .29688  35/64 = .54688  51/64 = .79688 |

| 1/16 = .06250   5/16 = .31250   9/16 = .56250  13/16 = .81250 |

| 5/64 = .07813  21/64 = .32813  37/64 = .57813  53/64 = .82813 |

| 3/32 = .09375  11/32 = .34375  19/32 = .59375  27/32 = .84375 |

| 7/64 = .10938  23/64 = .35938  39/64 = .60938  55/64 = .85938 |

| 1/8  = .12500   3/8  = .37500   5/8  = .62500   7/8  = .87500 |

| 9/64 = .14063  25/64 = .39063  41/64 = .64063  57/64 = .89063 |

| 5/32 = .15625  13/32 = .40625  21/32 = .65625  29/32 = .90625 |

|11/64 = .17188  27/64 = .42188  43/64 = .67188  59/64 = .92188 |

| 3/16 = .18750   7/16 = .43750  11/16 = .68750  15/16 = .93750 |

|13/64 = .20313  29/64 = .45313  45/64 = .70313  61/64 = .95313 |

| 7/32 = .21875  15/32 = .46875  23/32 = .71875  31/32 = .96875 |

|15/64 = .23438  31/64 = .48438  47/64 = .73438  63/64 = .98438 |

| 1/4  = .25000   1/2  = .50000   3/4  = .75000                 |

+---------------------------------------------------------------+

Figure 29. Fraction Table. Use to convert civil to decimal

measure.

[ 32                                                       QBALL ]

Chronograph Available (One Downrange Velocity) Method.

-----------------------------------------------------

A chronograph may be used to measure a bullet's downrange velo-

city, but two factors must be kept in mind. First, no matter how

near the muzzle the chronograph is placed, there is an inevitable

decrease in velocity. Second, there is a minimum safe distance at

which the chronograph should be placed.

QBALL provides a means to determine the muzzle velocity from a

velocity measured at a (relatively) short range. (This process is

variously known as "back-computing muzzle velocity" and "correcting

for instrument(al) error/variation.")

Select "From One Downrange <V>elocity" from the Infer Muzzle Velo-

city Menu.

QBALL will ask for a ballistic coefficient if one has not been en-

tered, the velocity, the range, and will prompt for the metro. QBALL

uses a default range of 5 yards/meters; you may select a range from

1 yard/meter to 100 yards/meters. (For safety's sake, a minimum of

three (3) yards/meters is suggested.) After a short delay, QBALL dis-

plays a calculated muzzle velocity.

Using the initial ballistic data from chapter 2, the velocity at

5 yards range is found to be

Range    R e m a i n i n g

Yards    Velocity   Energy

-----    -----------------

0      2200      1934

>>>>>     5      2187      1911    <<<<<

Figure 31 illustrates the data entry and result.

+-------------------------------------------------+

|   Range to Chronograph       (Yards)       5    |

|   Downrange Velocity           (FPS)    2187    |

|   Use Displayed Metro?    ([Y] or N)       Y    |

|                                                 |

|     Calculated Muzzle Velocity = 2200 FPS       |

|                                                 |

|            ===> press any key <===              |

+-------------------------------------------------+

Figure 31. Muzzle Velocity from One Downrange Velocity.

Press any key; if no bullet weight has been entered, QBALL will

ask for one, and go (or return) to the Computations Menu.

[ 34                                                       QBALL ]

4.4 Print Functions.

====================

You may obtain hard copy of range tables from within the "Choose

Next Output" menu, if the print function is activated, or of any

data display which is followed by the prompt:

+==============================+

| Print (........)? ([Y] or N) |

+------------------------------+

Figure 32. "Print?" prompt.

Press [Y] to print a data table. The prompt will be replaced with

the statement "Printing (......)" in highlighted text. QBALL sends

the data to the printer, and returns you to the previous menu.

4.5 Typical Printed Output.

===========================

The first range table which was computed in Chapter 2 looks like

this in printed form:

---------------------------------------------------------------

===============================================

>>> Ballistic Computations: Typical Firearm <<<

-----------------FILENAME.R01-----------------

Muzzle Velocity = 2200 FPS       Bal Coef (C1)   = .300

Bullet Weight =  180 Gr          Sectional Density = n/a

Temperature  =  59 deg F         Altitude  =    0 Feet

Barometric Pressure = 29.53"     Relative Humidity = 78%

===============================================

Range Table, Zeroed at [Bracketed] Range

----------------------------------------

Range  R e m a i n i n g  Total  Path/Sight Line  Defl'n  Time

Yards  Velocity   Energy   Drop  M.O.A.   Inches  10 MPH  Flght

-----  -----------------  -----  ---------------  ------  -----

0    2200      1934    0.00   0.00     -1.00    0.00  0.000

50    2068      1708    0.94   7.77      3.89    0.39  0.070

100    1939      1503    3.88   6.76      6.76    1.48  0.145

150    1816      1319    9.14   4.88      7.33    3.52  0.225

200    1700      1155   17.13   2.58      5.16    6.68  0.311

[  250    1589      1010   28.11   0.00      0.00   10.80  0.402 ]

300    1485       881   42.40  -2.82     -8.47   15.89  0.499

----------------------------------------

[ End of Table ]

---------------------------------------------------------------

Figure 33. Typical Range Table Printed Output.

For data tables, all data is recomputed for output to the print-

er; information which has scrolled past is printable. While the

header information differs, the printed data output formats are

virtually identical to the screen display.

Range tables saved to a disk file have exactly this same format.

[ 36                                                       QBALL ]

This Page For Notes.

[ 38                                                       QBALL ]

You may also <S>wap the unit of range measure between yards and

meters from the New or Changed Input Data Menu, accessed from the

Computations Menu.

Toggling the <G>rid merely changes the appearance of the graphics

screen when it is first called with any given set of data. You may

at that time change the graphics display; there is no provision to

turn graphics off completely.

If you toggle the <P>rinter off, all "Print (......)?" prompts

are suppressed,

====> Press any key <====

is displayed instead,

and all <P>rint...menu items are displayed in low-intensity video.

If <N>oises are toggled off, they are replaced with a short-

duration "flash" in the lower right corner of your screen (char-

acter-based screens, only.)

Press <ESC> to return to the Utilities Menu.

5.3  Configuring Display Colors.

================================

Press <C>. A special color configuration screen will appear.

+=============================+

|  Press [K]ey to increment:  |

|                             |

|   [F] - Foreground          |

|   [H] - Highlight color     |

|   [I] - Info Line color     |

|   [B] - Background color    |

|   [L] - Help Background     |

|                             |

|     press <ESC> to end...   |

+-----------------------------+

[Now Set-    FG = 15 HiLite = 10 Info = 7 BG = 1 Help = 7]

[[D]efault - FG = 15 HiLite = 10 Info = 7 BG = 1 Help = 7]

[Blk/[W]ht - FG =  7 HiLite = 15 Info = 7 BG = 0 Help = 0]

Figure 37. Set Display Colors Screen.

Press the appropriate key to "step" through the available colors

for the [B]ackground, main body text ([F]oreground), Highlighted

text ([H]ighlight), the Title/Info line (Low [I]ntensity), and the

He[L]p window background. The colors chosen are immediately shown

so that you will be able to see what they look like. When you've

found a combination that you like, press [ESC] to use the display

colors you have chosen, and return to the Utilities Menu. Note that

the currently chosen and the default color values are displayed at

the bottom of the color configuration screen. If you get in a

muddle, press [D] to return to the supplied default color settings.

If you'd like a white-on-black screen, press [W] for that option.

[ 40                                                       QBALL ]

5.5  Invoking a DOS Shell.

==========================

Press <D> to shell out to DOS.

When started, QBALL writes a special batch file, QBSHELIT.BAT, in

the current directory. When the <D> option is chosen, QBALL calls

QBSHELIT.BAT, which sets the DOS prompt like this:

[ Exit >> QBALL ] [drive:]\[directory] > _

and then invokes a secondary command

processor, COMMAND.COM. If you try to start QBALL from within the

shell, the computer will beep, and the following error message is

displayed:

QBALL is already loaded!

Depending on your own computer's configuration, you may or may

not remain in the shell; this is largely dependent upon how you

have COMSPEC set. See your DOS documentation for more details.

QBSHELIT.BAT is erased upon normal exit from QBALL. If, for some

reason, this error message is displayed when starting QBALL, check

for the presence of QBSHELIT.BAT, and erase it if necessary. See

page 43.

When you exit the shell, QBALL performs a series of file manipu-

lations, which are described in Chapter 7, File Handling, on page

43.

Type "exit" (+ [enter]) to return to QBALL.

5.6  Reading Range Table Files and Translating Graphics Files.

==============================================================

The <R>ead Range Table and <T>ranslate Graphics options are

discussed in detail in Chapter 8, QBALL's Associated Utilities,

which you will find on page 47.

[ 42                                                       QBALL ]

Option <G> and option <D> will reset the input data. If you

don't want to change the input data, press <ESC> to return to the

Main menu.

If you determine that the information displayed with the menu is

the data you want to use, and you don't need a picture, press <D>

to use the displayed data as the default.

Press <G> to display this trajectory graphic:

> Ballistic Computations: Typical Firearm <

+--------------------------------------------------------------+

| | 8                                                        8 |

|I|                         o   *                              |

| | 6                *                                       6 |

| |                                        *                   |

|N| 4                                                        4 |

| |        *                                                   |

| | 2                                                        2 |

|C|                                                            |

| |-0------------------------------------------------*-------0-|

| *                                                            |

|H|-2  [draw a smooth curve]                                -2 |

| |    [connecting the "*".]                                   |

| |-4                                                       -4 |

|E|                                                            |

| |-6                                                       -6 |

| |                                                            |

|S|-8                         YARDS                         -8 *

| |       50        100        150        200       250        |

+--------------------------------------------------------------+

|     [ --- Remaining Velocity/Energy, Current Round --- ]     |

+--------------------------------------------------------------+

|           >Current<     >EGA_SAV1<   +=======================+

|Max Ht :               7.43" @ 135 Yds| *** Graphics Menu *** |

|Min Ht :              -8.47" @ 300 Yds| --------------------- |

|C1 / Wt:              .300 / 180 gr   |<R>ecall <1> thru <2>  |

|Muz V/E:              2200 / 1934     |                       |

|Ter V/E:              1485 / 881      |                       |

|ZeroRg :                250 Yards     | --------------------- |

|Metro  :             59d0'29.53"78%   |<ESC>ape to Choose Next|

+--------------------------------------------------------------+

|        [ --- info line schematically indicated --- ]         |

+--------------------------------------------------------------+

Figure 40. Saved Trajectory Data Graphic Display

You may recall any of the other saved trajectory files for dis-

play at this point, but the input data is not reset. Press <R>ecall

and the <number> to display another trajectory.

QBALL will use the identifier, muzzle velocity, ballistic coeffi-

cient, bullet weight, metro, sight height, and range unit until

changed. You may change that data using the New/changed Data Input

routines, or you may recall another saved trajectory file.

[ 44                                                       QBALL ]

The data needed to display trajectory graphics is saved with one of

these filename formats: [filename].ETn (EGA-equipped computers) or

[filename].CTn (CGA-equipped computers). A [filename] may be user-

specified; the defaults are EGA_SAVn and CGA_SAVn, respectively.

The number "n" is never less than "1" and never greater than "F."

That's hexadecimal notation for "1" through "15;" QBALL will not

allow more than 15 trajectory graphics files. Why? Two reasons: it

allows you to choose a saved trajectory with one keypress; and,

fifteen different graphics trajectories displayed all at once make

for a pretty cluttered display.

EGA trajectory files average about 2450 bytes in length; CGA,

about 1400. That's a lot of space, and another reason to run QBALL

from a hard disk.

The rest of this section assumes EGA graphics; if you're working

with a CGA setup, for "EGA/ETn" read "CGA/CTn."

Suppose you have saved several trajectory files, [filename].ET1

through [filename].ETB--that's eleven files. Now suppose that you

delete [filename].ET3 through [filename].ET9; either from "bare"

DOS or from within QBALL's DOS shell. Now you have [filename].ET1,

*.ET2, *.ETA, and *.ETB remaining on/in your disk/directory. When

you start QBALL, or "exit" from QBALL's DOS shell, the remaining

trajectory files are renamed thus:

[filename].ET1 becomes [filename].ET1

[filename].ET2 becomes [filename].ET2

[filename].ETA becomes [filename].ET3

[filename].ETB becomes [filename].ET4

File renaming is completely automatic and transparent. All refer-

ences to saved trajectories, e.g., in menus, will now reflect "<1>

thru <4>."

Range Tables Saved to Disk.

---------------------------

Range tables saved to disk eat up lots of space, and do it fast!

For example, any table with a maximum range of 1760 yards and a

range increment of 1 yard--which is entirely possible--uses over

150k of disk space! Not only that, it's slow enough on a hard disk,

and almost unbearably so on a floppy. Use a hard disk, if available,

or a RAM disk if you can.

All range tables are named "[filename].Rnn, where [filename] may

be user specified; the default is [RGTAB_nn]. The number "nn" can

be anything from "01" to "99," thus allowing ninety-nine range

tables to be saved. (Right! If you've got the disk space!) If any

saved range tables are deleted, the remaining ones are renamed in

sequence, just as with graphics files. Again, the procedure is

automatic, and completely transparent.

[ 46                                                       QBALL ]

This Page For Notes.

[ 48                                                       QBALL ]

8.2 Graphics Translator (GRFXLATE.EXE).

=======================================

EGA-equipped computers ignore CGA-format saved trajectory data,

and vice versa. You may translate from one to the other so that

data saved with one graphics format may be viewed on the other.

In order to access the Graphics Translator from within QBALL,

both of the following conditions must be true: first, QBALL.EXE

and GRFXLATE.EXE are on/in the same disk/directory; and second,

if you have an EGA-equipped computer, then at least one

"[filename].CTn"...

or

if you have a CGA-equipped computer, then at least one

"[filename].ETn"...

...exists on/in that directory.

If either of these conditions is untrue, QBALL disables the

<T>ranslate a Graphics File option in the Utilities menu, i.e., the

option is displayed in low-intensity video.

The remainder of this section assumes that the current computer

--the one you're working on--is EGA-equipped. At the Utilities menu,

assuming that the option is activated, press "<T>." After a short

pause, the following screen will appear:

+=============================================================+

|          ===== =       = =  [                        ]      |

|          |   | |-+  -+ | |  [   C G A   TO   E G A   ]      |

|          |   | | | +-| | |  [                        ]      |

|          |___| |_| |_| | |        TRANSLATOR   1.0b         |

|       ----- |_----------------------------------------      |

+-------------------------------------------------------------+

.

CGA Graphics Files Present      EGA Graphics Files Present

--------------------------------  -------------------------------

CGA_SAV1.CT1                    EGA_SAV1.ET1

FILENAME.ET2

+---------------------------------------------------------+

| Press <1> to translate CGA Graphics file to EGA format. |

+---------------------------------------------------------+

[ESC] exits

Figure 42. QBALL's Graphics Translator

Press the key corresponding to the file extension of the CGA

file to translate, in this case, "1." (This one's "hotkeyed"--you

need not press [enter].) GRFXLATE will write the translated file to

"EGA_SAV3.ET3." If you have a full complement of EGA-format traj-

ectory files, "1" through "F" (hexadecimal notation--there are

fifteen such files permitted) GRFXLATE will request permission to

overwrite the existing EGA_SAV1.ET1. A "no" response at that time

returns to QBALL.

[ 50                                                       QBALL ]

This Page For Notes.

.

[ 52                                                       QBALL ]

caliber - For small arms, the diameter of the bore measured across

the lands. In practice, the diameter of the bullet. In artillery

and naval gun usage, a measure of length equal to bore diameter;

thus, a "fifty caliber" barrel would be 50 bore diameters long.

cartridge - Fixed ammunition, including case, powder, primer, and

bullet, for small arms. See also: round.

case - The (usually) brass component of a cartridge which contains

the powder, primer, and bullet prior to firing. See also: brass.

chamber - The breech, or rearmost portion of the barrel, which

contains and supports the cartridge prior to, during, and immedi-

ately following firing.

coefficient of form - A number relating the ballistic efficiency of

a given bullet shape to the shape of the projectile used to calcu-

late the ballistic table.

C1 - Formula symbol for ballistic coefficient.

cross wind angle - The angle between the wind direction and the

line of departure. Typically tabulated as ninety degrees, or "pure"

crosswind with no range vector.

drift - Technically, the distance a bullet will travel horizon-

tally due to its spin. This effect is usually quite small, amount-

ing to about 7 inches @ 1000 yards for a military M2 Ball (.30-'06)

cartridge. Often confused with wind deflection. See also: wind

deflection.

deflection - See: wind deflection.

downrange - Refers to the position of objects in space relative to

the direction of fire; point of aim and point of impact, for exam-

ple, are by definition downrange.

drop - The distance a bullet will fall due to the influence of

gravity. Drop is measured from the line of departure, not the line

of sight.

elevation - Vertical sight adjustment. Also, the (usually small)

angle between the axis of a firearm's bore and the horizontal.

energy - Here, the kinetic energy of a moving bullet, usually given

in ft-lbs. Calculated by multiplying the bullet mass by the square

of the velocity, and dividing by two.

F.P.S. (also FPS and Ft/Sec) - Feet per second. A unit of velocity.

(Properly a unit of speed, as velocity is a vector.)

Ft.-Lbs. (also FtLbs and Ft#) - Foot pounds. A unit of energy equal

to the effort required to lift one pound a vertical distance of one

foot.

[ 54                                                       QBALL ]

mass - Properly, the weight of an object divided by the accelera-

tion due to gravity. Commonly taken as synonymous with "weight."

maximum height - The greatest vertical distance the bullet rises

above the line of sight. Sometimes confused with midrange trajec-

tory since it typically occurs about halfway between muzzle and

target. Maximum height is usually somewhat higher and occurs some-

what further downrange than the midrange trajectory.

metallic sights - Sights containing no optical elements.

met - An abbreviation for "meter"; also, a super-abbreviation of

"Meteorological Conditions." See also: meter, metro, standard met.

meter (also metre) - A unit of length measure equal to 39.37

inches, 3.28083 feet, or 1.09361 yards.

metric - Having reference to length measurements based upon the

meter, its subdivisions, or its multiples.

metro - Short for Meteorological Conditions; the air temperature,

barometric pressure, gun altitude, and relative humidity existing

at the place and time of firing. See also: standard met.

midrange trajectory - The height of the trajectory at a point half-

way between the firearm's muzzle and the point of aim.

M.O.A. (also MOA) - Minute of angle or minutes of arc. Equal to

1/60 of an angular degree. At 100 yards, 1 M.O.A. is approximately

1.047 inches.

muzzle - The foremost point of a firearm's barrel; that part of a

firearm closest to the target.

muzzle energy - Foot-pounds of energy developed by a moving projec-

tile at the firearm's muzzle.

muzzle velocity - The speed of a projectile at a firearm's muzzle,

generally expressed in FPS.

N.R.A. (also NRA) - National Rifle Association. If you shoot, you

should belong to this organization.

ogive - The curve of a bullet's forward portion. Also, the radius

of this curve, usually expressed in calibers.

point-blank range - That range for which the bullet's path varies

from the line of sight by no more than a specified amount.

point of aim - That downrange point with which a firearm's sights

are aligned.

point of impact - That point which a bullet actually strikes. In

most cases, considered in relation to point of aim.

[ 56                                                       QBALL ]

stp - Standard Temperature and Pressure. See also: standard met.

string - A number of rounds fired at a common point of aim, often

for the purposes of sighting in or zeroing a firearm.

targeted range - That distance furthest from the muzzle where the

path of the bullet crosses the line of sight. (The "sighting in"

distance, or "zero range.")

time of flight - The time necessary for a bullet to travel from

the muzzle of a firearm to its point of impact or to any intermed-

iate range.

trajectory - The path of a moving projectile. Often expressed as

the number of inches above or below the line of sight.

windage - Horizontal, lateral sight adjustment. Also, the sight

adjustment required to compensate for crosswind effects at a given

range.

wind deflection - The amount of a projectile's horizontal lateral

motion attributable to the action of crosswind wind vector. Often

mistakenly called drift. See also: drift, windage.

yaw - Normally, a situation in which a bullet rotates on its axis

at a small angle to the line of flight.

zero - The sight setting producing coincidence of point of impact

and point of aim at a given range. Also, the process of determin-

ing that sight setting. See also: sighting in.

zero range - That range furthest from the muzzle at which the point

of aim and point of impact coincide.

[ 58                                                       QBALL ]

Muzzle Velocity.

----------------

This data is from the Speer Reloading Manual #11.

+-----------------------------------------------------+

| From Speer Reloading Manual. 180 gr bullet, C1=.30  |

|                                                     |

|                          Range in Yards             |

|            ---------------------------------------  |

|            Muz      50    100    150    200    250  |

|            ---------------------------------------  |

| Velocity   2400   2259   2122   1990   1863   1742  |

| Energy     2302   2039   1800   1582   1388   1213  |

| Trajectory -1.5    0.0   -0.2   -2.3   -6.7  -13.5  |

+-----------------------------------------------------+

Table B3. Data Taken From Speer Manual.

Using the zero range of 50 yards, a vertical difference of 0.2"

below line of sight at 100 yards, and standard metro, QBALL infers

the muzzle velocity to be 2411 FPS.

QBALL reports this data using the inferred muzzle velocity:

+-----------------------------------------------------+

| Calculated using QBALL.      180 gr bullet, C1=.30  |

|                                                     |

|                          Range in Yards             |

|             --------------------------------------- |

|             Muz      50    100    150    200    250 |

|             --------------------------------------- |

| Velocity    2411   2272   2137   2006   1880   1761 |

| Energy      2323   2062   1825   1609   1413   1239 |

| Trajectory -1.50   0.00  -0.20  -2.27  -6.47 -13.23 |

+-----------------------------------------------------+

Table B4. QBALL Data, Inferred Muzzle Velocity.

The maximum differences are:

+-----------------------------------------------------+

| QBALL vs Speer. Speer data taken as standard.       |

|                                                     |

|                          Range in Yards             |

|             --------------------------------------- |

|             Muz      50    100    150    200    250 |

|             --------------------------------------- |

| Velocity    0.5%   0.6%   0.7%   0.8%   0.9%   1.1% |

| Energy      0.9%   1.1%   1.4%   1.7%   1.8%   1.1% |

| Trajectory   0"     0"     0"    .03"   .23"   .27" |

+-----------------------------------------------------+

Table B5. Difference Table.

[ 60                                                       QBALL ]

If you have saved trajectory data with an earlier QBALL version,

you can convert the saved file by loading it into a plain-vanilla

ASCII text editor, and carefully modifying the lines above the

range/height data to match the pattern given on the preceding page.

Cautions: there must be an entry in every data position; other-

wise, QBALL will be hopelessly confused. In addition, make sure

that you save your modified file with the appropriate extension,

either ETn or CTn.

If properly done, such modified files are usable, but you may

find the trajectory plots are pretty ragged, as the range/height

data is not as precise.

+-------------------------------------------------+

| Hint:                                           |

|                                                 |

|   This whole process may be more work than it's |

| worth.   You might very well want to delete the |

| trajectory files generated by an earlier QBALL  |

| version, and start over from scratch.           |

|                                                 |

+-------------------------------------------------+

[ 62                                                       QBALL ]

Selected Bibliography (continued)

Winchester Ammunition Product Guide. East Alton: Winchester/Olin

Corporation, 1993.

Winchester Product Information and Ballistics Guide. East Alton:

Winchester/Olin Corporation, 1989.

Winchester Reloading Components Catalog. East Alton: Winchester

Group, Olin Corporation, 1992.

[ 64                                                       QBALL ]

+--------------------------------------------------------------+

|        Trajectory Comparisons  -  Conditions as Shown        |

|        ==============================================        |

|                   [ All at Standard Met ]                    |

|--------------------------------------------------------------|

|Range = 250 yards, zeroed at 50 yards; C1 =.12, MuzVel = 2100 |

|        Inches above/below line of sight at...(yards)         |

|        ---------------------------------------------         |

|        Muzzle     50       100       150       200       250 |

|        ------    ---      ----      ----      ----      ---- |

|QBALL    -1.5     0.0      -1.3      -6.4     -16.9     -34.3 |

|SPEER    -1.5     0.0      -1.3      -6.4     -17.0     -34.7 |

|HORNADY  -1.5     0.0      -1.2      -6.3     -16.7     -33.2 |

|--------------------------------------------------------------|

|Range = 400 yards, zeroed at 200 yards; C1 =.26, MuzVel = 2500|

|          Inches above/below line of sight at...(yards)       |

|          ---------------------------------------------       |

|          Muzzle       100        200        300        400   |

|          ------      ----       ----       ----       ----   |

|QBALL      -1.5        2.9        0.0      -13.0      -39.4   |

|SPEER      -1.5        3.0        0.0      -13.0      -39.6   |

|HORNADY    -1.5        3.0        0.0      -13.1      -39.7   |

|--------------------------------------------------------------|

|Range = 400 yards, zeroed at 200 yards; C1 =.50, MuzVel = 3300|

|          Inches above/below line of sight at...(yards)       |

|          ---------------------------------------------       |

|          Muzzle       100        200        300        400   |

|          ------      ----       ----       ----       ----   |

|QBALL      -1.5        1.1        0.0       -5.2      -15.1   |

|SPEER      -1.5        1.1        0.0       -5.2      -15.2   |

|HORNADY    -1.5        1.1        0.0       -5.3      -15.7   |

+--------------------------------------------------------------+

(Key: MuzVel = Muzzle Velocity  C1 = Ballistic Coefficient)

Table D2. Trajectory Comparisons Under Specified Conditions.

QBALL data rounded to nearest 0.1 inch.

An examination of the data in Table D2 shows that there is close

agreement between QBALL's results and published data. The greatest

variation is at the greatest ranges; this is to be expected. Since

the ballistic coefficients from the Hornady tables are not exactly

those chosen as typical, being "high-on-low" respectively, it is

also expected that the resulting bullet strikes will be too small,

"on", and too large. There is, however, substantial agreement between

the QBALL results and the data from both published tables. In gener-

al, QBALL gives excellent practical accuracy.

Inferred Muzzle Velocity.

-------------------------

As was found in chapter 3, muzzle velocities developed by use of

the bullet-drop method may not agree with known-correct data. The

following table illustrates the variation in resultant muzzle velo-

city using several different values for the bullet drop.

[ 66                                                       QBALL ]

This Page For Notes

[ 68                                                       QBALL ]

+--------------------------------------------------------------+

| Program: BALCALC.                                            |

| Input Data:                                                  |

|     Ballistic Coefficient (C1)....... .295                   |

|     Bullet Weight.................... 130 grains             |

|     Zero Range....................... 200 yards              |

|     Maximum Range.................... 500 yards              |

|     Range Increment..................  50 yards              |

|     Sight Height..................... 0.9 inches             |

|     Crosswind........................ -- no wind --          |

|     Metro............................ Standard               |

|--------------------------------------------------------------|

|Range R e m a i n i n g Total  Path/Sight Line Defl'n  Time   |

|Yards Velocity   Energy Drop   M.O.A.   Inches 0 MPH   Flght  |

|--------------------------------------------------------------|

|BALCALC Reports:                                              |

|                                                              |

|    0    3000     2597                   -0.9    0.0  0.00000 |

|   50    2838     2325  +-----++-----+    1.1    0.0  0.05141 |

|  100    2682     2076  | Not || Not |    2.0    0.0  0.10578 |

|  150    2531     1849  |Given||Given|    1.7    0.0  0.16334 |

|  200    2386     1643  +-----++-----+    0.0    0.0  0.22438 |

|  250    2245     1454                   -3.2    0.0  0.28920 |

|  300    2108     1283                   -8.1    0.0  0.35815 |

|  350    1977     1128                  -15.0    0.0  0.43163 |

|  400    1851      988                  -24.1    0.0  0.51005 |

|  450    1730      864                  -35.8    0.0  0.59389 |

|  500    1615      753                  -50.3    0.0  0.68364 |

|                                                              |

|QBALL Reports:                                                |

|                                                              |

|    0    3000     2598    0.00    0.00   -0.90   0.00  0.000  |

|   50    2838     2325    0.50    2.15    1.08   0.00  0.051  |

|  100    2682     2076    2.09    1.97    1.97   0.00  0.106  |

|  150    2531     1850    4.87    1.10    1.65   0.00  0.163  |

|  200    2386     1643    9.00    0.00    0.00   0.00  0.224  |

|  250    2245     1454   14.69   -1.29   -3.22   0.00  0.289  |

|  300    2109     1283   22.12   -2.72   -8.17   0.00  0.357  |

|  350    1977     1128   31.46   -4.29  -15.03   0.00  0.431  |

|  400    1850      988   42.94   -6.01  -24.03   0.00  0.508  |

|  450    1730      864   57.21   -7.96  -35.83   0.00  0.591  |

|  500    1616      754   74.44  -10.12  -50.59   0.00  0.684  |

+--------------------------------------------------------------+

Table F2. BALCALC and QBALL Compared

Summary of absolute differences. Remaining Velocity: 1 FPS; Re-

maining Energy: 1 FtLb; Path/Sight Line: 0.29 inches; Time of

Flight: 0.00105 seconds.

Available with BALCALC, unavailable with QBALL. Bullet Momentum in

Lb-Ft per second per second; Lead in feet for entered target

lateral speed.

[ 70                                                       QBALL ]

+--------------------------------------------------------------+

| Program: Barnes Ballistics.     (Barnes Bullets, Inc.)       |

| Input Data:                                                  |

|     Ballistic Coefficient (C1)....... .295                   |

|     Bullet Weight.................... 130 grains             |

|     Zero Range....................... 200 yards              |

|     Maximum Range.................... 600 yards              |

|     Range Increment.................. 100 yards              |

|     Sight Height..................... 0.9 inches             |

|     Crosswind........................ -- no wind --          |

|     Metro............................ Standard               |

|--------------------------------------------------------------|

|Range R e m a i n i n g Total  Path/Sight Line  Defl'n  Time  |

|Yards Velocity   Energy Drop   M.O.A.   Inches  0 MPH   Flght |

|--------------------------------------------------------------|

|BARNES BALLISTICS Reports:                                    |

|                                                              |

|    0    3000     2597  +-----++-----+   -0.90  0.0    0.0000 |

|  100    2682     2076  | Not || Not |    2.00  0.0    0.1061 |

|  200    2386     1643  |Given||Given|    0.00  0.0    0.2255 |

|  300    2109     1283  +-----++-----+    8.12  0.0    0.3593 |

|  400    1851      989                  -24.13  0.0    0.5115 |

|  500    1615      753                  -50.14  0.0    0.6842 |

|  600    1408      572                  -89.79  0.0    0.8851 |

|                                                              |

|QBALL Reports:                                                |

|                                                              |

|    0    3000     2598    0.00    0.00   -0.90  0.00   0.000  |

|  100    2682     2076    2.09    1.97    1.97  0.00   0.106  |

|  200    2386     1643    9.00    0.00    0.00  0.00   0.224  |

|  300    2109     1283   22.12   -2.72   -8.17  0.00   0.358  |

|  400    1850      988   42.94   -6.01  -24.03  0.00   0.509  |

|  500    1616      754   74.44  -10.12  -50.59  0.00   0.684  |

|  600    1407      572  119.18  -15.06  -90.37  0.00   0.883  |

+--------------------------------------------------------------+

Table F4. Barnes Ballistics and QBALL Compared

Summary of absolute differences. Remaining velocity, 1 FPS; re-

maining energy, 1 Ft-Lb; Path/Sight Line, 0.58 inches; Time of

Flight, .0025 seconds.

Available with BARNES BALLISTICS, unavailable with QBALL. Tabular

side-by-side comparison of two different loads.

----------

As can readily be seen from the preceding tables, QBALL is in

substantial (not to say "bang on"--pun intended) agreement with

several other computer ballistics programs. Other sources with

which QBALL is in substantial agreement include the Winchester

Product Information and Ballistics Guide (1989), and the Remington

Firearms and Ammunition Catalog (1990).

[ 72                                                       QBALL ]

Enter  50 <= Range <= 100: In inferring a muzzle velocity, range

for zero drop must be at least 50, and no more than 100 yards.

Enter  Range <= 1760 Yards

- or -

Enter  Range <= 1609 Meters: In either case, you're attempting to

enter a range which is outside QBALL's ability to use effectively.

(1760 yards = 1609 meters = 1 mile.)

Enter  0 < Range Increment <= Range: You're trying to enter a range

increment of zero, or to enter a range increment greater than the

maximum range chosen.

Enter  0 <= Relative Humidity <= 100: You're attempting to enter a

relative humidity value which is out of range. (Values outside

this range are, by definition, impossible.)

Enter  Second Range > First Range: When inferring a ballistic

coefficient, you're attempting to enter a second range which is

smaller than the first range you have already entered.

Enter  Second Velocity  < First Velocity: When inferring a

ballistic coefficient, you're attempting to enter a downrange

velocity which is larger than the velocity closer to the muzzle.

Enter  0.1 < Maximum Deviation <= 40: In the determination of point-

blank range, the maximum deviation must be greater than 0.1 inches

and less than 40 inches.

Enter  -2.0 <= Sight Height <= +5.0: You're attempting to enter a

value for sight height which is out of range.

Enter  -50 <= Temperature <= +125: You're attempting to enter a

temperature value which is out of range. (If you really need this

program to operate at Arctic/Saharan temperatures, let me know!

I'll customize one for you.)

Enter  300 <= Velocity <= 4500 FPS: You're trying to enter a velo-

city which is less than 300 FPS or greater than 4500 FPS.

Enter  Vertical Angle <= 45d

- or -

Enter  Height <= [feet]: In determining "hits high" for slant fire,

the vertical dimension must be less than 45d, or less than the

horizontal range expressed in [feet].

Enter  Wind Direction <= 360d: In determination of wind deflection

corrections, you're attempting to enter a wind direction which is

greater than one complete circle.

Enter  Wind Velocity <= 231 MPH: In determination of wind deflec-

tion corrections, you're attempting to enter a wind speed in excess

of the world record.

(Record set on Mount Washington, NH, in 1935.)

[ 74                                                       QBALL ]

This Page For Notes.

[ 76                                                       QBALL ]

This Page For Notes.

.

[ 78                                                       QBALL ]

[ F ]                         Inferring Ballistic

Files 1                                   Coefficients (cont)

automatic renaming 44              one downrange velocity 26

copying 1                          two downrange velocities 27

distribution 1                     actual muzzle velocity 27

naming 11 13 44                    Samples from published data 57

QBSHELIT.BAT 40 44               Inferred Muzzle Velocity 28-32

required 1                         from bullet drop 28ff

trajectory data 57                 from one velocity 32

trajectory incompatible 43 59      in C1 calculations 27

Filenames                          Information (Info) Line 4

Range tables 11 44

Trajectory graphics 11 44             [ L ]

Free Recoil Velocity, Energy 22    Limited use license i

data entry 22                    Lookup facility 7 23

examples of... 23

gun weight 22                         [ M ]

powder (propellant) weight 22    M.NDX 1 23

valid data limits 22             Maximum Deviation

Fraction table 30                    entering 19

display 20

[ G ]                         Menus

Glossary 49                          Choose Next Output 10

Graphics                             Color Configuration 38

auto-detect 3                      Computations 8

capture to .GIF et.al. 14          Graphics 13

displaying 11-16 42                Infer Data 25

translating 48                     Main 4

typical saved file 59              New Input Data 17

GRFXLATE.EXE 48                      Other Procedures 19

Group center, finding 31             Recall Trajectory Data 41

Switch Toggle 37

[ H ]                           Utilities 37

Hardware 1                         Metro 4

auto-detect graphics 1             changing 18

chronographs 25 28                 standard 4

recommended 1                      valid data limits 18

Header line                        Muzzle Velocity 7

default 4                          changing 18

weapon, cartridge identifier 17    entering 7

bullet maker and type 24           Inferring (calculating) 28-32

Help                                 valid data limits 18

from author vi

on-line 2                             [ N ]

New/changed Input Data 17

[ I ]                           Clean Sweep 5 17

Increment, range                     Single Inputs 17

(see: range increments)          Non-horizontal fire 21

Inferred Ballistic Data 25-31      Non-standard metro 7 18

Ballistic Coefficients 25-28

Muzzle Velocity 28-32                 [ O ]

Examples from published data 57  Opening screen 3

Inferring Ballistic                Other Ballistics Procedures 19

Coefficients 25-28            Free recoil velocity/energy 22