Stomper  3.05

Modemsharing-Software for Novell or Netbios

based networks

DOS/Windows

(C) 1994,1995 Pflug Datentechnik.

Hardware and software requirements for

installation:

-   network, preferably Personal NetWare,

Novell NetWare lite, NetWare >=3.11 or

Netbios network (Lantastic)

-   minimum DOS 3.3

-   Windows 3.1 if available

-   1 serial interface per modem server, if

possible FIFO buffered (UART 16550)

-   Processor depending on number of modem

servers on one computer, minimum AT

(80286)

1 The Concept

1.1 What is a modem-/ISDN-server?

1.2 Server-Client Operation

2 Installation

2.1 fast installation

2.2 Complete Installation

2.2.1 The Queuemanager

2.2.2 The modem server

2.2.3 The Client

2.2.4 Windows-Modemsharing

2.2.5 The ISDN server

2.2.6 The ISDN-client

3 Select Specific Modemservers By Different Ports

4 the Cooperative Mode

5 Queue Management

6 The Monitor

7 Programs Using INT 14 (BIOS)

8 Stomper And Windows For Workgroups / Windows 95

8.1 Microsoft Network

8.2 Novell Network

9 Installation On A Netware Fileserver (NLM-Version)

10 Applying Access Rights

Appendix A Suitable Programs

Appendix B NETINFO.EXE

Appendix C Tips

1. Telemate

2. Terminate

3. Telix For Windows

4. pcAnywhere

Appendix D Troubleshooting

Appendix E What is a FOSSIL driver?

Appendix F What is a network fossil driver?

Appendix G How to order

Appendix H Support

1   The Concept

~~~~~~~~~~~~~~~

1.1 What is a modem-/ISDN-server?

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The modemserver is a small program, that accesses the device directly, that

should be made available throughout the network.

There may be more than one modemserver (or ISDN-server) on one computer

oder there may be several servers spread over the whole network.

When a client accesses a modemserver, all communication between

communication software (terminal, fax, ...) and the modem is directed

through the network.

Stomper is responsible for repacking the data suitable for the network, to

send it and to convert it back. It appears to the user as if the modem was

attached directly to his local workstation.

Often the modemserver is compared to a printerserver. This is not quite

correct. The main difference is, that the printer server is "batch-driven".

This means, that a print job is placed into a temporary file, which is

read by the printer server whenever it is ready and the contents then are

sent to the printer.

A modemserver works online. All data is sent to the device immediately.

Additionally all line-control information is passed on.

1.2 Server-Client Operation

~~~~~~~~~~~~~~~~~~~~~~~~~~~

As soon as a client requests an available modem or an ISDN-adapter (this

happens automatically, when your communications software is started),

the client-program (NETFOS, NFCAPI, ...) searches for a modemserver and

initiates a session.

From this moment on, the server is not available for other stations. It is

reserved for this one client.

At the end of the session, the server will get released.

2   Installation

~~~~~~~~~~~~~~~~

2.1 fast installation

~~~~~~~~~~~~~~~~~~~~~

First of all, the queue manager

STMGR

is loaded on any available workstation in the network. This could be a

computer not integrated in the modem sharing, or a modem server.

If you intend to run Stomper in a Novell Netware 3.11 or higher network, we

strongly recommend to install STMGR.NLM on the fileserver rather than

installing the queuemanager on a DOS station. Refer to chapter Installation

on a NetWare fileserver (NLM-version) on page 28.

Please read chapter 4, waiting queue administration, in this context.

1.  Modemsharing

~~~~~~~~~~~~~~~~

To install a modemserver, load

STOMPER /Px    (x = number of the port ; 1 = COM1)

To install a modem client, load:

NETFOS /Px     (x = number of the port)

NETFOS now serves all applications, that use a FOSSIL

driver. For BIOS-INT14 or Windows-COMM.DRV,

read the next chapter.

2. ISDN-Sharing

~~~~~~~~~~~~~~~

To install an ISDN-server, load:

STISDN

To install an ISDN-client, load:

NFCAPI

Now your ISDN-applications can access NFCAPI(CAPI v1.1)

2.2 Complete Installation

~~~~~~~~~~~~~~~~~~~~~~~~~

Stomper either works with the IPX- or the Netbios protocol. In Novell

networks IPX is loaded already, so nothing else has to be done.

Others (for example Lantastic: AILANBIO), however, require the installation

of the Netbios driver. Most network types (NetWare/NetWare Lite,

Lantastic...) supply Netbios. With the use of Netbios Stomper becomes

independent of the available network.

2.2.1    The Queuemanager

~~~~~~~~~~~~~~~~~~~~~~~~~

The Queuemanager performs the following tasks:

1.  "Tracker", observes the status of all modem servers installed in the

network, containing the following information: Status of the modem

server (busy/free), the Client (Netfos) which is presently served,

number of bytes transmitted in a session.

This information is analysed and displayed for example by STMON, the

monitor program.

2.  "Queue manager", allocates free modem servers to the clients. In order

to avoid chaos, the queue manager can only exist once in the network.

When STMGR is loaded for a second time in the network, only the tracker

part will be loaded automatically.

3.  IPX routing. The transmitter must always know the destination network in

order to transport IPX packets over several network segments. STMGR

therefore gives his address to all routers in the network (the file

server can also work as a router and therefore adds this address to its

bindary). Stomper identifies this address by ordering it form the nearest

router. When the queue manager allocates a modem server to a client, the

network address of stomper is supplied to the client.

4.  Logging. If the option /L is specified, a log file will be generated,

that logs all actions.

5.  Stationrestrictions (only STMGR.NLM) See Chapter Applying access rights.

Command line parameters

If you intend to run Stomper in a Novell Netware 3.11 or higher network, we

strongly recommend to install STMGR.NLM on the fileserver rather than

installing the queuemanager on a DOS station. Refer to chapter Installation

on a NetWare fileserver (NLM-version) on page 28.

STMGR [/F] [/U] [/L.....] [/A]

/U   is given to remove an installed STMGR from the memory

/F   the queue manager uses netbios, even if IPX is loaded

/L  generates a logfile.

examples:

STMGR /LSTOMPER.LOG       generates STOMPER.LOG in the directory, in

which STMGR.COM resides.

STMGR /LC:\TEMP\ST.LOG    generates C:\TEMP\ST.LOG

/A  Allow "autoattach". See also: NETFOS /A

please note, that if you wish to create a logfile in a desired directory,

you must specify a complete path!

2.2.2    The modem server

~~~~~~~~~~~~~~~~~~~~~~~~~

The modemserver accesses the modem (or another device connected to the

RS-232-port).

Apart from the application data, the line-condition will be updated whenever

necessary. This condition includes DCD (Data Carrier Detect), DTR (Data

Terminal Ready), RI (Ring Indicator), DSR (Data Set Ready), RTS (Ready To

Send) and CTS (Clear To Send).

For DOS there are 2 variants of the modemserver:

STOMPERF.COM             uses a FOSSIL driver to communicate with the

hardware

Using the latter, makes it possible to share any communications-hardware in

the network, as long as there is an apropriate FOSSIL driver available

(like CFOS for ISDN).

For information on how to install a modemserver on a NetWare fileserver,

refer to chapter Installation on a NetWare fileserver (NLM-version)

Command line parameters

STOMPER [/Px] [/Na] [/Lxxxxxx] [/U] [/F] [/Qxx] [/Bxxxx] [/Caaa...] [/R]

The parameters are optional:

/P   selects the COM port for the modem,

for example /P2 for COM2. If omitted,

COM1 is selected automatically.

/U   is given to remove any already

installed STOMPER from the memory.

This is only possible while Stomper

is not active.

/F   STOMPER is told to use Netbios

exclusively (even if IPX is loaded)

/Q   for unconventional IRQs. /Q7 => IRQ7

/B   for unconventional ports. /B3E8  =>

Port 3E8

/C   Cooperative mode: see chapter 3,

Cooperative mode

/L   Lock Baud. Independent of the bps

rate chosen by the application

(terminal program,...), Stomper

always selects automatically the bps

rate of this parameter, for example

/L57600 (locked).

Attention: Windows applications

ignore this parameter!

/N   one single character is added to this

option (for example one letter or one

digit), giving the modem server an

individual identification. Each modem

server has to be identified

individually (several modem servers

on one computer have individual

characters).

/R   ensures RTS/CTS data flow control.

This is very helpful, for example if

you encounter problems when sending

with Winfax. Normally this option

does not influence any other

applications.

/Z   16550 FIFO buffer control

/Z0 FIFO off

/Z1 FIFO on 1 byte threshold

/Z4 FIFO on 4 byte threshold

/Z8 FIFO on 8 byte threshold (default)

/Z14FIFO on 14 byte threshold

It is not recommended to use this parameter!

Stomper normally selects an 8 byte threshold for the FIFO buffer, which

causes the least potential errors.

On very slow PCs (286 - 6 MHz), it may be necessary to use /Z1.

2.2.3    The Client

~~~~~~~~~~~~~~~~~~~

The modem client provides access to a modemserver. All applications that

can use a FOSSIL driver or BIOS-INT14 (option /I) can use it to access a

network modem.

Command line parameters

NETFOS [/Px] [/U] [/I] [/F] [/Naaa...] [/A|/A+]

/P   specifies which COM port will be

redirected, for example. /P2 for

COM2. COM1 is chosen automatically

when this parameter is not used.

/U   is given to remove an installed

NETFOS from the memory.

/I   This parameter is necessary to

redirect programs which communicate

using the BIOS-INT-14. With this

parameter given, Netfos finds out if

an application is able to serve a

fossil interface, and reacts

accordingly.

Without this parameter Netfos can

only serve Windows programs or

programs which support a fossil

driver.

/F   Netfos uses Netbios (even if IPX is

loaded)

/N   All modem servers wanted in the

network are listed here. When for

example. /N123 is given, the

queuemanager selects the first

available modem server from the list.

The priority is from left to right,

so that the queuemanager selects

STOMPER /N2 when STOMPER /N1 is

occupied.

This option is invalid (and

meaningless) In the 1-server version.

/A   "Autoattach"-mode. When a modemserver

is assigned to a user after he was

placed in the queue (waiting for a

modemserver to become available),

this option suppresses the message

"Connected to modemserver". So the

connection will be established

without interaction. For both /A and

/A+ STMGR must have been loaded with

the option /A as well!

/A+  Same as /A, but additionally the user

will be placed into the waitingqueue

automatically, in case there is no

modemserver available. This mode runs

completely unattended.

Note that the parameter /Px does not have to be identical for the two

drivers. For Stomper /Px defines the interface which is physically connected

with the modem. For Netfos /Px defines only the interface which is addressed

by the terminal program.

2.2.4    Windows-Modemsharing

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In the file SYSTEM.INI (Windows directory) change the following line in

the [boot]-section:

comm.drv=comm.drv

change into

comm.drv=stcomm.drv

Make sure that there is only one line of comm.drv=...!!

Attention!

After Winfax installations comm.drv is usually changed into wfxcomm.drv.

This can be replaced without problems.

STCOMM.DRV  should be placed on a DOS path or in a windows directory.

Alternatively, the whole path can be given.

2.2.5 The ISDN server

~~~~~~~~~~~~~~~~~~~~~

The ISDN server (STISDN) makes a CAPI accessible by the clients (NFCAPI).

An STISDN can only serve one client at a time. To let more than one client

access the CAPI at the same time, STISDN has to be loaded multiple times.

These can use the same CAPI, depending on how many applications this CAPI

can support (usually 2 to 4).

Command line parameters

STISDN [/Na] [/U] [/F] [/M...]

/U   is given to remove any already

installed STISDN from the memory.

This is only possible while STISDN is

not active.

/F   STISDN is told to use Netbios

exclusively (even if IPX is loaded)

/N   one single character is added to this

option (for example one letter or one

digit), giving the modem server an

individual identification. Each ISDN

server has to be identified

individually (several ISDN servers on

one computer have individual

characters).

/M...    defines, how much memory is

reserved for communications. The

default is 16384 Bytes. Values from

4000-65500 Bytes are accepted.

The optimum depends on the software

you will be using and the number of

concurrent ISDN-connections. The

default value usually is enough for 1

application with channel-bundeling or

two concurrent applications using

each a single ISDN-connection.

2.2.6  The ISDN-client

~~~~~~~~~~~~~~~~~~~~~~

The ISDN cclient (NFCAPI) provides applications with a virtual CAPI v1.1-

interface. Up to 4 applications can be serviced by the same NFCAPI.

Command line parameters

NFCAPI [/U] [/F] [/Naaa...] [/A|/A+]

/U   is given to remove an installed

NFCAPI from the memory.

/F   NFCAPI uses Netbios (even if IPX is

loaded)

/N   All ISDN servers wanted in the

network are listed here. When for

example, /N123 is given, the

queuemanager selects the first

available ISDN server from the list.

The priority is from left to right,

so that the queuemanager selects

STISDN /N2 when STISDN /N1 is

occupied.

This option is invalid (and

meaningless) In the 1-server version.

/A  "Autoattach"-mode. When a ISDN server

is assigned to a user after he was

placed in the queue (waiting for a

ISDN server to become available),

this option suppresses the message

"Connected to ISDN server". So the

connection will be established

without interaction. For both /A and

/A+ STMGR must have been loaded with

the option /A as well!

/A+  Same as /A, but additionally the user

will be placed into the waitingqueue

automatically, in case there is no

ISDN server available. This mode runs

completely unattended.

3   Select specific modemservers by different ports

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Example:

You have 5 modem-/ISDN-servers installed in your network:

two ISDN-lines with IDs 4 and 5

Of course you will want to use an ISDN-line, when you need an ISDN-line

and a modem line, when you need a modem line. You can do this by simply

entering the following command line:

NETFOS /P1(123) /P2(45)

When your Application addresses COM1, you will get connected to

modemserver 1, 2 or 3, whichever is available. When your application uses

COM2, one of th ISDN lines will be selected.

Note that there is no option /N specified, since this is completely replaced

by the values in brackets. You can specify up to 4 ports.

Of course you can use this method to distinguish different modemtypes as

well - ISDN was chosen as a typical example of different transport media.

4   The Cooperative Mode

~~~~~~~~~~~~~~~~~~~~~~~~

This mode - under certain circumstances - enables the use of another

software (for example a mailbox) on the modem server - sharing the port

cooperatively with Stomper.

Parameter: /Cxxxxx

xxxxx stands for any modem command. At the end of a session this command is

sent to the modem in order to re-initialize it. CR/LF is added to this

string (for example. ATZ), automatically.

Thus the modem server can run a mailbox, which will be stopped automatically

when a client in the network demands the modem. While a caller is logged

into the mailbox, Stomper is inactive.

In order to avoid fatal failures on the computer, please read the following

lines even if this sounds a bit technical:

For safety reasons a network-modem demand is rejected, when the CD signal

is active, for example, when someone is logged into the mailbox. Stomper

will also be blocked for another 60 seconds after the last ring.

This mode has to be handled with care, as it is quite rude to block an

application just like that, but this can be done for example with FrontDoor

and Remote Access.

As during the blocking of the modem the bps-rate could be changed, at the

end of such a session Stomper sends an optional command (see above, "/Caaa")

to the modem in order to set the correct bps rate. For this, the modem must

have an automatic baudrate identification.

In the cooperative mode Stomper MUST NOT BE LOADED in the high memory

area(UMB), due to the following:

The computer will be stopped at an appropriate time, that is in the moment

of a timer interrupt (produced by the computer 18 times per second).

"Appropriate" means:

1.  there is no hard disk or disk access

2.  Windows is inactive

3.  The program interrupted is placed in the memory above stomper and

under the high memory area from A000:0000 on.

No. 3 ensures that no IPX routine has been interrupted.

As Stomper depends on IPX, it would not dare to touch this without being

affected personally. As Stomper needs the IPX (or Netbios) drivers, they

are normally placed in a lower memory section or in an UMB. But if Stomper

has also been loaded in the high memory area, there is no certainty where

the network drivers are placed related to Stomper.

It is possible that the interrupt does not appear. This happens for example.

when the command line in DOS is used, which is mainly executed in

COMMAND.COM (below Stomper) or in BIOS (>A000). The use of the interface is

not affected in this case.

5   Queue Management

~~~~~~~~~~~~~~~~~~~~

In order to enable a precise access to one or several modem servers in the

network, the program

STMGR.COM

has to be loaded on one computer (any) in the network. STMGR.COM should

already be active in the network, before a Client (Netfos) or modem server

(Stomper) is loaded. Preferably this program should be installed on a

modem server. If you are working with Novell Netware, the best choice will

be the Fileserver (STMGR.NLM)

This program operates completely in the background and does not require

much memory.

The queue manager's strategies

When a client demands a modem server, the queue manager tries to allocate

a free modem server which is selected by NETFOS /Naaa...(from left to right).

In case all modem servers are busy, the client computer receives the

following message:

No modemserver available. Queue in?       Y/N

After confirming Y, the following message appears:

Queued in. Wait in foreground?         Y/N

Now there are two possibilities:

1.  "Wait in the background" (N) tells the application that no fossil

driver is installed (not recommended for pure BIOS INT14 applications).

The computer can be used and as soon as a modem server is free, the

following message appears:

Connected to modem server    (ENTER)

The queue manager marks the modem server as "reserved". The user has

1,5 minutes to start his application and to use the modem server.

2.  While waiting in the foreground, the following message appears

Waiting for modem server...    (ESC)

In the meantime the computer cannot be used for other applications.

Under windows this process is executed accordingly, but only the

application in use is paused.

6   The Monitor

~~~~~~~~~~~~~~~

The monitor (STMON.EXE) provides an overview over the modem servers

installed in the network and the proceedings can be observed. The monitor is

started by loading

STMON

As STMON displays the information obtained from the tracker, it depends on

STMGR.COM being loaded. If STMGR is not already in memory, the monitor will

load it automatically. For this, however, STMGR.COM must be either

-   in the same directory as STMON.EXE or

-   in the DOS path.

Attention: The monitor must not be started in a DOS box under windows!

The monitor displays ten sections on the screen, each containing information

of an individual modem server.

Field identifications:

-   Network address - individual address in the network which is preset by

the network adaptor. In order to obtain a better overview, a file called

NODES.LST

can be created, in which names are allocated to the network addresses.

An example for such a file:

924e32a  John's abacus

80ca23e5 venus

... allocating the names "John's abacus" and "venus" to the according

network addresses. John's abacus and venus will replace the cryptic

address names on the monitor.

-   Served interface - the serial interface served by the modem server

or CAPI

-   Status - status of the modem server.

Possible states:

- free           the modem server is available at the moment

- Ser. <Client>  the modem server is just serving the client named

<Client>

- Res. <Client>  the modem server is presently reserved for the client

<Client>.

- locally in use The modem server is presently used locally in the /"C"

mode         (for example a user in the mailbox)

-   Sent / received bytes - number of bytes transmitted in the current

session. This information will be held until the start of a new session.

-   Identification - the identification of the modem server (/Na).

7   Programs Using INT 14 (BIOS)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The fossil driver is to be understood as an extension to the BIOS-INT 14

interrupt. However, Stomper expects that the software used initializes the

fossil driver (not necessary with BIOS routines).

For programs only using BIOS-INT 14, NETFOS has to be loaded with the

parameter /I.

With pure INT 14 applications, Stomper always switches to RTS/CTS data flow

control.

Approximately 20 seconds after the end of an "INT-14 program" the modem

server becomes available for others again.

When using fax software, which communicates via INT 14, it is absolutely

necessary to lock Stomper on an appropriate rate. The ideal bps rate depends

on the modem used.

Standard modems transmit faxes with a rate of up to 14400 bps, or 19200 bps

on the phone line. In this case the bps rate has to be locked on 19200 bps.

Most modems accept a higher rate, for example 57600 bps, when transmitting

faxes.

For modems with a maximum baud rate of 9600 bps, Stomper should be locked

on 9600 bps (see parameter /L on page 6).

8   Stomper and Windows for Workgroups / Windows 95

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

A "real" Windows 95 version should be available december 95

WfW and Windows 95 normally load their own network drivers. This means, in

these cases, there are no network protocols available prior to loading

Windows.

In case you are running Netware or any network with DOS-drivers, you can

skip this chapter.

Create the file WINSTART.BAT in your Windows-directory, like

-----------------------------------

NETFOS /P1

-----------------------------------

The batchfile WINSTART.BAT will be executed by Windows after all

system-drivers have been loaded (including network drivers).

STCOMM.DRV should be loaded as described in chapter "Complete Installation"

Be aware that this version of Stomper only supports 16-bit applications!

New 32-bit software uses a different driver.

8.1 Microsoft Network

~~~~~~~~~~~~~~~~~~~~~

Make the following changes to your Windows network setup:

Defaultprotocol:   NetBEUI

NCBS=255

NetHeapSize=20

8.2 Novell Network

~~~~~~~~~~~~~~~~~~

Make the following changes to your Windows network setup:

Defaultprotocol:   IPX/SPX

SPX Connections = 60

9   Installation on a NetWare fileserver (NLM-version)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Copy STMGR.NLM and STOMPER.NLM to the SYS:SYSTEM directory.

To install STMGR.NLM on the fileserver, enter

load STMGR

at the console.

To install a modemserver, you will need to install an asynchonous

I/O-driver first. This requires the module AIO.NLM and any AIO-driver.

The appropriate AIO-driver for standard RS232 is AIOCOMX.NLM. You can

download AIO-drivers from Compuserve NOVLIB or from our BBS. Several

AIO drivers for Hostess, Digiboard, etc. are available.

For available commandline options recognized by STOMPER.NLM and STMGR.NLM,

refer to chapter Complete Installation on page 6.

Note the different format of STOMPER-option /P for port selection:

/P<hardwaretype>,<boardnumber>,<portnumber>

The three values are shown when you load the AIO-driver. If you are not

sure what to type, you can load STOMPER.NLM without the /P-option.

Stomper will let you select the desired port and then display the

appropriate commandline you should use in future.

If you posess a multi-modemserver version of Stomper, you can load

STOMPER.NLM multiple times.

If you type

unload stomper

at the system console, you will disable all running modemservers.

Caution!!

-   There is a bug in the Netware 3.x / 4.x -

SPX-protocol. This bug known to STOMPER.NLM and

Stomper tries hard to avoid problems that might

arise.

But wo do strongly advise you to install the

following Novell-patches:

PATCHMAN.NLM

SPXDDFIX.NLM

SPXFIX2.NLM

These three modules are contained in the

archive STRTL3.EXE, that can be downloaded free

from Compuserve NOVLIB or from our support BBS.

-   If you have problems loading AIO.NLM,

STOMPER.NLM and STMGR.NLM, and you get a

message like

"Public symbol IsColorMonitor not found",

then you have an old version of CLIB.NLM

(Netware 3.11). Just install an updated

version, that is part of the archive

LIBUP4.EXE (Compuserve NOVLIB or our support

BBS)

-   When loading AIOCOMX, do not use the

option "node=..." if possible

-   You should use AIOCOMX >= 2.11. It should

be loaded with the maxrate-option:

load aiocomx [...] maxrate=115200

-   The use of a FIFO (16550)-serial UART is

strongly recommended.

10  Applying access rights

~~~~~~~~~~~~~~~~~~~~~~~~~~

STMGR.NLM provides a mechanism to refuse access to all or specific

modemservers by username, stationaddress and/or network.

This mechanism is activated, as soon as STMGR.NLM finds a file named

QMREST.LST in the same directory, STMGR.NLM is contained in. If this file

is not present, no restrictions apply.

This file could look like this:

[stations]

928485

80A23E42     12Q

[nets]

1000002

1000003       1

[names]

supervisor

Peter         24

Rules:

-   there are 3 possible sections: [stations], [nets] and [names]

-   you can omit particular sections. There will be no restrictions for

these sections.

-   if a section is present, it defines  objects (stationaddresses,

usernames or networkaddresses), that have access to modemservers. All

objects of this sectiontype, that are not defined, will not have access

to any modemserver.

-   if only the object is entered in a line, it receives access rights to

all modemservers throughout the network. If you additionally enter a

list of modemserver-ids - separated by spaces or tabs from the objects

designation - the object will only have access to the specified

modemservers.

-   access rights are combined by logical and, which means, the user must

qualify by station, network and username (in case restrictions apply

respectively)

Typically the file QMREST.LST will only contain

a [names] or [nets] section.

Appendix A    Suitable Programs

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

All windows programs, using the functions (API) (provided by windows

COMM.DRV) for the interface addressing, are supported.

DOS:

Not every terminal program supports a fossil driver. An example for a

program with FOSSIL support is Telemate (In Options / Terminal, FOSSIL can

be selected)

For the use of Telemate note the following: in order to work with the dial

list, for each listing the fossil driver has to be selected individually.

To select the fossil support in Telemate, first the baud rate has to be

set to a maximum of 38400bps. Otherwise fossil will be ignored!

A short list of programs which have been tested with Stomper:

Name            Operating       Comments

system

------------------------------------------------------------------

Telemate        DOS             extensive and comfortable

FX-Term         DOS             Very easy to handle GUI, works with

RoboBoard.

Terminate       DOS             extensive (similar to Telemate). IEMSI

support

Dos/CIS         DOS             Compuserve information manager

Terminal        Windows         Terminal program included in windows packet

GoCIS           Windows         Offline Compuserve navigator

NavCIS          Windows         Offline Compuserve navigator

WinCIS          Windows         Compuserve information manager

PowerCom        Windows         terminal program

Telix for       Windows         extensive and comfortable terminal program

Windows

Transend        DOS/Windows     terminal / fax

RemoteAccess    DOS             BBS software

Frontdoor       DOS             Fido mailer

Binkley         DOS             Fido mailer

Crosspoint      DOS             Fido / Z net / UUCP - point system

Winfax          Windows         fax software

Bitfax          Windows         fax software

Unicom          Windows         comfortable terminal program

Most of these programs are available are available as shareware on our

Support BBS (see end of Document).

Appendix B    NETINFO.EXE

~~~~~~~~~~~~~~~~~~~~~~~~~

This a small diagnostic tool added to the packet, giving some information of

the network environment (received from netbios), for example

Permanent name:                         0000C0039A79   ; network address

Jumper:                                 40

POST result:                            00

Version:                                0.1

Minutes up:                             0

Number of CRC errors:                   0

Number of alignment errors:             0

Number of collisions:                   0

Number of aborts:                       1

Number of transmitted packets:          6

Number of received packets:             10

Number of retransmits:                  4

Number of out-of-buffer-counts:         0

Free NCBs:                              10

Max. NCBs:                              12

Active sessions:                        32

Max. packet size:                       1024

Names:                                  1

NFOSMPX1                                0

Attention! This tool can only be useful when Netbios is actually installed.

Otherwise no useful information will be displayed here! This information

is of no use for IPX.

Appendix C    Tips

~~~~~~~~~~~~~~~~~~

1.  Telemate

~~~~~~~~~~~~

Before you select FOSSIL, be careful to set the speed to not more than

38400 bps. Otherwise Telemate will reject the FOSSIL-Setting (without

displaying a message).

In case even this fails, the safest way is to edit TM.CFG manually. Just

change

Connection=Modem

to

Connection=Fossil

and make sure that the entry Port= is correct.

Also, FOSSIL has to be selected for every entry in the Dial-List.

2.  Terminate

~~~~~~~~~~~~~

The first time Terminate is set up for FOSSIL, it expects that a FOSSIL

driver is installed on COM1. This, of course, is not always given (e.g.

NETFOS installed on COM2).

The FOSSIL-support of Terminate is a little buggy here, and this may result

in a system-crash.

Workaround:

either use NETFOS on COM1 or

the first time, you switch Terminate to FOSSIL, do it with a regular FOSSIL

driver (like BNU or X00) installed. From then on you can use NETFOS.

If you don't have a FOSSIL driver handy, load

NETFOS /P1() /P2()

This installs NETFOS on both COM1 and COM2.

3.  Telix For Windows

~~~~~~~~~~~~~~~~~~~~~

Use the COM-Setting, not FOSSIL! In Windows it is always recommended to use

STCOMM whenever possible.

Apart from that, the FOSSIL support of early versions of TfW is buggy.

4.  pcAnywhere

~~~~~~~~~~~~~~

Use the plain INT14-Setting and do not set higher speeds than 9600 bps.

Otherwise pcAnywhere will not be able to initialize the port.

To achieve higher speeds, refer to the modemserver switch /L.

Appendix D    Troubleshooting

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Problem              possible source             solution / remarks

The terminalprogram  The terminalprogram might   If provided by the terminal

doesn't react - no   address the hardware        software, switch to Fossil

characters are       directly                    or INT 14.

transmitted                                      In case the software doesn't

support a software inter-

face,

there is no way of

redirecting port usage.

The terminalprogram is set  Call NETFOS /I when using

to INT-14, but Netfos       BIOS-INT14-routines.

wasn't loaded with the /I-

option.

the wrong port is being     set the com-port settings of

addressed.                  the terminal software to the

value specified in the

commandline of Netfos.

Characters are lost  A non-buffered serial       Use a buffered serial device

during transmission  hardware is used            (UART 16550). Such can

be obtained for a small fee

in most computer stores.

Stomper relies on buffered

communications devices

more than other COM-

software, since it has to

deal with many interrupt

types at once (timer,

network and serial).

no handshaking active       Usually you have to specify

the type of handshaking you

want to use in the setup of

your communications

software. In most cases it's

best to set hardware

handshaking (RTS/CTS) to

on and software

handshaking (XON/XOFF) to

off.

Stomper hangs in     Stomper was loaded in       Stomper cannot be loaded in

cooperative mode     high memory                 high memory when driven in

cooperative mode.

It works with DOS    STCOMM.DRV wasn't           Make sure, the line

but not with Windows installed or wasn't         comm.drv=stcomm.drv is

installed correctly         placed in the [boot]-section

of you SYSTEM.INI-file and

that there is only one

occurence of a line

beginning with "comm.drv="

Fax-sending causes   You are using a DOS-Fax-    In this case, Stomper sets

problems             software that uses the      the handshake-method to

Bios-INT 14 interface.      "Hardware". Fax

transmission with class 2

expects software flow

control. make sure that your

modem is set to hardware

flowcontrol explicitly

XON/XOFF handshaking        start Stomper with the

doesn't function properly   commandline option /R

Appendix E    What is a FOSSIL driver?

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

There are various possibilities for a DOS program to address serial

interfaces of a PC:

There is the hardware related method, communicating directly with the

interface components. This method is used very often because it's quick and

uncomplicated for the program user - no unnecessary thoughts about a

fossil driver and its implementation.

"Polite" programs use the 4 functions provided by the computer BIOS (INT 14):

Baud rate setting, character transmission, character receive and fetching

the interface status. This method has the advantage of being hardware

independent (only BIOS has to have the information how to communicate with

the hardware - the programmer isn't troubled with that). Big and decisive

disadvantage: under DOS, BIOS allows max. 9600 bps, which is out of date

today. Furthermore BIOS routines are generally quite slow.

Another possibility is the fossil driver, combining the advantages of the

solutions mentioned above - speed and hardware independence - troubling the

user with only one installation.

The fossil driver offers buffered interface communication as an additional

function to the BIOS, emulating the interface functions of BIOS. Data

arriving at the interface needn't be handled immediately by the software,

but are stored in a ring buffer. A program using BIOS for example, has to

handle every single character before the next arrives in order to avoid a

loss of data. With a fossil driver the data can be fetched and handled in

blocks.

Appendix F    What is a network fossil driver?

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

It's easy to exploit the advantage of a hardware independent program with

the use of a fossil driver. A suitable driver can pretend any digital

communication element against such a hardware independent program, for

example as COM2. The programm doesn't care how this works, as long as the

driver handles the mass of data.

Stomper uses this circumstance. A modem server is installed on a network

integrated computer linked to a modem. The modem server waits for a network

fossil driver connection, which again bluffs to be communicating with a

serial interface. At the same time a lot of instruction data and other data

are passed over the network, but don't worry, a normal network is hardly

concerned with that.

Appendix G - How to order

~~~~~~~~~~~~~~~~~~~~~~~~~

We have registration sites in many countries. Run the program

ORDER.EXE

to get information on pricing and registration sites. You will be prompted

to select your country.

Appendix H    Support

~~~~~~~~~~~~~~~~~~~~~

Pflug Datentechnik can be contacted via

Fax          49 / 7156 - 939005

e-mail       100340.1047@compuserve.com

Compuserve   Pflug 100340,1047

Fido         2:246/1009 (per Crashmail only, please!)

Mailbox  (see below)

Inquiries on the phone are not possible without prior consultation.

Current Stomper versions and several shareware/PD software can be

downloaded from our mailbox:

+49 / 7156 - 939006