���������������������������������������������������������������Ŀ

�                QMPCHART.EXE...Version 9309                    �

���������������������������������������������������������������Ĵ

�             A Quality Assurance Training Tool:                �

�      Statistics Committee of the QA Section of the PMA        �

���������������������������������������������������������������Ĵ

�         Bob Obenchain, CompuServe User [72007,467]            �

�        5261 Woodfield Drive North, Carmel, IN 46033           �

�����������������������������������������������������������������

Note:  PMA = Pharmaceutical Manufacturing Association

QMP = Quality Measurement Plan (Bellcore/AT&T Bell Labs)

The QMPCHART module makes QMP trend charts with detailed header/footer

titles and reporting period labels.  QMPCHART makes VGA graphical

displays of I-plots or Forecasts or Both.  QMPCHART is not a

user-friendly "self-paced learning" module with menus/prompts like the

QMP.EXE and QMPS.EXE modules.  QMPCHART works only with batch-input

files, which must be specified as command-line arguments to

QMPCHART.EXE.

QMPCHART is compiled using Microsoft C/C++, ver7.00, using the default

mode that will automatically detect and utilize a 8087/80x87 numeric

math co-processor if your MS-DOS personal computer has this chip.

Usage:

qmpchart infile [outfile]

where... "infile"  is a required command line parameter giving the name

of the batch input file (DESCRIBED BELOW), and

"outfile" is an optional command line parameter giving the

filename.ext for DETAILED output.  If no outfile is

specified, the output filename defaults to

"qmpchart.dtl".

Format for infile:

------------------

I                             Line  1: Trend Cart Type [ I, F, or B ]

1991                          Line  2: Year (or Years...1990-1991)

12                            Line  3: Number of Periods

J F M A M J J A S O N D       Line  4: String of Period Identifiers

Product Name                  Line  5: Header 1/Top of Trend Chart

Quadratic Loss                Lune  6: Header 2

Target Value = XX.XX          Line  7: Footer 1/Bottom of Trend Chart

Exp.Loss= XX  Var.Loss= XX    Line  8: Footer 2

6                             Line  9: History Window Length(Periods)

1                             Line 10: EFACTOR = 1 or < 1

0                             Line 11: Number of History Periods

9.15  3.94                   Line 12... Data PAIR for a Reporting

7.32  8.12                              Period (separated by one

7.78 12.65                              or more blanks.)

QMPCHART.EXE, ver.9309                               Page 2 of 7

9.61 17.53

9.61 19.41       Either...   Defect Expectancy & Observed Defects

9.61 31.29                       (when EFACTOR = 1, as here)

9.61 14.06

10.53  8.49        Or...      Sample Size & Observed Defects

10.07  9.10                       ( when 0.001 <= EFACTOR < 1.)

10.07  4.49                       In this latter case, Expectancy

9.61  2.49                       will be (Sample Size) * EFACTOR.

11.44  3.93

NOTES: Titles can have a maximum of 40 characters and are

automatically centered when transferred to the graphics screen.

Lines 9, 10 and 11 of the batch infile must always specify the

Window Width (usually 6 months or 16 lots), the E-factor, and

Number of Periods of History data that preceed the first

Present Period.

WARNING: A number of history periods equal to or greater than

the window width will be truncated to the window width minus

one.  This will probably cause the statistics plotted to lag

behind the reporting period identifiers specified on line 4.

An E-factor of 1 means that exact defect expectancies are

contained in the input file.  But, if 0 < E-factor < 1, then

the input file is assumed to contain "sample sizes" and the

corresponding expectancies will be computed by QMPCHART as

the product of the E-factor times these sample sizes.

Data pairs for History Periods start in row 12; the maximum

number of history periods allowed is the window size minus

one.  Historical data are not plotted on the trend chart

because they describe the previous "track record" of the

quality process (before the time period plotted at the left

extreme of the trend chart.)  Data from these History Periods

are used to estimate the mean and variance of the quality

process for all Present Periods that include these History

Periods within their QMP Window.

Data pairs for Present Periods follow those for the History

Periods.  If no previous history is provided, row 11 contains a

zero and the Present Period data pairs start in row 12.  The

maximum number of Present Periods that can be displayed is

about 24; the software will attempt larger numbers of periods,

but the charts are unlikely to be readable.

Contents of the QMPCHART outfile:

=================================

Details of QMPCHART.EXE results...batch input = [name of infile]

QMP Internal Parameter         x0 = 0.400000

QMP Internal Parameter         e0 = 0.400000

QMPCHART.EXE, ver.9309                               Page 3 of 7

QMP Internal Parameter         I0 = 1.000000

QMP Internal Parameter     gamma0 = 0.550000

QMP Internal Parameter maxgammasq = 2.200000

QMP Internal Parameter     sigma0 = 2.500000

QMP Internal Parameter     soundf = 1.000000

ChartType **I**  [I-plot or Forecast]

year(s) = **1991**

Periods = **12**

Labels  = **J F M A M J J A S O N D**

1st Head= **Product Name**

2nd Head= **Quadratic Loss**

1st Foot= **Target Value = XX.XX**

2nd Foot= **Exp.Loss= XX.XX     Var.Loss= XX.XX**

window size = **X**

Next, if data from "history periods" that preceed the first "plotting

period were included (i.e. positive value on line 11 of input file),

then the QMPCHART output file will include a line stating "history

periods = **X**" and then list the input defects and expectancies from

these history periods.

NOTE THE REVERSAL IN ORDER:  QMPCHART input files must give equivalent

expectancy first and the number of observed equivalent nonconformances

second.  But QMPCHART output files list observed nonconformances first

and expectancy second.

The above information is followed by a listing of the following 11

measures for each reporting period in an I-plot:

defct = equivalent defects

exptc = equivalent expectancy

q95 = lower 95% quality index for current period

q5 = upper  5% quality index for current period

best = best measure of current quality

pavg = process average quality over current window

indx = raw quality index for current period

symb = plotting symbol for current index

pvar = variance of current process average

bvar = variance of current best measure

pfac = P - factor

When QMPCHART makes a QMP Forecast plot instead of (or in addition to)

an I-plot, two more measures are also reported for each period:

f95 = lower 95% forecast index for next period

f5 = upper  5% forecast index for next period

QMPCHART.EXE, ver.9309                               Page 4 of 7

�����������������������������������������������Ŀ

�    Making Screen Dumps to Graphics Printers   �

�������������������������������������������������

If you have one of the following types of graphics printers connected

to your LPT1 parallel printer port, you can dump a copy of the VGA

graphics screen to your printer by pressing a single key as follows:

L or F10   ...dumps the screen to an HP Laserjet or DeskJet

(two portrait-mode charts per page)

I or F9    ...dumps to an IBM Graphics or ProPrinter

as well as to Epson LX and MX printers

E or F8    ...dumps to Epson FX, JX-80, or LQ-1500 printers

(landscape-mode, one chart per page)

P or F11   ...saves screen in PCX format (black, blue and red

on white) in a file with the same filename as the

"infile" but with the extension ".PCX".

The VGA screen will remain displayed while it is being dumped or

saved, but it will then disappear.  To return to DOS without making a

screen dump, press any other key (say, the SpaceBar or Enter key)

instead of any the eight keys listed above.

������������������������������������������Ŀ

�     Resetting QMP Internal Parameters    �

��������������������������������������������

If a file named QMP-PARM.SET is within the "current working directory"

from which QMPCHART is invoked, QMPCHART will read that file to

initialize QMP parameters.  Otherwise the following defaults will be

used:

���������������������������������������������������������������������Ŀ

�  0.4  =  e0, start-up equivalent expectancy                         �

�  0.4  =  x0, start-up equivalent defects = theta0 * e0              �

�  1.0  =  I0, start-up quality index                                 �

�  0.55 =  gamma0, prior mean of gammasq, the process variance        �

�  2.2  =  maxgammasq, 95% point of the prior distribution of gammasq �

�  1.0  =  sigma0, variables QMP prior sampling variance              �

�  1.0  =  QMPtunes, audible signaling time factor [0.0,2.0]          �

�����������������������������������������������������������������������

NOTE: Setting the QMPtunes time factor to 0.0 in a QMP-PARM.SET file is

the ONLY WAY to totally suppress QMPCHART "audible signaling."  A time

factor of 0.5 would attempt to speed the tunes up by a factor of 2; a

time factor of 2.0 (maximum allowed) would slow the tunes down by a

factor of 2.  The default setting for QMPtunes time (the 7-th and last

parameter specified in a QMP-PARM.SET file) is 1.0.  And 1.0 is the

QMPtunes time factor that will be automatically used when no

QMP-PARM.SET file is found within the current working directory.

QMPCHART.EXE, ver.9309                               Page 5 of 7

QMPtune's audible tones provide some not-particularly-subtle "aids-to-

interpretation" of the final I-plot on a QMP Trend Chart...

Case 1:  q95 <= 1     High Quality...CHARGE Ahead!!!

Case 2: q5<1 & q95>1  At Standard....no sound

Case 3:  q5  >  1     Below Normal...a DEATH in your

immediate family.

REFERENCES

HOADLEY, Bruce. (1981). "The Quality Measurement Plan, QMP."  Bell

System Technical Journal, 60, 215-273.

HOADLEY, Bruce. (1986). "QUALITY MEASUREMENT PLAN(QMP)."  Encyclopedia

of Statistical Sciences. (Kotz, Johnson and Read, Editors)

Volume 7, pages 393-398.  New York, John Wiley.

INTRODUCTION TO QUALITY TREND CHARTS.  Bell Communications Research,

Inc., January 1986 (18 page booklet.)

OBENCHAIN, Robert. (1992). "Regret Indices and Quantification of

Process Capability."  Eli Lilly, Statistical and Mathematical

Sciences.

QUALITY MEASUREMENT PLAN (QMP).  Bell Communications Research Technical

Reference, TR-TSY-000438.  Issue 1, April 1987.

APPENDIX ONE

�����������������������������������������������������������������������Ŀ

�                  QMP Trend Chart BENCHMARK Examples                   �

�����������������������������������������������������������������������Ĵ

� This Appendix presents eight canonical examples that cover a variety  �

� of different types of quality process behavior.  In addition to the   �

� graphical displays and verbal commentaries given here, the BENCHMARKS �

� include ASCII files of input data and output results that are used to �

� validate alternative QMP software systems.  We recommend that readers �

� first examine each Trend Chart and try to decide what information it  �

� conveys.  The reader can then confirm/guide his or her interpretation �

� by examining the verbal commentary associated with that Trend Chart.  �

�������������������������������������������������������������������������

Example One...Stable Compound

This is an example of a well controlled process where quality is

beinging maintained right at the Standard quality level.  There is

very little process variation.

Example Two...High Quality

This stable, well controlled process is running much better than

Standard; i.e. well within the threshold of present process

QMPCHART.EXE, ver.9309                               Page 6 of 7

capability.  From May through November, there were no defects

observed, as shown by the X's right at the Zero-Defect Limit.  There

is very little variation (either process or sampling) in the process.

Example Three...Process Jitter

This is an example of an unstable process...a process with

considerable sampling variability as well as process variability.

Each period the interval for CURRENT QUALITY is moves abruptly.  This

interval bounces from better than Standard to Standard to worse than

Standard, etc.  Process variability is more likely to cause this sort

of fluctuation than is sampling variation.  The trend over the last

five months hovers around 1.5 times Standard...even though October

does show a BEST MEASURE better than Standard.  This process has

severe quality problems.

Example Four...Out of Control

This process is literally Off-the-Chart.  The numbers at the bottom

indicate that the entire 90% confidence interval estimate for CURRENT

QUALITY is worse than 5 times Standard in June, August, and September.

This represents a severe quality problem that demands immediate

management action.

Example Five...Dramatic Decay

During February, March and April, this process was running above

Standard.  But quality started deteriorating toward Standard in May

and June.  Then quality plummeted in July, and the CURRENT INDEX

remained worse than two times Standard for the next four months.  From

May onward, there is increased process and sampling variation.  Even

though there is insufficient statistical evidence to say with 95%

confidence that quality is worse than Standard during these months,

there is a degradation, or downward trend, in quality.  As of

September, on the other hand, one can say with 95% confidence that

quality is running significantly worse than Standard.

Example Six...Questionable Quality

This process quickly drifts to between 1.5 and 2 times Standard,

showing considerable sampling variation.  Note that the interval

estimate for CURRENT QUALITY is entirely below Standard in April and

October indicating a quality problem.  During May through September,

the CURRENT QUALITY is probably below Standard at least part of the

time.  This is a fair interpretation even though the strenght of

evidence does not exceed 95% confidence for any single month between

May and September...all interval estimates cover the Standard value of

1.

Example Seven...Skip Lot Sampling

This is a well controlled process.  Very few defects have ever been

observed.  No samples were collected for this process in September or

QMPCHART.EXE, ver.9309                               Page 7 of 7

October; inspection resources are apparently concentrated on other,

more troublesome products during those months.  However, an occasional

check is needed to verify that the process has not changed

significantly.  Samples were again collected in November.

Example Eight...Subtle Window Effects

This example shows considerable sampling variation; the I-Plots tend

to be wide because defect expectancy is low most months.  Between

February and November, there is insufficient evidence to confirm that

quality for any particular month is worse than Standard.  But the LONG

RUN AVERAGE shows quality between 1 and 2 times Standard most of the

year.  The downward trend in the LONG RUN AVERAGE in September-October

combined with the below-standard signal for November should cause the

reader to be seriously concerned about the quality of product to be

produced in December.

ADVANCED COMPREHENSION QUESTIONS:

(1) Why did the LONG RUN AVERAGE move up in August when the CURRENT

SAMPLE INDEX was down from July?

(2) Why did the LONG RUN AVERAGE move downward in October when the

CURRENT SAMPLE INDEX was up from September?

These sorts of effects are explained by realizing that the LONG RUN

AVERAGE is calculated over the WINDOW of the 6 most recent months.

ANSWERS:

(1) Note that when August (with twice the expected number of defects)

entered the window, February (with more than FIVE times the

expected number of defects) exited out the back of the window.

Thus the LONG RUN AVERAGE moved upward in August primarily because

the very poor perfromance from Feburary became outdated as of

August.

(2) When October (with slightly more than the expected number of

defects) entered the window, April (with NO observed defects)

exited out the back of the window.  Note also that the I-Plot for

October is more narrow than those for most previous

months...meaning that a larger sample was taken in October (than

in April, May, or July when NO defects were observed).  Thus more

weight is assigned to October results in the current LONG RUN

AVERAGE than was given to April data in the previous LONG RUN

AVERAGE.  Both of these factors combine to move the LONG RUN

AVERAGE downward in October.