HyperDrive Disk-Based User Guide

Written by Jeff Duntemann

Software by Mark S. Zachmann

Last Revision 2/3/83

The HyperDrive software is explicitly placed in the Public Domain

as of 1/1/83.  You may distribute it freely without obligation to

Starside  Engineering  or  the  author,  Mark  Zachmann.   Please

understand  that,  aside from this documentation,  no support  of

this  package should be expected from either Starside Engineering

or  the author.   Full ASCII source for the program is  provided,

which   may  be  modified  and  reassembled  using  the  IBM   PC

macroassembler.

PLEASE NOTE!

The  typeset hardcopy documentation of HyperDrive which  Starside

Engineering  provides  as  a  service  to  its  customers  is   a

copyrighted expression of this document,  and we would appreciate

your  not  photocopying  the hardcopy version of  this  document.

Thank you!

Starside Engineering

PO Box 18306  Rochester NY 14618

(716) 461-1027

*   *   *   *

I.  CONTENTS OF THE DISTRIBUTION DISK

The  HyperDrive  distribution  disk contains at  least  four

files.  They are:

RAMDISK.ASM              Source for HyperDrive

INTERUPT.MAC             Macro for interrupts

HDD512.EXE               HyperDrive set up for 512K

HDS320.EXE               HyperDrive set up for 320K

There may be other files of one sort or another  (especially

configurations  for different combinations of size of ramdisk and

placement of ramdisk).

II.  SETTING UP YOUR PC TO RUN HYPERDRIVE

A  HyperDrive ramdisk may be sized to any multiple of  160K.

In  practice,  we have not had the memory to try anything  larger

than a 320K ramdisk.   You need at least 320K to run  HyperDrive.

(This  means a full 64K on the PC system board and an add-on 256K

RAM card.)  With a 320K RAM PC,  you may set up a 160K ramdisk in

the top half of your RAM,  leaving the lower 160K for ordinary PC

DOS programs.  This is the way the distributed file HDS320.EXE is

set up.

If  you have a 512K RAM card in your PC,  you may set  up  a

"double-sided"  320K ramdisk in the top 320K of RAM,  leaving the

remainder of lower memory for DOS programs.   This is the way the

distributed  file HDD512.EXE is set up.   Other combinations  are

possible and set up without a great deal of effort.  For example,

you  could configure a "single-sided" ramdisk up at the top of  a

512K PC system if you needed lots of lower memory.   In theory, a

"triple-sided"  (interesting notion) ramdisk is possible--try  it

if you have the RAM.   The second 'D' in HDD512.EXE refers to the

double-sidedness  of  the  ramdisk  it  creates.   In  a  similar

fashion, the 'S' in HDS320.EXE denotes a single-sided ramdisk.

The first thing you must do to set up a ramdisk is to inform

the  PC that you have one more disk drive than you used to.   The

System Board DIP switch 1 positions 7 & 8 indicate to the PC  how

many  disk drives are installed in it.   (Leave the other  switch

positions alone,  and be careful if you use a blunt instrument to

change  switch positions.   It's easy to bump something you don't

want to bump!)

For setting up a ramdisk as drive C,  set DIP switch 1  this

way:

12345678

|      X |

|X      X|

--------

(Switches  which are not shown in our diagram do not  affect

HyperDrive operation and should not be disturbed.)

For setting up a ramdisk as your fourth disk drive,  drive D

(assuming you have a drive C of some other sort) set DIP switch 1

this way:

12345678

|        |

|X     XX|

--------

See  page  2-28 of the Technical Reference Manual  for  more

discussion of the DIP switch settings.

Next,  you  must  "fool" the PC into thinking that  the  RAM

memory  you  are using for the ramdisk does not  exist.   PC  DOS

cannot  be  "aware"  that  ramdisk  RAM is  on-line  or  it  will

interfere  with  the  operation  of  HyperDrive.   You  must  set

positions 1-4 of System Board DIP switch 2 so that the top of RAM

indication it holds is below the RAM you are using for a ramdisk.

For example,  if you have 320K of RAM in your PC and want to

use  the top 160K for a ramdisk,  you must "tell" the PC that  it

has only 160K of RAM.  To do this, set switch 2 this way:

12345678

|  XX    |

|XX      |

--------

This  is  how  you  set this DIP switch  for  use  with  the

HDS320.EXE version of HyperDrive.

As another example,  if you have 512K in your PC and wish to

use  the  top  320K of RAM as a ramdisk,  you must  set  the  DIP

switches  to  inform the PC that it has only 192K of RAM to  work

with, since 512-320=192.

III.  RUNNING HYPERDRIVE

You invoke HyperDrive as you would any other .EXE file:

A>HDD512 (CR)

It will print a title message and immediately return control

to  DOS.   The ramdisk has been created.   Before you can use it,

you  must  format it.   It formats like any  other  disk  device.

Assuming you set it up to be drive C, you would type:

A>FORMAT C: (CR)

FORMAT will come back and tell you to insert a disk in drive

C;  ignore that request and press any key.   Once your ramdisk is

formatted, you can use it as you would any other disk drive.

You will find that putting assembler or compiler files on  a

ramdisk can greatly increase the speed of compiles and assembles,

since  such  programs  move data to and from disk a  great  deal.

Just remember to move anything you want to save (like a  compiler

output file) to a "real" diskette before you power down!

Hyperdrive  has  a feature which may save  you  considerable

aggravation  in  the  event of systems  crashes.   If  a  program

running  under  DOS  goes  into the Twilight Zone  and  you  must

reboot, you need not lose all files in your ramdisk.  (Of course,

if the program you were running went berserk and filled RAM  with

trash  before  it  died you're out of luck.)  HyperDrive  has  an

option  which will re-create the ramdisk in RAM without the  need

to reformat it.   With this option,  you can re-boot your PC  and

still  find your files waiting for you in your ramdisk.   This is

the "OLD" option.

To  re-build a ramdisk without destroying files  which  were

there before a re-boot, type:

A>HDD512 OLD (CR)

You  need not format the ramdisk after using the OLD option.

The  OLD option assumes that the ramdisk is still in  memory  and

formatted;  if  you  use  the OLD option without  there  being  a

formatted  ramdisk  already in RAM you will probably  crash  your

PC.

IV.  MODIFYING HYPERDRIVE

HyperDrive  is a means of mapping logical disk devices  onto

other  logical disk devices,  or onto areas of memory.   You  can

alter  this  mapping  either  by changing  the  source  file  and

reassembling it (if you own the Macroassembler) or using DEBUG to

change the assembled object code.

Using  DEBUG is probably easier.   One caution:   You  can't

modify  a file with an .EXE extension using DEBUG.   First RENAME

the HyperDrive program file to have a different  extension,  like

(for example) HDD512.ZZZ.  Then load it into DEBUG:

A>DEBUG HDD512.ZZZ (CR)

Display the 32 bytes from offset 03C0 through 03DF:

-D03C0-03DF (CR)

We  have  added some delimiters to the hex dump  you  should

see:

|  1  A: 2  |

052B:03C0  8E C0 8E DB 87 FE C3 00-00 00 00|00 00 00 01|01

0     1

1 B: 2  |  1  C: 2  |  1  D: 2  |

052B:03D0  00 01 01|00 40 00 68|FF-FF FF FF|00 00 00 00 00

0    1     0     1     0     1

The  16  bytes  from  03CB to 03DA are a map  of  your  disk

system.   Each  group of two bytes identifies one side of a  disk

device.   The first 2 bytes is the map for Drive 0, Side 0, which

you know as side 1 of drive A:.  The second two bytes are the map

for Drive 0 Side 1.   The third set of two bytes are the map  for

Drive 1 Side 0 (the first side of the B: drive) and so on.

Notice the four bytes at offset 03D3:  00 40 00 68.  This is

the  map for both sides of the C:  disk drive.   It is actually a

pair of memory segment addresses.   It means that Drive 2 Side  0

is  mapped into RAM at segment 4000 and Drive 2 side 1 is  mapped

into  RAM at 6800.   (For those of you not familiar with  Intel's

memory address notation,  the least significant byte of a  2-byte

address offset or segment always comes first in memory.)

The  important  thing  to notice about the map is  that  the

first  four byte pairs (encompassing disk drives A:  and B:)  map

onto themselves.   That is,  Drive 0,  Side 0 is mapped as 00 00.

You could remap Drive 0, Side 0 into RAM by replacing the pair of

bytes at 03CB with a segment address.  If you were to replace the

byte at 03CC with 24H, you would see this:

|  1  A: 2  |

052B:03C0  8E C0 8E DB 87 FE C3 00-00 00 00|00 24 00 01|01

0     1

1 B: 2  |  1  C: 2  |  1  D: 2  |

052B:03D0  00 01 01|00 40 00 68|FF-FF FF FF|00 00 00 00 00

0    1     0     1     0     1

This  would  map side 1 of disk A:  into RAM  as  a  ramdisk

existing  at segment 2400.   (This is not recommended unless  you

know  what you're doing and what the consequences might be.)  You

could also map Disk 0,  Side 0 onto another disk drive.   Suppose

you changed the map to look like this:

|  1  A: 2  |

052B:03C0  8E C0 8E DB 87 FE C3 00-00 00 00|01 01 00 01|01

0     1

1 B: 2  |  1  C: 2  |  1  D: 2  |

052B:03D0  00 01 01|00 40 00 68|FF-FF FF FF|00 00 00 00 00

0    1     0     1     0     1

This  would  map side 1 of disk A:  onto side 2 of disk  B:,

which  is,  again,  something  you shouldn't do unless  you  have

really  thought it through and understand what's  going  on.   We

show  you these possibilities mostly to give you a flavor for how

disk device mapping works and what it does.

Once you have altered the map the way you want it,  save the

program back out to disk and RENAME it as an .EXE file.   (If you

recall,  we changed the file extension to .ZZZ to allow DEBUG  to

work properly with it.)

If you have no place to put a disk device,  say,  the second

side  of  a single-sided ramdisk,  or all of drive D in a  three-

drive  system,  fill  the unnecessary positions in the  map  with

FF's.  (We have done this in the above examples for drive D.)  If

you do a hex dump of HDS320.EXE, you will see that drive 2 side 1

(A.K.A.  Drive  C side 2--confusing,  isn't it?) is  filled  with

FF's.   This informs HyperDrive not to attempt to format the half

of the ramdisk which doesn't exist.

It's yours,  folks.   Use it in good health.   Pass it on to

those  who  don't have it.   Tell them you got it  from  Starside

Engineering.   Get  our catalog of other goodies which (while not

free) are cheaper than you might expect!

##########################################################