BALANCING THE SURVEY                page 1.

The  operation  of distributing corrections to eliminate the error

of closure is called "balancing the survey", or balancing the tra-

verse".  A very satisfactory discussion of the theory  and  recom-

mended methods is contained in the reference text book:

SURVEYING THEORY AND PRACTICE

by

Raymond E. Davis, M.S., C.E., D.Eng.

Francis S. Foot, E.M.

Joe W. Kelly, B.S.

Fifth Edition

McGraw-Hill Book Company

Copyright 1966

The need for balancing the survey arises whenever the  results  of

the  surveyor's measurements must be reported out and submitted to

the  critical  scrutiny of others.  At such time it becomes impor-

tant that the reported or depicted information be exactly selfcon-

sistent, since any inconsistency or self-contradiction that  might

be  discoverable  in the survey would make it vulnerable to criti-

cism.  Therefore, the professional land survyor, when he is  about

to  release  his work where it will be subject to public scrutiny,

is careful to eliminate from it any detectable inconsistencies. In

other words, he is entitled to "take the Fifth Amendment."  He  is

not  expected  to  testify against himself, or to submit data that

would appear to be self-contradictory.

Balancing the survey accomplishes two things:

1.- It discovers mistakes or errors in field measurements,  which  can

be reduced by additional or repeated measurements in the field.

2.- It  eliminates  controversy  by concealing the residual errors, to

eliminate any apparent contradictions.

When  progressing   around  a  traverse,  there  is  invariably  a

cumulative  error,  which shows up as a failure to close.  It is a

reasonable assumption that the error is not the fault of  any  one

measurement, but rather should be partitioned more or less propor-

tionally  amongsts other measurements of the survey.  At least, if

it were so partitioned, no one measurement would  be  particularly

hurt.   On  the other hand, the experienced land surveyor may have

an intuition  about  where  he  feels  the  error  would  best  be

concealed.  This is a matter of professional judgement, and is not

to be dictated by even a computor program.

There is really no magic way or best way to distribute the errors.

Remember,  you  are  the surveyor; the quality of the work is your

responsibility.  Who else is there to say that you are wrong?

When  the surveyor does not elect to assign the corrections on the

basis of his personal judgement, he may  wish  to  distribute  the

errors  proportionally to the several sides of the survey, so that

only minute corrections are alloted to each.

BALANCING THE SURVEY                page 2.

In this discussion that follows the term SURVEY shall be taken  to

mean  the  measurement  of  a  tract  of land shaped as a polygon,

enclosed by several straight sides.  The SURVEY shall be taken  to

designate  the  progress  commencing  at  the  P.O.B.   (point  of

beginning) and proceding around the entire tract, and returning to

the P.O.B.

The term TRAVERSE shall be taken to  mean  the  measurement  of  a

single side; alternatively this can also be referred to as a CALL.

A  `traverse' or a `call' in every case is a bearing (azimuth) and

a distance, which can be represented mathematically as a `VECTOR'.

The surveyor begins at the P.O.B. or point  #1,  and  proceeds  to

measure  angle and distance to the next point, proceding thus from

point to point until he returns to his established P.O.B.

A survey should have three or more sides; in every case  you  will

have one more calculated point than there are sides.  For the sake

of this discussion let us think in terms of a FOUR SIDED SURVEY.

If the survey has four sides there will  be  four  traverses,  and

there  will  be  five  pairs  of coordinates calculated.  You will

begin with a pair of coordinates for  point  #1  either  given  or

assumed.   When  you  have  completed  the  circuit, you will have

measured back to the P.O.B., but you will arrive there  with  cal-

culated values of coordinates that will not quite match the values

for  point  #1.   Therefore you will assign it point #5.  Point #1

and point #5 will represent the same P.O.B. on the ground, but the

calculated coordinates will not likely match,  because  your  best

efforts are not perfect. (You may use any point No's; I am  merely

using #1 through #5 as an example.)

FIRST:  If  the field notes contain the necessary information, you

should find the algebraic sum of all the deflection  angles  going

around the traverse. This sum should be exactly 360 deg.

This same concept can be expressed another way.  The  sum  of  the

interior  angles  of  a  polygon  = (n - 2)�180 deg, where n = the

number of sides.

Each  angle,  large or small, is subject to the same reading error

or least count.  Therefore  the  total  angular  error  should  be

divided  by the number of angles, and each angle corrected by that

amount so that the adjusted sum is exactly the number expected. On

the otherhand certain angles may have been long established in the

surveying community; you will need to be  guided  by  professional

discretion.

SECOND:  you  should  use  COGO  and run the traverses (DEFLECTION

ANGLE or otherwise) around the survey establishing  the  tentative

coordinates,  northings  and eastings, of your designated numbered

points.  Beginning at point #1, CLEAR the  accumulator  registers,

and  quickly step around the survey from point to point, using the

INVERSE traverse, from point #1 to #5 and close from point  #5  to

#1.  Read  and  make  a  note  of  the  closing  CALL, azimuth and

distance; this is the absolute error; this is the unwanted VECTOR.

BALANCING THE SURVEY                page 3.

When  you  INVERSE  back  to  your  beginning  point,  where   you

originally  cleared  the  accumulators,  COGO  will  automatically

display the enclosed area and the perimeter.  Read and make a note

of the computed perimeter of the survey.

With your pocket calculator, divide the perimeter by the  absolute

error  distance;  this  quotion  is  the  PRECISION  RATIO of your

survey.  Let this number be your guide as to whether  or  not  you

should make further efforts to discover other errors in your field

work.

The absolute error is a VECTOR, i.e. an azimuth and a distance, as

is each of the other calls in the survey.  It is portions of  this

vector  that  you will wish to distribute and apportion to certain

of the other vectors.  Since you will most probably  have  already

adjusted  the  angles  of  the survey in the first step above, you

will  not  want  to  disturb  those  angles  in  distributing  the

remaining error.  Your best bet will be to make a scale drawing (a

vector  diagram)  of  the survey, and a magnified scale drawing of

the absolute error vector.  From this you should be  able  to  see

which  of  the  traverses  could best absorb portions of the error

distance with minimum relative change.  (At  this  point  you  may

wish  to send the field crew back out to more carefully check some

of their measurements.)

Each adjustment, plus or minus, that you make to the lenght of one

of  the  traverse  sides,  will make a corresponding change in the

error vector.  Judicious choices on your part will permit  you  to

shrink  the  remaining  error to insignificance.  The longer sides

should be your best candidates for adjustment.

The error vector is the vector resultant of the  adjustments  that

you  wish  to  make.   With  your  parallel  straight edge, draw a

zigzag or broken line of vectors,  each  PARALLEL  to  a  traverse

side  to  be  adjusted,  connecting  from  beginning to end of the

error vector. Obviously there is more than one  way  to  do  this.

There  should  be some clues to tell you which of the traverses is

more probably to be of questionable precision.  At  any  rate  you

will  thus  be  able to apportion the error amongst several of the

sides.  Having laid this out to scale, you can now add or subtract

the measured component to each of the corresponding sides.  If the

error moves in the same direction as the corresponding side, it is

to  be  added numerically to that side; if it moves counter, it is

to be subtracted from that side.

Having calculated the adjusted length  of  each  of  the  traverse

distances,  you  may  again  run  through  the processes described

above, and this  time  the  absolute  error  may  be  sufficiently

reduced, or you may wish to try again.  Finally, when you have re-

duced  the  error  to  a  negligible quantity, you may abandon the

readjusted point, which we have called #5, and the  survey  closes

with precision.

Having established and balanced  the  principal  corners  of  your

survey,  you  may  proceed  with COGO to subdivide and compute the

points that must be laid out within or  adjacent  to  your  survey

tract.

#