INSTALLATION OF PC-FOTO(C)

PC-FOTO (C)COPYRIGHT 1988 FOTO 64, INC. ALL RIGHTS RESERVED

PC-FOTO and it support programs are distributed on one disk.

It may be installed and used on either two disk drives or on

a hard drive. PC-FOTO was not designed for use on machines with

single disk drive. Operations with a single disk drive are not

recommended unless both program and data files are situated on

the same disk. This might be feasible with a hard drive or

on higher capacity disk drives.

Programs are also included to support its operation on machines

containing both the IBM Monographic adapter and the Color

Graphics Adapter. These systems usually have both monographic

and color monitors.

Protect your master disks..!! PC-FOTO is not copy protected,

however, it is designed so that individual routines will not

function with out the presence of the other files.

Make back up copies using the DOS "diskcopy.com" command. These

should be verified as exact copies using the DOS "diskcomp.com"

program. Put you original disks in a safe spot for future use.

Be sure to place write protect tabs on your masters before copying

them. Use your back up  copies for the creation of your working

Program disks.

Format one disk with the /V/S options. To do that with your DOS

disk in drive A: enter the following command.( A>FORMAT B:/V/S )

You will be prompted to place disks to be formatted in Drive B:

and strike any key. When you are prompted for the Volume name

enter the following :PCFOTO22.The format program will when

finished indicate if there are any bad sectors. There must be no

bad sectors on the disk.

To make your working programs follow the following instructions.

DISK DRIVE INSTALLATION

Make your System working disk by placing Master Back Up Disks in

drive A:(BE SURE IT IS WRITE PROTECTED) and place your newly

formated disk in drive B:. Enter the following command at the A

prompt. ( A>COPY *.* B: ). When finished label your PC-FOTO

Program System disk.

This is your working program disk. Before running PC-FOTO format

a new disk with out the /S option for the data drive. Copy the

files "TEST0001.100. TEST0001.350 AND TEST0001.220" from the

master working disk. These files are sample label files which

will be helpful as you become aquatinted with the operation of

PC-FOTO.

If PC-FOTO is to be operated from a hard drive all files

from the working disk should be copied to the same

directory. To do this it will necessary for a directory to be

created on the hard disk and the PC-FOTO files copied to it.

Set your default drive to the hard disk. This is usually drive

C:. At the C> prompt enter the following statement - MD FOTO.

This will create the directory PCMAIL. Enter CD\FOTO enter the

new directory. Place the PC-FOTO System Disk in Drive A:. Enter

COPY A:.*.*. This will copy the system disk files to hard disk.

The PC-FOTO disk contains the two batch files FOTO.BAT and

FOTOD.BAT. These files contain instructions to the computer

on the sequence of file loading. It is important that the

PC-FOTO files be loaded in proper order.

FOTO.BAT is to be used on installations which have only a

graphics adapter and monitor. If you have a rare dual system

containing both monographic and  color graphics adapter

(Dual Monitors) you will need to use FOTOD.BAT.

To start PC-FOTO set your drive to the one containing your

system disk. In the case of a hard disk set it to the PC-FOTO

directory. If you have a single adapter system enter at the

prompt (a>) FOTO. The Speedscreen Module will be loaded and the

PC-FOTO program will start.

If you have a dual adapter system enter FOTO if the system is

operating from the Color Graphics Adapter. It the system is

operating from the monographic adapter enter FOTOD. The later

file will switch the system to the Color Graphics adapter and

load the Speedscreen Module and start PC-FOTO. In a dual

adapter systems PC-Mail must be operated from the Color Graphics

Adapter.

When PC-FOTO starts the Introductory screen will be displayed.

To proceed strike the space bar or enter "h" for help.. The

PC-FOTO main menu screen will be displayed. To proceed it will

be necessary to place the data disk you created earlier in the

drive to receive data (usually B:). Enter this drive letter "B".

PC-FOTO will then validate your drive choice and proceed to the

menu selections. If the drive choice is not valid, or if the

drive is not ready instructions will be given.

PC-FOTO is designed as an interactive - self prompting program.

Many screen and prompts will lead you through the routine.

It is highly recommended that when PC-FOTO is run for the first

time that the HELP program be read. This will assist you is

becoming aquatinted the program and it operation.

Just enter "H" from the INTRODUCTORY SCREEN.

Every effort has been made to produce a reliable - trouble free

program in PC-FOTO. Foto 64, Inc. or the author assumes no

responsibility for problems arising from program use. Your use

of the program supports this understanding. Should you encounter

any bugs please note the conditions and send the information to

Carl F. Miescke at P.O. Box 2130  Saratoga, CA 95070. This feed-

back will assist in improving PC-MAIL.

PC-FOTO (C)1988 - CARL F. MIESCKE - All Rights Reserved

OTHER PROGRAMS BY CARL F. MIESCKE

PC-MAIL(2.6): a mail address management program permitting

sorting and printing on up to ten fields. Prints on 16 sizes of

labels one to five labels wide. Allows for screen searches on up

to ten fields or combination of fields. Prints envelopes, file

cards,name tags, mailers, rosters, telephone lists, data lists,

and special parcel labels. Provides special programs for file

and disk management.

PC-POSTCARD(2.1): A program permitting the design,printing, and

saving of post cards for mailing. Permits message design and

printing, return address and permit mail stamp, and addressing

of cards from createrd address data files. Supports the creation

of new data files and screen searches of these files. PC-

POSTCARD and PC-MAIL data files are fully compatable.

PC-DISK LABEL(2.0): This program permits the design, printing,

and storage of labels for 5 1/4" floppy disks. Up to five lines

of data are permitted. The labels are printed on 1" x 5" label

stock and are easy to read from a distance.

FOR FURTHER INFORMATION CONTACT

FOTO 64, INC.

P O BOX 2130

SARTATOGA, CA. 95070